3GPP TSG-RAN WG2 Meeting #130	R2-2503454

St.Julians, Malta, 19th – 23rd May, 2025

Agenda Item:	8.5.4

Source: 	OPPO

Title:  	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

# Introduction

In Rel-19 NES WID [1], the objective on adaptation of common signal/channel transmissions is captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution would like to provide our considerations on the objective of Rel-19 NES.

# Discussion

# RACH Adaptation

### 2.1.1 Applicable RA types for RACH adaptation

From R2 perspective, what we need to discuss includes whether the RACH adaptation mechanism applies to different types of RACH resources (e.g., CBRA/CFRA, 2/4-step RA type, feature/feature combination-specific RACH resources). Based on the following agreements made in R2#129 meeting, we further discuss the leftover issues on the applicable RA types for RACH adaptation.

RAN2 starts CBRA for RACH adaptation. RAN2 starts 4-step RACH adaptation. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. From R2 perspective, RACH adaptation is not modelled as RA feature(s). From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

In R2#129 meeting, RACH adaptation is excluded for RACH resources for (legacy) MSG1-based SI request, however, it is still unclear whether RACH adaptation applies to RACH resources for OD-SIB1 request introduced in Rel-19 NES O2 or not. We understand that it is straightforward that the conclusion should also be extended to OD-SIB1. But it is good to further clarify this.

- R2 confirms not applying RACH adaptation to RACH resources for OD-SIB1 introduced in Rel-19 NES.

For 4/2-step RACH, 4-step RA type is supported as baseline. On top of 4-step RA type, CBRA/CFRA and feature (combination)-specific RACH resource have been discussed. However, it is quite open on the support of 2-step RA type, as well as the combination with CFRA/CBRA and feature (combination)-specific RACH resource, as shown below.

Table 1 Status of RACH adaptation applicability

|  |  | 4-Step RA type | 2-step RA type |
| --- | --- | --- | --- |
| CBRA | RACH towards serving cell (including BFR case) | No remaining issue | FFS |
|  | RACH towards neighboring cell > RACH for HO > RACH for LTM > RACH for C-HO/C-LTM | FFS | FFS |
| CFRA | RACH towards serving cell (including BFR case) | Discussed in [POST129b] [122] | FFS (NOT applicable for BFR) |
|  | RACH towards neighboring cell > RACH for additional PCI > RACH for early UL sync > RACH for HO > RACH for LTM > RACH for C-HO/C-LTM | Discussed in [POST129b] [122], except the C-HO/C-LTM case | FFS (NOT applicable for > RACH for additional PCI > RACH for early UL sync > RACH for LTM (MAC CE triggered) |

- R2 has not started to discuss on the support of 2-step RA type, with and without feature (combination)-based RACH resource partitioning, considering the late stage of this release.

From the technical perspective,

1/ For CFRA with 2-step RA type, considering CFRA with 2-step RA type is only supported for handover, and the network does not configure CFRA resources for 4-step and 2-step RA types at the same time, whether RACH adaptation is supported for CFRA with 2-step RA type or not depends on the support of RACH adaptation for CFRA for L3 HO initiated by RRC signalling discussed in the post-meeting email discussion. Based on our understanding of RACH adaptation for CFRA for L3 HO, we understand CFRA with 2-step RA type is also not supported.

2/ For CBRA with 2-step RA type,

- network can configure both 2-step and 4-step RA types at the same time, and an RSRP threshold is used by the UE to select between 2-step and 4-step RA types. The impact on DCI design, e.g., indicating 2-step and/or 4-step RA type in DCI, needs to be considered, which may need additional R1 effort.

- Besides the MsgA PRACH resources (via rach-ConfigCommonTwoStepRA-r16) for transmission of MsgA preamble, network needs to configure the MsgA PUSCH resources (via msgA-PUSCH-Config-r16) for transmission of MsgA payload in 2-step random access type procedure. In case of additional MsgA PRACH resources provided, the configuration of the associated additional MsgA PUSCH resources also needs to be discussed. In detail, it is still unclear which parameters in msgA-PUSCH-Config-r16 are needed for the configuration of additional MsgA PUSCH resources, which may need additional effort for both R1 and R2.

Considering there are still many technical issues needed to discuss if supporting 2-step RA type for RACH adaptation, we propose not to support 2-step RA type for RACH adaptation.

- R2 does not consider RACH adaptation for 2-step RA type.

For CFRA towards neighboring cell, there are still leftover issues not touched in the post-meeting email discussion [POST129b] [122].

In the email discussion, only “CFRA for HO initiated by RRC signalling (via RRCReconfiguration with ReconfigurationWithSync)” was discussed. The intention was to cover the CFRA for network-initiated handover to the target cell. The CFRA resources (i.e., rach-configGeneric from the rach-ConfigDedicated) of the target cell are configured via reconfigurationWithSync in RRCReconfiguration. However, CFRA for C-HO/C-LTM was missed. For CFRA for C-HO, the CFRA resources of candidate cells are configured via conditionalReconfiguration in RRCReconfiguration. For CFRA for C-LTM, the CFRA resources of candidate cells are configured via ltm-Config in RRCReconfiguration.

For CFRA for C-HO/C-LTM, we understand there are same concern, considering they are all related to availability of additional RACH resources in neighboring cell, i.e., UE does not know the availability of neighboring cell, so may not be sure whether the additional RACH resources are available or not when initiate the RACH. If we leave this to network, it relies on the assumption that network has good knowledge of when the RACH would be initiated. Otherwise, neighboring cell has to update the RACH resources when the availability status changes, which may lead to inter-node signaling, i.e., R3 impact. Therefore, we propose to further exclude CFRA for C-HO/C-LTM, or not to pursue specification impact to enable it.

- R2 does not pursue spec impact to enable RACH adaptation for CFRA for C-HO/C-LTM.

Another issue is still for CFRA towards neighboring cell where the CFRA resources come from the target/candidate cell, including

1/ CFRA for LTM cell switch initiated by MAC entity itself (via an LTM Cell Switch Command MAC CE),

2/ CFRA for HO/C-HO/C-LTM.

For these CFRA cases, as shown below, the CFRA resources are optionally configured by rach-configGeneric (contained by occasions) from the rach-ConfigDedicated. If rach-configGeneric (contained by occasions) is not present in rach-ConfigDedicated, rach-configGeneric from rach-configCommon would be used.

RACH-ConfigDedicated ::= SEQUENCE { cfra CFRA OPTIONAL, -- Need S ra-Prioritization RA-Prioritization OPTIONAL, -- Need N ..., [[ ra-PrioritizationTwoStep-r16 RA-Prioritization OPTIONAL, -- Need N cfra-TwoStep-r16 CFRA-TwoStep-r16 OPTIONAL -- Need S ]] } CFRA ::= SEQUENCE { occasions SEQUENCE { rach-ConfigGeneric RACH-ConfigGeneric, ssb-perRACH-Occasion ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL -- Cond Mandatory } OPTIONAL, -- Need S resources CHOICE { ssb SEQUENCE { ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource, ra-ssb-OccasionMaskIndex INTEGER (0..15) }, csirs SEQUENCE { csirs-ResourceList SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource, rsrp-ThresholdCSI-RS RSRP-Range } }, ..., [[ totalNumberOfRA-Preambles INTEGER (1..63) OPTIONAL -- Cond Occasions ]], [[ msg1-RepetitionNum-r18 ENUMERATED {n2, n4, n8, spare1} OPTIONAL -- Cond 4StepCFRArep ]] } CFRA field descriptions. occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP..

As mentioned in our previous paper, if network would like to configure additional RACH resources for CFRA, it can simply configure the rach-configGeneric in rach-ConfigDedicated same as the one for additional RACH resources, which is fully up to network implementation without any specification impact.

However, the issue is in case that CFRA is configured but rach-configGeneric is referred to the one from rach-configCommon (i.e., rach-configGeneric is not present in rach-ConfigDedicated). If there are both legacy RACH resources and additional RACH resources configured in rach-configCommon, UE still has chance to use the additional RACH resources configured for CBRA. There are same concern as discussed above, i.e., UE does not know the availability of neighboring cell, so may not be sure whether the additional RACH resources are available or not when initiate the RACH.

One straightforward way is to further clarify that in this case UE has to use the legacy RACH resources.

- For CFRA towards neighbouring cell (during HO or LTM), if rach-configGeneric is not present in rach-ConfigDedicated, R2 does not consider RACH adaptation, i.e., legacy RACH resources are used in this case.

Similar issue occurs for CBRA towards neighboring cell itself. If there are both legacy RACH resources and additional RACH resources provided by a target/candidate cell, the situation is same as the case above, and same way is suggested to adopt, i.e., UE should not use the additional RACH resources to initiate CBRA towards neighboring cell.

- For CBRA towards neighboring cell (during HO or LTM), R2 does not consider RACH adaptation, i.e., legacy RACH resources are used in this case.

### 2.1.2 RRC remaining issues for RACH adaptation

Regarding how to configure additional RACH resources for Rel-19 NES UEs in 38331, there are three options:

- Option-1: add the parameters for additional RACH resources within the same RACH-ConfigGeneric used for legacy RACH resources,

- Option-2: add another RACH-ConfigGeneric for additional RACH resources within the same RACH-ConfigCommon for legacy RACH resources,

- Option-3: add another RACH-ConfigCommon in BWP-ConfigCommon for additional RACH resources.

Based on the Rel-19 higher layers parameters list sent by R1 [2], the parameters for additional RACH resources include

- prach-ConfigurationIndex, which exists in legacy RACH-ConfigGeneric,

- msg1-FrequencyStart, which exists in legacy RACH-ConfigGeneric,

- msg1-FDM, which exists in legacy RACH-ConfigGeneric,

- prach-SubsetMask-Index-Adaptation, which is a new parameter and its location is FFS,

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB, which exists in legacy RACH-ConfigCommon.

Considering ssb-perRACH-OccasionAndCB-PreamblesPerSSB has already existed in legacy RACH-ConfigCommon, if we go for Option-1, it is weird that for a parameter, the legacy one is in RACH-ConfigCommon while the new one is in RACH-ConfigGeneric. Therefore, we tend to exclude Option-1, and are open to discuss between Option-2 and Option-3.

- For the configuration of additional RACH resources, R2 discusses the following options:
Option-1: add the parameters for additional RACH resources within the same RACH-ConfigGeneric used for legacy RACH resource; 
Option-2: add another RACH-ConfigGeneric for additional RACH resources within the same RACH-ConfigCommon for legacy RACH resources;
Option-3: add another RACH-ConfigCommon for additional RACH resources.

For feature (combination) specific RACH partitioning, the legacy RACH resource can be applied to all existing features, i.e., RedCap, SDT, Slicing, and CE, and their feature combinations, but for additional RACH resources introduced by Rel-19 RACH adaptation mechanism, CE and its related feature combinations are excluded, considering it is not reasonable to configure both NES and CE features for UE’s SCell.

- CE and its related feature combinations are not supported for additional RACH resources introduced by RACH adaptation.

The issue lies in the disparity between the legacy RACH resources and the new additional RACH resources introduced in R19. If network needs to allocate RACH resources to the feature of CE (or CE-related feature combinations) using the legacy RACH resources, it will face a significant constraint with the new additional RACH resources considering it cannot be configured to support CE, if both types of RACH resources tend to share the same feature or feature combination-based RACH partition scheme. It is unclear how to enable feature(-combination) based RACH resource partitioning for the two types of RACH resources, i.e., legacy RACH resources and new additional RACH resources.

Two approaches could be considered for the two types of RACH resources, as follows:

- utilizing two different feature(-combination)-based RACH partitioning schemes including/not including the support of CE-related feature(-combination), or

- utilizing the same feature(-combination)-based RACH partitioning scheme for them, and additional indication to address the specific exclusion of CE-related features in the new additional RACH resources.

- R2 discusses the approaches to enable feature(-combination)-based RACH resource partitioning for the legacy RACH resources and the new additional RACH resources, including 
1) two different feature(-combination)-based RACH partitioning schemes,
2) a same feature(-combination)-based RACH partitioning scheme.

### 2.1.3 MAC remaining issues for RACH adaptation

Another remaining issue in MAC is how to calculate RA-RNTI to avoid RAR differentiation issues, in case of additional RACH resources introduced due to RACH adaptation.

Note that in Rel-19 SBFD, the additional ROs for SBFD are introduced, and the similar issue on RA-RNTI formula enhancement for the additional ROs for SBFD is also being discussed. For the potential solution in SBFD, the additional RACH would take a different f-id compared to legacy RACH based on a configurable offset in SIB, but would also rely on the same RA-RNTI formula. Therefore, for the two new RACH resources of SBFD and NES, it is still risky for space overlapping, and related RA-RNTI has to be solved, unless we can differentiate the s-id/t-id combination for new RACH resources of SBFD and NES as well. We understand the coordination with SBFD on RA-RNTI calculation is required, but the cross-feature issue due to the different features in the same release could be handled in the maintenance phase, as we did before.

- R2 de-prioritizes the discussion of NES and SBFD co-existence, and leave the related issue, if any (e.g., RA-RNTI design for additional RACH), to maintenance phase.

Although RA-RNTI calculation is specified in 38321, the main solution is within R1 scope, e.g., separate search space for additional PRACH resources can be easily used to solve this issue (as R2 discussed for the RACH separation issue since R15), or RA-RNTI formula enhancement (which invites more solutions since it has to be backwards compatible with legacy UEs).

Considering the last stage of this release, we understand it is hard to go for the separate search space since it requires more R1 effort, while the next meeting is the last R1 meeting in this release.

- R2 discusses whether to leave the RAR differentiation issue caused by the additional RACH resources to R1 or network implementation.

If R2 tends to solve this RAR differentiation issue caused by the additional RACH resources within R2, it seems RA-RNTI formula enhancement is the only feasible choice from R2 perspective.

The current RA-RNTI and MSGB-RNTI formulas are shown as follows,

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

The MSGB-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as: MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2 where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The RA-RNTI is calculated as specified in clause 5.1.3.

where the value range of RA-RNTI is from 1 to 17920, while the value range of MSGB-RNTI is from 17921 to 35840. Both RA-RNTI and MSGB-RNTI share the same space from 1 to 65536.

For RA-RNTI formula enhancement for the additional PRACH resources, as shown in the following figure, there are two solutions:

Solution-1: Introduce a new value range different from the ones for RA-RNTI (1-17920) and MSGB-RNTI (17921-35840). For example, using a fixed offset on the RA-RNTI formula, i.e., new RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80 x 8 x 4, which starts from 35841 to 53760.

Solution-2: within the value range of RA-RNTI (1-17920), rely on f-id to differentiate between the additional RACH resources and the legacy RACH resources, assuming there are no more than 8 ROs for the same s-id/t-id/ul-carrier-id, as mentioned in the following R1 agreement. For example, f_id for the additional RACH resources is offset by the number of legacy RO in frequency domain or by a fixed/configured offset.

R1#120 Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

For Solution-1, since the new value range in the RNTI space is used, the risk of insufficiency on the RNTI space should be analysed. As shown in the figure above, the occupied space depends on whether 2-step RACH is supported and whether SUL is supported.

If additional RACH resources are only supported for 4-step RA type, the RNTI space from 1 to 65536 would be sufficient. Only in the case of supporting 2-step RA type, the RNTI space may have chances to be insufficient.

Besides, considering the coordination with Rel-19 SBFD, if the additional ROs introduced in Rel-19 SBFD also adopt similar ways as Solution-1 and assuming 2-step RACH is not supported for NES RACH adaptation, we understand no risk for the RNTI space overlapping, since Rel-19 SBFD only supports SBFD configuration for TDD band, while SUL band is not a TDD band, thus the additional RNTI space needed is only 8960 (i.e., 14 x 80 x 8).

For Solution-2, two understandings are further discussed by companies, i.e., either the enhancement is only applied to the additional RACH resources which are overlapped with legacy ones in time domain, or it is applied to all additional RACH resources regardless of the overlapping. In our understanding, RA-RNTI calculation depending on the overlapping status should be avoided since it adds complexity from UE perspective. The most straightforward way is to apply to all additional RACH resources. In detail, taking the following two cases as examples,

1/ if legacy RACH resources (4 ROs in frequency domain) and additional RACH resources (4 ROs in frequency domain) are not overlapping, so RA-RNTI for additional RACH resources starts from f-id = 4, as shown in the figure below.

2/ if legacy RACH resources (4 ROs in frequency domain) and additional RACH resources (8 ROs in frequency domain) are overlapping (no matter which 4 ROs of the additional RACH resources are overlapping with legacy ROs), then considering the overlapping 4 ROs of the additional RACH resources are invalidated, the RA-RNTI for usable ROs also starts from f-id = 4, as shown in the figure below.

- If No for P7, R2 further discusses the two solutions below:
Solution-1: introduce new value range different from ones for RA-RNTI and MSGB-RNTI using a fixed offset on the legacy RA-RNTI formula, i.e., new RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80 x 8 x 4;
Solution-2: within the value range of RA-RNTI, legacy RACH resources use f_id values from 0 to N-1 as in legacy, while RA-RNTI for additional RACH resources (excluding the invalidated ROs due to overlapping with legacy RACH resources) adopt f_id values from N.

### 2.1.4 Remaining issues in LS (R2-2503312)

Another issue is related to the LS received from R1 [3], as shown below.

# 1 Overall description RAN1 would like to inform RAN2 of the following agreement related to DCI-based PRACH adaptation. Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs # 2 Actions To RAN2: ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

For the usage of one bit from the Bits 5-8 within the Short Message field, from R2 perspective, no showstopper is foreseen. Thus, it is straightforward to confirm this and further discuss the R2 impacts based on this.

- R2 confirms that the usage of one bit from the Bits 5-8 within the Short Message field for adaptation indication in DCI format 1_0 with P-RNTI is feasible.

Besides, as shown below, in the same meeting, for additional RACH (de)activation indicated in DCI 1_0 with P-RNTI, R1 has also agreed to introduce a validity duration configured via higher layer signalling.

R1#120bis Agreement For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. Agreement - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

One further issue is that, as shown in the following figure, if a UE camps on this cell after the first modification period (i.e., MP#0 in the figure) where the additional RACH resources being activated, i.e., enters into the cell at MP#1/2 in the figure, the UE will not know the (de)activation status of the additional RACH resources since this UE misses the activation indication via DCI 1_0 with P-RNTI in the first SI modification period.

We understand R2 may discuss whether to handle this issue or not.

If not handling this issue, network still only sends activation indication via DCI 1_0 with P-RNTI in the first SI modification period where the additional RACH resources are being activated (i.e., in MP#0 only). UE determines the (de)activation status of the additional RACH resources based on this. If UE misses it, UE considers the additional RACH resources invalid. In this way, UE needs to bear the consequences, i.e., UE cannot use the available additional RACH resources of this validity duration.

If R2 determines to solve this issue, there are two candidate approaches:

One approach is to consider that network always send activation indication via DCI 1_0 with P-RNTI in each SI modification period where the additional RACH resources are activated (i.e., in MP#1, MP#2 and MP#3), so UE can determine the (de)activation status of the additional RACH resources by receiving any one of DCIs during the validity duration.

However, in this way, it limits NW implementation to having to send DCIs in each SI modification period of the validity duration, which causes negative impacts on network power saving, and is inconsistent with the purpose of NES.

Meanwhile, from UE perspective, the introduced new validity duration parameter would be useless. UE does not know when the validity duration starts, since the validity duration only starts from the first SI modification period, but UE does not know whether the received activation indication is in the first SI modification period or not. Taking the figure above as an example, UE receiving this indication in MP#1 and MP#2 does not know that the start of the 3 MP is actually in MP#0 rather than in MP#1 and MP#2.

Therefore, we doubt the feasibility of this approach.

Another approach is to also broadcast the (de)activation status of the additional RACH resources for this kind of cells in SIB1. In detail, network can reuse the introduced new validity duration parameter to indicate the remaining validity duration in units of SI modification periods of the additional RACH resources (e.g., indicate 3 in MP#1, 2 in MP#2, 1 in MP#3). In this way, the newly camping UE would read the value of remaining validity duration in SIB1 to know how long the additional RACH resources would be valid. And if the additional RACH resources are not valid, the remaining validity duration would be set as inactive (e.g., using optionality bit, a code-point = 0, or a separate active/inactive bit). And considering the remaining validity duration changes in each SI modification period, the change of the remaining validity duration would not lead to SI modification notification to avoid unnecessary overhead.

Based on the analysis above, we propose to discuss the following options on this issue.

- R2 discusses the options to handle the UE who camps on the cell after the first modification period where the additional RACH resources are being activated (i.e., camp at MP#1 or MP#2 in the figure):
Option-1: Not handle this issue;
Option-2: Handle this issue, e.g., reuse the new validity duration parameter to indicate the remaining validity duration of the additional RACH resources.

Another general issue is for the update mechanism of the additional RACH configuration in SIB1, i.e., if the additional RACH configuration in SIB1 changes (changes between absent/present, or change of related configuration), whether that would lead to SI modification notification (Bit-1 within the Short Message field in DCI 1_0 with P-RNTI) or not.

If relying on SI modification notification, the legacy UE who is not capable of the additional RACH feature will also be activated, although the additional RACH resources are only useful for Rel-19 NES UE. Considering the activation indication (one bit from the Bits 5-8 within the Short Message field) is introduced in DCI 1_0 with P-RNTI for Rel-19 NES UE, the additional false alarm for the legacy UE has chance to be avoided by the activation indication.

We understand it is coupled with the issue of whether UE is required to read SIB1 to acquire additional RACH resource configuration upon reception of the activation indication via DCI 1_0 with P-RNTI or not. If yes for this issue, no need to handle that via SI modification, and vice versa.

- R2 discusses whether UE is required to read SIB1 to acquire additional RACH resource configuration upon reception of the activation indication via DCI 1_0 with P-RNTI or not.

- If Yes for P10, the update of the additional RACH configuration does not result in system information change notifications in Short Message. Otherwise, it does.

  - SSB Adaptation

In R2#129bis meeting, the following agreements on SSB adaptation were made.

Agreements on common channel adaptation 4. Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. 5. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 6. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

One open issue on SSB adaptation in RRC open issue list is how to capture the procedure text and RRC configuration of L3 RRM for SSB adaptation, with consideration of RAN4/RAN1 input. Here we further discuss the L3 RRM for SSB adaptation.

Considering that R2 defined a L3 measurement framework in the following manner, i.e., the L3 measurement configuration of SSB supporting SSB burst periodicity switching has to be carried via the same MO,

The network applies the procedure as follows: […] - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

it seems easier to perform a combined L3 measurement and generate a single MR. But that is to be further evaluated by R4, and R2 only needs to focus on MO configuration and MR report.

- For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO.

If R2 would like to progress more, as shown in the following R1 conclusions,

R1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

R1#118bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

R1#119 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

R1#120 Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell.

the remaining issue on servingCellMO configuration granularity could be considered, i.e., whether to apply this to per-cell MO (i.e., servingCellMO in ServingCellConfig) only or per-BWP MO (i.e., servingCellMO in BWP-DownlinkDedicated) as well.

- For SSB adaptation, R2 further discusses whether to support it for NCD-SSB configured in BWP-DownlinkDedicated as well.

# Paging Adaptation

One leftover issue mentioned in the EN of the RRC running CR is whether to introduce a separate pei-ConfigBWP for paging adaptation or not, as shown below.

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

Note that pei-ConfigBWP includes the PEI-dedicated search space configuration (i.e., pei-SearchSpace), and PEI-O location (i.e., firstPDCCH-MonitoringOccasionOfPEI-O). We understand that the intention of the current agreement to introduce a separate PEI configuration has already covered a separate PEI-O location, therefore, the main issue is whether the pei-SearchSpace needs to be separate or not. From R2 perspective, no motivation to introduce a separate pei-SearchSpace, so a separate PEI-O location is sufficient instead of a separate pei-ConfigBWP.

- No motivation to introduce separate pei-SearchSpace from R2 perspective.

- R2 clarifies that a separate PEI-O location for Rel-19 NES UEs is sufficient, and no need to introduce a separate PEI search space.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Applicable RA types for RACH adaptation

- R2 confirms not applying RACH adaptation to RACH resources for OD-SIB1 introduced in Rel-19 NES.

- R2 has not started to discuss on the support of 2-step RA type, with and without feature (combination)-based RACH resource partitioning, considering the late stage of this release.

- R2 does not consider RACH adaptation for 2-step RA type.

- R2 does not pursue spec impact to enable RACH adaptation for CFRA for C-HO/C-LTM.

- For CFRA towards neighbouring cell (during HO or LTM), if rach-configGeneric is not present in rach-ConfigDedicated, R2 does not consider RACH adaptation, i.e., legacy RACH resources are used in this case.

- For CBRA towards neighboring cell (during HO or LTM), R2 does not consider RACH adaptation, i.e., legacy RACH resources are used in this case.

RRC remaining issues for RACH adaptation

- For the configuration of additional RACH resources, R2 discusses the following options:
Option-1: add the parameters for additional RACH resources within the same RACH-ConfigGeneric used for legacy RACH resource; 
Option-2: add another RACH-ConfigGeneric for additional RACH resources within the same RACH-ConfigCommon for legacy RACH resources;
Option-3: add another RACH-ConfigCommon for additional RACH resources.

- CE and its related feature combinations are not supported for additional RACH resources introduced by RACH adaptation.

- R2 discusses the approaches to enable feature(-combination)-based RACH resource partitioning for the legacy RACH resources and the new additional RACH resources, including 
1) two different feature(-combination)-based RACH partitioning schemes,
2) a same feature(-combination)-based RACH partitioning scheme.

MAC remaining issues for RACH adaptation

- R2 de-prioritizes the discussion of NES and SBFD co-existence, and leave the related issue, if any (e.g., RA-RNTI design for additional RACH), to maintenance phase.

- R2 discusses whether to leave the RAR differentiation issue caused by the additional RACH resources to R1 or network implementation.

- If No for P7, R2 further discusses the two solutions below:
Solution-1: introduce new value range different from ones for RA-RNTI and MSGB-RNTI using a fixed offset on the legacy RA-RNTI formula, i.e., new RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80 x 8 x 4;
Solution-2: within the value range of RA-RNTI, legacy RACH resources use f_id values from 0 to N-1 as in legacy, while RA-RNTI for additional RACH resources (excluding the invalidated ROs due to overlapping with legacy RACH resources) adopt f_id values from N.

Remaining issues in LS (R2-2503312) for RACH adaptation

- R2 confirms that the usage of one bit from the Bits 5-8 within the Short Message field for adaptation indication in DCI format 1_0 with P-RNTI is feasible.

- R2 discusses the options to handle the UE who camps on the cell after the first modification period where the additional RACH resources are being activated (i.e., camp at MP#1 or MP#2 in the figure):
Option-1: Not handle this issue;
Option-2: Handle this issue, e.g., reuse the new validity duration parameter to indicate the remaining validity duration of the additional RACH resources.

- R2 discusses whether UE is required to read SIB1 to acquire additional RACH resource configuration upon reception of the activation indication via DCI 1_0 with P-RNTI or not.

- If Yes for P10, the update of the additional RACH configuration does not result in system information change notifications in Short Message. Otherwise, it does.

SSB adaptation

- For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO.

- For SSB adaptation, R2 further discusses whether to support it for NCD-SSB configured in BWP-DownlinkDedicated as well.

Paging adaptation

- No motivation to introduce separate pei-SearchSpace from R2 perspective.

- R2 clarifies that a separate PEI-O location for Rel-19 NES UEs is sufficient, and no need to introduce a separate PEI search space.

# 4. Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

[2] R2-2503314, LS on Rel-19 higher layers parameters list Post RAN1#120bis, RAN1

[3] R2-2503312, LS on DCI-based PRACH adaptation, RAN1