3GPP TSG-RAN WG2 Meeting #130	R2-2503461

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.3

Source: 	CATT

Title: 	Discussion on open issues for CB-Msg3 EDT

Document for: 	Discussion and Decision

Introduction

This contribution further discusses the open issues for CB-Msg3 EDT based on the open issue list in [1], and some other remaining open issues that need to be addressed to finalize this feature.

Discussions on open issues in [1]

- MAC-2: CB-RNTI calculation

Open issue MAC-2: CB-RNTI calculation Issue description: RAN2 had agreement to use CB-RNTI to scramble Msg3 and monitor Msg4. The CB-RNTI is derived from the transmit resource for the transmission window. However, the detail of calculation of CB-RNTI is still FFS. RAN2#129bis agreement: - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1. It has been captured as editor’s note.

Based on the agreement so far, it seems that the CB-RNTI is a per transmission window RNTI, i.e. a common one applied to all the CB-Msg3 transmissions within the window. The problem here is whether there is a need to further differentiate the CB-RNTI corresponding to different transmission windows. During discussion in [1], some companies proposed to have one specified CB-RNTI, e.g. FFF8, whereas some other companies commented that such specified CB-RNTI will lead to the false alarm issue happened across different transmission windows (e.g. in the case a Msg4 monitoring window covers multiple transmission windows) and still want to have the CB-RNTI calculated for each transmission window respectively.

Since anyway UE will relay on the UE ID included in the Msg4 as the response to each UE sending CB-Msg3 EDT, using one specified RNTI can still work. By contrast, if ones wants to calculate a CB-RNTI in a per transmission window manner, it needs to be further discussed which specific resource included in a transmission needs to be used to calculate the corresponding CB-RNTI (e.g. the last/first resource included in the transmission window). Also, from the moment that we concluded a single RNTI for the Msg4 monitoring, the false alarm issue is already not regarded as a critical issue by companies. So we think at this stage, perhaps the support a specified CB-RNTI is simplest way.

Proposal 1 (MAC-2): RAN2 discusses whether to support one specified CB-RNTI value in the specification.

- MAC-7: Whether the HARQ operation is applicable to transmit CB-Msg3

Open issue MAC-7: whether the HARQ operation is applicable to transmit CB-Msg3. Issue description: In legacy Random Access, the HARQ operation is appliable to transmit the Msg3. The HARQ process 0 will be used. The HARQ process will obtain the MAC PDU to transmit from Msg3 buffer when the UL grant is available. A retransmission of Msg3 can be triggered by a NACK. How and whether the HARQ operation is applicable to transmit CB-Msg3 has not been discussed. It has been captured as editor’s note.

It is true, as commented in [1] by some companies, that the CB-Msg3 EDT cannot enjoy the scheduling based HARQ operation as legacy Msg3 retransmission in RACH, because the retransmission now is performed by the transmission of replica whose resources are also selected by the UE itself. However, our understanding to this issue is whether, after the new transmission has already been performed by the first replica, the transmission(s) of the MAC PDU by the subsequent replica(s) are regarded as new transmission or retransmission in the procedures of HARQ operation (i.e. 5.4.2/5.4.2.1). As a simple example: assume that the UE selects the resources for 2 replicas' transmission; after the MAC PDU for a new transmission is built and transmitted as the 1st replica, whether the MAC PDU's transmission as the 2nd replica is treated as a new transmission or a retransmission.

From our perspective, we first think the MAC PDU transmissions as the subsequent replica(s) (other than the first replica) is obviously the retransmission of the MAC PDU. The resources of the subsequent replica(s) cannot be regarded as resource for new transmission, because otherwise the MAC entity needs to obtain MAC PDU for new transmission from the MUX entity and override the existing MAC PDU waiting for replica transmissions.

To this end, we propose that RAN2 discusses whether it is possible to treat the subsequent replicas' transmissions (other than the new transmission in first replica) for a MAC PDU as retransmissions, with the UL grant corresponding to a subsequent replica as the UL grant for retransmission.

Proposal 2 (MAC-7): HARQ operation (i.e. 5.4.2) also includes the transmission procedure for CB-Msg3 EDT. RAN2 discusses for a given MAC PDU whether the transmission on the resource(s) selected for subsequent replica(s) (other than the first replica) can be treated as "retransmission" in HARQ operation.

- MAC-9: Whether NW/UE processing time is needed when determine the Msg4 monitoring starts.

Open issue MAC-9: NW/UE processing time is needed or not. Issue description: In RAN2#129, it has been agreed that the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. FFS NW/UE processing time is needed or not. RAN2#129bis agreement: - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). It has been captured as editor’s note.

Till now, we do not see strong need from companies on support such "process time". For simplicity and progress of this feature, we propose to not consider any processing time.

Proposal 3 (MAC-9): No need to introduce NW/UE processing time for the Msg4 monitoring window start.

- MAC-10: FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT.

Open issue MAC-10: FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. Issue description: In RAN2#129, it was discussed that whether it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. Under some configuration of CB-Msg3 resource, this option can reduce the latency of reception of CB-Msg3 response. Also, to possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring. RAN2#129bis agreement: - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring. It has been captured as editor’s note.

Technically speaking, we do share the view from companies that support of this enhancement will have benefit on latency reduction, ensuring coverage, avoiding false alarm, etc. On the other hand, considering the huge number of remaining FFS and the limited progress of this feature, we would rather go for simplicity, in order for an on-time completion of the whole WI.

To this end, we propose to not pursue this configurability at this stage.

Proposal 4 (MAC-10): RAN2 no longer pursues the enhancement for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT.

- MAC-11: Whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution

Open issue MAC-11: Whether L2 ACK is supported. Issue description: In RAN2#129, it was discussed that whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. This discussion was postponed to the next meeting. It has been captured as editor’s note.

Similar to the majority's views in [1], we also think a L2 ACK (without RRC message included in Msg4) can be supported to terminate the CB-Msg3 EDT procedure for NB-IoT CP solution which does not necessarily involve the state transfer as in UP solution.

Proposal 5 (MAC-11): CB-Msg4 without RRC message is allowed as a complete response to CB-Msg3 in CP solution.

- MAC-12: FFS how the multiplexing is organized for CB-MSG4

Open issue MAC-12: FFS how the multiplexing is organized. Issue description: In RAN2#129bis, we confirmed the working assumption that one CB-Msg4 can target multiple UEs simultaneously. And how the multiplexing is organized is FFS. RAN2#129bis agreement: - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized..

- MAC-13: FFS new MAC PDU format for CB-Msg4

Open issue MAC-13: FFS the Msg4 structure including Contention Resolution Identities, C-RNTIs, Backoff parameters, TA information in the CB-Msg4. Issue description: We had agreement to include the Contention Resolution Identities, C-RNTIs, Backoff parameters, TA information in the CB-Msg4. How this information is included in the Msg4 (e.g., the format) and the structure of Msg4 is unknown. RAN2#128 agreement: - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) RAN2#129bis agreement: - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4 - Backoff information could be included in CB-Msg4..

We think issues in MAC-12 and MAC-13 are relevant, so we discuss them together. The design of the MAC PDU format depends on the information that needs to be included. Based on the RAN2 agreement, now the following information is agreed:

- Contention Resolution Identity

- C-RNTI

- Timing alignment information

- Backoff information

- RRC messages

- User data (if any)

Based on the agreed information, we think we can introduce the following elements into the CB-Msg4 MAC PDU:

- One Backoff information MAC CE

For simplicity, only one BI MAC CE is sufficient in a CB-Msg4 MAC PDU.

- A Contention Resolution Identity MAC CE

This MAC CE is UE specific. It includes a Contention Resolution identity (included in CB-Msg3) targeting a specific UE, and (optionally) includes C-RNTI and TA information. Note that this design is imitating the design of successRAR MAC CE in NR 2-step RACH.

- MAC SDU

The MAC SDU is UE specific. It is optionally included, and can include RRC message and/or User data. Note that a MAC SDU targeting a UE should be associated with a Contention Resolution Identity MAC CE identifying the corresponding UE.

Therefore, we propose the following elements included in the CB-Msg4 MAC PDU format.

Proposal 6 (MAC-12/13): CB-Msg4 MAC PDU includes the following elements:

- One Backoff information MAC CE;

- One or more Contention Resolution Identity MAC CEs, with each MAC CE including a Contention resolution Identity, C-RNTI (optionally) and timing alignment information (optionally);

- (Optionally) One or more MAC SDUs, with each SDU including RRC signaling or User data and associated with a Contention Resolution Identity MAC CE.

- MAC-14: FFS for the detail of HARQ operation on CB-Msg4.

Open issue MAC-14: FFS for the detail of HARQ operation on Msg4. Issue description: We had agreement that HARQ feedback is adopted to acknowledge Msg4. Since one CB-Msg4 can target multiple UEs simultaneously, there could be multiple HARQ feedbacks for one CB-Msg4. How the PUSCH resources are provided to support multiple HARQ feedbacks is unknown. Whether the ACK or NACK should be used also needs further discussion. RAN2#129bis agreement: - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.)..

This information involves how to allocate the ACK/NCK feedback resources to multiple UEs targeted by the same CB-Msg4. Some companies suggested in [1] to follow a similar design as successRAR in NR 2-step RACH. However, we wonder whether it is eventually up to RAN1 o decide how to allocation on ACK/NCK feedback resources for this kind of CB-Msg4 multiplexing response to multiple UEs. So we suggest asking RAN1 to do the related design.

Proposal 7 (MAC-14): It is up to RAN1 how to allocate the ACK/NCK feedback resources for a CB-Msg4 multiplexing responses to multiple UEs.

- MAC-15:  What should be the UE behavior (e.g. the can initiate the legacy 4-step RA) when the CB-Msg3 procedure fails.

Open issue MAC-15: Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. Issue description: Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails was discussed in the RAN2#129bis. Most companies agreed. But when the CB-Msg3 procedure failes, which upper layer should be notified is still unknown. This issue was postponed to the next meeting. It has been captured as editor’s note.

We think when CB-Msg3 procedure fails, the MAC should indicate the problem to RRC which then further decides whether to trigger 4-step RACH. But what we would like RAN2 to further clarify is whether such legacy 4-step RA includes legacy EDT or not.

Proposal 8 (MAC-15): When CB-Msg3 EDT procedure fails, MAC indicates to the upper layers (RRC) which determines whether to fall back to legacy 4-step RA. RAN2 further clarifies whether legacy 4-step RA includes legacy EDT.

- MAC-17: Whether to allow multiple TBSs as in EDT.

Our understanding is that since the MCS associated with a CE level should be configured with a fixed value, and the resource unit in the resource configuration per CE level is also fixed, then if the UE performs the CB-Msg3 EDT procedure at a given CE level, the TBS would be a fixed value determined by the associated MCS and resource unit associated with this CE. So the issue is not crystal-clear to us, and thus may need further clarification. No proposal is given for this issue.

- MAC-18: How to model the CB-Msg3 response window (i.e. MSG4 monitoring window) ? Should it be a timer as in legacy RA response window, and what should be the value range.

There seems to be no big difference in using a window or timer, to handle the right time that the UE needs to monitor CB-Msg4 response from the NW. This is purely Stag-3 issue, so we think it is OK to leave it to MAC CR rapporteur to use a proper Spec modelling.

Proposal 9 (MAC-18): Up to MAC running CR Rapporteur to decide whether to use a timer or window definition to handle CB-Msg4 reception (which is a Spec modelling issue).

Conclusion

In this contribution, we discussed remaining open issues for CB-Msg3 EDT procedure in [1]. Our proposals are as follows:

Proposal 1 (MAC-2): RAN2 discusses whether to support one specified CB-RNTI value in the specification.

Proposal 2 (MAC-7): HARQ operation (i.e. 5.4.2) also includes the transmission procedure for CB-Msg3 EDT. RAN2 discusses for a given MAC PDU whether the transmission on the resource(s) selected for subsequent replica(s) (other than the first replica) can be treated as "retransmission" in HARQ operation.

Proposal 3 (MAC-9): No need to introduce NW/UE processing time for the Msg4 monitoring window start.

Proposal 4 (MAC-10): RAN2 no longer pursues the enhancement for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT.

Proposal 5 (MAC-11): CB-Msg4 without RRC message is allowed as a complete response to CB-Msg3 in CP solution.

Proposal 6 (MAC-12/13): CB-Msg4 MAC PDU includes the following elements:

- One Backoff information MAC CE;

- One or more Contention Resolution Identity MAC CEs, with each MAC CE including a Contention resolution Identity, C-RNTI (optionally) and timing alignment information (optionally);

- (Optionally) One or more MAC SDUs, with each SDU including RRC signaling or User data and associated with a Contention Resolution Identity MAC CE.

Proposal 7 (MAC-14): It is up to RAN1 how to allocate the ACK/NCK feedback resources for a CB-Msg4 multiplexing responses to multiple UEs.

Proposal 8 (MAC-15): When CB-Msg3 EDT procedure fails, MAC indicates to the upper layers (RRC) which determines whether to fall back to legacy 4-step RA. RAN2 further clarifies whether legacy 4-step RA includes legacy EDT.

Proposal 9 (MAC-18): Up to MAC running CR Rapporteur to decide whether to use a timer or window definition to handle CB-Msg4 reception (which is a Spec modelling issue).

Reference

- R2-2504526	Remaining MAC open issues in IoT NTN