3GPP TSG-RAN WG2 Meeting #130	R2-2503481

Malta, 19th – 23rd May 2025

Agenda Item:	8.2.2

Source: 	Xiaomi

Title:  	Remaining open issues on A-IOT paging procedure

Document for:	Discussion and Decision

# 1	Introduction

In the email discussion [POST129bis][017][AIoT] 38.391 Running CR (Huawei), A-IoT MAC CR Rapporteur listed open issues for Paging, Random access and data transmission.

In this contribution, we discuss the following paging related open issues:

- Subgroup: Transaction ID

  - Issue 1-2: transaction ID

-	Subgroup: Paging message content

  - Issue 1-3: Paging ID length field

  - Issue 1-4: AO number field

  - Issue 1-5: Paging content for CFRA

-	Subgroup: Others

  - Issue 1-6: Paging ID visibility

# 2	Discussion

## 2.1 Subgroup: Transaction ID (issue 1-2)

### 2.1.1 Issue 1-2: transaction ID

As listed by Rapporteur, the issue is

| Issue 1-2: transaction ID | Whether/how to specify how the reader generate Transaction ID, and the size - Relevant agreements: - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size - Status in running CR: captured as Editor’s Note in 6.2.1.1. |
| --- | --- |

The issue was discussed in email discussion [1]. To our understanding, the only thing RAN2 needs to do is to agree the purpose of the transaction ID, i.e., avoid duplicated response from the same device for the same service and the size of it, no matter whether it is generated by CN or reader, and no matter whether it is generated based on correlation ID or not since the details shall be discussed and decided by RAN3.

Regarding the size, to our understanding, to avoid the case that a device misses paging due to charging, the repetition times should be longer enough. Longer ID size needs more resources for the transmission from the reader side and more power consumption for the reception and decoding from device side. Therefore 2-4 bits should be sufficient and as shorter as better.

Note: RAN2 has excluded 1 bit solution at RAN2 129 meeting.

Proposal 1 (Issue 1-2): RAN2 to agree the purpose of transaction ID is to avoid duplicated response from the same device for the same session/service. The size of transaction ID is 4.

Proposal 1.1 (Issue 1-2): Send LS to RAN3, CC SA2, inform them of the need of transaction ID (avoid duplicated response from the same device for the same service) and the size (4bits), and ask them to decide how to generate the transaction ID.

### 2.1.2 Association between the paging identifier and the AS-ID

[1] also proposed to discuss how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.). To our understanding, it is related how gNB and CN maintain the UE context, e.g., whether separate NGAP ID for different devices or same NGAP ID for a group of devices. It should be discussed in RAN3 instead of RAN2. RAN3 agreed to introduce NGAP Device ID based per session per device association between gNB and AIOTF in case of Command after Inventory. Therefore RAN2 does not need to consider/resolve this issue.

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device. - The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE.

Proposal 2: per session per device association between gNB and AIOTF has been concluded in RAN3. No RAN2 action is needed.

## 2.2 Subgroup: Transaction ID (Issue 1-3, 1-4, 1-5)

### 2.2.1 Issue 1-3 Paging ID length field

As listed by Rapporteur, the issue is

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g. value range, how many bits, format design - Relevant agreements: - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - RAN2 sent LS to CT4 and SA2 in R2-2503197 asking for their feedback on the above agreement, for RAN2 to determine the field for paging ID length. - Note: SA2 already agreed the filtering information and captured it in clause 5.8 in 23.369, this may enable some extent of RAN2 discussion before their feedback. - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. |
| --- | --- |

As captured in TS23.369, the AIOT device permanent ID consists of fixed part, and variable part.

NOTE 3: The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6] and TS 29.xxx [xx]. The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits. NOTE 4: The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

SA2 also sent the LS S2-2504296 to CT4, ask them to consider the length of the ID.

Regarding the RAN2 agreements, there are two different understandings:

Option 1: Paging ID length field indicates the type of Paging ID

The assumption of the option 1 is, the length of paging ID could be different for different use cases, e.g., different factor owner, etc. Therefore, we only need to indicate the mapping of it. However, the risk is that more use cases may come later, it is difficult to predict the number of different paging ID length.

Option 2: Paging ID length field indicates the real length of Paging ID

With this option, RAN2 only needs to know the maximum length of paging ID, or reserve 1 more bit on top of it. It is simple from design perspective.

In RFID,

- The typical ID length for TID is 96 bits (12 bytes) to 240 bits (30 bytes).

- The typical ID length for EPC is 96 bits (12 bytes), but may extend to 128 (16 bytes) or 256 bits (32 bytes).

- For some special scenario, the maximum length could be 512 bits (64 bytes)

Therefore, the paging ID length field can refer to the maximum length 512 bits (64 bytes). Another question is, whether we should indicate the length in bit level (9bits) or byte level (6 bits). As agreed in last RAN2 meeting, RAN2 has agreed the assumption that for both R2D message and D2R message the upper layer data SDU to be contained in the AIoT MAC PDU is byte-aligned. To our understanding, Paging ID is similar to Device ID which is the upper layer data. Therefore byte level should be sufficient, i.e. introduce 6 bits Paging ID length field to indicates maximum 64 bytes Paging ID length.

Proposal 3 (Issue 1-3): Paging ID is byte aligned. The size of Paging ID length field is 6 bits to indicate the maximum 64 bytes Paging ID.

Proposal 3.1 (Issue 1-3): Send LS to SA2 and CT4 to inform them of RAN2 agreements “Paging ID is Byte aligned, and max size is 64bytes”.

### 2.2.2 Issue 1-4: AO number field

As listed by Rapporteur, the issue is

| Issue 1-4: AO number field | How to indicate the number of access occasions, e.g. the maximum number, the length of field, format design. - Relevant agreements: - the A-IoT paging message can include a number of msg1 resources - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. |
| --- | --- |

As captured in draft A-IoT MAC CR, the number of access occasion is to indicate the total number of access occasions within a paging round, i.e., between two paging messages.

The A-IoT MAC entity shall: 1> randomly select an access occasion for transmission of the Random ID message among the access occasions configured in A-IoT Paging message; Number of Access Occasions: This field indicates the number of access occasions.

According to RAN1 discussion, from time domain, X (the number of slot) could be 1 or 2, but 2 is only applicable for Msg1 transmission. From frequency domain, Y (the max number of devices to be multiplexed in the frequency domain of an access slot) could be 4 or 6 (FFS in RAN1).

From RAN2 perspective, we need to consider two factors, total number of devices in a reader and the max number of devices to be multiplexed in the frequency domain of an access occasion.

MaxNOfAO=2164 or 6

Therefore MaxNoOfAO is 14 bits or 15 bits if we want to consider max 65535 devices and if the Ymax is 4 or 6.

Proposal 4 (Issue 1-4): MaxNoOfAO is 14 bits or 15 bits in order to address max 65535 devices, assuming Ymax is 4 or 6.

### 2.2.3 Issue 1-5: Paging content for CFRA

As listed by Rapporteur, the issue is

| Issue 1-5: Paging content for CFRA | Whether paging in CFRA can omit the CBRA related fields, such as transaction ID, Indication of Paging ID present/absence, Number of access occasions in Paging message. - In last meeting, there was a discussion whether transaction ID is needed for CFRA in Paging message. Then during the CR drafting, the Rapp identifies other fields like no paging ID indication, number of access occasions are not useful for CFRA. Since RAN2 also agreed to introduce an explicit indication to indicate CFRA and CBRA, it should be feasible to have different fields in the paging message for CFRA and CBRA. Thus, this can be discussed further. - Status in running CR: captured as Editor’s Note in 6.2.1.1. |
| --- | --- |

For paging triggered CF access, the target device id will be included in paging message, and the reader will know whether the target device has completed the service or not. If not, network can send paging again to trigger the device to re-access with the device ID. The transaction ID is not needed. As also mentioned by Rapporteur, the number of access occasions, Indication of Paging ID present/absence are also not needed for CFRA.

Another question is how to indicate the absent of these fields. We fully agree with Rapporteur’s view that “Since RAN2 also agreed to introduce an explicit indication to indicate CFRA and CBRA, it should be feasible to have different fields in the paging message for CFRA and CBRA.” If RA type is contained in the Paging message before “Indication of Paging ID presence/absence”, “Transaction ID” and “Number of Access Occasions”, the device can identify the subsequent content of the paging message is different that is same as we introduce two individual messages for CBRA and CFRA indicated by R2D Message Type+RA Type.

| CBRA - R2D Message Type: This field indicates the message type. See the Table 6.1-1. - RA Type: This field indicates CBRA - Indication of Paging ID presence/absence: This field indicates whether Paging ID is present or absent. - Length of Paging ID: This field indicates the Paging ID length information when Paging ID field is present. - Paging ID: xxx - Transaction ID: xxx - Number of Access Occasions: This field indicates the number of access occasions. - D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling. | CFRA - R2D Message Type: This field indicates the message type. See the Table 6.1-1. - RA Type: This field indicates CFRA - Length of Paging ID: This field indicates the Paging ID length information when Paging ID field is present. - Paging ID: xxx - D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling. |
| --- | --- |

Proposal 5 (Issue 1-5): For paging message, “transaction ID”, “the number of access occasions”, “Indication of Paging ID present/absence” are not contained for CFRA.

Proposal 5.1 (Issue 1-5): For paging message, the message content for CBRA and CFRA is distinguished by RA Type.

## 2.3 Subgroup: Others (Issue 1-6)

### 2.3.1 Issue 1-6: Paging ID visibility

As listed by Rapporteur, the issue is

| Issue 1-6: Paging ID visibility | Whether Paging ID is invisible or visible to MAC. - Relevant agreements: - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - From the previous discussion, there are some motivations to make paging ID visible to MAC: - 1. Reader can operate on the paging ID for further sub-grouping. The Rapp understands this can be considered as an enhancement from reader side for better system efficiency. From device side, since there is an explicit indication for CBRA and CFRA, the device (even in multi-device CFRA) can determine how to perform random access instead of paging ID/group ID. In this case, such visibility is not an essential function. And according to guidance from chair lady, such enhancement can be considered with lower priority. - 2. Reader can associate the paging ID/device ID and AS ID for a given device within a service request. The Rapp understands according to RAN3 LS R3-252481, reader will allocate NGAP device ID for each device and maintain the per-session per-device context, via which the reader can associate the command receiving from the NG interface with the AS ID assigned for a device, i.e. such device management/association does not rely on the paging ID/device ID. - 3. The Temp ID may have impact on this visibility discussion. The Rapp understands SA3 has not concluded on the solution of Temp ID. But majority seems think this Temp ID is maintained/managed between CN and device, since they already concluded there is no AS security in A-IoT. Therefore, no RAN2 discussion is needed before SA3 further inputs. |
| --- | --- |

Companies have different understanding on visibility.

From reader side:

Understanding 1: visibility means the reader needs to understand whether it is for single or group devices;

Understanding 2: visibility means the reader can change the ID;

From device side:

Understanding 3: visibility means the device A-IoT MAC layer can identify whether the ID is for itself or not;

To our understanding, the paging ID is the data from upper layer. It would be good to handle it together with other upper layer data. Same as what Rapporteur captured in the draft A-IoT specification, the paging ID shall be checked by upper layer. Regarding reader side, as we discussed before, there is not strong motivation for the reader to change the ID. Considering nothing to be changed, the open issue 1-6 can be closed.

Proposal 6 (Issue 1-6): Visibility means the reader cannot change the Paging ID, no spec impact. the Issue 1-6 can be closed.

# 3	Conclusion

In this contribution, we discussed the procedures and signalling for the initial access in A-IOT and have corresponding proposals.

Subgroup: Transaction ID:

Proposal 1 (Issue 1-2): RAN2 to agree the purpose of transaction ID is to avoid duplicated response from the same device for the same session/service. The size of transaction ID is 4.

Proposal 1.1 (Issue 1-2): Send LS to RAN3, CC SA2, inform them of the need of transaction ID (avoid duplicated response from the same device for the same service) and the size (4bits), and ask them to decide how to generate the transaction ID.

Proposal 2: per session per device association between gNB and AIOTF has been concluded in RAN3. No RAN2 action is needed.

Subgroup: Paging message content:

Proposal 3 (Issue 1-3): Paging ID is byte aligned. The size of Paging ID length field is 6 bits to indicate the maximum 64 bytes Paging ID.

Proposal 3.1 (Issue 1-3): Send LS to SA2 and CT4 to inform them of RAN2 agreements “Paging ID is Byte aligned, and max size is 64bytes”.

Proposal 4 (Issue 1-4): MaxNoOfAO is 14 bits or 15 bits in order to address max 65535 devices, assuming Ymax is 4 or 6.

Proposal 5 (Issue 1-5): For paging message, “transaction ID”, “the number of access occasions”, “Indication of Paging ID present/absence” are not contained for CFRA.

Proposal 5.1 (Issue 1-5): For paging message, the message content for CBRA and CFRA is distinguished by RA Type.

Subgroup: Others:

Proposal 6 (Issue 1-6): Visibility means the reader cannot change the Paging ID, no spec impact. the Issue 1-6 can be closed.

# Reference

[1] R2-2502211 Email discussion report: [POST129][035][AIoT] Paging

[2] R2-2503998 Summary of A-IoT MAC open issues