3GPP TSG-RAN WG2 Meeting #130	R2-2503490

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discovery and relay (re)selection for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the discovery and relay (re)selection for multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

Open issue RRC-5: NotificationMessageSidelink message handling

For the NotificationMessageSidelink message handling, the following agreements were made in RAN2 #129b meeting:

RAN2#129 bis

- When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

- When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

- FFS detailed information in the indication and child UE handling.

- FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

- FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

For the first FFS on the notification forwarding at the intermediate relay, there are 2 options listed in the open issue list:

When an intermediate relay UE receives a NotificationMessageSidelink message from the parent, it has two options Option 1 – Forward a copy of the original notification (with the same cause) to its child UEs. Option 2 – Generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment.

As analyzed by the rapporteur, Option 1 risks causing duplicate notifications due to redundant message propagation. In contrast, Option 2 is preferred because its notification mechanism operates as a per-hop indication. This approach avoids duplicate forwarding and ambiguity, ensuring each Child UE can promptly act based on the most relevant and current context of its Parent Relay UE (e.g., cell changes or PC5 connection states). For intermediate Relay UEs, upon receiving a notification:

- If in RRC CONNECTED state, they will first trigger RRC re-establishment.

- If in RRC IDLE/INACTIVE state, they will either maintain or release the PC5 connection with their Parent Relay UE.

Following these actions (RRC re-establishment, cell change, or PC5 release), the intermediate Relay UE generates a new notification message reflecting its latest status (e.g., cell change or PC5 release) and notifies the Child UEs accordingly.

The intermediate relay UE handles the received notification message before transmitting notification message to its connected child UE can avoid duplicated notification.

(RRC-5) Upon receiving NotificationMessageSidelink from parent relay, the intermediate Relay UE handles the received notification message first (e.g., trigger RRC Reestablishment, cell change or PC5 release) and then generates NotificationMessageSidelink to its connected child UE(s) based on the latest status.

The FFS in the second agreement are about whether and how to signal relay reselection at the intermediate Relay UE to the Child UE.

For “whether”, the key argument is whether the new path’s link-related information (e.g., cell ID, hop number…) remains identical to the old path and continues to satisfy the relay reselection criteria at the remote UE defined by both the AS and upper layer. The following criteria are defined by AS and upper layer:

Table 1. Relay (re)selection criteria

| AS criteria | Cell ID/PLMN ID | Intermediate relay cannot know the criterion at the Remote UE, it can only check whether cell ID/PLMN ID is changed. |
| --- | --- | --- |
|  | PC5 RSRP quality | Doesn’t need to be checked by the intermediate relay for the remote |
| Upper layer criteria | Hop count | Intermediate relay can know |
|  | Accumulated QoS | Intermediate relay cannot know the criterion at the Remote UE. |

As highlighted in the table above, the intermediate Relay UE has no visibility into the relay reselection criteria (e.g., cell ID, PLMN ID, hop count, accumulated QoS metrics) configured at the Remote UE. Consequently, it cannot assess whether a new link meets the Remote UE’s suitability requirements. Therefore, notifications about path changes must always be propagated to ensure the Remote UE can independently evaluate and act on updated network conditions.

The path switching at the intermediate relay may cause change of serving cell ID/PLMN ID, hop number, and accumulated QoS information, which may invalidate both AS/upper layer relay (re)selection criteria at the child UE.

The intermediate Relay UE cannot validate the suitability of a new path for multiple downstream Remote UEs, as it lacks visibility into their relay (re)selection criteria. Consequently, notifications must always sent.

For how to send the indication, there are the following 3 options:

- Option-1: follow single-hop U2N Relay mechanism, rely on NotificationMessageSidelink;

- Option-2: rely on upper layer discovery message, i.e., the discovery message transmitted according to the new link;

- Option-3: rely on upper layer link modification message.

For the NotificationMessageSidelink, it is an existing signalling defined in R17/R18 used to notify Remote UEs of link failures or changes. It includes specific cause values for distinct scenarios (e.g., radio link failure, handover), for this relay reselection case, the PC5 connection to the source Parent Relay UE must always be released, regardless of the reselection trigger (e.g., RLF, mobility), so the simplest way would be to reuse the legacy cause value used in R18 U2U Relay for the PC5 connection release (relayUE-PC5-RLF).

Since all relay reselection triggers necessitate releasing the PC5 link to the source Parent Relay UE, the legacy relayUE-PC5-RLF cause value (defined in R18) can be reused to signal this event.

For the discovery message, based on TS 23.304, for Model-A discovery, the discovery transmission has dependency on broadcast at the parent UE, i.e., it is transmitted only if received from the parent UE. For Model-B discovery, the solicitation message is only initiated by the remote UE, without know the path change status, remote UE cannot know when to trigger the Model-B discovery.

Model-A: <…> 3. The 5G ProSe Intermedidate UE-to-Network Relay creates a corresponding discovery entry based on the received 5G ProSe UE-to-Network Relay Discovery Announcement message, if the discovery message meets the AS layer criteria and the Hop-Count is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message. <…> Model-B: <…> 2a. The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Relay Discovery Solicitation message additionally contains following IEs compared with that in clause 6.3.2.3.3: an indication that multi-hop relay is supported, Hop-Count and Hop-Limit (optional). The Target Info may contain the User Info ID of the UE-to-Network Relay and Intermediate UE-to-Network Relay(s). 3a. If an indication that multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message. <…>

Neither Model-A nor Model-B discovery transmissions are fully initialized by the intermediate Relay UE, i.e., depends on whether/when the discovery procedure is initialized at the Last Relay or Remote UE. So cannot reliably indicate path updates.

At last, for the Link modification information, as captured in TS 23.304, it is not clear whether it can be applied to L2 case, and whether it can support all the cases that may happen for L2 Relay.

Figure 6.4.3.8.3-2: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays with Root Relay change When the 5G ProSe Intermediate Relay-1 has lost the connection or has an imminent connection loss/deteriorated connection with its parent 5G ProSe Intermediate Relay or the 5G ProSe UE-to-Network Relay, the 5G ProSe Intermediate Relay-1 performs relay reselection by using the relay discovery procedure in clasue 6.3.2.5. For Layer-3 UE-to-Network relay, if the 5G ProSe Intermediate Relay-1 decides a new 5G ProSe UE-to-Network Relay is used after the relay reselection, IP address/prefix changes may happen. In this case, the Intermediate UE-to-Network Relay-1 notifies its child relay(s) and/or Remote UE(s) about the change of Root 5G ProSe UE-to-Network Relay before sending Remote User ID and Remote UE Info to its parent Relay. The 5G ProSe Intermediate Relay-1 sends Link Modification Request message to its child relay(s) and Remote UE(s) including an indication that the Root UE-to-Network Relay is changed. The nofitication triggers the Remote UE to request for a new IP address/prefix from the new Root 5G ProSe UE-to-Network Relay.

It is not clear whether the link modification defined in SA2 is only applied to L3 U2N Relay, and whether it can support all the cases that may happen for L2 U2N Relay.

Given we only have two meetings, there is no time for checking with SA2 on the applicability to L2 U2N Relay and completing any design needed.

With only two meetings left, there is no time for the LS to check with SA2 on the applicability to L2 U2N Relay and complete the design.

Based on the above analysis, the below table summarizes the key issues of each option

Table 2. Analysis of signalling to indicate the path change information

| Option | Can be used for L2? | Spec impact | Can support all cases? |
| --- | --- | --- | --- |
| Option-1: NotificationMessageSidelink | Yes | Minor since both legacy signalling and cause value can be reused | Yes |
| Option-2: discovery | Yes | Have impact to SA2 specification | No, since the discovery message is not fully initialized by the intermediate relay UE. |
| Option-3: link modification information | Not sure | Have impact to SA2 specification | Not sure |

Based on the analysis, the most workable and easy way forward is to use NotificationMessageSidelink as in legacy.

(RRC-5) In all relay reselection scenarios, upon PC5 link release between parent relay and the grand-parent relay, the parent relay notifies its child node(s) via NotificationMessageSidelink with the legacy cause value relayUE-PC5-RLF.

Open issue RRC-6: Discovery Model B with no PC5 Link in multi hop scenario

For the PC5 threshold condition, RAN2 agree no PC5 threshold condition is needed for Model-A and Model-B when the PC5 link has been established. And for the case when the PC5 link between the intermediate relay and its parent is not established in Model-B, PC5 threshold condition is agreed, but FFS whether this case is a valid case

Agreements:

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For L2 Relay case, a UE with a SIM card always needs to find a cell for camping (i.e., paging/SIB reception). For a UE who can act as intermediate relay which is out of coverage, no matter it is acting as intermediate relay (with remote UE connected) or not yet, it needs to rely on a relay link for SI/Paging/data reception/transmission, which means it firstly acts as a remote UE.

The following procedures describe the reasonable UE (with SIM card) behavior which is captured in TS 38.304, the UE should always search for cell upon switching on.

When a UE is switched on, a public land mobile network (PLMN) or a SNPN is selected by NAS. For the selected PLMN/SNPN, associated RAT(s) may be set, as specified in TS 23.122 [9]. The NAS shall provide a list of equivalent PLMNs or a list of equivalent SNPNs, if available, that the AS shall use for cell selection and cell reselection. With cell selection, the UE searches for a suitable cell of the selected PLMN or selected SNPN, chooses that cell to provide available services, and monitors its control channel. This procedure is defined as "camping on the cell". <…> The purpose of camping on a cell in RRC_IDLE state and RRC_INACTIVE state is as follows: a) It enables the UE to receive system information from the PLMN or the SNPN. b) When registered and if the UE wishes to establish an RRC connection or resume a suspended RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped. c) If the network needs to send a message or deliver data to the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNA (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in the corresponding set of areas. The UE will then receive the paging message and can respond. d) It enables the UE to receive ETWS and CMAS notifications. e) It enables the UE to receive MBS broadcast services. f) It enables the UE to receive MBS multicast services in RRC_INACTIVE state.

Figure 1. Procedure for a switch-on UE

For L2 Multi-hop U2N Relay, the intermediate relay UE first acts as a remote UE for SI/Paging/data reception/transmission.

In other words, in L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves the remote UE no matter for Model-B or Model-A. Which means the FFS case is not reasonable for L2 multi-hop U2N Relay.

In L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves a remote UE regardless of Model-B or Model-A. Which means the FFS case (PC5 link not established) is not a reasonable case for L2 multi-hop U2N Relay.

(RRC-6) For L2 U2N Relay, RAN2 consider the intermediate relay UE always establishes connection with a parent relay by acting as a remote UE first.

Conclusion

We have the following observations:

Observation 1	The intermediate relay UE handles the received notification message before transmitting notification message to its connected child UE can avoid duplicated notification.

Observation 2	The path switching at the intermediate relay may cause change of serving cell ID/PLMN ID, hop number, and accumulated QoS information, which may invalidate both AS/upper layer relay (re)selection criteria at the child UE.

Observation 3	The intermediate Relay UE cannot validate the suitability of a new path for multiple downstream Remote UEs, as it lacks visibility into their relay (re)selection criteria. Consequently, notifications must always sent.

Observation 4	Since all relay reselection triggers necessitate releasing the PC5 link to the source Parent Relay UE, the legacy relayUE-PC5-RLF cause value (defined in R18) can be reused to signal this event.

Observation 5	Neither Model-A nor Model-B discovery transmissions are fully initialized by the intermediate Relay UE, i.e., depends on whether/when the discovery procedure is initialized at the Last Relay or Remote UE. So cannot reliably indicate path updates.

Observation 6	It is not clear whether the link modification defined in SA2 is only applied to L3 U2N Relay, and whether it can support all the cases that may happen for L2 U2N Relay.

Observation 7	With only two meetings left, there is no time for the LS to check with SA2 on the applicability to L2 U2N Relay and complete the design.

Observation 8	For L2 Multi-hop U2N Relay, the intermediate relay UE first acts as a remote UE for SI/Paging/data reception/transmission.

Observation 9	In L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves a remote UE regardless of Model-B or Model-A. Which means the FFS case (PC5 link not established) is not a reasonable case for L2 multi-hop U2N Relay.

We have the following proposals:

Proposal 1	(RRC-5) Upon receiving NotificationMessageSidelink from parent relay, the intermediate Relay UE handles the received notification message first (e.g., trigger RRC Reestablishment, cell change or PC5 release) and then generates NotificationMessageSidelink to its connected child UE(s) based on the latest status.

Proposal 2	(RRC-5) In all relay reselection scenarios, upon PC5 link release between parent relay and the grand-parent relay, the parent relay notifies its child node(s) via NotificationMessageSidelink with the legacy cause value relayUE-PC5-RLF.

Proposal 3	(RRC-6) For L2 U2N Relay, RAN2 consider the intermediate relay UE always establishes connection with a parent relay by acting as a remote UE first.