3GPP TSG-RAN WG2 Meeting #130	R2-2503503

St. Julian’s, Malta, 19 – 23 May, 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Remaining issues on PWS support for NB-IoT

Agenda Item:	8.9.4

Document for:	Discussion and decision

# Introduction

The WID [1] supports PWS for NB-IoT with the following descriptions:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In the RAN2#129 meeting, the following agreements were achieved:

Agreements:

1.	Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

2.	Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

3.	Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

4.	We don’t introduce a new SIB for carrying geographical information for emergency areas

In the RAN2#129bis meeting, the following agreement was achieved:

Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

In this contribution, we provide our consideration on how to ensure PWS message delivery in a time efficient manner.

# Discussion

## 2.1 SIB1-NB skipping for PWS reception in RRC Idle

It is important to note that there are specific requirements for delivery of ETWS message, as outlined in TS 22.268 [2]:

-	Quick Warning Notification delivery after the occurrence of Earthquake or Tsunami.

Earthquake and Tsunami propagate very fast. The duration time between the actual occurrence of the disaster and its arrival is very short. The order of the duration time is around seconds or minutes at most. Therefore the Warning Notifications shall be delivered quickly to the users in the emergency impacted area so that they could take any actions to escape from danger.

The amount of data included in the ETWS primary notification is limited to a few bytes to ensure efficient delivery. However, the ETWS secondary notification and CMAS notification are expected to contain elements that instruct the UE to take specific actions, which may involve larger data volume:

-	Event Description

-	Area Affected

-	Recommended Action

-	Expiration Time (with time zone)

-	Sending Agency

Besides, additional elements may be included, based on regulatory requirements. In such cases, the message may need to be delivered in multiple segments, which could result in longer latency.

It is also observed that there is growing necessity to ensure that the public needs the capability to receive the alerts, warnings and critical information regarding emergencies timely and accurately. This is essential for taking prompt actions to avoid serious injury, property loss or even threat to life.

There is an FFS issue from the last meeting on whether SIB1-NB can be skipped when acquiring the PWS related SIB(s). From our point of view, it is beneficial to skip SIB1-NB to decrease the corresponding waiting period and processing time. The most straightforward approach would be for the NW to send the scheduling information in advance and indicate whether the PWS SIBs are being broadcast or not, similar to NR on-demand SI si-BroadcastStatus indication. If the PWS SIBs are not being broadcast, the UE would just store the scheduling information. When the ETWS/CMAS notification is sent via paging, UE can then directly use the stored scheduling information for the acquisition of the PWS SIBs.

Proposal 1: (RRC-5) NW can choose to send the scheduling information of PWS SIBs in advance and indicate whether the PWS SIBs are being broadcast or not similar to NR on-demand SI si-BroadcastStatus indication.

## 2.2 PWS reception across different cells

It may be difficult for the UE to receive all the PWS segments from one serving cell due to limited serving time per cell in NTN, especially in the earth moving cell scenario. According to TS 36.331, when a UE moves to a new cell, the UE discards the previously buffered PWS segments. If a cell change occurs frequently, the latency of delivering all PWS segments could be extended significantly and hard to estimate, also resulting in increased UE power consumption. What is worse, a UE may miss some segments already after switching to a new cell and there might not be enough time for the UE to receive all the segments in this cell, causing PWS reception failure.

In order to increase the probability of timely delivering all PWS segments for one PWS message, it is beneficial to introduce inter-cell PWS segment reception. Specifically, RAN could indicate to the UE the PWS area where it can assemble the PWS segments from different cells to avoid PWS reception latency or failure. The UE would then decide whether to keep the previously buffered PWS segments or discard them after switching to a new cell, depending on whether the source cell and the target cell belong to the same PWS area.

Fig1. the target area for continued PWS reception across cells

Proposal 2: (RRC-6) A UE is allowed to assemble the PWS segments from different cells within an area.

# Conclusion

This contribution discussed the support of PWS for NB-IoT. Proposals are summarized as follows:

Proposal 1: (RRC-5) NW can choose to send the scheduling information of PWS SIBs in advance and indicate whether the PWS SIBs are being broadcast or not similar to NR on-demand SI si-BroadcastStatus indication.

Proposal 2: (RRC-6) A UE is allowed to assemble the PWS segments from different cells within an area.

# References

RP-243278, Revised WID: Non-Terrestrial Networks (NTN) for IoT Phase 3.

3GPP TS 22.268, Public Warning System (PWS) requirements