3GPP TSG-RAN WG2 Meeting #130	R2-2503507

St. Julian’s, Malta, May 19th - 23rd, 2025

Agenda item:		8.7.5

Source:		Samsung

Title:			Discussion on RLC Enhancements for XR

Document for:	Discussion & Decision

# 1 Introduction

In RAN2#129bis, following agreements were made on RLC enhancements:

Agreements on RLC enhancements Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

Also, RAN2 reached following agreements for PDCP SN gap reporting for mobility scenario, considering the Rel-19 AM RLC enhancements:

PDCP SN gap reporting in context of R19 RLC enhancements - From RAN2 point of view, for Rel-18 alternative 3 should not be supported. RAN2 doesn’t see a big problem to solve for Rel-18. - RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives. - The session chair indicates that it can be treated in RLC enhancement AI

In this paper we discuss the following:

- Relevant Stage-3 issues for R19 RLC enhancements

- PDCP SN gap reporting during mobility in R19

- Impact of R19 RLC enhancement on PDCP SN gap report triggering

# 2 Discussion

## 2.1  Handling polling when SDU is discarded

Open Issue: RLC-11: FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

As mentioned in above text-box, this open RLC issue needs to be addressed. We understand the ongoing polling procedure is not necessary when the indicated RLC SDU for discard has sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded. Therefore, if the t-PollRetransmit is running, it should stop and reset t-PollRetransmit. Otherwise, there would be an ambiguity for the polling procotol as there is no suitable SDU to continue polling operation.

Proposal 1: When the indicated RLC SDU for discard has a sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded, stop and reset t-PollRetransmit, if running. (Adopt TP 1)

## 2.2  Successful delivery indication in case of fake ACK

##### 5.2.3.1.1 General … When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU; - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet.

When a discard indication is received from upper layer for an RLC SDU, the transmitting side of an AM RLC entity considers the RLC SDU as obsolete and stops transmission or retransmission for the RLC SDU. Further, upon receiving fake ACK for the obsolete RLC SDU, the transmitting side of an AM RLC entity moves its transmission window and thereby, obsolete SDU is discarded. However, the question is whether an indication of successful delivery should be  sent or not to the upper layer in this case? The purpose of this indication at upper layer (PDCP) is to determine whether retransmission of the PDCP SDU is to be performed. However, as the PDCP SDU corresponding to obsolete RLC SDU has already been discarded by PDCP, no such purpose exists. Therefore, there is no need to provide an indication of successful delivery to the upper layers for an obsolete RLC SDU.

Proposal 2: Upon receiving ACK for an obsolete RLC SDU, the transmitting side of an AM RLC entity does not send an indication of successful delivery of the RLC SDU to the upper layers. (Adopt TP 2)

## PDCP SN gap reporting for AM DRBs during Mobility

Previosuly RAN2 discussed an issue raised by RAN3 LS. Issue concerns to the PDCP SN gap report during mobility. In this scenario, UE may discard some PDCP SDUs before handover and informs the source gNB through a PDCP SN gap report. Now, the source gNB during the handover operation forwards the buffered packets along with the SNs for the PDCP packets that are not received in the SN STATUS TRANSFER message to the target gNB. However, target gNB can not recover those PDCP SDUs through retransmissions of not received packets which are already discarded by the UE. Also target gNB is not aware about the discard of those PDCP SDUs and this will lead to excesive delay due t-reordering operations.

In RAN2#129, it was agreed to discuss the issue and potentially address in the context of R19 AM RLC enhancements. Some companies think that the issue can be solved by allowing a re-transmisison or a re-submission of the PDCP SN gap report after handover to the target gNB that was earlier sent to the source gNB during mobility. However, such approach breaks the basic tenet of radio protocols wherein a Control PDU is neither stored nor retransmitted.

Instead, we think it is possible to consider a new trigger for PDCP SN gap report after handover that allows the reporting of the discard information including the previous set of discarded PDCP SDUs. More precisely, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility (e.g. prior to PDCP entity re-establishment) but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity should trigger a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. Further, a separate configuration “sn-GapReportMobility” can be used for R19 AM DRBs for this purpose, to distinguish from the legacy configuration.

Proposal 3: RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 3):

- For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

- A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

## 2.4  Impact of R19 RLC enhancements on PDCP SN gap report triggering

TS 38.323 For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. ….

In legacy R18, upon receiving a discard indication for a RLC SDU from PDCP, a RLC SDU is either fully submitted to lower layer, or fully discarded at RLC (when no segment is submitted to lower layer).

With R19 AM RLC enhancements (i.e. stopReTxDiscardedSDU), we can have a new scenario where RLC SDU may be incompletely submitted to lower layer. The obsolete SDUs (incompletely received SDUs, if any) are discarded at the receiving side of AM RLC entity after t-RxDiscard expiry. That is, the receiving PDCP entity may not receive the obsolete SDUs. However, the receiving PDCP entity also does not get to know about the discard signalling for such incompletely submitted SDUs in PDCP SN gap report. This causes the reoderering delays to creep in.

Therefore, with R19 AM RLC enhancements in place, we have an issue about the highlighted legacy phrase “the discarded PDCP SDU(s)” in the third condition as it does not address the new scenario. More precisely, for R19 specifications, the third condition should be enhanced to “at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” while triggering a PDCP SN gap report.

The benefit is that the PDCP SN gap report can early and accurately notify more discard information to the peer receiving PDCP entity and it will prevent unnecessary re-ordering delay.

Proposal 4: RAN2 to discuss if R18 condition for triggering PDCP SN gap report “the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” needs an update in context of R19 AM RLC enhancements (i.e. stopReTxDiscardedSDU).

Proposal 5: Based on P4, RAN2 to agree if in context of R19 AM RLC enhancements (i.e. stopReTxDiscardedSDU), condition for triggering PDCP SN gap report should be updated as: (Adopt TP4)

- at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

Proposal 1: When the indicated RLC SDU for discard has a sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded, stop and reset t-PollRetransmit, if running. (Adopt TP 1)

Proposal 2: Upon receiving ACK for an obsolete RLC SDU, the transmitting side of an AM RLC entity does not send an indication of successful delivery of the RLC SDU to the upper layers. (Adopt TP 2)

Proposal 3: RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 3):

- For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

- A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

Proposal 4: RAN2 to discuss if R18 condition for triggering PDCP SN gap report “the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” needs an update in context of R19 AM RLC enhancements (i.e. stopReTxDiscardedSDU).

Proposal 5: Based on P4, RAN2 to agree if in context of R19 AM RLC enhancements (i.e. stopReTxDiscardedSDU), condition for triggering PDCP SN gap report should be updated as: (Adopt TP4)

- at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers

# 4 Annex: Text Proposals

TP 1

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall: - if the indicated RLC SDU for discard has a sequence number = X equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit.

TP 2

TS 38.322 ##### 5.2.3.1.1 General … When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU (excluding RLC SDU for which the transmission and retransmission are stopped); - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet. …

TP 3

TS 38.323 ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReportMobility in TS 38.331 [3]), when a PDCP SN gap report was previously transmitted during mobility but its succesful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall trigger a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. If a PDCP SN gap report is triggered for these discarded PDCP SDU(s), the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with these discarded PDCP SDU(s); - if more than one PDCP SDU is discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first of these discarded PDCP SDU(s) up to and including the last of these discarded PDCP SDU(s), rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '1' for these discarded PDCP SDU(s); - setting in the discard bitmap field as '0' for all other PDCP SDU(s). - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

TP 4

TS 38.323 For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxDiscardedSDU is set to enabled for AM DRB and at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers; otherwise, the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. ….