3GPP TSG-RAN WG2 Meeting #130											R2-2503510

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.4.2

Source: 	Sharp

Title:  	Open Issues on DSR Enhancements

Document for: 	Discussion

# Introduction

In RAN2#129bis meeting, RAN2 made the following agreements:

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

This document discusses remaining open issues on DSR enhancements.

# Discussion

## Empty DSR Issue

In RAN2#129bis meeting, RAN2 discussed how to avoid the case that there is no delay-reporting data for DSR, but DSR has been triggered. The conclusion was that it is reasonable that at least one configured reporting threshold should be no lower than the DSR triggering threshold. The remaining issue is whether we need to capture it in the specification.

In our view, this is about the network configuration that a sensible network implementation can configure the triggering threshold and reporting thresholds appropriately. Even if there is nothing to report, there will be no critical issue at the UE side. It means that no specification impact is expected.

It is up to NW implementation to configure thresholds to avoid the empty DSR issue (no specification impact).

## PDCP/RLC Data Volume Calculation for DSR MAC CE

In RAN2#129bis meeting, RAN2 discussed for which portion of delay-reporting data PDCP control PDU, SDUs/PDUs to be transmitted should be included but it was not concluded. The PDCP running CR [1] has the following editor’s note:

Editor’s Note: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

The RLC running CR [2] has similar editor’s notes as follows:

Editor’s Note: Same as PDCP open issue: it is FFS which delay-reporting RLC data volume shall consider RLC data PDUs to be retransmitted. Editor’s Note: It is FFS which delay-reporting RLC data volume shall consider STATUS PDU to be transmitted.

We may have the following options on the table:

- Option 1: Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest configured dsr-ReportingThreshold (i.e., i=1).

- Option 2: Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the LCG of the PDCP entity.

- Option 3: Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

Considering that Control PDU and PDUs/SDUs to be retransmitted are prioritized over only the initial transmission from the same PDCP entity. The transmission of data from different PDCP entities relies on the LCH priority other than the PDU type. The delay requirement of Control PDU and PDUs/SDUs to be retransmitted should be the same as the delay requirement of the delay-reporting PDCP SDU from the same PDCP entity. It is inaccurate to always consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest confiugred dsr-ReportingThreshold (Option 1) or the smallest dsr-ReportingThreshold for the LCG (Option 2). In addition. Option 1 will result in signalling overhead if the LCG does not have delay-reporting PDCP SDUs associated with smallest configured dsr-ReportingThreshold. Thus, we see that Option 3 is straightforward and reasonable.

Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

When multiple PDCP entities belong to the same LCG, the buffered data in a PDCP entity are purely either Control PDU or PDUs/SDUs to be retransmitted. We can consider two cases:

- Case 1: Only Control PDU

- Case 2: Control PDU and PDUs/SDUs to be retransmitted or only PDUs/SDUs to be retransmitted.

Control PDU is prioritized over the delay-reporting PDCP SDU within the same PDCP entity. If there is no delay-reporting PDCP SDU in the PDCP entity (even if other PDCP entity belonging to the same LCG has delay-reporting PDCP SDU), Control PDU of the PDCP entity should not be considered as PDCP data volume for DSR.

If there is no delay-reporting PDCP SDUs in a PDCP entity, Control PDU is not considered as PDCP data volume for DSR.

PDUs/SDUs to be retransmitted may need to be treated similarly to Control PDU. Thus, we propose:

If there is no delay-reporting PDCP SDUs in a PDCP entity, PDUs/SDUs to be retransmitted are not considered as PDCP data volume for DSR.

We see that the same principle can apply to RLC data volume calculation for DSR.

RLC data volume calculation and PDCP data volume calculation on Control PDU and PDUs/SDUs to be retransmitted are consistent with each other.

In addition, in current RLC running CR [2], the definition of delay-reporting RLC SDU relies on the indication from PDCP. But the non-delay reporting RLC SDU is defined independently in RLC. Based on the definition, all the descriptions related to non-delay reporting PDCP SDU are duplicately captured for non-delay reporting RLC SDU in RLC, which makes the specification look more complicated.

From RLC perspective, it does not need to distinguish delay-reporting RLC SDU and non-delay-reporting RLC SDU. RLC can simply rely on current delay reporting indication from PDCP to evaluate RLC data volume for DSR. For example, if dsr-ReportNonDelayCriticalData is configured, PDCP indicates both delay-reporting PDCP SDUs and non-delay reporting PDCP PDUs associated with the i-th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. Otherwise, PDCP only indicates the delay-reporting PDCP SDUs associated with the i-th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. Based on the existing delay-reporting indication from PDCP, the non-delay-reporting information can be invisible to RLC and all the description related to non-delay reporting RLC SDU in RLC spec can be removed.

If dsr-ReportNonDelayCriticalData is configured, PDCP indicates non-delay reporting PDCP PDUs associated with the i:th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold.

Remove all the description related to non-delay reporting RLC SDU from RLC specification.

The corresponding TPs for proposal 6 and proposal 7 are provided in the Annex.

## DSR Configuration

The next issue is issue 7 in the open issue list [3] as showed below:

Issue7: FFS how to handle the following agreement in RAN2#129 "If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold”

In RAN2#129 meeting, following agreements were made:

- FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

The FFS was not confirmed in RAN2#129bis meeting, but it captured in the MAC Running CR [4] after RAN2#129bis meeting and no companies disagreed the implementation of this part.

So, it is a common understanding that New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds and RAN2 can confirm it.

RAN2 to confirm that New DSR MAC CE is always used when at least one LCG is configured with multiple thresholds.

Based on the understanding, it is obvious that the second agreement means if at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold.

If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold.

# Conclusions

Based on the discussion above, RAN2 is requested to agree the following proposals:

- It is up to NW implementation to configure thresholds to avoid the empty DSR issue (no specification impact).

Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

If there is no delay-reporting PDCP SDUs in a PDCP entity, Control PDU is not considered as PDCP data volume for DSR.

If there is no delay-reporting PDCP SDUs in a PDCP entity, PDUs/SDUs to be retransmitted are not considered as PDCP data volume for DSR.

RLC data volume calculation and PDCP data volume calculation on Control PDU and PDUs/SDUs to be retransmitted are consistent with each other.

If dsr-ReportNonDelayCriticalData is configured, PDCP indicates non-delay reporting PDCP PDUs associated with the i:th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold.

Remove all the description related to non-delay reporting RLC SDU from RLC specification.

RAN2 to confirm that New DSR MAC CE is always used when at least one LCG is configured with multiple thresholds.

If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold.

# Reference

[1] R2-2503696	PDCP running CR for R19 XR, LG Electronics Inc. (Rapporteur)

[2] R2-2503620	RLC running CR for R19 XR, vivo

[3] R2-2503788	Summary of [POST129bis][503][XR] RRC running CR and open issues (Huawei), Huawei, HiSilicon

[4] R2-2503360	Introduction to R19 XR enhancements, Qualcomm Incorporated

# Annex

TP for TS 38.323

--------------------------------------------Start Change of TS38.323---------------------------------------

## 5.15	Data volume calculation for delay status reporting

For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume:

-	the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

The transmitting PDCP entity provides a delay-critical indication for the PDCP Data PDU to lower layers when:

-	the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-critical PDCP SDU; or

-	the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-critical PDCP SDU.

For the purpose of MAC delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold:

-	the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	[if i = 1], the PDCP Control PDUs;

-	[if i = 1], for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	[if i = 1], for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Editor’s Note: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

If dsr-ReportNonDelayCriticalData is configured, the transmitting PDCP entity shall further consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold:

-	the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i.

The transmitting PDCP entity provides a delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers when:

-	the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

-	the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

-	if dsr-ReportNonDelayCriticalData is configured, the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

-	if dsr-ReportNonDelayCriticalData is configured, the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

If the transmitting PDCP entity is associated with at least two RLC entities, when indicating the delay-critical PDCP data volume or delay-reporting PDCP data volume to a MAC entity for DSR triggering and Buffer Size calculation (as specified in TS 38.321 [4]), the transmitting PDCP entity shall:

-	if the PDCP duplication is activated for the RB:

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume excluding the PDCP Control PDU to the MAC entity associated with the RLC entity other than the primary RLC entity activated for PDCP duplication;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume as 0 to the MAC entity associated with RLC entity deactivated for PDCP duplication;

-	else (i.e. the PDCP duplication is deactivated for the RB):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the split secondary RLC entity;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume as 0 to the MAC entity associated with RLC entity other than the primary RLC entity and the split secondary RLC entity;

-	else:

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume as 0 to the MAC entity associated with the RLC entity other than the primary RLC entity.

--------------------------------------------End Change of TS38.323---------------------------------------

TP for TS 38.322

---------------------------------------------Start Change of TS38.322---------------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Data field element: An RLC SDU or an RLC SDU segment that is mapped to the Data field.

Delay-critical RLC SDU: RLC SDU corresponding to a PDCP PDU indicated as delay-critical by PDCP (see TS 38.323 [4]).

Delay-reporting RLC SDU: a delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]).

Editor’s Note: The terminology is to be aligned with other specifications.

[not related part are omitted]

## 5.5	Data volume calculation

For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume:

-	RLC SDUs and RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs that are pending for initial transmission;

-	RLC data PDUs that are pending for retransmission (RLC AM).

For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume:

-	delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs pending for initial transmission, and containing a del ay-critical RLC SDU or a delay-critical RLC SDU segment;

-	RLC data PDUs that are pending for retransmission (RLC AM).

For the purpose of MAC delay status reporting, the UE shall evaluate the delay-reporting RLC data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting RLC data volume associated with the i:th dsr-ReportingThreshold:

-	delay-reporting RLC SDUs and delay-reporting RLC SDU segments that associated with the i:th dsr-ReportingThreshold and have not yet been included in an RLC data PDU, and are not considered as delay-reporting RLC data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	RLC data PDUs pending for initial transmission, and containing a delay-reporting RLC SDU or a delay-reporting RLC SDU segment associated with the i:th dsr-ReportingThreshold, and are not considered as delay-reporting RLC data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	[if i=1,] RLC data PDUs that are pending for retransmission (RLC AM).

Editor’s Note: Same as PDCP open issue: it is FFS which delay-reporting RLC data volume shall consider RLC data PDUs to be retransmitted.

In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting, as part of delay-critical RLC data volume for MAC delay status reporting, and as part of [the first (i.e. i=1)] delay-reporting RLC data volume for MAC delay status reporting.

Editor’s Note: It is FFS which delay-reporting RLC data volume shall consider STATUS PDU to be transmitted.

--------------------------------------------End Change of TS38.322---------------------------------------