3GPP TSG-RAN WG2 Meeting #130	R2-2503514

St. Julians, Malta, May 19th – May 23rd, 2025

Agenda item:	8.3.2

Source:	Qualcomm Incorporated

Title:	AI/ML for mobility Configuration and Functionality management

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #129-bis meeting [1], the final set of conclusions were drawn from the simulation studies on AI/ML for mobility. Also, RAN2 initiated the study of LCM aspects and specification impacts for AI/ML for mobility, with a view towards converting the SI into a WI and determining the scope of the WI in Rel-20. A set of basic agreements were reached on the LCM aspects and specification impacts for AI/ML for mobility [1].

Leading to RAN2#130, a few companies including Qualcomm, worked on submitting R2-2503780 [3] to this meeting. While we do agree with the intent of that contribution, which was to clarify that intra-frequency spatial domain prediction is included in the scope of this study item, we believe that a better classification of the procedures will lead to a better study on the impact to the specification of AI/ML for mobility in general.

In this contribution, we continue the discussion on LCM aspects and specification impacts for AI/ML for mobility, focusing on:

- Configurations of AI/ML for mobility focusing on the main input/output,

- Other inference configuration and reporting for UE-side model.

# Configurations of AI/ML for mobility

## Configuration principles

RAN2#129bis made the following agreement:

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

Since we’re following the same baseline as for the AIML for NR Air Interface, we understand the agreement means that:

Proposal 1: The standard will define the AIML mobility feature input and output and not the model input and output.

However, looking back at the sub-use cases studied so far, we find that they were defined in terms of the MODEL input/output, which can be summarized in this table:

Table 1: Sub-Use-Case as defined by the model input and output

The table has an empty lower triangle because it doesn’t make sense to walk backward from a cell to a beam measurement and from L3 to L1 measurements.

If we concern ourselves with only the FEATURE output, the table simplifies to:

Table 2: Sub-Use-Case as defined by the model input and the feature output

Furthermore:

- For the UE side models:

  - The UE can only measure at L1 (SSB and CSI/RS).

    - L3-Beam and L3-Cells are derivatives of L1 measurements

  - The AIML for NR Air Interface work has agreed NOT to define the input to the models,

- For the NW side models, we are not going to define the input either.

Then, in general, the only input we need to consider is the L1-Beam, which translates to the presence of the SSB or CSI-RS over the air.

The table becomes:

Table 3: Sub-Use-Case as defined by the feature input and output

In the discussion for specification impact, we condense sub-use-cases 1,2&3 and 4,5,6, and we talk instead of whether we’re predicting L3-Beam or L3-Cell RRM, or event prediction.

Proposal 2: In the discussion for specification impact, the main outputs of the AIML for mobility are L3-Beam, L3-Cell RRM, or event prediction.

The event prediction had been a separate output from the start and is listed in the proposal for completeness.

## RRM measurement and Event prediction

Now that we have simplified the input and output of the feature, we still have to spell out whose cells are we measuring for the input and what cells qualities are being predicted.

A table is helpful to show how [3] approached the problem:

| Feature Output Feature Input | RRM for Cell A | RRM for Cell B |
| --- | --- | --- |
| Single Cell A | Proposal 1a | Proposals 2/3 (Not supported) |
| Cells A-M | Proposal 1b |  |

Table 4: Use Cases as viewed in [3]

That paper ([3]) aimed at establishing the inclusion of spatial prediction and we supported that effort. This paper aims to expand on that discussion to pave the way to a good analysis of standard impact.

Proposal 3: “Intra-frequency prediction” refers to when the frequencies of the measured and predicted cells of the AI/ML for mobility feature are the same. “Across-frequency prediction” refers to the case when they are different.

Proposal 4: “Intra-cell prediction” refers to when the same cell is measured and predicted for the AI/ML for mobility feature. “Across-cell prediction” refers to the case when a group of cells is measured and the RRM of, or an event involving, one or more of these cells is being predicted.

Proposal 5: For specification work in AI/ML for mobility, study the following configurations:

|  | Feature Output Feature Input | Serving Cell (A) | RRM or Event for Cell B | Event across A&B |
| --- | --- | --- | --- | --- |
| Single | Serving Cell (A) | Intra-frequency, intra-cell serving prediction | N/A | N/A |
|  | Single Cell B on F1 | N/A | Intra-frequency, intra-cell neighbor prediction | N/A |
| “Cluster” | Cells A&B on F1 | Intra-frequency, across-cell serving prediction | Intra-frequency, across-cell neighbor prediction | Intra-Frequency, across-cell event Prediction |
|  | Cells (A on F1) & (B on F2) | Across-frequency, across-cell serving prediction | Across-frequency, across-cell neighbor prediction | Across-frequency, across-cell event Prediction |

Table 5: Configurations to study

For all the configurations, a UE-side model or a network-side model can be used.

As a side note, “L1” beam prediction within a cell has been studied in R18/R19 AI/ML for Beam Management (BM) prediction and should not be considered for specification in AI/ML for mobility nor is the subject of this paper.

In the rest of the paper, we are concerned with L3 beam prediction, L3 cell quality prediction or event prediction.

### Intra-frequency, intra-cell serving cell RRM prediction

Definition #1 Intra-frequency, intra-cell serving cell RRM prediction: Measure some beams of the serving cell to predict beam or cell measurements of the serving cell now (spatial) or in the future (temporal), or a combination of the above; see Figure 1 for an example.

Figure 1: Intra-frequency, intra-cell serving cell RRM prediction

While we showed these 3 options in Figure 1 (spatial, temporal and a combination), we will not to do so in the next figures, even though it is understood that all 3 options are possible in all other cases.

### Intra-frequency, intra-cell neighbor cell RRM prediction

Definition #2 Intra-frequency, intra-cell neighbor cell RRM prediction: Measure some beams of the neighbor cell to predict beam or cell measurements of the same neighbor cell now (spatial) or in the future (temporal), or a combination of the above; see Figure 2 for an example.

Figure 2: Intra-frequency, intra-cell neighbor cell RRM prediction

### Intra-frequency, across-cell serving cell RRM prediction

Definition #3 Intra-frequency, across-cell serving cell RRM prediction: Measure some beams of cells belonging to a cluster, for beam or cell measurement prediction of the serving cell, now (spatial) or in the future (temporal), or a combination of the two. Figure 3.

Figure 3: Intra-frequency, across-cell serving cell RRM prediction

### Intra-frequency, across-cell neighbor cell RRM prediction

Definition #4 Intra-frequency, across-cell neighbor cell RRM prediction: Measure some beams of cells belonging to a cluster, for beam or cell measurement prediction of the neighbor cell belonging to the cluster, now (spatial) or in the future (temporal), or a combination of the two.

Figure 4: Intra-frequency, across-cell neighbor cell RRM prediction

### Intra-Frequency, across-cell event prediction

Definition #5 Intra-frequency, across-cell event prediction: Measure some beams of cells belonging to a cluster, for event prediction, e.g., for events A3, A4, now (spatial) or in the future (temporal), or a combination of the two. Figure 5 shows an example.

Figure 5: Intra-frequency, across-cell event prediction

### Across-frequency, across-cell serving cell RRM prediction

Definition #6 Across-frequency, across-cell serving cell RRM prediction: Measure some beams of cells belonging to a cluster, for beam or cell measurement prediction of the serving cell, now (spatial) or in the future (temporal), or a combination of the two. Table 5 shows an example where the cluster consists of the serving cell and another cell in a different frequency.

Figure 6: Across-frequency, across-cell serving cell RRM prediction

### Across-frequency, across-cell neighbor cell RRM prediction

Definition #7 Across-frequency, across-cell neighbor cell RRM prediction: Measure some beams of cells belonging to a cluster, for beam or cell measurement prediction of the neighbor cell, now (spatial) or in the future (temporal), or a combination of the two.

Figure 7: Across-frequency, across-cell neighbor cell RRM prediction

### Across-frequency, across-cell event prediction

Definition #8 Across-frequency, across-cell event prediction: Measure some beams of cells belonging to a cluster, for event prediction, e.g., for events A3, A4, now (spatial) or in the future (temporal), or a combination of the two.

Figure 8: Across-frequency, across-cell eventcell RRM prediction

During the simulation evaluation for across-frequency RRM prediction, the scenario that was studied was of collocated cells in the same sector, but at different frequencies (frequency layers). The simulation results showed that the RRM prediction results were of reasonable accuracy, and that generalization was possible. We therefore propose below that across-frequency RRM prediction should be considered for specification in AI/ML for mobility.

Proposal 6: For configurations #6, #7, RAN2 considers for specification the scenario where the predicted cell is collocated with the cells that are measured. The evaluation of scenarios where the predicted cell is not collocated with the cells that are measured, can be left to the WI phase.

# Inference configuration and reporting for UE-side model

During the R19 AI/ML for BM prediction discussion, the issue of network-side additional conditions was discussed and in the RAN2 #126 meeting [2], an agreement was made on the signaling of such conditions for consistency of inference and data collection/training configurations provided by the network to the UE, for UE-side models.

RAN2 #126 [2] agreement:

For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.

For AI/ML for mobility also, for UE-side models, the network needs to ensure consistency between the inference and data collection/training configurations provided to the UE. The solution to ensure consistency can be an associated ID, as was agreed for R19 AI/ML for BM prediction, the associated ID in this case denoting a transmitted beam pattern.

Consistency is needed because, between the data collection time and the inference time, the following may have changed:

- The operator may have added or removed cells,

- The operator may have adjusted the relative transmit power of cells, or the desired offsets for an equivalent legacy mobility, or

- The network may have two reference configurations for NES (high capacity vs low power consumption). Each could be assigned an associated ID.

Proposal 7: For UE-side model for AI/ML for mobility, associated IDs can be introduced for consistency between the inference and data collection/training configurations provided by the network.

## RRM measurement prediction

The exact configuration provided depends on the configuration, and we discuss at a high level the configurations for each of the configurations we have discussed above.

We also discuss the RRM prediction reporting configuration and the contents of the predicted report for each configuration.

Proposal 8: Configuration for #1 – Intra-frequency, intra-cell RRM prediction for a serving cell:

- Serving cell frequency, Serving cell ID,

- Set B of serving cell beams to measure.

- Prediction Window (PW) length, for the case of temporal prediction.

Proposal 9: Prediction reporting configuration for #1 – Intra-frequency, intra-cell RRM prediction for a serving cell:

- Periodicity in case of periodic reporting,

- Temporal prediction report case:

  - The period and number of instances within the PW length that the UE can report.

Proposal 10: The RRM prediction report for #1 – Intra-frequency, intra-cell RRM prediction for a serving cell contains:

- Temporal prediction report case:

  - Predicted cell measurements at future periodic time instances, and the number of such time instances.

- Spatial prediction report case:

  - Predicted cell measurement at the current time.

Proposal 11: Configuration for #3 – Intra-frequency, across-cell RRM prediction for a serving cell:

- For each cell of input cluster:

  - Cell frequency, Cell ID,

  - Set B of beams to measure,

- Serving cell frequency, Serving cell ID,

- For beam prediction:

  - Set A of serving cell beams.

Proposal 12: The prediction reporting configuration for #3 is similar as that for #1.

Proposal 13: The RRM prediction report for #3 – Intra-frequency, across-cell RRM prediction for a serving cell, contains:

- Temporal prediction report case:

  - Predicted beam and/or cell measurements at future periodic time instances, and the number of such time instances,

- Spatial prediction report case:

  - Predicted beam and/or cell measurements at the current time.

Proposal 14: The configuration for inference and reporting, and contents of the prediction reports, for configurations #2 and #4 for a neighbor cell is similar as that for a serving cell.

Proposal 15: Configuration for #7– Across-frequency, across-cell RRM prediction for a neighbor cell:

- Neighbor cell frequency, Neighbor cell ID,

- Set A of neighbor cell beams,

- For each cell of input cluster:

  - Measured cell frequency, Measured cell ID,

  - Set B of measured cell beams.

The configuration for reporting and contents of the prediction reports for configuration  #7 is similar as that for a serving cell.

## Measurement Event prediction

Proposal 16: For Event prediction, besides the configuration for RRM measurement, the gNB provides the UE with a list of events to predict.

During the simulation evaluation, an option (Option 3) that was considered for event prediction reporting for A3 or A5 is that the UE transmits a report at the time when the entering condition of the event is met by actual measurements, and that the entering condition is satisfied for TTT duration by predicted measurements. The gNB may configure the UE for providing such reports.

Proposal 17: For Event A3 or A5, the gNB can configure the UE for event prediction reporting when the entering condition is met by actual measurements, and the entering condition is satisfied for TTT duration by predicted measurements.

Proposal 18: An Event prediction report includes the expected time of occurrence of event and probability of event occurrence within an indicated time window around the expected time.

# Conclusions

Based on the discussion above, we recommend RAN2 to discuss the proposals and observations below.

Proposal 1: The standard will define the AIML mobility feature input and output and not the model input and output.

Proposal 2: In the discussion for specification impact, the main outputs of the AIML for mobility are L3-Beam, L3-Cell RRM, or event prediction.

Proposal 3: “Intra-frequency prediction” refers to when the frequencies of the measured and predicted cells of the AI/ML for mobility feature are the same. “Across-frequency prediction” refers to the case when they are different.

Proposal 4: “Intra-cell prediction” refers to when the same cell is measured and predicted for the AI/ML for mobility feature. “Across-cell prediction” refers to the case when a group of cells is measured and the RRM of, or an event involving, one or more of these cells is being predicted.

Proposal 5: For specification work in AI/ML for mobility, study the following configurations:

|  | Feature Output Feature Input | Serving Cell (A) | RRM or Event for Cell B | Event across A&B |
| --- | --- | --- | --- | --- |
| Single | Serving Cell (A) | Intra-frequency, intra-cell serving prediction | N/A | N/A |
|  | Single Cell B on F1 | N/A | Intra-frequency, intra-cell neighbor prediction | N/A |
| “Cluster” | Cells A&B on F1 | Intra-frequency, across-cell serving prediction | Intra-frequency, across-cell neighbor prediction | Intra-Frequency, across-cell event Prediction |
|  | Cells (A on F1) & (B on F2) | Across-frequency, across-cell serving prediction | Across-frequency, across-cell neighbor prediction | Across-frequency, across-cell event Prediction |

Table 5: Configurations to study

Proposal 6: For configurations #6, #7, RAN2 considers for specification the scenario where the predicted cell is collocated with the cells that are measured. The evaluation of scenarios where the predicted cell is not collocated with the cells that are measured, can be left to the WI phase.

Proposal 7: For UE-side model for AI/ML for mobility, associated IDs can be introduced for consistency between the inference and data collection/training configurations provided by the network.

Proposal 8: Configuration for #1 – Intra-frequency, intra-cell RRM prediction for a serving cell:

- Serving cell frequency, Serving cell ID,

- Set B of serving cell beams to measure.

- Prediction Window (PW) length, for the case of temporal prediction.

Proposal 9: Prediction reporting configuration for #1 – Intra-frequency, intra-cell RRM prediction for a serving cell:

- Periodicity in case of periodic reporting,

- Temporal prediction report case:

  - The period and number of instances within the PW length that the UE can report.

Proposal 10: The RRM prediction report for #1 – Intra-frequency, intra-cell RRM prediction for a serving cell contains:

- Temporal prediction report case:

  - Predicted cell measurements at future periodic time instances, and the number of such time instances.

- Spatial prediction report case:

  - Predicted cell measurement at the current time.

Proposal 11: Configuration for #3 – Intra-frequency, across-cell RRM prediction for a serving cell:

- For each cell of input cluster:

  - Cell frequency, Cell ID,

  - Set B of beams to measure,

- Serving cell frequency, Serving cell ID,

- For beam prediction:

  - Set A of serving cell beams.

Proposal 12: The prediction reporting configuration for #3 is similar as that for #1.

Proposal 13: The RRM prediction report for #3 – Intra-frequency, across-cell RRM prediction for a serving cell, contains:

- Temporal prediction report case:

  - Predicted beam and/or cell measurements at future periodic time instances, and the number of such time instances,

- Spatial prediction report case:

  - Predicted beam and/or cell measurements at the current time.

Proposal 14: The configuration for inference and reporting, and contents of the prediction reports, for configurations #2 and #4 for a neighbor cell is similar as that for a serving cell.

Proposal 15: Configuration for #7– Across-frequency, across-cell RRM prediction for a neighbor cell:

- Neighbor cell frequency, Neighbor cell ID,

- Set A of neighbor cell beams,

- For each cell of input cluster:

  - Measured cell frequency, Measured cell ID,

  - Set B of measured cell beams.

The configuration for reporting and contents of the prediction reports for configuration  #7 is similar as that for a serving cell.

Proposal 16: For Event prediction, besides the configuration for RRM measurement, the gNB provides the UE with a list of events to predict.

Proposal 17: For Event A3 or A5, the gNB can configure the UE for event prediction reporting when the entering condition is met by actual measurements, and the entering condition is satisfied for TTT duration by predicted measurements.

Proposal 18: An Event prediction report includes the expected time of occurrence of event and probability of event occurrence within an indicated time window around the expected time.

# References

[1] Chair notes, RAN2 #129-bis meeting.

[2] Chair notes, RAN2 #126 meeting.

[3] R2-2503780, Support of Spatial Domain Prediction for AI Mobility