3GPP TSG RAN WG2 Meeting #130                                                            R2-2503519

# St Julian's, Malta, May 19th - 23rd, 2025

Agenda item:	8.2.3

Source:	Ofinno

Title:	Discussion of issues 2-4, 2-6, 2-11, 1-4 and a new issue

Document for:	Discussion and decision

# Introduction

This document discusses the following issues identified during email discussion [POST129bis][017][AIoT] [1]: [issue 2-4] [issue 2-11] [new issue] on the success/failure of Msg3, [issue 1-4] on Msg1 resource information, and [issue 2-6] on Msg1/2 association. For reference, Rapp’s details captured on the issues here discussed are copied in Annex.

# Discussion

## [Issue 2-4] [Issue 2-11] [New issue] Success/failure of Msg3

Issue 2-4 focuses on how to determine the CBRA failure/contention resolution failure, and Issue 2-11 focuses on whether to use a new/explicit R2D message as an explicit NACK message. During the email discussion [POST129bis][017][AIoT][1], companies shared the view that a new message could indeed be defined, but it should also be discussed whether any of the already introduced R2D messages could be used for example as an implicit ACK/NACK indication. In addition, a New Issue is suggested to further discuss the success and failure scenarios for any D2R message sent, even after Msg3 (until the A-IoT procedure is completed). In our understanding, all these issues and their potential solutions are inter-connected.

So far, the NACK based mechanism is only agreed for Msg3 during CBRA, as shown in below RAN2 agreements:

[Agreements (RAN2#128)] 1. RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID 2. If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported). [Agreements (RAN2 #129)] 1. NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. [Agreements (RAN2 #129)] 1 For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). 2 For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging 3 For CFRA, NACK feedback and re-access is not supported. FFS how to achieve 4 FFS on end of procedure.

RAN2 needs to still discuss how to define the CFRA procedure success, failure, and ongoing. Based on CFRA’s agreements, we could consider the D2R transmission as successful as soon as it is transmitted, however this was not explicitly discussed as RAN2 #129bis meeting discussion focused on whether the failure/re-access indication was defined. Therefore, for CFRA, it is unclear when/how/whether the device can consider its D2R transmission as successful or failed. Similarly, for CBRA, RAN2 #129bis meeting discussion focused on how the device determines the CBRA failure/contention resolution failure. In addition, if the device misses the reader’s NACK indication or re-transmission request, the device might be in a limbo/stuck state.

[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] For CFRA, after sending D2R message, it is unclear whether the A-IoT device can consider this transmission immediately as successful. For CBRA, after sending Msg3, the A-IoT device might be in a limbo/stuck scenario if it misses the corresponding failure/re-access indication.

During email discussion [POST129bis][017][AIoT] [1], Rapp also clarified that the retransmitted Msg2 and the NACK feedback both aim to indicate failure of Msg3 with different device behaviours expected from device side, i.e. (1) retransmitted Msg2 would trigger retransmission of Msg3; (2) NACK feedback would trigger re-access (i.e., retransmission of Msg1).

| Ofinno | Agree with comment | We are ok with the proposal for this issue 2-11. One thing to note is that the retransmitted Msg2 may also be considered as the R2D NACK feedback message. Rapp: Just to clarify, as vivo explained above about how this NACK indication works, the device will trigger re-access after receiving the NACK feedback, but retransmitted Msg2 is supposed to trigger retransmission of Msg3, in this case the device does not need to start from Msg1, which should be the difference between re-access and retransmission. |
| --- | --- | --- |

[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] For CBRA, network could request, to the device, different behaviours upon detecting a failure of Msg1/Msg3: (1) re-access (i.e., retransmission of Msg1), or (2) retransmission of Msg3. For scenario (2), the usage of retransmitted Msg2 is already agreed. The open question is how re-access is informed to the device (which seems preferable to create a new R2D NACK message instead of other means e.g., also reusing Msg2).

RAN1 and RAN2 agreed the following for the monitoring of Msg2:

[Agreement (RAN1#119)] For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring: Option 1: Predefined; Option 2: Indicated in the R2D transmission triggering random access. Note: Option 1 and Option 2 may not be mutually exclusive. [Agreement (RAN1#119)] A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. • Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. • Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 • Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 [Agreements (RAN2 #128)] - RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID - If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported). - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages [Agreements (RAN2#129bis)] - A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. - Same Msg2 format is used for initial transmission and retransmission of Msg2.

RAN1 has agreed to define a TD→Rmin as the time that the device does not need to monitor for R2D message after transmission of D2R message which could be the start time of the monitoring window of A-IoT Msg2 if agreed. The reader can re-transmit the A-IoT Msg2 to specific device(s) in case of failure reception of the A-IoT Msg3. There are two cases when the reader doesn’t receive the A-IoT Msg3 and re-transmits the A-IoT Msg2.

- When the A-IoT device transmits the A-IoT Msg3, but the reader fails to receive the A-IoT Msg3.

- When the A-IoT device fails to receive the A-IoT Msg2 and doesn’t transmit the A-IoT Msg3.

The reader may not distinguish between those two cases and transmits the re-transmitted A-IoT Msg2 either way (i.e., in both two cases).

[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] A guard time TD→Rmin is defined by RAN1 as the time that the device does not need to monitor for R2D message after transmission of D2R message. This was discussed in relation to Msg1/2 but RAN1 agreement/TS seems general for any D2R message.

Multiple approaches could be considered to determine whether Msg3 are successfully transmitted and potentially even decoded by the reader. This document discusses two simple approaches: timer-based and Msg2-based.

### Timer-based approach to trigger success/failure of Msg3

The companion document [3] explains a simple mechanism for the A-IoT device to determine how the transmission of the D2R message is or can be considered as successful or failed. Similar behaviour could also be applied to Msg1 for CFRA and Msg1/Msg3 for CBRA as it is shown in below Figure 1 as well as the D2R message transmitted during a A-IoT command procedure.

Figure 1. Timer-based mechanism to determine if Msg 1 and Msg 3 are or can be considered successful or failed

The points to highlight of the mechanism shown in Figure 1 for CFRA and CBRA are as follows:

- Point (A) Upon transmission of the Msg1 or Msg3, a new short timer TD2R_ACK is started immediately (steps [C], [E1] and [E2] in Figure 1) or after certain time, referred as TOffset (steps [C’], [E1’] and [E2’] in Figure 1).

    - FFS if TOffset is needed (which represents a delay to start the new short timer TD2R_ACK after sending D2R message) and if so, FFS whether this TOffset is same or different to TD→Rmin (defined in TS 38.291).

- Point (B) While the new timer TD2R_ACK is running, device’s behavior upon reception of the applicable R2D messages (e.g., Msg2 in steps [D1] or [D2] in Figure 1) would be defined case by case on A-IoT MAC spec. For example:

    - After transmission of Msg.1:

      - If Msg2 is received addressed to the device, Msg1 is considered successful (and device can move to transmit Msg3). Based on current RAN2 agreements, this would only apply to CBRA.

      - If Paging is received for the device, re-access would be triggered. Note that this behaviour might vary as it is dependent to end of procedure open FFS.

    - After transmission of Msg.3

      - If the retransmitted Msg2 is received addressed to the device, Msg3 is considered failure (and device can move to re-transmit Msg3). Based on current RAN2 agreements, this would only apply to CBRA.

- Point (C) We can define, per case by case on A-IoT MAC spec, that the expiry of the new timer TD2R_ACK means that the device considers implicitly the success or failure of previous Msg1 or Msg.3.

### Msg2-based approach to trigger success/failure of Msg3

The Msg2 (e.g., including the retransmitted Msg2) can schedule transmission of Msg3 to a device. During the SI phase, RAN2 already agreed that Msg2 could also be used to trigger retransmissions of Msg3 if the failure of Msg3 transmission is detected for specific devices. Therefore, it could be straightforward to use a retransmitted Msg2 to provide explicit/implicit status information of previous Msg3 transmission/reception. This explicit/implicit status information of Msg3 transmission could be defined as an success or failure for Msg3 as shown in below Figure 2.

Figure 2. Msg2-based mechanism to determine if previous Msg 3 are or can be considered successful or failure

Figure 2 depicts an example in which after Msg3 transmission, the retransmitted Msg2 is used to indicate failure (step [D2]) as well as success (step [D3]) of previous Msg3 transmissions. This success/failure indications could be defined per device (e.g., based on the device AS ID or the Msg1/Msg3 resource index used by the device), and if signalling overhead were a concern, optimizations could also be considered e.g. per group of devices or implicit.

Summarizing, we suggest that RAN2 consider enabling at least one of them above mechanism to be able to determine the success or failure of Msg.3 transmission.

- [Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] To determine the success or failure of a transmitted Msg3, to consider whether to define a timer-based approach and/or Msg.2-based approach.

- [Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] [Timer-based approach] To consider defining a new short timer (TD2R_ACK) to determine the success (as well as failure) of previously transmitted Msg3 which works as follows:

  - Point (A) Upon transmission of the Msg3, TD2R_ACK is started immediately or after certain time (TOffset). FFS on TOffset (if it’s needed and its relationship with TD→Rmin defined in TS 38.291).

  - Point (B) While TD2R_ACK is running, the device’s behaviour upon reception of the applicable R2D messages (e.g., Msg2 or paging) is defined case by case in A-IoT specification (including for both, failure and success).

  - Point (C) Upon expiry of TD2R_ACK, the A-IoT device considers previous Msg3 as successful or failed.

  - Point (D) To also apply this TD2R_ACK to any D2R messages sent as part of the A-IoT inventory and command procedure (including Msg1).

- [Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] [Msg.2-based approach] To consider defining the usage of retransmitted Msg2 to determine the success (as well as failure) of previously transmitted Msg3 which works as follows:

  - Msg2 is used to indicate success or failure of previously transmitted Msg3 (per UE basis). FFS whether this success indication can also be defined per group of devices and/or implicit.

## [Issue 1-4] Msg1 resource information

The following agreements were made in RAN1 and RAN2 in relation to Msg1 transmission and resources:

[Agreement (RAN1 #120)] Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4. [Agreement (RAN1#120bis)] For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. • All devices support the above • Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. • FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. • FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact • FFS: how to define the end of the R2D transmission in 9.4.1 At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. [Working assumption (RAN1#120bis)] For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. [Agreements (RAN2 #129)] The A-IoT paging message can include a number of msg1 resources From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field [Agreements (RAN2 #129)] - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

RAN1 agreements would require that signaling is able to provide information associated to both time and frequency. RAN1 still needs to discuss some details (e.g. values of the time offset(s)) but RAN2 could start discussing the corresponding signaling. Moreover, the RAN2 running CR to A-IoT MAC TS [2] captured the following:

6.2.1.1	A-IoT Paging message

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1.

-	RA Type: This field indicates CBRA or CFRA.

-	Indication of Paging ID presence: This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0).

-	Length of Paging ID: This field indicates the Paging ID length information when Paging ID field is present.

-	Paging ID: xxx

-	Transaction ID: xxx

-	Number of Access Occasions: This field indicates the number of access occasions.

-	D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling.

Editor’s Note:   FFS if CFRA can omit the fields of transaction ID, Indication of Paging ID present/absence, Number of access occasions.

Editor’s Note:  FFS the length of transaction ID.

The slotted-ALOHA is the baseline for A-IoT random access procedure [1]. In the A-IoT random access, radio resources available for A-IoT Msg1 are divided into one or more time-frequency resources, and an A-IoT device randomly selects one of the time-frequency resources to transmit an A-IoT Msg1. A time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device) is defined as an access occasion (i.e., A-IoT Msg1 resources), and a set of access occasion(s) for different A-IoT device(s) is scheduled via an R2D message [1]. For example, the R2D message includes a paging message and/or a separate R2D trigger message for the A-IoT random access procedure.

When the R2D message triggers A-IoT devices to respond during the A-IoT random access procedure, an A-IoT device selects (e.g., randomly) one access occasion from the set of access occasion(s) scheduled via the R2D message. By allocating multiple access occasions, the reader can distribute A-IoT devices across these occasions to reduce the likelihood of collisions. The total number of access occasions is a key parameter for A-IoT devices to properly participate in slotted-ALOHA based A-IoT random access procedure. To be able to perform the selection, the A-IoT device must know the total number of access occasions scheduled by the R2D message.

There are implicit or explicit ways to indicate the total number of access occasions (i.e., A-IoT Msg1 resources). For example, the total number of access occasions is determined by Q = X×Y, where X and Y indicate the total number of TDMed access occasions (e.g., A-IoT Msg1 time resources) and the total number of FDMed access occasions (e.g., A-IoT Msg1 frequency resources), respectively. The R2D message (e.g., A-IoT paging message) can include one or more fields indicating at least one value of Q, X, or Y. Any value of Q, X, or Y can be also predefined.

In RAN2#129, RAN2 agreed that RAN2 re-uses the subsequent paging message to trigger re-access and there is no need to differentiate Msg1 resource for initial access vs re-access. Thus, the subsequent A-IoT paging message to trigger re-access can also include one or more fields indicating at least one value of Qre-access, Xre-access, or Yre-access, where Qre-access, Xre-access, and Yre-access indicate total number of access occasions (i.e., A-IoT Msg1 resources) for re-access, the total number of TDMed access occasions (e.g., A-IoT Msg1 time resources) for re-access, and the total number of FDMed access occasions (e.g., A-IoT Msg1 frequency resources) for re-access, respectively. This subsequent A-IoT paging message may adjust/update (Q, X, or Y) to (Qre-access, Xre-access, or Yre-access) based on a number of detected collisions, a number of identified/unidentified A-IoT devices, etc.

- [Issue 1-4] [Msg1 resource information] To define “D2R scheduling info” (provided in the A-IoT paging message) in terms of (a) the total number of access occasions, (b) the total number of TDMed access occasions, and/or (c) the total number of FDMed access occasions.

## [Issue 2-6] Msg1/2 association

This issue [2-6] was also discussed as part of email discussion [POST129bis][017][AIoT] [1] and views are not aligned among majority of the companies. RAN1 and RAN2 have made the following related agreements:

[Agreement (RAN1 #118b)] RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception. [Agreement (RAN1 #119)] For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies the following for the starting time for Msg2 monitoring - For Option 1, where a PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device, - Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. - Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. - For Option 2, where a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources. [Agreement (RAN1 #119)] For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring - Option 1: Predefined - Option 2: Indicated in the R2D transmission triggering random access Note: Option 1 and Option 2 may not be mutually exclusive. [Agreement (RAN1#120)] Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2. [Agreement (RAN1#120bis)] For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. • Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. • Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 • Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 [Agreements (RAN2#129bis)] - A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

A-IoT Msg1 includes at least one 16-bit random ID generated by the A-IoT device. When the A-IoT device identifies the start of its own access occasion, it transmits the random ID via the A-IoT Msg1 to the reader [1]. The reader responds to A-IoT Msg1 with the successfully received random ID in A-IoT Msg2. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, the A-IoT device considers the contention resolution as successful. The A-IoT Msg2 also contains parameters related to A-IoT Msg3 transmission.

RAN2 should consider the impact of the progress of RAN1 related to the agreements above. For example, an A-IoT device receives an A-IoT Msg2 (including a response to an A-IoT Msg1 transmission) which occurs after one or more other A-IoT Msg1 transmissions by other A-IoT devices. This case can occur when multiple access occasions (TDMed and/or FDMed) for multiple A-IoT Msg1 transmissions are allocated before the A-IoT device receives the A-IoT Msg2. In this case, echoing the random ID of the A-IoT device in A-IoT Msg2 is not enough to achieve the contention resolution since another A-IoT device could transmit the same random ID via a different access occasion before the A-IoT device receives the A-IoT Msg2. Furthermore, the number of A-IoT Msg2s (e.g., random IDs) included/multiplexed in the A-IoT Msg2 transmission may not be the same as (e.g., be less than) the total number of access occasions for A-IoT Msg1 transmissions, e.g., due to number of access occasions being larger than the number of paged A-IoT devices, due to A-IoT Msg1 transmission failure, or due to contention resolution failure, etc. As a result, if multiple A-IoT Msg2s (e.g., random IDs) are included/multiplexed in the A-IoT Msg2 transmission, an A-IoT devices cannot determine which A-IoT Msg2 among the multiple A-IoT Msg2s is corresponding to its Msg1 transmission.

Similar issue can happen for the case of re-transmitted A-IoT Msg2. According to RAN2 agreement during SI phase, re-transmitted A-IoT Msg2 can only include the random IDs of failed devices. If the re-transmitted A-IoT Msg2 includes multiple random IDs, echoing the random ID of the A-IoT device in A-IoT Msg2 is also not enough to achieve the contention resolution, regardless of whether multiple access occasions are TDMed or FDMed.

An association or mapping between an access occasion for A-IoT Msg1 and a corresponding A-IoT Msg2 can resolve this issue. We can consider two options as below:

- Option 1: the A-IoT Msg2 transmission includes a bitmap indicating the association or mapping between an access occasion for A-IoT Msg1 and a corresponding A-IoT Msg2. For example, each bit of the bitmap is associated with a respective time-frequency resource of A-IoT Msg1. If a value of a bit field in the bitmap is set to a first value (e.g., a value ‘0’), an A-IoT Msg2 (e.g., random ID) corresponding to an A-IoT Msg1 transmitted via an access occasion associated with the bit field is not present in the A-IoT Msg2 transmission; Otherwise (e.g., the value of the bit field is set to a second value, e.g., a value ‘1’), the A-IoT Msg2 (e.g., random ID) corresponding to the A-IoT Msg1 is present in the A-IoT Msg2 transmission. Thus, the A-IoT device can identify, based on the value of the bit field, whether the A-IoT Msg2 (e.g., random ID) corresponding to its A-IoT Msg1 is present or not and, if present, which A-IoT Msg2 among multiple A-IoT Msg2s in the A-IoT Msg2 transmission is for its A-IoT Msg1.

- Option 2: the A-IoT Msg2 transmission includes an indication (e.g., index) of time-frequency resource of an access occasion for the A-IoT Msg1. For example, an A-IoT Msg2 corresponding to the A-IoT Msg1 transmitted via a time-frequency resource includes an indication of the time-frequency resource (e.g., time-frequency resource index), a time resource indication of the time-frequency resource (e.g., time resource index), or a frequency resource indication of the time-frequency resource (e.g., frequency resource index). Indexing the time-frequency resources may be predefined. Based on the indication, the A-IoT device can identify which A-IoT Msg2 among multiple A-IoT Msg2s in the A-IoT Msg2 transmission is for its A-IoT Msg1. We can also consider the A-IoT Msg2 transmission multiplexing A-IoT Msg2s corresponding to A-IoT Msg1 transmissions via the same time resource with different frequency resources. In this case, a time resource indication (e.g., time resource index) is common for all A-IoT Msg2s and can be present one time in the A-IoT Msg2 transmission.

Thus, we have the following proposal:

- [Issue 2-6] [Issue 4-1] [Msg1/2 association] To discuss the association or mapping between an access occasion for A-IoT Msg1 and the corresponding A-IoT Msg2 in a PRDCH corresponding to multiple A-IoT Msg1 received from different devices. For this, to discuss if A-IoT Msg2 includes option (1) bitmap indication, or option (2) an indication (or index) time-frequency resource of an access occasion for the A-IoT Msg1.

# Conclusion

The observations captured are the following:

Observation 1.	[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] For CFRA, after sending D2R message, it is unclear whether the A-IoT device can consider this transmission immediately as successful. For CBRA, after sending Msg3, the A-IoT device might be in a limbo/stuck scenario if it misses the corresponding failure/re-access indication.

Observation 2.	[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] For CBRA, network could request, to the device, different behaviours upon detecting a failure of Msg1/Msg3: (1) re-access (i.e., retransmission of Msg1), or (2) retransmission of Msg3. For scenario (2), the usage of retransmitted Msg2 is already agreed. The open question is how re-access is informed to the device (which seems preferable to create a new R2D NACK message instead of other means e.g., also reusing Msg2).

Observation 3.	[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] A guard time TD→Rmin is defined by RAN1 as the time that the device does not need to monitor for R2D message after transmission of D2R message. This was discussed in relation to Msg1/2 but RAN1 agreement/TS seems general for any D2R message.

The proposals captured are the following:

Proposal 1.	[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] To determine the success or failure of a transmitted Msg3, to consider whether to define a timer-based approach and/or Msg.2-based approach.

Proposal 2.	[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] [Timer-based approach] To consider defining a new short timer (TD2R_ACK) to determine the success (as well as failure) of previously transmitted Msg3 which works as follows:

Proposal 2.1.	Point (A) Upon transmission of the Msg3, TD2R_ACK is started immediately or after certain time (TOffset). FFS on TOffset (if it’s needed and its relationship with TD→Rmin defined in TS 38.291).

Proposal 2.2.	Point (B) While TD2R_ACK is running, the device’s behaviour upon reception of the applicable R2D messages (e.g., Msg2 or paging) is defined case by case in A-IoT specification (including for both, failure and success).

Proposal 2.3.	Point (C) Upon expiry of TD2R_ACK, the A-IoT device considers previous Msg3 as successful or failed.

Proposal 2.4.	Point (D) To also apply this TD2R_ACK to any D2R messages sent as part of the A-IoT inventory and command procedure (including Msg1).

Proposal 3.	[Issue 2-4] [Issue 2-11] [new issue] [Success/failure of Msg3] [Msg.2-based approach] To consider defining the usage of retransmitted Msg2 to determine the success (as well as failure) of previously transmitted Msg3 which works as follows:

Proposal 3.1.	Msg2 is used to indicate success or failure of previously transmitted Msg3 (per UE basis). FFS whether this success indication can also be defined per group of devices and/or implicit.

Proposal 4.	[Issue 1-4] [Msg1 resource information] To define “D2R scheduling info” (provided in the A-IoT paging message) in terms of (a) the total number of access occasions, (b) the total number of TDMed access occasions, and/or (c) the total number of FDMed access occasions.

Proposal 5.	[Issue 2-6] [Issue 4-1] [Msg1/2 association] To discuss the association or mapping between an access occasion for A-IoT Msg1 and the corresponding A-IoT Msg2 in a PRDCH corresponding to multiple A-IoT Msg1 received from different devices. For this, to discuss if A-IoT Msg2 includes option (1) bitmap indication, or option (2) an indication (or index) time-frequency resource of an access occasion for the A-IoT Msg1.

# Reference

- R2-2503998, Summary of A-IoT MAC open issues, Huawei, HiSilicon, May 2025.

- R2-2503997, A-IoT MAC running CR, Huawei, HiSilicon, May 2025.

- R2-2503520, Discussion of new issue: success/failure of R2D message(s), Ofinno, May 2025.

- Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, EPC Radio-Frequency Identity Generation-2 UHF RFID Standard, Release 3.0.

# Annex

Details captured by Rapporteur on the issues discussed on this document (as part of[POST129bis][017][AIoT] 38.391 Running CR [1]) are included also here for easier reference.

## Issue 2-4

| Issue 2-4: CBRA failure detection | How to determine CBRA failure/contention resolution failure. - In SI, RAN2 agreed to support re-access in case of contention resolution failure as capture in TR 38.796. In WI phase, RAN2 has agreed to re-use the subsequent paging message to trigger re-access, but has not discussed how to determine CBRA failure/ contention resolution failure. - Status in running CR: captured as Editor’s Note in 5.5. | To be discussed by company contributions |
| --- | --- | --- |

## Issue 2-11

| Issue 2-11: explicit message for NACK | Whether to use a new/explicit R2D message - There is an FFS in the following agreement, but according to the online discussion in last meeting, the Rapp feels companies already consider this NACK message as a separate message, thus marks this as straightforward issue, and companies can check the proposal in 2.2. - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. - Status in running CR: captured as Editor’s Note in 5.5. | Straightforward |
| --- | --- | --- |

## New Issue

| Ofinno | We suggest discussing further the success and failure scenarios one by one with their corresponding handling for any D2R message sent after Msg3 until the A-IoT procedure is concluded. |
| --- | --- |

## Issue 1-4

| Issue 1-4: AO number field | How to indicate the number of access occasions, e.g. the maximum number, the length of field, format design. - Relevant agreements: - the A-IoT paging message can include a number of msg1 resources - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. | To be discussed by company contributions |
| --- | --- | --- |

## Issue 2-6

| Issue 2-6: number indication of echoed random IDs in Msg2 | Whether/how to indicate the number of echoed random IDs included in Msg2. - RAN2 agreed that A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices, but there is no discussion on whether/how to indicate the number of echoed random IDs. The Rapp understands this can be considered as signaling design/stage3 issue which should be quite straightforward. Companies can check the proposal in 2.2. - Status in running CR: not captured yet. | Straightforward |
| --- | --- | --- |