3GPP TSG RAN WG2 Meeting #130                                                            R2-2503520

# St Julian's, Malta, May 19th - 23rd, 2025

Agenda item:	8.2.4

Source:	Ofinno

Title:	Discussion of new issue: success/failure of D2R message(s)

Document for:	Discussion and decision

# Introduction

This document discusses the following New Issue identified during email discussion [POST129bis][017][AIoT] [1] on the success/failure of D2R message (with details captured on the issue copied in Annex A for easier reference).

# Discussion

## [New issue] Success/failure of D2R message

We suggested a New Issue during email discussion [POST129bis][017][AIoT] [1] to further discuss the success and failure scenarios for any D2R message sent even after Msg3 (until the A-IoT procedure is concluded). The solution here discussed could also address other open issues, such as [issue 2-4] as discussed in our companion document [2]. The following RAN2 agreements are taken for the NACK based mechanism in relation to CBRA and CFRA:

[Agreements (RAN2#128)] 1. RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID 2. If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported). [Agreements (RAN2 #129)] 1. NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. [Agreements (RAN2 #129)] 1 For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). 2 For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging 3 For CFRA, NACK feedback and re-access is not supported. FFS how to achieve 4 FFS on end of procedure.

RAN2 has still not concluded on whether a device needs any mechanism or timer to be used for the  device to confirm the success or failure after transmitting its D2R message. According to RAN2 agreements of previous meetings, there is no way to know whether a D2R transmission is successful from the device point of view (PoV). The first option that we can have in Rel. 19 A-IoT may be that the device should always consider a D2R transmission successful immediately after the D2R transmission. The second option may be that the device should wait until a next R2D message to implicitly or explicitly identify a success or failure of the previously transmitted D2R message. The first option might be simple from the device PoV but might not be very optimum if the reader needs to request the device to retransmit the previously transmitted D2R message at some point in the future. The second option might be simple from the network PoV but might not be ideal from the device PoV as the device might require keeping the previously transmitted D2R message in limbo for long time, e.g. until next/applicable R2D message is received or the ongoing A-IoT procedure is terminated (as success or failure). If so, this would not be ideal from power consumption PoV.

After sending any D2R message, the A-IoT device is not aware whether or when to consider this message successful. This may impact negatively to the A-IoT device (e.g., if it keeps the previously transmitted D2R message until next applicable R2D message is received or the A-IoT procedure is terminated), or to the network (e.g., if the A-IoT device always considers a transmission of a D2R message as successful upon its transmission).

In addition, it is important to clarify that RAN2 agreed not to define a timer in relation to the failure or re-access scenario. However there has no explicit discussion on whether a timer may be used by a device to trigger an implicit success of a previously transmitted D2R message.

It is important to keep also in mind, that RFID also specifies the usage of similar timer/behaviour between consecutive transmissions and receptions defining, when applicable, the minimum, nominal and maximum values. For example, corresponding RFID table/figure [3] which provides linking timing relation and parameters are included in Annex B for reference. It can be observed how RFID technology already defines multiple/different timers for the diverse scenarios and pairs of transmission/reception.

RFID technology already defines multiple/different timers for the diverse scenarios and pairs of transmission/reception (as captured in §6.3.1.6.5 of RFID specification).

Summarizing, in our understanding, the usage of a short time can be a simple approach to solve the concerns explained above. Figure 1 provides an overview of this simple mechanism that could be used by the A-IoT device to determine when the transmission of the D2R message is or can be considered successful via a short timer, referred as TD2R_ACK.

Figure 1. Mechanism to determine if the D2R message is or can be considered successful

The points to highlight of the mechanism shown in Figure 1 are as follows:

- Point (A) Upon transmission of the D2R message (steps [C], [E] and [H] in Figure 1), a new short timer TD2R_ACK is started immediately or after certain time, referred as TOffset (steps [C’], [E’] and[H’] in Figure 1).

    - FFS if TOffset is needed (which represents a delay to start the new short timer TD2R_ACK after sending D2R message) and if so, FFS whether this TOffset is same or different to TD→Rmin (defined in TS 38.291).

- Point (B) While the new timer TD2R_ACK is running, device’s behavior upon reception of the applicable R2D messages (e.g., step [D] in Figure 1) would be defined case by case on A-IoT MAC specification [4] (e.g., whether it means success/failure and whether it may trigger re-access, continue/end of procedure, etc).

- Point (C) Expiry of the new timer TD2R_ACK (e.g., step [F] and [I] in Figure 1) means that the device considers implicitly the success or failure of the previously transmitted D2R message.

- [New issue] To define a mechanism for the device to determine when the transmission of the D2R message is or can be considered successful or failure via a new short timer (TD2R_ACK) which works as follows:

  - Point (A) Upon transmission of the D2R message, TD2R_ACK is started immediately or after certain time (TOffset). FFS on TOffset (if it’s needed and its relationship with TD→Rmin defined in TS 38.291).

  - Point (B) While TD2R_ACK is running, the device’s behaviour upon reception of the applicable R2D messages is defined case by case in A-IoT specification (including for both, failure and success).

  - Point (C) Upon expiry of TD2R_ACK, the device considers previously transmitted D2R message as successful or failed.

# Conclusion

The observations captured are the following:

Observation 1.	After sending any D2R message, the A-IoT device is not aware whether or when to consider this message successful. This may impact negatively to the A-IoT device (e.g., if it keeps the previously transmitted D2R message until next applicable R2D message is received or the A-IoT procedure is terminated), or to the network (e.g., if the A-IoT device always considers a transmission of a D2R message as successful upon its transmission).

Observation 2.	RFID technology already defines multiple/different timers for the diverse scenarios and pairs of transmission/reception (as captured in §6.3.1.6.5 of RFID specification).

The proposals captured are the following:

Proposal 1.	[New issue] To define a mechanism for the device to determine when the transmission of the D2R message is or can be considered successful or failure via a new short timer (TD2R_ACK) which works as follows:

Proposal 1.1.	Point (A) Upon transmission of the D2R message, TD2R_ACK is started immediately or after certain time (TOffset). FFS on TOffset (if it’s needed and its relationship with TD→Rmin defined in TS 38.291).

Proposal 1.2.	Point (B) While TD2R_ACK is running, the device’s behaviour upon reception of the applicable R2D messages is defined case by case in A-IoT specification (including for both, failure and success).

Proposal 1.3.	Point (C) Upon expiry of TD2R_ACK, the device considers previously transmitted D2R message as successful or failed.

# Reference

- R2-2503998, Summary of A-IoT MAC open issues,, Huawei, HiSilicon, May 2025.

- R2-2503519, Discussion of issues 2-4, 2-6, 2-11, 1-4 and a new issue, Ofinno, May 2025.

- Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, EPC Radio-Frequency Identity Generation-2 UHF RFID Standard, Release 3.0.

- R2-2503997, A-IoT MAC running CR,, Huawei, HiSilicon, May 2025.

# Annex A

Details captured by Rapporteur on the issues discussed on this document (as part of[POST129bis][017][AIoT] 38.391 Running CR [1]) are included also here for easier reference.

## New Issue

| Ofinno | We suggest discussing further the success and failure scenarios one by one with their corresponding handling for any D2R message sent after Msg3 until the A-IoT procedure is concluded. |
| --- | --- |

# Annex B

Link timing scenarios and parameters captured by RFID specification, §6.3.1.6.5 [3], are copied below for reference.