3GPP TSG RAN WG2 Meeting #130                                                            R2-2503523

# St Julian's, Malta, May 19th - 23rd, 2025

Agenda item:	8.7.6

Source:	Ofinno

Title:	XR Rate Control

Document for:	Discussion and decision

# Introduction

In this contribution, we would like to discuss the following issues listed in the MAC open issues list [1]:

- [MAC-12] How to indicate in the Rate Control MAC CE that a query from UE is for available bit rate

- [MAC-08] Handling of triggered UL rate queries (e.g. multiplexing, transmission and cancelation, etc)

- [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows

- [MAC-07] Format of the Rate Control MAC CE

# Discussion

## Query an available bit rate with no desired bit rate

Based on the RAN2 agreements, a Rate Control MAC CE may be used by the UE to query an available bit rate. Two cases of queries have been agreed:

- Case 1: The UE queries an available bit rate by indicating a desired bit rate.

- Case 2: The UE queries an available bit rate with no desired bit rate.

For the Case 1, it could be like the legacy Recommended Bit Rate (RBR) MAC CE, whereby the UE explicitly indicates a desired bit rate via a bit rate field of the Rate Control MAC CE. If so, the bit rate field could use a predefined index from the bit rate table as specified in the MAC specification.

For the Case 2, however, the method for querying with no desired bit rate requires further discussion. In the legacy RBR procedure, while using the RBR MAC CE, the UE must always provide a desired bit rate, for two reasons:

- The legacy RBR MAC CE has a fixed size structure, meaning the bit rate field is always included.

- Index 0 in the bit rate table for the RBR, according to Note 1 of the bit rate index table in specification, is reserved only for the gNB to indicate “no new recommendation on bit rate is given”. Consequently, index 0 is not available to the UE for indicating “no desired bit rate.”

Given that RAN2 has agreed to explicitly support the Case 2 in Release-19, a solution for the UE to query an available bit rate with no desired bit rate must be introduced.

In this contribution, we propose three options (which are also shown in Figure 1) for further RAN2 discussion:

- Option 1: Use a specific index (e.g., index 0 or a reserved index) in the bit rate table.

  - A specific index (e.g., index 0 or a reserved one) in the bit rate table can be specifically used to indicate a query for an available bit rate with no desired bit rate. For this, the behaviour of the specific index needs to be clearly specified in the bit rate table for this usage (e.g., by a Note).  If the UE indicates any other index value, it would imply the query with an indicated desired bit rate.

- Option 2: Introduce a single-bit field (e.g., D/Desired bit rate field) in the Rate Control MAC CE.

  - A single-bit field/flag can be introduced in the Rate Control MAC CE. For example, the single-bit field/flag can indicate whether the UE queries an available bit rate with no desired bit rate or queries an available bit rate by indicating a desired bit rate.

- Option 3: Introduce an additional Rate Control MAC CE format which does not include bit rate field.

  - Define a separate Rate Control MAC CE format (e.g., a dedicated eLCID) explicitly used for UE queries with no desired bit rate. This new format could exclude the bit rate field in the MAC CE, reducing MAC CE size. For this, the existing Rate Control MAC CE format including the bit rate field would continue to serve the UE query with a desired bit rate.

Figure 1

[MAC-12] RAN2 to discuss and down-select one of the following options to indicate that the bit rate query is for an available bit rate with no desired bit rate:

- Option 1: Use a specific index (e.g., index 0 or a reserved index) in the bit rate table:

  - The specific index (e.g., index 0 or reserved index) in the bit rate table specifically to indicate a query for an available bit rate with no desired bit rate.

- Option 2: Introduce a single-bit field/flag in the Rate Control MAC CE:

  - The single-bit field/flag can indicate whether the UE queries an available bit rate with no desired bit rate or queries an available bit rate by indicating a desired bit rate.

- Option 3: Introduce an additional Rate Control MAC CE format which does not include bit rate field.

## Handling of the bit rate query procedure

According to the recent RAN2 agreements and the ongoing MAC running CR [1], a bit rate query procedure has been introduced. This procedure allows the UE to trigger a query to transmit the Rate Control MAC CE for an available/recommended bit rate. However, the detailed handling of the bit rate query procedure has not been thoroughly discussed or defined yet. Specifically, the following aspects require further consideration when the bit rate query procedure is triggered:

- When should the UE generate a Rate Control MAC CE onto allocated UL resources?

- What is the UE behavior if the allocated UL resources are insufficient to accommodate the Rate Control MAC CE?

- Under which conditions should the triggered bit rate query procedure be cancelled?

Regarding the generation of the Rate Control MAC CE, it is straightforward to reuse an approach consistent with legacy MAC operation. Specifically, if the MAC entity has received UL resources allocated for the new transmission, it should first determine whether a bit rate query has been triggered (and has not yet been cancelled). If there is a triggered bit rate query (has not yet been cancelled), the MAC entity shall further verify whether the allocated UL resources are sufficient to accommodate the Rate Control MAC CE along with its corresponding subheader. If these UL resources are sufficient, the MAC entity shall instruct the multiplexing and assembly procedure to generate and transmit the Rate Control MAC CE.

In case the allocated UL resources are insufficient to accommodate the Rate Control MAC CE and its associated subheader, it seems necessary to trigger the Scheduling Request (SR) to be able to transmit the Rate Control MAC CE as soon as possible. In legacy RBR procedure, a SR is not triggered when the allocated UL resources are insufficient to accommodate a RBR MAC CE. For XR Rate Control, we believe introducing an SR triggering mechanism for the Rate Control MAC CE is justified. With XR Rate Control, the XR application can dynamically adjust its codec bit rate and encoding parameters to ensure more precise uplink congestion management and QoE optimization. This is achieved by monitoring the network conditions and adapting the application's behavior accordingly, such as reducing the resolution or frame rate when the network is congested. In another scenario, when the UE triggers a bit rate query, it may imply that an immediate adjustment of the data rate might be required. For instance, the XR application may request an increased data rate, or current uplink conditions might be insufficient to fulfil the XR requirements. Therefore, promptly transmitting the bit rate query seems crucial to allow the network to determine and convey a suitable available/recommended bit rate as quickly as possible.

[MAC-08] For the triggered bit rate query, when MAC entity has allocated UL resources for a new transmission:

- (1) If the allocated UL resources can accommodate the Rate Control MAC CE (plus its subheader as a result of LCP), the MAC entity generates the Rate Control MAC CE;

- (2) If the allocated UL resources cannot accommodate the Rate Control MAC CE (plus its subheader as a result of LCP), the MAC entity triggers a Scheduling Request (SR) for the Rate Control MAC CE.

As for the cancellation of a triggered bit rate query procedure, we propose reusing similar cancellation conditions as those specified for the legacy RBR procedure. That is, when the MAC entity receives UL resource allocation for a new transmission and determines that the allocated UL resources are sufficient to accommodate the Rate Control MAC CE and its associated subheader, the MAC entity shall generate and transmit the Rate Control MAC CE. Subsequently the MAC entity cancels the corresponding triggered bit rate query procedure. Additionally, when a MAC reset occurs, the MAC entity shall cancel any triggered bit rate query procedures that remain pending.

[MAC-08] A triggered bit rate query is cancelled when:

- (1) MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate the Rate Control MAC CE (plus its subheader as a result of LCP);

- (2) MAC Reset.

## Format of Rate Control MAC CE

XR services typically comprise multiple distinct, yet interdependent QoS flows that collectively deliver a unified and cohesive user experience. For example, a VR session may involve individual QoS flows for video, audio, and haptic feedback. Similarly, an AR application might encompass QoS flows for rendered graphics, real-world video pass-through, spatial audio, and uplink sensor data, etc. Given this, it becomes beneficial to communicate recommended bit rate adjustments for multiple related QoS flows simultaneously within a single MAC CE.

In particular, coordinated bit rate adjustments across related QoS flows are essential to maintain synchronization (e.g., preserving audio and video alignment) or to ensure consistent QoE when network conditions change. For instance, when the uplink capacity becomes constrained, the network may need to recommend a balanced bit rates reduction simultaneously across AR video and sensor-data QoS flows, instead of handling each QoS flow individually.

Additionally, using a single Rate Control MAC CE to carry recommended bit rate adjustments for multiple QoS flows has the potential benefit of reducing MAC PDU overhead. Each MAC CE, irrespective of the size of its payload, requires an individual MAC subheader. Thus, if multiple XR-related QoS flows require concurrent bit rate updates, transmitting a single Rate Control MAC CE that bundles these updates is typically more resource-efficient than sending multiple Rate Control MAC CEs individually.

[MAC-05] A single Rate Control MAC CE can indicate one or multiple QoS flows.

When a single Rate Control MAC CE is used to indicate recommended bit rates for multiple QoS flows simultaneously, identifying these QoS flows via a bitmap offers certain advantages compared to explicitly listing each QFI. Specifically, using a bitmap provides a more compact and efficient way to signal which subset of the pre-configured QoS flows is included in the recommendation. Each bit in the bitmap can correspond to a particular pre-configured QoS flow, with the bit value indicating whether or not a recommended bit rate is provided for that QoS flow in the Rate Control MAC CE. This approach is particularly beneficial when the total number of configurable QoS flows is reasonable (e.g., 8 or 16 QoS flows), allowing the bitmap to be efficiently represented within only a few octets. Instead of explicitly including multiple full-length QFIs (e.g., 6 bits) in the Rate Control MAC CE, only a single bit per QoS flow is needed within the bitmap. Consequently, recommended bit rate values are included exclusively for the QoS flows indicated by bits marked in the bitmap, significantly reducing the signalling overhead.

[MAC-07] If the Proposal 4 is agreed, define a bitmap to identify QoS flows(s) within the Rate Control MAC CE. FFS on the size of the bitmap.

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	[MAC-12] RAN2 to discuss and down-select one of the following options to indicate that the bit rate query is for an available bit rate with no desired bit rate:

- Option 1: Use a specific index (e.g., index 0 or a reserved index) in the bit rate table:

  - The specific index (e.g., index 0 or reserved index) in the bit rate table specifically to indicate a query for an available bit rate with no desired bit rate.

- Option 2: Introduce a single-bit field/flag in the Rate Control MAC CE:

  - The single-bit field/flag can indicate whether the UE queries an available bit rate with no desired bit rate or queries an available bit rate by indicating a desired bit rate

- Option 3: Introduce an additional Rate Control MAC CE format which does not include bit rate field.

Proposal 2	[MAC-08] For the triggered bit rate query, when MAC entity has allocated UL resources for a new transmission:

- (1) If the allocated UL resources can accommodate the Rate Control MAC CE (plus its subheader as a result of LCP), the MAC entity generates the Rate Control MAC CE;

- (2) If the allocated UL resources cannot accommodate the Rate Control MAC CE (plus its subheader as a result of LCP), the MAC entity triggers a Scheduling Request (SR) for the Rate Control MAC CE.

Proposal 3	[MAC-08] A triggered bit rate query is cancelled when:

- (1) MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate the Rate Control MAC CE (plus its subheader as a result of LCP);

- (2) MAC Reset.

Proposal 4	[MAC-05] A single Rate Control MAC CE can indicate one or multiple QoS flows.

Proposal 5	[MAC-07] If the Proposal 4 is agreed, define a bitmap to identify QoS flows(s) within the Rate Control MAC CE. FFS on the size of the bitmap.

# Reference

[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)