3GPP TSG-RAN WG2 #130 	R2-2503527

St Julian’s, Malta, 19th - 23rd May 2025

Agenda Item:	8.2.3

Source:	KT Corp.

Title:	Discussion on random access aspects of Ambient IoT

Document for:	Discussion/Decision

WID/SID:	Ambient_IoT_solutions

1	Introduction

The New WID for Ambient IoT was approved in RAN #107 meeting [1]:

WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this document, we would like to address the potential issues to support the randomly resource selection for MSG1 and MSG1, starting time for MSG1 transmission for A-IoT devices and propose our views on A-IoT Random Access.

2	Discussion

2.1	Randomly Resource Selection for MSG1

Here are the agreements related to the scheduling of MSG1 which were made in the previous RAN2#129, RAN2#129b meeting. RAN2 agreed that device(s) randomly selects among FDMA occasions as baseline. However, there was no reached an agreement on whether/how to specify the device detailed behaviour of randomly selecting the MSG1 resource based on the R2D trigger message.

RAN2#129 Agreements (Scheduling of MSG1 Transmissions) - The A-IoT paging message can include a number of msg1 resources Agreements (MSG1 Resource Selection for FDMA) - RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

RAN2#129bis Agreements on msg1 - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. Agreements on new R2D message - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number.

In case of CBRA, device(s) should randomly select resource for MSG1 transmission among the access occasions. This is because in CBRA, there is no information on which access occasion is assigned per device ID.  So, if a device randomly selects one of multiple access occasions without any rule, there is a possibility of collisions. In addition, at the last RAN1#120bis meeting, RAN1 made an agreement that the reader would provide up to two-time domain resources for MSG1 transmission.

To reduce the probability of collisions when devices randomly select resource, device can implicitly select based on specific rules. Figure 1 is an example of resource allocation by using the number of MSG1 resources. For example, device(s) implicitly select one resource among time/frequency access occasions based on the modular operation of both device ID and the total number of access occasions. Then, device can select one resource in order of time-first or frequency-first based on the result of a modulo operation. Alternatively, if an access occasion index is provided for each access occasion, the device can select a resource without conflict based on the result of the modulo operation and the access occasion index.

Figure 1. Random resource selection for MSG1

In order to reduce the collision probability, RAN2 support including indication bit in new R2D message to instruct the device(s) how to select the one resource among access occasions.

2.2	Reception of (re-)Msg2

In the previous RAN2#128 and RAN2#129 meeting, RAN2 agreed that re-transmission of Msg2 and NACK based mechanism for at least Msg3 as shown in below. We would like to discuss how and when a A-IoT reader (re-)transmits MSG2 and NACK to specific device(s).

Agreements (RAN2#128) - RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID - If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

As shown in figure 2, According to the agreement from the previous RAN2#129 meeting, it was agreed that the reader would re-transmit MSG2 if the reader doesn’t receive MSG3 from that ID.

Figure 2. Problem due to duplicate retransmission of Msg2 or Msg3 feedback by reader

As shown in figure 2, According to the agreement from the previous RAN2#129 meeting, it was agreed that the reader would re-transmit MSG2 if the reader doesn’t receive MSG3 from that ID. The device can expect to receive MSG2 after sending Msg1. If the device successfully receives Msg2, it can transmit Msg3. However, if the device did not receive the initial MSG2, it can expect to receive re-transmitted MSG2. But there was no monitoring configuration for receiving the re-transmitted MSG2.

If the reader decides to re-transmit Msg2 to the specific devices that failed to transmit Msg3, the device may need a separate monitoring window to receive (re-)Msg2. Therefore, we would like to discuss the monitoring windows for receiving the (re-)Msg2 such as Random-Access Response (RAR).

RAN2 to discuss the monitoring windows for receiving the (re-)Msg2 such as Random Access Response (RAR).

3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	In order to reduce the collision probability, RAN2 support including indication bit in new R2D message to instruct the device(s) how to select the one resource among access occasions.

Proposal 2	RAN2 to discuss the monitoring windows for receiving the (re-)Msg2 such as Random Access Response (RAR).

4	References

RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc., RAN#107

RAN2#129 chairman’s notes.

RAN2#129bis chairman’s notes.

RAN1#120 chairman’s notes.

RAN1#120bis chairman’s notes.

TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR” V19.0.0 (2024-12).