3GPP TSG-RAN WG2 Meeting #130	R2-2503534

St. Julians, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on functionality management

Agenda Item:	8.3.2

Document for:	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper discusses the functionality management for the UE-sided model, including the applicability procedure, inference, and report configurations.

- Applicability procedure

At the RAN2 #129bis meeting, the following were agreed for the applicability procedure [2],

Agreements

The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework.

For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

FFS whether association ID is required and should provide motivation on per use case bases.

Figure 1: General procedure for applicability report and measurement configurations

Figure 1 shows the general procedure for the applicability report and measurement configurations and activations. The major difference between this use case and other AI/ML for PHY ones is that different UE capabilities and measurement/report configurations should be considered during each step. During Step 1~2, UE capabilities corresponding to the RRM measurement or measurement event prediction should be reported. During Step 3, the contents of the signaling for AI/ML for PHY use cases, including beam management and CSI prediction, are based on RAN1 signalings under the CSI framework, which aims to notify the UE of the candidate CSI report configurations. For AI/ML for mobility, the contents are based on the RRM measurement signalings, which should be studied in RAN2. Therefore, we have the following proposals,

Proposal 1

- Step 1~5 of the LCM procedure for the UE-sided model for beam management can be reused after the following revisions,

  - In Step 1~2,

    - Under the enquiry, the UE reports capabilities corresponding to the RRM measurement or measurement event prediction.

  - In Step 3,

    - For Option A, RRM measurement configurations can be used for the inference configurations.

    - For Option B, OtherConfig can be used for the inference-related parameter configurations.

  - In Step 4, UE reports the applicability/inapplicability of the inference or inference-related parameter configurations.

  - In Step 5, RRM measurement configurations can be used for the inference configurations for Option B.

- RAN2 study the detailed contents of the signalings in each step.

- Inference and report configurations

At the RAN2 #129bis meeting, the following were agreed for the inference and report configurations [2],

Agreements

For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details.

For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details.

Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

For AI/ML for PHY use cases, such as beam management and CSI prediction, all related specification impacts are in RAN1, where the CSI report is reused for the inference result report. For AI/ML for mobility, RAN2 needs to study specification impacts in detail for each scenario and/or sub-use case because the predictions are about the layer 3 RRM measurements or events.

At the last meeting, the RSRP was considered the baseline quantity of the study. In practical deployments with multiple frequency bands, the RSRQ is also an important measurement quantity to support the inter-frequency cell selection and handovers. Therefore, the RSRQ prediction also benefits the NW performance in measurement overhead reduction or HO performance enhancements. Both RSRP and RSRQ should be considered as the baseline of the study for the specification impacts of RRM measurement predictions.

For the RRM measurement or measurement event prediction with the UE-sided model, UE should know which measurements can be measured, i.e., the corresponding reference signals are actually transmitted, and which measurements are to be predicted without the opportunity to measure. This information should be configured to the UE before the activation of the UE-sided model.

We have the following proposals based on the above discussions,

Proposal 2

- Input/output of the inference

  - For report quantity, RSRQ should also be considered as a baseline for inter-frequency handover.

  - For inference, the measurements that a UE needs to actually measure and to infer should be configured for the UE.

After the UE obtains the predictions, how the UE processes them and reports them should be specified to align the understanding of reported values between the NW and UE. For legacy RRM measurements, a Layer 3 filtering operation is specified and controlled by the NW. After the RRM measurement predictions are considered, the UE has both measured and predicted measurements in hand. How UE processes them to obtain the filtered measurements should be specified to maintain a common understanding of the reported values.

For measurement event predictions, the NW also needs to know whether a measurement event reported from the UE is predicted or measured. There are two options to support such a feature: One is to allow the UE to report the predictions in an existing measurement event, while additional indications in the events should be further studied in RAN2. The other is to define new measurement events dedicated to the predictions. In either option, the NW can distinguish whether a UE reported event is based on the predictions and make the handover decisions without misunderstandings.

In summary, we propose the following,

Proposal 3

- Inference result report

  - For the inference report of RRM measurement prediction and indirect measurement event prediction, how UE processes the measured and predicted values for Layer 3 filtering should be studied and specified.

  - For RRM measurement prediction and measurement event prediction, enhancements on measurement events are necessary to support UE reporting predicted measurement events. The following two options can be studied,

    - Option 1: Reuse the current events and allow UE to report the predictions. FFS how to indicate the event is a prediction.

    - Option 2: Define new events to accommodate the UE predictions.

- Conclusion

In this contribution, we have the following proposals,

Proposal 1

- Step 1~5 of the LCM procedure for the UE-sided model for beam management can be reused after the following revisions,

  - In Step 1~2,

    - Under the enquiry, the UE reports capabilities corresponding to the RRM measurement or measurement event prediction.

  - In Step 3,

    - For Option A, RRM measurement configurations can be used for the inference configurations.

    - For Option B, OtherConfig can be used for the inference-related parameter configurations.

  - In Step 4, UE reports the applicability/inapplicability of the inference or inference-related parameter configurations.

  - In Step 5, RRM measurement configurations can be used for the inference configurations for Option B.

- RAN2 study the detailed contents of the signalings in each step.

Proposal 2

- Input/output of the inference

  - For report quantity, RSRQ should also be considered for inter-frequency mobility.

  - For inference, the measurements that a UE needs to actually measure and to infer should be configured for the UE.

Proposal 3

- Inference result report

  - For the inference report of RRM measurement prediction and indirect measurement event prediction, how UE processes the measured and predicted values for Layer 3 filtering should be studied and specified.

  - For RRM measurement prediction and measurement event prediction, enhancements on measurement events are necessary to support UE reporting predicted measurement events. The following two options can be studied,

    - Option 1: Reuse the current events and allow UE to report the predictions. FFS how to indicate the event is a prediction.

    - Option 2: Define new events to accommodate the UE predictions.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #129bis Chair Notes”, Apr., 2025.