3GPP TSG-RAN WG2 Meeting #130	R2-2503536

St. Julians, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on performance monitoring

Agenda Item:	8.3.5

Document for:	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper discusses the performance monitoring for both UE-sided and NW-sided models.

- Performance monitoring for the UE-sided model

At the RAN2 #129bis meeting, the baseline framework has been agreed for performance monitoring [3],

Agreements

For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case.

During the Rel-18 study for the AI/ML beam management, the following performance monitoring types are listed for the UE-sided model [2],

- Type 1

  - The decisions of LCM operations are made by the NW and indicated to the UE.

  - Option 1 (NW-side performance monitoring)

    - UE sends a report to NW (for the calculation of the performance metric at NW).

    - The report is at least configured/triggered by the NW.

  - Option 2 (UE-assisted performance monitoring)

    - UE calculates the performance metrics.

      - UE either reports the metrics or sends an event based on the metrics.

- Type 2 (UE-side performance monitoring)

  - The UE calculates performance metrics and makes decisions on the LCM operations.

There are several issues with the Type 2 performance monitoring for AI/ML for mobility use cases. Because the performance of the AI/ML models for HO significantly impacts the user experiences and the NW resource scheduling, the LCM operations decided by UE may cause severe issues for the NW operations from unexpected UE behaviors. Secondly, the NW can obtain some UE HO-related information, like the HO occasions, ping-pong HOs, and time-of-stays without UE reports. The NW should be able to control the UE-sided models if the NW detects issues regarding the handover. Therefore, we propose the following,

Proposal 1

- Support only Type 1 Option 1 (NW-side performance monitoring)  and/or Type 1 Option 2 (UE-assisted performance monitoring), i.e.,  decisions of LCM operations are made by the NW, and Type 2 (UE-side monitoring) is not supported.

  - Type 1-Option 1 (NW-side performance monitoring)

    - UE sends a report to NW (for the calculation of the performance metric at NW).

    - The report is at least configured/triggered by the NW.

  - Type 1-Option 2 (UE-assisted performance monitoring)

    - UE calculates the performance metrics.

      - UE either reports the metrics or sends an event based on the metrics.

Regarding the performance metrics, reusing the intermediate KPIs used in RAN2 evaluations for these topics is straightforward. The average differences of RSRP/RSRQ/SINR can be considered for RRM measurement predictions. The same intermediate KPIs can also be adopted for the indirect measurement event prediction because the indirect prediction is based on the RRM measurement predictions.

For the measurement event predictions, including both indirect and direct ones, intermediate KPIs such as F1-score have been adopted for the RAN2 evaluations. However, how UE collects the ground-truth tags for calculating metrics such as the F1-score or miss/false prediction rates should be further investigated.

Another option for monitoring measurement event predictions is to adopt handover performance metrics, such as the HoF rate, Short-ToS rate, or Ping-pong rate. The NW can obtain these metrics and identify abnormal tendencies. In summary, our proposals for the performance metrics of monitoring are,

Proposal 2

- RAN2 needs to study specification impacts in detail for each scenario and/or sub-use cases:

  - For RRM measurement prediction and indirect measurement event prediction,

    - Intermediate KPIs about the prediction error (e.g., average RSRP differences) can be used as the performance metric for monitoring.

  - For direct and indirect measurement event prediction, study the following options,

    - Option 1: Use intermediate KPIs related to the event detection, such as F1-score, miss/false prediction rate, for monitoring.

    - Option 2: Use the HO KPIs such as HoF rate, Short-ToS rate, PingPong rate, etc.

- Performance monitoring for the NW-sided model

In the AI/ML for PHY sessions, RAN2 agreed to focus on the data collection part for the NW-sided model, and the gNB is responsible for monitoring its own performance. RAN2 will specify performance monitoring only based on RAN1 inputs, if any.

No specification impacts have been identified so far for AI/ML use cases with NW-sided models, e.g., AI/ML beam management with NW-sided models. The NW can configure the UE to measure and report the information necessary for monitoring the model's performance. The situation does not change for the AI/ML for mobility use case with NW-sided models. For mobility use cases we studied, only layer 3 measurements and procedures are involved. Therefore, RAN2 can conclude the specification impacts by itself as follows,

Proposal 3

- RAN2 concludes that there is no specification impact for performance monitoring of the NW-sided model.

- Conclusion

In this contribution, we have the following observations,

Proposal 1

- Support only Type 1 Option 1 (NW-side performance monitoring)  and/or Type 1 Option 2 (UE-assisted performance monitoring), i.e.,  decisions of LCM operations are made by the NW, and Type 2 (UE-side monitoring) is not supported.

  - Type 1-Option 1 (NW-side performance monitoring)

    - UE sends a report to NW (for the calculation of the performance metric at NW).

    - The report is at least configured/triggered by the NW.

  - Type 1-Option 2 (UE-assisted performance monitoring)

    - UE calculates the performance metrics.

      - UE either reports the metrics or sends an event based on the metrics.

Proposal 2

- RAN2 needs to study specification impacts in detail for each scenario and/or sub-use cases:

  - For RRM measurement prediction and indirect measurement event prediction,

    - Intermediate KPIs about the prediction error (e.g., average RSRP differences) can be used as the performance metric for monitoring.

  - For direct and indirect measurement event prediction, study the following options,

    - Option 1: Use intermediate KPIs related to the event detection, such as F1-score, miss/false prediction rate, for monitoring.

    - Option 2: Use the HO KPIs such as HoF rate, Short-ToS rate, PingPong rate, etc.

Proposal 3

- RAN2 concludes that there is no specification impact for performance monitoring of the NW-sided model.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- 3GPP TR 38.843 v2.0.0, Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface (Release 18), Dec., 2023.

- RAN2 Chair (InterDigital), “RAN2 #129bis Chair Notes”, Apr., 2025.