3GPP TSG-RAN WG2 Meeting #130	R2-2503548

St. Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.1.3

Source: 	Fujitsu

Title: 	Discussion on NW side Data Collection

WID/SID:	NR_AIML_air

Document for:	Discussion and Decision

Introduction

In the last several meetings [1][2][5][6], there were discussions on the NW side data collection of beam management use cases and several agreements were achieved, while the discussion for positioning is still pending. In this contribution, we would like to discuss the remaining issues of the detailed solutions for NW side data collection of beam management and positioning use cases for model training.

Discussion

Activation/Deactivation for Data Logging and Reporting

Some remaining issues are listed below:

[RAN2 #127] 1 As the baseline approaches, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

As for the activation/deactivation, they are optional for the data collection procedure. UE will start/stop the data collection procedure upon configuration, and multiple events/conditions can be discussed to guarantee the flexibility to dynamically control the data collection procedure. Therefore, additional dynamic activation/deactivation signaling may not be necessary.

Proposal 1 Additional dynamic activation/deactivation may not be necessary since events/conditions can be designed to dynamically control the data collection procedure.

Contents of Data Collection

The contents of data collection have been discussed for three meetings, and the agreements [2] are shown below:

Agreements 1. For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit 2. New SRB can be configured for NW-side data collection (with lower priority)

In the parallel discussion in RAN1, it has been agreed that a time gap will be configured by RRC to indicate the timing information among reported beam samples of AI/ML BM Case-2 for inference. Therefore, we proposed to have similar configuration for data collection as well since unified frameworks such as CSI-MeasConfig will be used regardless of the purpose of the measurements.

Proposal 2 Similar to inference configuration, the timing gap should be configured in the CSI-ReportConfig for data collection as well.

One potential issue for data collection is that, unlike inference, the measured data will not be reported immediately and will be stored in UE buffer and reported later via L3 signaling, so it is important to let the NW know the boundaries among data collected for different functionalities/models/features/FGs. One solution is to include functionality related index into the reporting with the data samples so that NW can distinguish different data.

Proposal 3 In order to support the reporting of multiple configurations, functionality related index may be reported so that NW can distinguish the samples per functionality/model/feature/FG.

Data Collection for Positioning

It has been agreed in RAN1 that the collected data used for training comprises two parts:

Agreement [RAN1 #116bis] For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement [RAN1 #117] For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for the same location associated with Part B. Note: the association can be discussed

We would like to discuss the data collection for NW-side positioning models (case 3a/3b) for the following three aspects:

- Part A

- Part B

- Association between Part A/B

Case 3a

Part A

The input data generation (Part A) is indicated in TR38.843[3]:

From Section 7.1.4 of TR38.843 - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified: - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP

So, no matter which entity is used for case 3a model training, there may not be any RAN2 specification impact for part A data collection:

- Training at gNB: There will be no data transfer among entities, so no specification impact.

- Training at LMF: gNB->LMF data transmission is left for RAN3 discussion.

Observation 1 There may not be RAN2 impact for Part A data collection of case 3a.

Part B

The label data generation (Part B) is given in the following agreement from RAN1:

Agreement [RAN1 #116bis] For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

Again, there seems no RAN2 impacts at all if LMF will be the only entity to generate part B data.

- Training at gNB: LMF->gNB data transmission is left for RAN3 discussion.

- Training at LMF: There will be no data transfer among entities, so no specification impact.

Observation 2 There may not be RAN2 impact for Part B data collection of case 3a.

Association Between Part A/B

There might be some RAN2 impacts for the association between Part A and Part B for case 3a, since these two parts of data may be associated by the same UE/PRU, if model is trained at LMF may need to request some assistance information from UE/PRU to help the LMF identify different UE/PRUs. However, the detail for this part is far beyond current RAN2 discussion progress and cannot be proceeded unless other WGs have more input.

Observation 3 RAN2 may not be able to figure out the impact on association between Part A/B data collection for case 3a, until there is more input from other WGs.

Proposal 4 RAN2 suspends the discussion on training data collection for case 3a and wait for other WGs.

Case 3b

Part A

The input data generation (Part A) is indicated in TR38.843[3]:

From Section 7.1.4 of TR38.843 - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified: - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP

Specifically, for case 3b, gNB reports SRS-based channel measurements to LMF as part A.

In legacy positioning specification, NRPPa framework defined in TS38.455 is used for communication between gNB and LMF, and LPP framework defined in TS37.355 is used for communication between UE/PRU and LMF, thus it is natural to reuse these two frameworks for the collection of part A data for case 3b.

It is noticed that the channel measurement report based on SRS/PRS in NRPPa/ LPP has already been supported for legacy positioning method. In NRPPa, the procedure measurement information transfer is defined as “a procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements.” [4]:

From section 8.5 of TS38.455

Once upon the data collection triggered, LMF can configure gNB to report measurement results, and the reporting contents can be filtered by LMF, measurement quality and time stamp which defined in section 9.1.43 and 9.1.42 respectively can be re-used to report together with the channel measurements in TRP measurement results IE.

In current RAN2 discussion, it is suggested that RAN3 to provide more information on whether the signalling and procedure need to be extended or enhanced for case 3b part A data collection, meanwhile, it is suggested that RAN1 to provide more information on whether the collected contents need to be extended or enhanced, e.g., the type of the channel measurement, the format of the quality indicator and so on.

Proposal 5 To support part A data collection of case 3b, existing NRPPa procedure (e.g., Measurement information transfer) can be re-used.

Proposal 6 For the existing NRPPa procedure of part A data collection of case 3b, it is up to RAN3 on the potential enhancements of signalling and procedure, whilst it is up to RAN1 on the potential enhancements of collected data contents.

Part B

The label data generation (Part B) is given in the following contents from TR and working assumptions from RAN1:

Working Assumption [RAN1 #117] For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location. - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. From Section 7.2.4 of TR38.843 For LMF-side model, the LMF is the termination point for training data.

Case 3b is direct AI/ML positioning, therefore, their model output (ground truth labels for training) are UE locations, from the above-listed working assumptions, PRU/Non-PRU UE with estimated location/LMF are three common sources to provide part B data for case 3b.

- PRU: According to the definition in TS38.305, PRU is a special type of UE with known location, the detailed ways of PRU deployment have not been defined in specifications, however, the locations of the PRU will be known to LMF without introducing additional signalling and procedure.

- Non-PRU UE with estimated location: The data privacy has been discussed in RAN1 so the paths to deliver the estimated UE locations to LMF are pending for further study. It may either be transmitted via RAN-transparent way or follow the LPP location information transfer procedure.

- LMF: It refers to LMF obtain the ground truth labels by other means, e.g., GNSS.

Basically, LMF will be in charge of determining the ways of part B data collection of case 3b, it may choose to collect data by stored PRU information, from UE reports or by other implementation ways.

Proposal 7 LMF can choose the appropriate ways to collect part B data for case 3b. One feasible way to collect estimated location of non-PRU UE is via LPP procedures such as location information transfer, details pending for further RAN1 conclusions.

Association Between Part A/B

According to the above agreements, Part A and Part B of one data sample should be associated in the following two ways:

- For a same UE (PRU or Non-PRU UE): LMF can configure the contents of the measurement reporting, in legacy specification, part A (channel measurement) and part B (UE locations) can be reported together in the same reporting IE. Otherwise, LMF has the freedom to match the reported contents by itself, e.g., it can start two parallel LPP sessions towards one UE with two different positioning methods, then encapsulate part A from one session and UE location from another session into one data sample, the decisions on how to associate are up to LMF, and it can give more configurations if it needs UE to report more information to help its association, RAN2 can wait for RAN1 for the details.

- For a same location associated with Part B: LMF may know whether two UE/PRUs are grouped into “same location,” so it may classify the groups by itself. Similar to the above, LMF may give more configurations for UE/PRU to report assistance information (e.g., some indications on the adjacency), again, RAN2 can wait for RAN1 for the details.

Proposal 8 LMF may associate part A and part B data from the same PRU/UE for case 3b by at least the following ways:

- LMF configures UE/PRU to report part A/B together in the same LPP reporting.

- LMF configures UE/PRU to report part A/B separately, but with assistance information to combine two reports. The assistance information may be either from UE reporting, or from LMF locally.

The details are left for further RAN1 input.

Proposal 9 LMF may associate part A for a same location associated with Part B for case 3b by its definition on “same location”. There may be potential assistance information from the UE to help LMF make decisions, but the details are left for further RAN1 input.

Conclusion

In this contribution, we discuss the potential solutions for the NW-side data collection for model training in beam management and positioning. The following observations and proposals are considered:

Beam Management:

Proposal 1 Additional dynamic activation/deactivation may not be necessary since events/conditions can be designed to dynamically control the data collection procedure.

Proposal 2 Similar to inference configuration, the timing gap should be configured in the CSI-ReportConfig for data collection as well.

Proposal 3 In order to support the reporting of multiple configurations, functionality related index may be reported so that NW can distinguish the samples per functionality/model/feature/FG.

Positioning:

Observation 1 There may not be RAN2 impact for Part A data collection of case 3a.

Observation 2 There may not be RAN2 impact for Part B data collection of case 3a.

Observation 3 RAN2 may not be able to figure out the impact on association between Part A/B data collection for case 3a, until there is more input from other WGs.

Proposal 4 RAN2 suspends the discussion on training data collection for case 3a and wait for other WGs.

Proposal 5 To support part A data collection of case 3b, existing NRPPa procedure (e.g., Measurement information transfer) can be re-used.

Proposal 6 For the existing NRPPa procedure of part A data collection of case 3b, it is up to RAN3 on the potential enhancements of signalling and procedure, whilst it is up to RAN1 on the potential enhancements of collected data contents.

Proposal 7 LMF can choose the appropriate ways to collect part B data for case 3b. One feasible way to collect estimated location of non-PRU UE is via LPP procedures such as location information transfer, details pending for further RAN1 conclusions.

Proposal 8 LMF may associate part A and part B data from the same PRU/UE for case 3b by at least the following ways:

- LMF configures UE/PRU to report part A/B together in the same LPP reporting.

- LMF configures UE/PRU to report part A/B separately, but with assistance information to combine two reports. The assistance information may be either from UE reporting, or from LMF locally.

The details are left for further RAN1 input.

Proposal 9 LMF may associate part A for a same location associated with Part B for case 3b by its definition on “same location”. There may be potential assistance information from the UE to help LMF make decisions, but the details are left for further RAN1 input.

Reference

[1] R2_127bis_ChairNotes_EOM.

[2] R2_129bis_ChairNotes_final.

[3]TR38.843, V2.0.0, 3rd Generation Partnership Project, Technical Specification Group Radio Access Network. Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), Dec 2023.

[4]TS38.455.

[5]R2_128_ChairNotes_EOM.

[6]R2_129_ChairNotes_final.