3GPP TSG-RAN WG2 Meeting #130	R2-2503550

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source: 	Fujitsu

Title:	Discussions on AIoT Paging

Document for:	Discussion and Decision

- Introduction

According to the objective specified in the New WID [1], RAN2 needs to specify A-IoT Paging to enable DO-DTT and DT data transmission. At previous RAN2 meetings, some agreements related to A-IoT Paging are reached [2] [3]:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Agreements

- FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

- RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

- Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

This contribution will provide further considerations on the Ambient IoT Paging (including subsequent paging).

- Discussions

During Rel-19 SI phase, RAN2 has made some agreements and reported them in [4]. According to the WID, the definitions provided in the TR and decisions made during the Rel-19 SI are taken into this WI. Therefore, the discussions can be started from the decision already made in the TR.

2.1 Determination to skip the response

In the Rel-19 TR, it is captured that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. Based on a transaction ID, the A-IoT device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before).

Regarding the determination to skip sending the response, the A-IoT device should compare the stored transaction ID with the received one in the A-IoT paging message. In this way, the A-IoT device will store the transaction ID if it had successfully responded the A-IoT paging message associated with the transaction ID. Alternatively, the A-IoT device also stores the status, i.e. whether it has successfully responded the A-IoT paging message associated with a transaction ID and takes the stored transaction ID and the corresponding status into account to determine whether to skip sending the response. We prefer the former slightly since less information will be stored. Also, if a transaction ID is stored before it responded the A-IoT paging message successfully, previous transaction ID associated with already successfully responded A-IoT paging will be replaced. Thus, it increases the possibility that the device initiates A-IoT random access to respond A-IoT paging which has already been responded successfully. So, if a new transaction ID in the A-IoT paging message is received, the A-IoT device will store the received transaction ID and release it when the procedure fails. Conversely, if a same transaction ID is received, the A-IoT device will skip sending the response to the A-IoT paging message.

Proposal 1: To reduce the stored information, the A-IoT device releases the stored Transaction ID when the procedure fails.

Proposal 1bis: The A-IoT device will skip subsequent paging with the same Transaction ID as its stored Transaction ID.

2.2 Content and format for A-IoT paging

According to RAN2 agreements and A-IoT MAC running CR [5], A-IoT paging message at least contains:

- R2D Message Type: at least two bits to indicating Paging, Access Occasion Trigger, Random ID Response, Upper Layer Data Transfer

- RA type: one bit to indicate whether it is CFRA or CBRA

- Transaction ID: FFS the size, more than one bit

- Number of Access Occasions: at least for CBRA

- Indication of Paging ID presence

- Length of Paging ID: as small as possible

- Paging ID (upper layer data)

One open issue for CFRA is whether paging in CFRA can omit the CBRA related fields, such as Transaction ID, Indication of Paging ID presence, Number of access occasions in Paging message. In our view, some of the CBRA related fields in paging message can be omitted for CFRA to save overhead, including Indication of Paging ID presence, Number of access occasions. For Transaction ID, it will be discussed in 2.3.

Proposal 2: Paging for CFRA can omit some of the CBRA related fields, including Indication of Paging ID presence, Number of access occasions.

Considering the relevance, these CBRA related fields should be included in the A-IoT Paging message after RA type field.

Proposal 2bis: RA type field is present before the CBRA related fields, if present, including Indication of Paging ID presence and Number of Access Occasions.

Regarding Number of Access Occasions field, there are some aspects for discussion. For example, whether a fixed number of bits or variable length will be used for this field. If it has fixed size, the size can be one or two bytes, i.e. 8 or 16 bits. If the supported range of the number of access occasions is quite wide and overhead reduction in some scenarios is desired, then variable length for this field can be supported. If it is variable, length of the Number of Access Occasions field needs to be indicated in the A-IoT Paging message.

Proposal 3: RAN2 to discuss the length for Number of Access Occasions field is variable or not. The length needs to be indicated in the A-IoT Paging message if it is variable.

RAN2 assumed that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. During the email discussion [6] the need for visibility in the MAC layer was discussed. It seems that the following use cases require the paging identifier visible in A-IoT MAC layer:

- Case 1: the paging identifier is used to associate the AS ID with the A-IoT device

- Case 2: AS grouping

For case 1, the issue can be discussed in AI 8.2.4. Basically, we think that the issue can be solved by e.g. deterministic D2R resource or 16-bit random number in Msg1. For case 2, AS grouping incurs more complexity and standization effors. Considering the timeline, at least AS grouping will not be supported in this release. So,  it is not required that the paging identifier is visible the A-IoT MAC Layer.

Proposal 4: RAN2 to confirm the current assumption that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.

As agreed, paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/ “filtering criteria”. Considering forward compatibility to support multiple paging identifiers, a reserved field is required. The reserved field is used to indicate whether there are more combinations of paging ID length information and paging identifier since paging ID length information is always included together with the paging ID field.

Proposal 5: For forward compatibility, A-IoT Paging message contains a reserved field to indicate whether there is more information on paging ID length and corresponding paging identifier.

2.3 Multi-reader scenario

One open issue in A-IoT paging is to consider potential multi-reader scenario. If a device gets a new service request while one procedure is still ongoing, whether/how to specify device behaviour or leave it to implementation, and design the end of procedure if needed.

At RAN2#129bis, the device behaviour if it gets a new service request while one procedure is still ongoing was discussed. Two options were considered:

- Option A: Ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

- Option B: terminate the ongoing procedure and respond to the latest request.

In our view, option B will cause issues such as ping-pong issue with device possibly responding to a wrong reader. More detailed analysis on the two options may refer to a co-sourcing contribution [7] submitted to this meeting.

Proposal 6: Support Option A: Ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure.

One implication from Proposal 6 is that a device needs to know whether R2D messages are addressed to itself or not. For Msg2 there is Random ID to identify a device; for R2D Command message, AS ID is used to identify the device. The only message that needs to be considered is the R2D trigger message. We propose that the Transaction ID is included in every R2D trigger message to let the device to associate it with the current procedure.

Proposal 7: Transaction ID is included in every R2D Trigger message to avoid overhearing unintended R2D trigger messages in a multi-reader scenario.

With Option A, it is necessary to design end of procedure, since a device needs to be aware of whether the current procedure is still ongoing or not. Many approaches have been proposed during previous discussions on AS ID validity, such as an explicit indication from reader. AS ID validity issue has close relation with end of procedure indication. Explicit indication for end of procedure is straightforward, but it is preferred to use existing signaling to achieve the same purpose. R2D trigger message and subsequent paging are two candidate R2D messages as end of procedure indication. For this, RAN2 needs to discuss whether a procedure for a device can be finished in one RFID-like slot as started by a paging/R2D trigger message or not.

In the TS38.300 running CR from RAN3 [8], the Inventory procedure and Command procedure are described using basic communication between the gNB and the A-IoT CN node. It also states that the Command Request for a device from the CN to the gNB may happen in parallel of the Inventory procedure involving other A-IoT devices. From RAN2 point of view, there is currently no restriction on the exact timing of the Command procedure after Inventory. However, in order to decide the signaling for the end of procedure, it is necessary to discuss whether to restrict the Command procedure of a specific device to be in the same “slot” of its Inventory procedure. Figure 1 illustrates the overall Command procedure involving A-IoT CN, RAN reader and A-IoT devices. For simplicity, only two devices are shown in the figures assuming that they select two different access occasion sets. Figure 1a and 1b show the two options in the AS procedure respectively:

-	Option 1: all the message transmission for the command procedure targeting for one device will be finished in the same “slot” as the random access procedure, where the “slot” boundary is indicated by paging/R2D trigger message.

-	Option 2: the message transmission for the command procedure targeting for one device can be carried out in a different “slot” from the random access procedure.

a) Option 1

b) Option 2

Figure 1 Overall Command procedure

Proposal 8: For CBRA, RAN2 to discuss the following options on Command procedure:

- Option 1: all the message transmission for the command procedure targeting for one device will be finished in the same “slot” as the random access procedure, where the “slot” boundary is indicated by paging/R2D trigger message.

- Option 2: the message transmission for the command procedure targeting for one device can be carried out in a different “slot” from the random access procedure.

For option 2, the command procedure may happen in any time after the inventory procedure, then the subsequent paging may be used as an end of procedure indication. However, it is not appropriate to use the subsequent paging since the device cannot determine whether this paging is coming from the same reader or a different reader. There is still the issue about overhearing paging messages from other reader while the current procedure is ongoing. Option 1 has no such issue since subsequent R2D trigger message can be used as end of procedure indication. The R2D trigger message includes a Transaction ID (if P7 is agreed), so the device knows it is for the current procedure (coming from the serving reader). For this reason, option 1 is preferred.

Proposal 8bis: RAN2 to agree on option 1 in P8 (all the message transmission for the command procedure targeting for one device will be finished in the same “slot” as the random access procedure, where the “slot” boundary is indicated by paging/R2D trigger message). The next R2D trigger message after Msg1 transmission can be used as the end of procedure indication.

In last meeting, there was a discussion whether Transaction ID is needed for CFRA in Paging message. For CFRA, NACK feedback and re-access is not supported. It is true that Transaction ID is not needed for re-access purpose. However, for the same reason as CBRA procedure, it is also necessary to have an end of procedure indication in multi-reader scenario. For example, if one device is performing a CFRA for reader A and receives a paging from reader B, it will never know whether this new paging is from the same reader to re-page it or it is from a different reader, since it has no status on failure or success.

Based on above analysis, to have a uniform design for CBRA and CFRA, R2D trigger is also used for CFRA and it can be used purely as an end of procedure indication. The reader transmits an R2D trigger message when it determines the CFRA procedure completed, i.e., successful or failed. Like above CBRA related conclusions, the Transaction ID should be included in the R2D trigger message. Therefore, the Transaction ID is needed for CFRA in Paging message.

Proposal 9: Transaction ID is needed for CFRA in Paging message.

Proposal 10: For CFRA, R2D trigger message with Transaction ID is introduced as an end of procedure indication. Reader transmits an R2D trigger message when it determines the CFRA procedure completed, i.e., successful or failed.

- Conclusion

In this contribution, we had some discussions on the Ambient IoT Paging and made the following observation and proposals:

Determination to skip the response

Proposal 1: To reduce the stored information, the A-IoT device releases the stored Transaction ID when the procedure fails.

Proposal 1bis: The A-IoT device will skip subsequent paging with the same Transaction ID as its stored Transaction ID.

Content and format for A-IoT paging

Proposal 2: Paging for CFRA can omit some of the CBRA related fields, including Indication of Paging ID presence, Number of access occasions.

Proposal 2bis: RA type field is present before the CBRA related fields, if present, including Indication of Paging ID presence and Number of Access Occasions.

Proposal 3: RAN2 to discuss the length for Number of Access Occasions field is variable or not. The length needs to be indicated in the A-IoT Paging message if it is variable.

Proposal 4: RAN2 to confirm the current assumption that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.

Proposal 5: For forward compatibility, A-IoT Paging message contains a reserved field to indicate whether there is more information on paging ID length and corresponding paging identifier.

Multi-reader scenario

Proposal 6: Support Option A: Ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure.

Proposal 7: Transaction ID is included in every R2D Trigger message to avoid overhearing unintended R2D trigger messages in a multi-reader scenario.

Proposal 8: For CBRA, RAN2 to discuss the following options on Command procedure:

- Option 1: all the message transmission for the command procedure targeting for one device will be finished in the same “slot” as the random access procedure, where the “slot” boundary is indicated by paging/R2D trigger message.

- Option 2: the message transmission for the command procedure targeting for one device can be carried out in a different “slot” from the random access procedure.

Proposal 8bis: RAN2 to agree on option 1 in P8 (all the message transmission for the command procedure targeting for one device will be finished in the same “slot” as the random access procedure, where the “slot” boundary is indicated by paging/R2D trigger message). The next R2D trigger message after Msg1 transmission can be used as the end of procedure indication.

Proposal 9: Transaction ID is needed for CFRA in Paging message.

Proposal 10: For CFRA, R2D trigger message with Transaction ID is introduced as an end of procedure indication. Reader transmits an R2D trigger message when it determines the CFRA procedure completed, i.e., successful or failed.

# Reference

- RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#107, March 2025

- RAN2 Chair notes, RAN2 Chair (InterDigital), RAN2#129

- RAN2 Chair notes, RAN2 Chair (InterDigital), RAN2#129bis

- 3GPP TR38.769, “Study on solutions for ambient IoT (Internet of Things)”

- R2-2503997, “A-IoT MAC running CR”, RAN2#130, May 2025

- R2-2502211, “Email discussion report: [POST129][035] [AIoT] Paging”, RAN2#129bis, April 2025

- R2-2504112, “Details of option A for overall A-IoT MAC procedure”, RAN2#130, May 2025

- R3-252493, “(BL CR to 38.300) Introduction of Ambient IoT”, RAN3#127bis, April 2025