3GPP TSG-RAN WG2 Meeting #130	R2-2503553

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.4

Source: 	Fujitsu

Title: 	Adaptation of common signal or channel

Document for:	Discussion and decision

Introduction

Rel-19 NES WID was updated and approved in RAN#105 [1]. The WID describes the following objective on the adaptation of common signal/channel transmissions.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we would like to discuss and provide our views on the adaptation of common signal/channel transmissions.

Discussion

Adaptation of SSB in time domain

The following agreements about SSB adaptation were made in RAN2 #129bis meeting [2].

Agreements on RACH adaptation@RAN2#129bis 4. Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. 5. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 6. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

For SSB adaptation, the remaining issue is whether SSB adaptation has impacts on RLM and BFD/BFR. According to the current specification, RLM is performed in PCell rather than SCell, and BFD reference resource is restricted to 1-port CSI-RS. Thus, SSB adaptation in SCell has no impact on RLM and BFD. For BFR, an SSB resource identified by ssb-Index can be configured as an BFR reference resource. However, it has been agreed the time domain positions of SSBs within a burst remain unchanged during SSB adaptation, ensuring that ssb-Index remains consistent before and after SSB adaptation. Therefore, SSB adaptation has no impacts on BFR.

Proposal 1: RAN2 to confirm that SSB adaptation has no impact on RLM and BFD/BFR

Adaptation of PRACH in time domain

The following agreements about PRACH adaptation were made in RAN2 #129 meeting [3].

Agreements on RACH adaptation@RAN2#129 1. Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed. 2a. RAN2 starts CBRA for RACH adaptation. 2b. RAN2 starts 4-step RACH adaptation. 3. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. 4. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. 5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s). 5b. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaptation in time domain. 6. Will follow legacy mechanism regarding how to select RACH resource.

The first issue to be discussed is whether to support 2-step RA for PRACH adaptation. It should be noted that 4-step and 2-step are configured by separate parameters in current specification. Therefore, additional PRACH resource needs separate configurations for 4-step and 2-step RA, respectively. Furthermore, PUSCH configuration for 2-step RA, as well as the fallback procedure from 2-step RA to 4-step RA require further discussion. Given the time constraints and specification impacts, it is proposed that PRACH adaptation initially supports 4-step RA in Rel-19, while the support for 2-step RA may be deferred to future release.

Proposal 2: Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19.

The second issue to be discussed is Random Access (RA) resource selection. The existing RA procedure is illustrated in Figure 1. First, the UE selects the RA resource set based on RA resource configuration and feature availability determination. Then, the UE chooses an SSB based on the SS-RSRP, followed by randomly selection of a preamble associated with the selected SSB and selected RA preamble group (i.e., group A or group B). After that, the UE determines the next available PRACH occasion (RO) corresponding to the selected SSB.

Figure 1. Illustration of legacy RA resource selection procedure

Although RAN2 agreed to follow legacy mechanism for RA resource selection, it remains unspecified whether legacy and additional PRACH resources are the same RA resource set.

In RAN1 #120bis meeting, it was agreed to support configuration of additional PRACH resources within RACH-ConfigCommon in SIB1, which is also used to configure the legacy PRACH resources. For example, within RACH-ConfigCommon, RACH-ConfigGeneric is configured for legacy PRACH resources, and a newly defined parameter, like RACH-ConfigGeneric-r19, is configured for additional PRACH resources or RACH-ConfigGeneric can be enhanced to indicate additional PRACH resources. Given this shared configuration framework, we understand that additional PRACH resources can be configured in the same RA resource set with legacy PRACH resources.

Proposal 3: RAN2 to confirm that additional PRACH resources, if configured and activated, should be in the same RA resource set with legacy PRACH resources.

In addition, RAN1 has agreed to have separate configurations for the number of SSBs per RO and CB-PreamblesPerSSB for additional PRACH resources and legacy PRACH resources. Note that these two parameters are configured in the same RRC field ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

Agreement@RAN1#120 - Separate configuration of number of SSB per RO is supported Agreement@RAN1#120bis Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB

Thus, it is possible that for the same SSB, the preambles corresponding to additional PRACH resources differ from that corresponding to legacy PRACH resources. An example of such preamble assignment is shown in Figure 2. This example assumes the following configurations: for additional PRACH resource configuration, the number of SSB per RO is 1 and CB-PreamblesPerSSB is n8, while for legacy PRACH resource configuration, the number of SSB per RO is 2 and CB-PreamblesPerSSB is n4. The example also assumes that the total number of preambles used for CBRA and CFRA is 64 for both configurations, and there are two transmitted SSBs, SSB#0 and SSB#1. As illustrated in this figure, for the SSB#1, additional PRACH configuration assigns preamble #0~#7 while legacy PRACH configuration assigns preamble #32~#35.

Figure 2. An example of preamble assignment

Observation 1. With separate configuration of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, it is possible that for the same selected SSB, the preambles assigned by additional PRACH resource configuration differ from those assigned by legacy PRACH resource configuration.

However, the current RA resource selection mechanism does not account for the above situation. Referring to the example shown in Figure 2, if a UE selects SSB#1 and performs RA resource selection based on this SSB, it is possible that the UE selects preamble #0 and then legacy RO#0 as the next available RO. Consequently, the NW mistakenly identifies the SSB selected by the UE as SSB#0.

Proposal 4: RAN2 to discuss how to handle the RA resource selection when there are different preamble assignments for additional PRACH resource configuration and legacy PRACH resource configuration.

Furthermore, in the existing RA procedure, when the UE fails to receive an RAR, it will perform preamble retransmission until either successfully receiving an RAR or reaching the maximum configured number of preamble attempts (preambleTransMax), at which point the RA procedure fails.

Regarding RA resource selection, if legacy and additional PRACH resources are configured within the same RA resource set, the UE would randomly select RA resources from this resource set. However, since the additional PRACH resources may be activated or deactivated, we propose that the UE should first verify the availability of these additional PRACH resources before performing RA resource selection. This ensures the UE only attempts to access currently active PRACH resources during RA procedure.

Proposal 5: In the RA procedure, the UE should verify the availability of the additional PRACH resources at the beginning of RA resource selection.

Conclusion

In this contribution, we have some discussions about the adaptation of common signals or channels, and the following proposals are made:

Proposal 1: RAN2 to confirm that SSB adaptation has no impact on RLM and BFD/BFR.

Proposal 2: Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19.

Proposal 3: RAN2 to confirm that additional PRACH resources, if configured and activated, should be in the same RA resource set with legacy PRACH resources.

Observation 1. With separate configuration of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, it is possible that for the same selected SSB, the preambles assigned by additional PRACH resource configuration differ from those assigned by legacy PRACH resource configuration.

Proposal 4: RAN2 to discuss how to handle the RA resource selection when there are different preamble assignments for additional PRACH resource configuration and legacy PRACH resource configuration.

Proposal 5: In the RA procedure, the UE should verify the availability of the additional PRACH resources at the beginning of RA resource selection.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson, RAN#105 meeting.

- RAN2#129bis NES session chair's note

- RAN2#129 NES session chair's note