3GPP RAN WG2 Meeting #130	R2-2503561

Malta, Malta May 19th – 23rd, 2025

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Remaining issues on Functionality based LCM for BM

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #129bis meeting, RAN2 reached the following agreements.

- Agreements on Option B RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - - To take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior Agreement on Applicability reporting 1 Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec 2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. 3 No prohibit timer is introduced Agreement on UE data collection configuration 1 The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. 2 Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

This contribution aims to address issued regarding applicable functionality reporting and data collection for UE sided training.

2. Discussion

Remaining Open issues

Open issue RRC-1: Cause of inapplicability

Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec. This issue refers to the RAN2#129bis agreement: Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec. The issue is captured as an editor’s note in the running CR, clause 5.3.5.3. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The core purpose of inapplicability reporting is to allow the network to understand whether a previously applicable functionality or configuration can no longer be used. However, it is important to distinguish between cases where this cause information is truly helpful. Two main cases can be identified:

- Case 1: Temporary inapplicable functionality: The UE temporarily cannot activate relevant functionality (e.g., due to temporal condition/limitation), but the configuration itself remains valid.

- Case 2: Model is not available: This case indicates that the UE does not have an available model. The cause of inapplicability is directly related to the network’s need to release the associased configuration.

In Case 1, since the network is expected to maintain the configuration regardless of the temporary inapplicability, including a cause value provides limited benefit. In case 2, by contrast, a simple and clear cause value (e.g., release) can assist the network in deciding whether to release the associated configuration.

Observation 1. [Open issue RRC-1] It is important to distiguish between cases where the cause information is truly helpful.

- Case 1: Temporary inapplicable functionality : since the network is expected to maintain the configuration regardless of the temporary inapplicability, including a cause value provides limited benefit

- Case 2: Model is not available: A simple and clear cause value (e.g., release) can assist the network in deciding whether to release the associated configuration.

Therefore, the UE includes a cause value only when the configuration needs to be released, such as when a required model is no longer available. This cause value does not necessarily reflect the detailed reason (e.g., model availability status), but rather indicates the UE’s intended action, such as requesting the release of the configuration.

Proposal 1. [Open issue RRC-1] UE includes a cause value (e.g., release) only when requesting configuration release, e.g., due to model unavailability

When a configuration remains valid and the network’s goal is merely to activate available functionalities, explicitly reporting non-applicable functionalities without a clear cause provides little to no benefit. In such scenarios, the network can already infer what is inapplicable by omission, based solely on the applicable functionalities reported. Reporting both applicable and inapplicable functionalities simultaneously not only adds redundant information but also increases signaling overhead and implementation complexity.

Therefore, unless there is a specific reason, such as requesting configuration release, reporting applicable functionalities alone is sufficient.

Observation 2. [Open issue RRC-1] When there is no reason for inapplicability, reporting both applicable and non-applicable functionalities may be redundant and introduce signalling overhead.

Proposal 2. [Open issue RRC-1] Inapplicable functionalities should only be reported only when specific cause (e.g., release) exist

Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI

Issue description: It is not yet clarified what the content (if any) of the UAI configuration should be, to enable the UE to report applicability in UAI, e.g. applicability updates/changes as agreed for option A. For instance, this configuration could be just a flag or could contain further information. The issue is captured as an editor’s note in the running CR, clause 6.3.4. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

There is no fundamental difference between initial applicability reporting and update reporting in terms of the target configuration. For example, for Option A, the CSI-ReportConfig that includes inference-related set A/set B settings will already be the subject of the initial applicability report, and those same configurations will also be the target of updates. Since UAI is merely the signaling mechanism used for delivering updates, a simple flag to permit update reporting is sufficient.

Proposal 3. [Open issue RRC-2] A simple indication in otherConfig is sufficient to enable applicability reporting via UAI; no additional configuration content is needed

Open issue RRC-3: UE data collection request

Issue description: For UE-side data collection, the details of UAI signaling for the UE start/stop request and especially the following aspects are not yet clarified: - how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW - where/what the NW provides as candidate configurations - what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.). This issue refers to the RAN2#129bis agreements: The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request. The issue is captured as an editor’s notes in the running CR, clause 5.7.4.3, 6.2.2, and 6.3.4. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

For UE data collection, the UE may initiate requests for RS configuration in two possible ways:

- RS request based on configured CSI report configuration used for data collection

- RS request based on configured CSI related configuration

A similar discussion is observed in the context of applicability report. In ongoing RAN1 and RAN2 discussions, two options are being considered for how a UE may inform the network of preferred CSI-related configuration.

Observation 3. [Open issue RRC-3] To request the UE preferred RS configuration, a similar discussion is ongoing in the context of the applicability report.

Therefore, it is worth considering whether the mechanism used for applicability reporting could also be reused for UE-initiated RS configuration requests for data collection purposes.

In Option A, where the UE informs the network of applicable configurations among already configured CSI report configurations, the same mechanism could serve as a simple approach to indicate a preferred RS configuration. In contrast, Option B, which allows the UE to request a specific RS configuration, would likely require further input from RAN1 regarding how such a request can be supported within the CSI framework.

Following the RAN1#121 meeting, RAN1 has initiated further offline discussions [5], leading to the consideration of two additional detailed options under Option B.

- Option1, which involves providing information related to Set A and Set B for inference, specifically the associated IDs for Set A and Set B (AssociatedIdforSetA-r19 / AssociatedIdforSetB-r19) and/or size of Set A and Set B (as a new RRC parameter)

- Option2, which involves providing information related to Set A and Set B for inference, specifically the associated IDs for Set A and Set B (AssociatedIdforSetA-r19 / AssociatedIdforSetB-r19) and/or the resources for Set A and Set B (resourceForSetA-r19 / resourceForSetB-r19)

For training data collection, collecting measurement results for both Set A and Set B is essential to gather input and ground-truth data. Therefore, while the detailed signaling for Option B depends on further RAN1 input, the Option B mechanism could be a feasible candidate to support UE-initiated requests for preferred CSI-related configurations, including RS resources for data collection.

Observation 4. [Open issue RRC-3] In Option A, the UE informs the network of applicable configurations among already configured CSI report configurations. The same mechanism could serve as a simple approach to indicate a preferred RS configuration.

Observation 5. [Open issue RRC-3] For training data collection, collecting measurement results for both Set A and Set B is essential to gather input and ground-truth data. In Option B could be a feasible candidate to support UE-initiated requests for preferred CSI-related configurations, including RS resources for data collection

Issue. Where/what the NW provides as candidate configurations & What the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.)

In addition to the flag indicating that UE requests are allowed, the network may also provide RS configuration-related settings. The method and parameters by which the network provides candidate configurations for data collection depend on the selected option, as described below:

- For Option A like mechanism, NW will provide the full RS configuration used for data collection through CSI-ReportConfig, which may be identical to that NW provides full inference configuration through CSI-ReportConfig. Upon receiving a full RS configuration used for data collection, the UE may inform the list of preferred RS configuration among which NW already configured via RRCReconfigurationComplete message or UAI.

- For Option B like mechanism, the NW will provide the parameters related to the data collection through DataCollectionPreferenceConfig-r19 in OtherConfig. Based on the parameter information received from the NW, the UE requests a specific RS configuration that it prefers, and the NW configures CSI-ReportConfig to provide the specific RS configuration requested by the UE.

Issue. How to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW

To indicate a preferred RS configuration in UAI, the following methods can be adopted for each option:

- For Option A like mechanism, the UE may inform the list of preferred RS configurations among which NW already configured CSI report configuration used for data collection. For the list of preferred RS configurations, an IE like the ApplicabilityReportList-r19, which is defined for applicability report, may be defined and used.

- For Option B like mechanism, the UE may inform the specific preferred RS configuration (i.e., ID of DataCollectionPreferredConfiguration) based on the further input for applicability report from RAN1 regarding the CSI framework.

Proposal 4. [Open issue RRC-3] For the UE request configuration, the applicability report mechanism can be re-used

- Option A-like mechanism: The network provides a full RS configuration for data collection via CSI-ReportConfig. The UE indicates its preferred RS configuration(s) from among the already configured CSI report configurations.

- Option B-like mechanism: The network provides data collection-related parameters via OtherConfig. The UE explicitly indicates a preferred RS configuration. Further RAN1 input is needed to support Option B.

Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

Issue description: RAN2#129 agreed that for periodic CSI reporting, the UE autonomously activates the applicable functionality upon sending the applicability report via RRCReconfigurationComplete in Step 4: If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).. Further, RAN2#129bis agreed: 2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.. Thus, the UE will maintain a periodic CSI report configuration that is inapplicable, but it is unclear whether/how a periodic CSI report is activated upon a change of applicability, from inapplicable to applicable, since there is no legacy procedure to activate such a configuration (other than the initial sending of the RRC configuration). One way to solve this situation is by network implementation, namely the network can de-configure a periodic CSI report configuration after the UE initially reports it as inapplicable via RRCReconfigurationComplete. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

If the network de-configures a periodic CSI report configuration after the UE initially reports it as inapplicable via RRCReconfigurationComplete, this can prevent future activation of that functionality when it later becomes applicable. Therefore, the network may choose to retain configurations related to inapplicable functionalities, so that the functionality can be re-activated when the UE later reports it as applicable.

Observation 6. [Open issue RRC-4] If the network de-configures a periodic CSI report configuration after the UE initially reports it as inapplicable, this can prevent future activation of that functionality when it later becomes applicable. Therefore, the network may choose to retain configurations related to inapplicable functionalities, so that the functionality can be re-activated when the UE later reports it as applicable

When the UE reports updated applicability via UAI, the timing for activating the corresponding functionality (e.g., inference-based reporting) becomes unclear. RAN2 agreed that, in the case of initial configuration using RRC signaling, the UE autonomously activates applicable functionality upon sending the applicability report in RRCReconfigurationComplete. In this case, the network is aware that the applicability report has been delivered as the UE always responds to RRCReconfiguration with RRCReconfigurationComplete.

However, in the case of UAI-based reporting, the network may not be aware whether the UAI message was delivered. If the UE autonomously activates functionality upon sending applicability report via UAI, it can result in a synchronization mismatch between the UE and the network. To avoid such inconsistencies, a different activation mechanism should be considered for UAI-based reporting. For example, the UE may activate the functionality only after ensuring the UAI was successfully transmitted, such as upon receiving an acknowledgment from the network.

Proposal 5. [Open issue RRC-4] UE activates a functionality updated as applicable via UAI after confirming that the UAI was successfully transmitted, i.e., upon receiving an acknowledgment from the network

Open Issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

Issue description: It is ambiguous what the UE should report to the NW when its periodic beam prediction configuration becomes inapplicable and whether the model would output anything at all when if, e.g., the input distribution no longer matches what is expected. We need to determine whether the configuration should cease reporting, or send the input (measurements) to the NW once it becomes inapplicable, or continue sending (inaccurate) predictions. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

As per the RAN2#128 agreement, the UE does not autonomously deactivate a functionality when it becomes inapplicable. Instead, the network is responsible for deactivating the active functionality based on the applicability report received from the UE.

If the UE continues to transmit prediction-based reports despite the functionality becoming inapplicable, it may result in incorrect beam management decisions from the network. To prevent this, it is proposed that the UE maintains the active configuration but switches to legacy measurement-based reporting.

Specifically, instead of reporting results for resource set A inferred from resource set B (i.e., inference-based reporting), the UE should fall back to legacy reporting based directly on the measurement of resource set A itself. This ensures the network receives valid and accurate measurement results even when the prediction functionality is no longer applicable.

Proposal 6. [Open issue RRC-5] When a functionality becomes inapplicable, the UE reports measurements based on the actual RS resources in Resource Set A, rather than using inference results derived from Resource Set B

Open issue RRC-7: Applicability reporting for option B in RRCReconfigurationComplete

Issue description: It is FFS whether the applicability report for option B (sets of inference related parameters) can be included in RRCReconfigurationComplete (or if it can only be included in UAI). This issue refers to the following RAN2#129bis agreement, based on which the overall design for option B requires further input from RAN1: Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior. Given the agreement above, it is the Rapporteur’s understanding that the procedures for option B may depend on the RAN1 list of parameters for option B, since based on this list RAN2 will confirm whether the configuration for option B is sent in otherConfig or not. For instance, if the option B configuration is indeed sent in otherConfig, it may be more suitable to send the applicability report in UAI. However, if the parameters for option B are sent within the CSI measurement framework, RRCReconfigurationComplete may be more suitable to send the applicability report. Thus, if further procedures for option B are discussed, it is suggested to taken into account whether these procedures may depend on the list of inference related parameters from RAN1. The issue is captured as an editor’s note in the running CR, clause 5.3.5.3. Proposed resolution: Companies may provide contributions to the following meeting to resolve the issue..

For Option A, the initial applicability report is conveyed via RRCReconfigurationComplete message, and subsequent updates are sent using UAI message. This approach was chosen to support fast initial reporting without requiring additional signaling. However, the signaling method for Option B remains unclear. Although Option B differs in how configurations are handled, it similarly requires the UE to report applicability to the network. Therefore, to minimize configuration delay, Option B would also benefit from using the same initial reporting approach as Option A, i.e., through RRCReconfigurationComplete.

Therefore, adopting a unified mechanism, where the initial applicability report for Option B is also conveyed via RRCReconfigurationComplete, and subsequent updates are handled via UAI, can improve consistency and reduce initial configuration latency.

Proposal: [Open issue RRC-7] For Option B, use RRCReconfigurationComplete for initial reporting and UAI for updates, as in Option A.

Open issue RRC-8: Coexistence between option A and option B

Issue description: Considering that both Option A and B are to be specified, we need to discuss how the 2 options co-exist, whether there is any interaction between them etc. For instance, it is not clear whether both option A and option B can be configured or not. This can affect signaling design of ApplicabilityReportList. If further procedures for option B are discussed, it is suggested to taken into account whether these procedures may depend on the list of inference related parameters from RAN1. Proposed resolution: Companies may provide contributions to the following meeting to resolve the issue.

From a signaling perspective, there is no inherent issue with supporting both options concurrently. According to the current running CR, applicability for Option A is indicated using the CSI-ReportConfigId. If Option B is introduced, a corresponding IE to indicate its applicability could simply be added to the Applicability Report.

However, the motivation for simultaneous use of both options within a single (sub) use case remains unclear. Option A and Option B represent different trade-offs between signalling simplicity and configuration efficiency. In scenarios where Option A is already configured, the necessity of Option B becomes questionable. Therefore, for at least one (sub) use case, using only a single option may be more efficient and reasonable.

Proposal. [Open issue RRC-8] For at least one (sub) use case, only a single option, i.e., option A or option B, is used

Open issues requiring RAN1 inputs

Open issue RRC-9: Definition of ‘applicable AI/ML functionality’

Issue description: How to update the definition of ‘applicable AI/ML functionality’ in clause 3.1, e.g. replace 'functionality', and align it with RAN1 specs and with TS 38.300. The issue is captured as an editor’s note in the running CR, in clause 3.1. The running CR also contains an example initial definition. Proposed resolution: Suggest to wait for RAN1 progress.

RAN1 is not expected to specify the term ‘functionality’ in their specifications. In practice, the applicability report only provides information on applicable CSI report configurations (Option A) or related CSI settings(Option B), rather than reporting on abstract AI/ML “functionalities.” Therefore, the notion of “applicable functionality” is unnecessary and may even be misleading in the context of the current specification.

Therefore, the definition of ‘applicable AI/ML functionality’ from clause 3.1 needs to be removed, as the term is not used in a meaningful or consistent way and is not aligned with RAN1 specifications.

Proposal. [Open issue RRC-9] Remove the definition of ‘applicable AI/ML functionality’ from clause 3.1, as the term is not used in a meaningful or consistent way and is not aligned with RAN1 specifications.

Open issue RRC-11: How to configure RS configuration for UE sided data collection within CSI-ReportConfig

Issue description: According to RAN1 agreements, a CSI-ReportConfig without an actual CSI report can be configured for UE data collection purposes. Since the reportConfigType field is mandatory in CSI-ReportConfig, further discussion is needed on how to handle this situation. For example, instructing the UE to ignore the legacy reportConfigType could be considered. Proposed resolution: Suggest to wait for RAN1 to provide the list of parameters for AI/ML beam management.

For CSI-ReportConfig without CSI reporting, a way to prevent reporting is needed. To do so, a new reportConfigType-r19 with “none” can be introduced to ignore the legacy reportConfigType without suffix (mandatory config), as in Appendix 4.1. In this case, UE can proceed with logging, ignoring the related report resource settings (e.g. reportSlotConfig, pucch-CSI-ResourceList).

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

…skipped…

[[

reportConfigType-v19                CHOICE {

none                                    NULL,

...,

}

]]

Proposal 17. [Open issue RRC-11] To introduce new reportConfigType-r19 with “none” in the CSI-ReportConfig to prevent CSI reporting

3. Conclusion

Remaining Open issues

Open issue RRC-1: Cause of inapplicability

Observation 1. [Open issue RRC-1] It is important to distiguish between cases where the cause information is truly helpful.

- Case 1: Temporary inapplicable functionality : since the network is expected to maintain the configuration regardless of the temporary inapplicability, including a cause value provides limited benefit

- Case 2: Model is not available: A simple and clear cause value (e.g., release) can assist the network in deciding whether to release the associated configuration.

Proposal 1. [Open issue RRC-1] UE includes a cause value (e.g., release) only when requesting configuration release, e.g., due to model unavailability

Observation 2. [Open issue RRC-1] When there is no reason for inapplicability, reporting both applicable and non-applicable functionalities may be redundant and introduce signalling overhead.

Proposal 2. [Open issue RRC-1] Inapplicable functionalities should only be reported only when specific cause (e.g., release) exist

Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI

Proposal 3. [Open issue RRC-2] A simple indication in otherConfig is sufficient to enable applicability reporting via UAI; no additional configuration content is needed

Open issue RRC-3: UE data collection request

Observation 3. [Open issue RRC-3] To request the UE preferred RS configuration, a similar discussion is ongoing in the context of the applicability report.

Observation 4. [Open issue RRC-3] In Option A, the UE informs the network of applicable configurations among already configured CSI report configurations. The same mechanism could serve as a simple approach to indicate a preferred RS configuration.

Observation 5. [Open issue RRC-3] For training data collection, collecting measurement results for both Set A and Set B is essential to gather input and ground-truth data. In Option B could be a feasible candidate to support UE-initiated requests for preferred CSI-related configurations, including RS resources for data collection

Proposal 4. [Open issue RRC-3] For the UE request configuration, the applicability report mechanism can be re-used

- Option A-like mechanism: The network provides a full RS configuration for data collection via CSI-ReportConfig. The UE indicates its preferred RS configuration(s) from among the already configured CSI report configurations.

- Option B-like mechanism: The network provides data collection-related parameters via OtherConfig. The UE explicitly indicates a preferred RS configuration. Further RAN1 input is needed to support Option B.

Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

Observation 6. [Open issue RRC-4] If the network de-configures a periodic CSI report configuration after the UE initially reports it as inapplicable, this can prevent future activation of that functionality when it later becomes applicable. Therefore, the network may choose to retain configurations related to inapplicable functionalities, so that the functionality can be re-activated when the UE later reports it as applicable

Proposal 5. [Open issue RRC-4] UE activates a functionality updated as applicable via UAI after confirming that the UAI was successfully transmitted, i.e., upon receiving an acknowledgment from the network

Open Issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

Proposal 6. [Open issue RRC-5] When a functionality becomes inapplicable, the UE reports measurements based on the actual RS resources in Resource Set A, rather than using inference results derived from Resource Set B

Open issue RRC-7: Applicability reporting for option B in RRCReconfigurationComplete

Proposal: [Open issue RRC-7] For Option B, use RRCReconfigurationComplete for initial reporting and UAI for updates, as in Option A.

Open issue RRC-8: Coexistence between option A and option B

Proposal. [Open issue RRC-8] For at least one (sub) use case, only a single option, i.e., option A or option B, is used

Open issues requiring RAN1 inputs

Open issue RRC-9: Definition of ‘applicable AI/ML functionality’

Proposal. [Open issue RRC-9] Remove the definition of ‘applicable AI/ML functionality’ from clause 3.1, as the term is not used in a meaningful or consistent way and is not aligned with RAN1 specifications.

Open issue RRC-11: How to configure RS configuration for UE sided data collection within CSI-ReportConfig

Proposal 17. [Open issue RRC-11] To introduce new reportConfigType-r19 with “none” in the CSI-ReportConfig to prevent CSI reporting

4. Reference

[1] R2-2504350 - RRC open issues AIML PHY