3GPP TSG-RAN WG2 Meeting #130	 R2- 2503568

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	8.4.2

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129b meeting, the following agreements are made:

LP-WUS is supported with eDRX, FFS on exact impact if any

Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X.

X is based on 32 subgrouping number. Details can be discussed in the running CR.

Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping.

Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS:

        - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1.

Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR.

All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations.

Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation.

Use measured value for all LR measurement based entry/exit condition evaluation.

In this contribution, we further discuss the remaining issue for subgrouping, configuration aspect and mobility.

- 2 Discussion

- 2.1 LP-WUS subgrouping

Based on RAN2 agreement, now we have the following LP-WUS UE-ID based subgrouping formula:

SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID),

where:

N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1

Ns: number of paging occasions for a PF

Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1

UE_ID: 5G-S-TMSI mod X, X is FFS

lp-SubgroupsNumForUEID and lp-SubgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively

We almost finish UE-ID based subgrouping except the design on value of X, here we discuss how to design value of X.

First of all, let’s take a look at the principle of value X by taking paging calculation as an example:

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

- 5G-S-TMSI mod 4096

else:

- 5G-S-TMSI mod 1024

It can be seen that for normal DRX scenario, the UE_ID could be [0 ~ 1023], which means 10 bits are needed to represent UE_ID. The reason why value X is 1024 is to make sure UE-ID in the formula can be fully used, in other word, we need to make sure that all UEs (5G-S-TMSI) with different UE-ID can be allocated into every PF/PO/subgroup, i.e., no PF/PO/subgroup remains unused. With this understanding, the value of UE-ID should not be less than the max [N * Ns] – 1, for this case, i.e., 256 * 4 – 1 = 1023 (why minus 1 is due to modulo operation, start with 0). While for eDRX case, since the paging cycle can be extended from 256rf to 1024rf, the max N (i.e., oneT) should be 1024, therefore, X should be [1024 * 4] = 4096.

Now back to LP-WUS discussion. Same principle is that to make sure UE-ID for LP-WUS can be fully used, UE-ID should not be less than [N*Ns*Np* lp-SubgroupsNumForUEID]. Otherwise, some subgroup ID (e.g., the last few subgroup ID) will never be used or assigned to UEs. Keep this in mind, we further discuss different scenarios including eDRX case and PEI case.

Based on RAN2 agreement, eDRX is supported for LP-WUS and value X calculation is based on lp-SubgroupsNumForUEID = 32, therefore, we can have the following scenarios:

N should be based on 1024 if eDRX is applied while N should be based on 256 if eDRX is not applied;

Np should be 8 if PEI is configured and UE supports while Np should be 1 otherwise.

Based on this understanding, we can have the following table:

Table-1: value of X considering different scenarios

| UE-ID with LP-WUS subgrouping | If PEI is not configured by NW or not supported by UE | If PEI is configured by NW and supported by UE |
| --- | --- | --- |
| If eDRX is not used | X = 32768 (256*4*32) 15bits | X = 262144 (256*4*8*32) 18bits |
| If eDRX is used | X = 131072 (1024*4*32) 17bits | X = 1048576 (1024*4*8*32) 20bits |

But having said this, one question is that do we really need to differentiate value of X for different cases? In our understanding, there is no problem if UE just considers using the max X for UE-ID calculation, i.e., only specifying [UE_ID: 5G-S-TMSI mod X, where X is 1048576 (20 bits)] in the spec, UE-ID can still work well. The reason is that as long as there is no PF/PO/subgroup remaining unused, same/similar allocation mechanism is also assumed based on the mechanism of formula and modulo operation. For example, for non-eDRX case, no matter whether the UE is supporting PEI or not (based on UE capability), all UEs could just calculate UE-ID based on X = 262144 (18 bits) instead of X = 32768 (15 bits), actually same effects will be assumed. Noted that same principle is also applied for eDRX case. But since we have already differentiated eDRX for PEI, take PEI mechanism is also a way forward.

Proposal-1: RAN2 to make down selection between the following options for value of X:

Option-1: UE_ID: 5G-S-TMSI mod X, where X is 1048576 (20 bits).

Option-2: UE_ID: 5G-S-TMSI mod X, where X is 1048576 (20 bits), if eDRX is applied; otherwise, X is 262144 (18 bits).

- 2.2 Configuration aspect (RRC-12)

For enabler/disabler of LP-WUS monitoring, it means the NW can indicate LP-WUS capable UE whether needs to apply LP-WUS feature or not when the UE is related to RRC_IDLE/INACTIVE. If without introducing this enable/disable indication, normally all LP-WUS supporting UEs need to apply LP-WUS feature (i.e., monitoring LP-WUS when entry condition is fulfilled) as long as the NW is broadcasting LP-WUS configuration in system information, this is not ideal and there are following reasons to support dedicated enable/disable indication:

1. Paging false alarm

When large amount of UEs are monitoring LP-WUS, once one UE is paged, unnecessary LP-WUS reception to other UEs is increased. Therefore if NW can control part of UEs (instead of all LP-WUS supporting UEs) monitor LP-WUS and other part of UEs do not monitor LP-WUS, it can provide flexibility and avoid potential serious paging false alarm issue.

2. Latency requirement

For LP-WUS monitoring UE, the paging delay is increased compared with normal paging monitoring UE when the paging is initiated after LP-WUS monitoring occasion and before paging monitoring occasion. If NW want one specific LP-WUS capable UE not to enter LP-WUS monitoring (e.g., considering latency), the NW can send this dedicated enable/disable indication to this UE, which provides flexibility and avoids latency issue.

3. Power consumption

The purpose of introducing LP-WUS is for power saving, but if the transition between MR and LR is frequent for some specific UEs, the power saving could be overkilled. For example, if the paging probability is relatively high, the UE better not to monitor LP-WUS even though this UE supports LP-WUS feature, otherwise, every time when this UE is monitoring LP-WUS, it will wake up its MR soon, which cannot be acceptable. For such UEs, it could just directly monitor PEI/PO. Therefore, NW can, based on implementation (e.g., information from paging probability), to determine whether such kind of UEs need to apply LP-WUS or not.

Some companies could be concerning about mobility issue, in our understanding, the UE who receives dedicated enable indication in RRCRelease can still monitor LP-WUS in different cell when the UE reselects to neighbour LP-WUS cell, i.e., supporting mobility case. As long as the LP-WUS is enabled by RRCRelease, the UE follows the baseline behaviour, i.e., the UE is supporting LP-WUS and NW is providing LP-WUS configuration in system information, then the UE with enable indication could just use LP-WUS among cells. From RAN2 point view, there is no impact, the only thing needs to do is RAN3 need to consider that for CN paging, NG-RAN node need to inform CN of the current UE LP-WUS state via Ng interface, e.g., NG-AP UE Context Release Complete message, so that CN can properly initiate NGAP paging message with/without LP-WUS. For RAN paging, no spec impact as the NG-RAN which is aware of the current UE LP-WUS state can send Xn paging message with/without LP-WUS capability, for example, if LP-WUS for one UE is disabled, the anchor gNB will send Xn paging message without LP-WUS capability to neighbour gNB.

Our understanding is that only minor spec impact is assumed in RAN3. Since this is related to RAN3 discussion, we may inform RAN3 of this decision.

Based on the above reasons, we propose:

Proposal-2(RRC-12): support dedicated enable/disable indication in RRCRelease to indicate whether the UE needs to apply LP-WUS feature or not during RRC_IDLE/INACTIVE.

Proposal-3(RRC-12): the UE who receives dedicated enable indication in RRCRelease still can apply LP-WUS feature in different LP-WUS supporting cell, i.e., mobility case is supported.

One more issue is that for OFDM-based LR, it has been agreed that such UE may perform LR measurements based on either SSB or LP-SS. Since the LR measurement results may be used for evaluating entry/exit condition of monitoring, relaxation, offloading, which reference signalling is measured should be clear as it may have impact on evaluation precision and threshold selection. In our understanding, there could be two alternatives, one is under network control, i.e., NW to indicate which RS should be measured for OFDM-based LR, so that NW flexibility is provided. Another alternative is that SSB is always used for condition evaluation if the SSB-based condition is available, as SSB measurement is more reliable and robust compared with LP-SS.

Proposal-4: down-select between the following solutions for OFDM-based LR which is capable of measuring both SSB and LP-SS:

Solution-1: introduce one indication (e.g., RS-Type) in LP-WUS configuration to indicate which reference signal (SSB or LP-SS) is used for condition evaluation by the UE.

Solution-2: SSB is always used for condition evaluation if the SSB-based condition is available.

- 2.3 Mobility

Regarding mobility (i.e., cell reselection), since measurement results of both serving cell and neighbour cell can still be derived during LP-WUS monitoring, it is expected that cell reselection can be supported for a LP-WUS capable UE during LP-WUS monitoring. And one of the reasons why mobility should be supported is that we are expecting the UE could stay in LP-WUS monitoring as much as possible as long as there is no paging/data/signal arrival, which is beneficial for UE power saving.

Fig.1 Cell reselection for LP-WUS monitoring UE

One example for intra frequency is that, in legacy the UE performs cell reselection based on the best ranked cell. However if the best ranked cell is not supporting LP-WUS meanwhile some other cells supporting LP-WUS feature are also good enough for camping (e.g., Cell-A in the example), by legacy way, the LP-WUS capable UE needs to exist from LPWUS monitoring and camps on the best ranked cell (e.g, Cell B), which is not friendly for power saving. Therefore some solutions could be considered to let UE perform cell reselection to select one cell which supports LP-WUS feature. For example, neighbour cell LP-WUS supporting information can be provided by serving cell, then the UE can, based on this information, optimize its cell reselection decision.

Proposal-5: RAN2 is suggested to consider optimization on cell reselection, e.g., LP-WUS supporting information of neighbour cell is provided in serving cell to optimize cell reselection.

- 3 Conclusions

Proposal-1: RAN2 to make down selection between the following options for value of X:

Option-1: UE_ID: 5G-S-TMSI mod X, where X is 1048576 (20 bits).

Option-2: UE_ID: 5G-S-TMSI mod X, where X is 1048576 (20 bits), if eDRX is applied; otherwise, X is 262144 (18 bits).

Proposal-2(RRC-12): support dedicated enable/disable indication in RRCRelease to indicate whether the UE needs to apply LP-WUS feature or not during RRC_IDLE/INACTIVE.

Proposal-3(RRC-12): the UE who receives dedicated enable indication in RRCRelease still can apply LP-WUS feature in different LP-WUS supporting cell, i.e., mobility case is supported.

Proposal-4: down-select between the following solutions for OFDM-based LR which is capable of measuring both SSB and LP-SS:

Solution-1: introduce one indication (e.g., RS-Type) in LP-WUS configuration to indicate which reference signal (SSB or LP-SS) is used for condition evaluation by the UE.

Solution-2: SSB is always used for condition evaluation if the SSB-based condition is available.

Proposal-5: RAN2 is suggested to consider optimization on cell reselection, e.g., LP-WUS supporting information of neighbour cell is provided in serving cell to optimize cell reselection.

- 4 References