3GPP TSG-RAN WG2 Meeting #130	 R2- 2503570

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	8.4.4

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129b meeting, the following agreements are made:

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used.

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback.

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously.

If configured, the UE can signal a preferred time offset via UAI signalling.

Ask RAN1 for further information regarding their conclusions.

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

In this contribution, we further discuss remaining issue.

- 2 Discussion

- 2.1 LP-WUS monitoring occasion collision (MAC-2)

Currently we agreed that for LP-WUS option 1-1, DCP is taken as baseline, therefore, the following conditions highlighted below could trigger drx-onDurationTimer:

2>	else if LP-WUS monitoring is configured:

3>	if lpwus_PDCCHMonitoringTimer is not configured (i.e., LP-WUS Option 1-1):

4>	if LP-WUS indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

4>	if all LP-WUS monitoring occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last LP-WUS occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4):

5>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

We can see from running CR that two conditions will trigger drx-onDurationTimer, the first one is straightforward that the UE receives LP-WUS indication, while for the latter one, the condition means when the configured LP-WUS monitoring occasions occurred in active time considering some actions or during measurement gap, window for BFR, the UE can not monitor LP-WUS so that is not aware of whether there is wake-up indication or not, in this regard, the UE could just blindly start drx-onDurationTimer for safety.

When it comes to LP-WUS option 1-2, the running CR is shown as below:

1>	if LP-WUS monitoring is configured and the lpwus_PDCCHMonitoringTimer for this DRX group is configured (i.e., LP-WUS Option 1-2):

2>	if LP-WUS indication is received from lower layer indicated to start lpwus_PDCCHMonitoringTimer, as specified in TS 38.213 [6]:

3> start lpwus_PDCCHMonitoringTimer from the beginning of the subframe indicated from lower layer.

For option 1-2, the LP-WUS cycle is configured independently from C-DRX cycle, and LP-WUS could trigger a new timer after its monitoring occasion with an offset. Here only one condition is specified, i.e., once the UE receives LP-WUS indication, the UE starts the new timer introduced for LP-WUS. In our understanding, same logic as option 1-1 should also be applied. At least for the case, when LP-WUS option 1-2 monitoring occasions occurred in Active Time, the UE does not monitor LP-WUS based on agreement, therefore the UE is not aware of whether there is wake-up indication or not, in this regard, the UE could just start this new timer [lpwus_PDCCHMonitoringTimer] as like DCP mechanism for safety. Below takes Retransmission Timer as an example.

Note that in addition to Active Time collision, there could be other cases, e.g., measurement gap, BWP switching, RAR window monitoring for BFR, since it could have different behavior from DCP mechanism, can be further FFS.

Fig.1 one example for collision case

Proposal-1(MAC-2): for option 1-2, take option 1-1 as baseline, if all LP-WUS monitoring occasion(s) in time domain occurred in Active Time, the UE starts new timer [lpwus_PDCCHMonitoringTimer] from the beginning of the subframe indicated from lower layer. FFS other cases.

- 2.2 CSI reporting for option 1-2

In previous meeting, we agreed that For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. This is referred to DCP mechanism, but it is not complete in our understanding.

Fig.2 UE reports periodic CSI during the configured drx-onDurationTimer

Based on the agreement, it can be seen from Fig.1 that UE can report periodic CSI/L1-RSRP during the configured drx-onDurationTimer even though this timer will never be started. In legacy, why we need this control of CSI reporting is that when the drx-onDurationTimer is not running due to DCP is not detected, the UE still can report periodic CSI/L1-RSRP. In fact, the general principle should be as long as the UE is within the active time duration but not started due to wake-up signalling, CSI reporting can be enabled by NW indication. When it comes to LP-WUS option 1-2, not only the legacy drx-onDurationTimer but also the new introduced active timer triggered by LP-WUS should be considered, i.e., lpwus_PDCCHMonitoringTimer, if the new active timer is not started due to LP-WUS option 1-2 is not detected, the network can configure (maybe reuse the same indication) whether the UE can report periodic CSI/L1-RSRP or not during the new timer duration when this new timer is not running, this is exactly to refer to DCP mechanism. The diagram can be seen from Fig.2.

Note that, in legacy behaviour, CSI reporting is based on CSI-ReportConfig (slot-level) / CSI-ResourceConfig and DRX active time, as long as the UE is in Active Time, CSI/L1-RSRP reporting can be enabled based on CSI configuration.

Fig.3 network can configure whether the UE can report CSI or not during new timer

Proposal-2: for option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the new timer [lpwus_PDCCHMonitoringTimer] when this timer is not running.

- 3 Conclusions

Proposal-1(MAC-2): for option 1-2, take option 1-1 as baseline, if all LP-WUS monitoring occasion(s) in time domain occurred in Active Time, the UE starts new timer [lpwus_PDCCHMonitoringTimer] from the beginning of the subframe indicated from lower layer. FFS other cases.

Proposal-2: for option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the new timer [lpwus_PDCCHMonitoringTimer] when this timer is not running.

- 4 References