3GPP TSG-RAN WG2 Meeting #130	 R2-2503571

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	8.1.3

Source: 	NEC

Title:  	Discussion on NW-side data collection

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129b meeting, the following agreements related to NW-side data collection are made:

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection  (with lower priority)

In this contribution, we further discuss remaining issues.

- 2 Discussion

- 2.1 Logging aspect (RRC-31)

Normally when the UE has collected enough data, the NW could explicitly release the data collection configuration. Besides explicit way, we think some mechanism from logged MDT can be referred, i.e., maximum logging duration, or maximum data size required by NW could be considered to allow UE to release the configuration, in this way, NW-UE signalling interaction could be reduced, i.e., without NW additional release but both NW and UE side are aligned with the configuration. For example, the NW could configure the UE to perform data collection for a specific duration, e.g., 2 hours at night, when the duration is expired, the data collection configuration could be released by UE and NW is aware of this situation. Another example, the NW could configure the UE to perform data collection up to X MB data, and the configuration could be released once the UE has reported data up to X MB data.

Proposal-1(RRC-31): maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already logged data up to max duration/size, the UE releases the configuration.

Furthermore, it is not necessary for UE to always retain logged data in its buffer, the following events could trigger UE to clear buffer or release logged data:

1. Data collection configuration is released;

2. The UE has already reported the logged data.

Proposal-2(RRC-31): the following events could trigger UE to clear its buffer:

1. Data collection configuration is released;

2. The UE has already reported the logged data.

- 2.2 Reporting aspect

Time stamp (RRC-21)

In last RAN2#129b meeting, we discussed whether to include time stamp information to label each logged data for temporary domain prediction. Some companies think there is no need to introduce explicit time stamp information as periodical logging can include this information, even for event-triggered logging, event instance could be differentiated by different report list, therefore NW can be aware of the time stamp information implicitly.

However in our understanding, both explicit and implicit can work but the efficiency of implicit time stamp information is doubted. Even though periodical logging is assumed, there could be some gap between logged data, for example, if one UEInformationResponse message is not sufficient to carry all logged data, then multiple UEInformationResponse is needed, in this case, continuous logged data for same use case can be divided into different reporting message. And also it is not clear that whether the dynamic start/stop of logging (due to e.g., event, HO, low power) could impact on time stamp information. Therefore, for safety or future-proof reason, we suggest to consider absolute/relative time stamp mechanism which is already introduced for MDT to label logged data.

Proposal-3 (RRC-21): for data collection temporal domain case, absolute/relative time stamp mechanism should be introduced.

Buffer threshold

In last RAN2#129b meeting, RAN2 has agreed that the following reason could trigger data availability indication:

o	Full buffer being reached (if configured)

o	Buffer threshold being reached (if configured).

o	Low power (if configured)

For the first one – full buffer, it is clear that as long as the UE reaches its buffer limitation (capability), data availability indication should be initiated.

For the third one – low power, it is up to UE implementation how to detect low power, once the UE internally detects low power issue, data availability indication is initiated.

For second one – a buffer threshold is introduced, once the UE logged data reaches this threshold, the data availability indication should be initiated. One question is about how to define this threshold in RRC ASN.1, based on company understanding, it could be a specific size or percentage. In our understanding, since different UE may have different buffer size capability, percentage is not accurate when NW only want some specific size of logged data. In this sense, threshold should be based on specific size, e.g., KB, MB.

Proposal-4: buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB, MB instead of percentage.

Multiple UEInformationResponse

Now we discuss UE information procedure, in previous meeting, RAN2 agreed that the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. Considering the logged data size could be huge, a further availability indication in UEInformationResponse is promising. But there is still one drawback, in legacy, once receiving UEInformationRequest from NW, the UE shall transmit UEInformationResponse including the recorded information (e.g., logged measurement) requested by NW. For data collection, the UE may have already logged a large amount of training data, one message may not be enough to carry all of training data at one time. But if NW further send the sequent multiple UEInformationRequest messages to UE, signal overhead is increased, in this sense, the UE could send multiple UEInformationResponse to respond the corresponding UEInformationRequest, and each UEInformationResponse can indicate data availability indication and remaining data size. By this way, unnecessary signal overhead can be saved when a large amount of training data need to be reported.

Note that the current segmentation of UL DCCH message is not applied to UEInformationResponse.

Proposal-5: the UE can send multiple UEInformationResponse messages to respond the same one UEInformationRequest message when large amount of training data need to be reported and UEInformationResponse could include the data availability indication and/or the remaining data size.

- 3 Conclusions

Proposal-1(RRC-31): maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already logged data up to max duration/size, the UE releases the configuration.

Proposal-2(RRC-31): the following events could trigger UE to clear its buffer:

1. Data collection configuration is released;

2. The UE has already reported the logged data.

Proposal-3 (RRC-21): for data collection temporal domain case, absolute/relative time stamp mechanism should be introduced.

Proposal-4: buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB, MB instead of percentage.

Proposal-5: the UE can send multiple UEInformationResponse messages to respond the same one UEInformationRequest message when large amount of training data need to be reported and UEInformationResponse could include the data availability indication and/or the remaining data size.

- 4 References