3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503579

St. Julian, Malta, 19th – 23rd May, 2025

Source:	CATT

Title:	Consideration on CSSF optimization

Agenda Item:	8.19

Document for:	Discussion and Decision

Introduction

An LS on CSSF optimization for NR RRM Phase 5 from RAN4 was received in RAN2#129bis meeting [1]. RAN2 discussed the issue online but no conclusion was achieved yet. In this contribution, our consideration with RAN4 progress is provided.

Discussion

It is observed that FR2-1 SSB based RRM requirements allow excessively long delay for UE in L3 measurement, according to the large Rx beam sweeping factor and/or CSSF factor as specified in corresponding requirement tables of TS38.133. Therefore, CSSF optimization is one of the objectives in NR Radio Resource Management (RRM) Phase 5 WI [2] to reduce FR2-1 SSB based L3 measurement delay reduction for connected mode, as shown below.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

According to RAN4 LS [1], in order to reduce FR2-1 SSB based L3 measurement delay for connected mode, UE measurement procedure is enhanced via optimizing CSSF outside gap in CA/DC. The CSSF optimization allows the UE to measure only one serving CC per band if multiple serving CCs are configured for measurement in the same band.

Observation 1: The CSSF optimization introduced in RAN4 allows the UE to measure only one serving CC per band if multiple serving CCs are configured for measurement in the same band, which can reduce FR2-1 SSB based L3 measurement delay for connected mode.

The CSSF optimization that only measures one serving CC per band is solution 1 of CSSF optimization in RAN4. And in RAN4#114bis meeting, only one agreement on solution 1 was achieved as follows [3]:

Agreement: - the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios. - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only - Details of whether/how the CSSF factor is modified can be left to CR discussion.

Network indication of a specific SCC per-band for measurement in RAN4 has following possible options. But no agreement was made.

  - Option 1: dynamic indication based on MAC CE.

  - Option 2: semi-static indication based on RRC signaling.

  - Option 3: leave to RAN2 to decide.

Considering there might be no conclusion in RAN4, RAN2 is supposed to participate in this discussion. Since the motivation of this CSSF optimization is to reduce measurement delay, and it also should be aligned with NW scheduling flexibility (e.g., according to the overload rearrangement of serving carriers), it’s beneficial to apply MAC CE based dynamic indication to configure a specific SCC per-band for measurement, which is also aligned with the SCC activation/deactivation mechanism.

Proposal 1: Introduce a new MAC CE to indicate the specific SCC per-band for CSSF optimization.

Conclusion

According to section 2, we observe and propose:

Observation 1: The CSSF optimization introduced in RAN4 allows the UE to measure only one serving CC per band if multiple serving CCs are configured for measurement in the same band, which can reduce FR2-1 SSB based L3 measurement delay for connected mode.

Proposal 1: Introduce a new MAC CE to indicate the specific SCC per-band for CSSF optimization.

Reference

- R2-2501739, LS on CSSF optimization for NR RRM Phase 5, RAN4 LS

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN#105

- R4-2504916, AdHoc minutes for [114bis][208][209] NR_RRM_Ph5