3GPP TSG RAN WG2 #130                                                                                 R2-2503586

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	MHI Enhancement for SCG Activation and Deactivation

Agenda Item:	8.10.3

Document for:	Discussion and Decision

Introduction

In RAN2#129bis meeting, MHI enhancement for SCG activation and deactivation was discussed and the following agreement was achieved [1].

Agreement: UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

In this contribution, we will continue to discuss the detail about how to log the absolute time the UE has spent in PSCell with SCG activated.

Discussion

In last meeting, RAN2 has agreed that UE reports the absolute time it has spent in PSCell with SCG activated in MHI. However, an issue needs to be clarified is how to log the absolute time information in case of the SCG is activated/deactivated multiple times when connected to the SCG, an example in Figure 1 below.

Figure 1 SCG activation/deactivation when UE is connected to the SCG

In Figure 1, the SCG activation/deactivation when UE is connected to the SCG is described as follow:

T1: UE accesses the SCG1 with activated SCG1;

T2/T4: SCG1 is deactivated due to no data transmission is needed on SCG1;

T3: SCG1 is activated due to data transmission is needed on SCG1;

Based on above, it can be seen that the time the UE has spent in SCG1 with SCG activated includes two periods of time i.e., T1~T2 and T3~T4. How to log the two periods of time in MHI needs to be discussed. The following two options can be considered:

Option 1: logging each time duration of SCG activation for a PSCell in chronological order;

Option 2: logging the accumulated time for SCG activation for a PSCell;

For Option 1, it is a straight way to give all the details on SCG activation/deactivation procedure, but it will increase UE logging complexity. For Option 2, it is a simpler way, but the network cannot get the detailed information about SCG activation/deactivation. From our perspective, we prefer Option 2 to reduce the complexity.

Proposal 1: UE logs the accumulated time for SCG activation for a PSCell in MHI if SCG activation/deactivation are performed multiple times for the PSCell.

Conclusion

According to the discussion in section 2, the proposal is summarized as follows:

Proposal 1: UE logs the accumulated time for SCG activation for a PSCell in MHI if SCG activation/deactivation are performed multiple times for the PSCell.

Reference

- R2-2503302, RAN2_129bis_Meeting_Report