3GPP TSG RAN WG2 #130                                                                                  R2-2503588

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on LCM for UE-sided model for BM use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis meeting, RAN2 discussed the LCM for UE-sided model for beam management use case, including applicability reporting and UE side data collection, and achieved the agreements as follows [1].

Applicability reporting

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

UE side data collection

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In this contribution, we will further discuss the leftover issues for the applicability reporting and UE side data collection respectively.

- Discussion

2.1 Applicability reporting

In the beginning, RAN2 discussed applicable functionality reporting related issues, some agreements were made and sent to RAN1 in LS [2], some questions also were included in the LS for checking RAN1’s understanding. After RAN2#128 meeting, RAN2 receives the reply LS [3] from RAN1, and in this reply LS, RAN1 provides some replies to these issues of RAN2 in LS [2]. In the next two RAN2 meetings, RAN2 discussed and achieved some agreements based on RAN1’s reply LS [3], but there are still some FFS issues which need to be discussed. In this section, we continue to discuss these FFS issues, and for the convenience of discussion, the signalling procedure figure of applicable functionality reporting is also placed here (see figure 1 below).

Figure1: Applicable functionality reporting signalling procedure

For Step 3

- NW-side additional condition

In [2], RAN2 pointed out that these agreements of applicable functionality reporting agreed in RAN2 is based on the assumption “NW-side additional condition is assumed as associated ID.”, and further asked RAN1 whether the assumption is correct or not. Based on the issue, RAN1 gave the reply as follow in [3].

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Observation 1: RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Another issues proposed by RAN2 in [2] e.g., “Is NW-side additional condition functionality specific?”, or “whether NW-side additional condition is mandatory or optional” etc., RAN1 did not give the clear answer and some details about NW-side additional condition are still under discussion in RAN1. RAN2 needs to wait for RAN1’s further discussion.

Proposal 1: RAN2 assumes NW-side additional condition as associated ID in RAN2’s discussion, more details can wait for RAN1.

- Content of otherConfig for enabling applicability reports in UAI

In the current running CR [4], an editor's note is captured as follow in OtherConfig field descriptions.

applicabilityReportConfig Configuration for the UE to indicate the applicability of configurations subject to the applicability determination procedure. Editor's Note: FFS the content (if any) of the UAI configuration to enable the UE to report applicability.

There are two options can be considered to address the issue, one option is only configuring one flag to indicate the applicability reporting in UAI, it is up to UE implement to associated to the CSI reporting configuration, but the granularity of the flag needs to be considered, e.g., per sub use case. Another option is configuring the list of CSI-ReportConfigId in OtherConfig to indicate the applicability reporting in UAI, UE reports applicability of these CSI-ReportConfigId in UAI when the applicability change occurs. For reducing configuration signaling overhead, we prefer the former.

Proposal 2: (RRC-2) Introduce only a flag in otherConfig to indicate the applicability reporting in UAI.

- Coexistence of Option A and Option B

In previous meeting, it was agreed to support both Option A and Option B for applicable functionality reporting. However, it is not clear whether to support to configure Option A and Option B simultaneously. We understand Option A and Option B is for different scenarios, e.g., Option A can be used for the small number of functionality, and Option B can be used for large number of functionality. It is up to network implement to configure Option A or Option B to UE, Option A and Option B will not be configured simultaneously e.g., for the same sub use case.

Proposal 3: For the same sub use case, Option A and Option B are not configured simultaneously.

For Step 4

- Applicability reporting for Option B

In last meeting, RAN2 assumed UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Considering that RAN2 has agreed that for option A, upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete, UAI can be sent to update applicability. For option B (if supported), similar mechanism can be considered, not matter the inference related parameters are provided in otherConfig or CSI-ReportConfig, i.e., UE reports the initial applicability via RRCReconfigurationComplete message when receive the inference related parameters in RRCReconfiguration, and UAI can be sent to update applicability.

Proposal 4: (RRC-5) For option B, UE reports the initial applicability via RRCReconfigurationComplete when receive the inference related parameters in RRCReconfiguration, and UAI can be sent to update applicability.

- Non-applicable functionalities reporting

In RAN2#129bis meeting, RAN2 agreed to report a simple cause value of inapplicability together with inapplicability reporting, FFS how to define this simple cause related to model availability and how we capture it in the spec. From our perspective, a simple cause value of inapplicability that whether the model is available or not for the functionality is enough which to indicate no model in device or model is not applicable to network, no further information is needed.

Proposal 5: (RRC-1) A simple cause value of inapplicability that whether the model is available or not is indicated to network together with inapplicability reporting, no further information is needed.

For Step 5

- Inference configuration contents in Step5

For option B, for performing inference, the network needs to send the full inference configuration (CSI-ReportConfig) to UE after receiving the applicability reporting. For the contents of inference configuration in Step5, RAN1 replied in [3] that RAN1 will discuss and design later. RAN2 can wait for RAN1’s discussion and progress.

Q6: What is the content of inference configuration in Step 5? Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Proposal 6: For option B, NW needs to configure CSI-ReportConfig for inference configuration in Step5. The contents of inference configuration in Step5 are up to RAN1.

After the UE receives the inference configuration in Step 5, the UE does not need to report the applicability again as it has been reported in Step 3, if the applicability change occurs, the UE can trigger UAI reporting to inform network, e.g., to update the inference configuration in Step 5.

Proposal 7: For option B, UE does not need to report the applicability after receiving the inference configuration in Step 5.

- Applicable functionalities activation

In RAN2#129 meeting, for option A, RAN2 agreed the following agreements about the applicable functionalities activation.

Agreements - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: Semi-persistent reporting, activated by MAC CE/DCI Aperiodic CSI reporting, activated by DCI

For option B, considering that the network needs to send the full inference configuration (CSI-ReportConfig) to UE after receiving the applicability reporting, therefore, for periodic CSI reporting, the UE activates the applicable functionalities when receiving the full inference configuration of the applicable functionality in Step5, for semi-persistent and aperiodic CSI report, legacy CSI framework is used, i.e., semi-persistent reporting is activated by MAC CE/DCI, aperiodic CSI reporting is activated by DCI.

Proposal 8: For option B, the UE activates the applicable functionalities as follow:

- Periodic CSI report is activated when receiving the full inference configuration of the applicable functionality in Step5;

- Semi-persistent and aperiodic CSI report are activated follow legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

In [2], RAN2 proposed an issue that if more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? In [3], RAN1 gave the following agreement, RAN1 agreed that for beam management use case, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Proposal 9: If more than one functionality is configured in Step 3 or Step 5, multiple/all applicable functionalities can be activated.

2.2 UE side data collection

In last meeting, RAN2 agreed that the UE can request the UE side data collection configuration by UAI, and the request information can include the following.

- An indication on start/stop of data collection;

- Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Based on the agreement, network needs to provide a list of candidate configuration before UE requests data collection, the candidate configuration can be provided by the following two options, both options can be considered.

Option 1: a list of candidate configuration is provided in otherConfig together with the indication of allowing UE requests;

Option 2: UE requests and reports preferred configuration based on the current CSI measurement configuration.

Proposal 10: (RRC-3) RAN2 to discuss the following two options for the candidate configuration provided from network to UE:

Option 1: a list of candidate configuration is provided in otherConfig together with the indication of allowing UE requests;

Option 2: UE requests and reports preferred configuration based on the current CSI measurement configuration.

When the UE requests the data collection and sends the preferred configuration in UAI message to network, the index of the preferred configuration from a list of candidate configuration provided by NW is included in the request message for reducing signaling overhead.

Proposal 11: (RRC-3) The index of the preferred configuration from a list of candidate configuration provided by NW is included in the request message.

In addition, an FFS issue was left in RAN2#128 meeting, i.e., whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration. From our perspective, it seems to be unnecessary that the UE reports the indicator to network to indicate UE can’t perform data collection based on received configuration. On the one hand, the UE can provide the sufficient information in the data collection request procedure, to make sure the network can get the sufficient information to configure the effective data collection configuration. Even if the configuration from network is invalid, the UE can send the request again to network to request the suitable configuration. One the other hand, if the UE can’t perform data collection based on received configuration, e.g., low power or low memory, the UE can stop the data collection, and continue to collect data when the UE has high power or sufficient memory again.

Proposal 12: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

- Conclusion

According to the discussion in section 2, we propose:

Applicability reporting

Observation 1: RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID.

Proposal 1: RAN2 assumes NW-side additional condition as associated ID in RAN2’s discussion, more details can wait for RAN1.

Proposal 2: (RRC-2) Introduce only a flag in otherConfig to indicate the applicability reporting in UAI.

Proposal 3: For the same sub use case, Option A and Option B are not configured simultaneously.

Proposal 4: (RRC-5) For option B, UE reports the initial applicability via RRCReconfigurationComplete when receive the inference related parameters in RRCReconfiguration, and UAI can be sent to update applicability.

Proposal 5: (RRC-1) A simple cause value of inapplicability that whether the model is available or not is indicated to network together with inapplicability reporting, no further information is needed.

Proposal 6: For option B, NW needs to configure CSI-ReportConfig for inference configuration in Step5. The contents of inference configuration in Step5 are up to RAN1.

Proposal 7: For option B, UE does not need to report the applicability after receiving the inference configuration in Step 5.

Proposal 8: For option B, the UE activates the applicable functionalities as follow:

- Periodic CSI report is activated when receiving the full inference configuration of the applicable functionality in Step5;

- Semi-persistent and aperiodic CSI report are activated follow legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

Proposal 9: If more than one functionality is configured in Step 3 or Step 5, multiple/all applicable functionalities can be activated.

UE side data collection

Proposal 10: (RRC-3) RAN2 to discuss the following two options for the candidate configuration provided from network to UE:

Option 1: a list of candidate configuration is provided in otherConfig together with the indication of allowing UE requests;

Option 2: UE requests and reports preferred configuration based on the current CSI measurement configuration.

Proposal 11: (RRC-3) The index of the preferred configuration from a list of candidate configuration provided by NW is included in the request message.

Proposal 12: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

- References

- R2-2503302, RAN2_129bis_Meeting_Report

- R2-2407848 LS on applicable functionality reporting for beam management UE-sided model

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

- R2-2502903 - Draft running RRC CR for AIML PHY, Ericsson