3GPP TSG RAN WG2 #130	                                                                             R2-2503594

St Julian, Malta, 19th – 23rd May, 2025

Source:	CATT, Turkcell

Title:	Discussion on performance monitoring

Agenda Item:	8.3.5

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis meeting, the following agreements were made regarding performance monitoring for AI mobility.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) 7 For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In this contribution, we discuss the performance monitoring of RRM measurement prediction and measurement event prediction.

- Discussion

  - General

NW sided AI model

In the RAN2 #126 meeting, the following agreement has been reached for BM For NW sided AI model:

The gNB is responsible for monitoring its own performance. RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

Regarding AI mobility, the performance monitoring of NW sided AI model also should be up to the NW implementation.

Proposal 1: For network sided AI model, the performance monitoring should be implemented at NW without any spec impact.

For intra-frequency temporal domain case A, the UE is required to perform real-time reporting of measurement results to support inference, and the real-time measurement results can be used for performance monitoring. However, for intra-frequency temporal domain case B, inter-frequency and spatial domain prediction, where measurement result(s) is not reported at every time instance, the true measurement result for performance monitoring can be collected by UE and reported to NW. Hence, it is proposed that for calculating the performance metric for network sided model, UE needs to report benchmark/reference to network.

Proposal 2: For calculating the performance metric for network sided model, UE needs to report benchmark/reference to network.

UE sided AI model

Regarding BM, RAN1 defines the following options for UE-sided model.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

And during RAN1#117 meeting, RAN1 reached an agreement to support Type 1 performance monitoring including both Option 1 and Option 2 for performance monitoring of UE-sided model.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Observation 1: For BM with a UE-side AI/ML model, the following performance monitoring operations have been agreed:

- Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW)

- Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

Since Type 2 performance monitoring has not yet been excluded by RAN1 and considering that the monitoring can be performed by network and UE side, it is feasible for the UE to indicate, request, or report to the NW for performance monitoring. Therefore, it is proposed that:

Proposal 3: RAN2 starts the study of performance monitoring for AI mobility of UE-sided model including the following options:

- Performance monitoring is triggered by the network:

- Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW)

- Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

- Performance monitoring is indicated/requested/reported by the UE.

  - Metrics of UE-side AI/ML model

In this section, we discuss the metrics of UE-side AI/ML model on performance monitoring that is triggered by the network.

RRM measurement prediction

In the existing specification, MeasQuantityResults includes RSRP, RSRQ and SINR, for RRM measurement prediction, actual RSRP, RSRQ and SINR can be reported for performance monitoring. In RAN2 #127bis meeting, in addition to the actual RSRP, the average RSRP value over the entire window (PW) is also agreed to be provided in the simulation results. Considering reporting the average of actual RSRP is friendly to the signaling overhead and can be used to reflect the model accuracy, it is proposed that the average of actual RSRP, RSRQ, and SINR over the entire PW window can be reported for performance monitoring.

Agreement Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used. The companies should at least provide the results of only one value it should be the last value at the end of the PW. We will add two columns in the spreadsheet to capture the last value and the average value.

In the RAN2 #126 meeting, in the section of simulation assumptions and evaluation methodology for RRM measurement prediction, the prediction accuracy metric for RRM measurement cell level prediction have been discussed with the following agreement:

3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases

1 average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged

Based on the above agreement, it can be seen that RSRP difference and average RSRP difference are taken as prediction accuracy metric in the simulation. Similarly, RSRP difference and average RSRP difference are taken as prediction accuracy metric for performance monitoring.

Proposal 4: For RRM measurement prediction, RAN2 to study the following options which can be used for performance monitoring metric :

- Option 1: average of actual RSRP, RSRQ, and SINR over PW window and average of predicted RSRP, RSRQ, and SINR over one PW window;

- Option 2: average RSRP, RSRQ and SINR difference over PW window.

Measurement event prediction

During the RAN2 #128 meeting, it was agreed on the following definitions for F1 score, true event prediction, false event detection and missed event detection for indirect/direct measurement event prediction.

Agreement 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’

Basically, the F1 score a metric used to evaluate the performance of classification models. Hence, it is proposed that:

Proposal 5: For measurement event prediction, F1 score can be used for performance monitoring metric.

According to the definition of F1 score, it can be seen that F1 score is the formula changing with n1, n2 and n3 which are greatly impacted by the maximum EDT. During the simulation phase, it was also observed that the value of missed event detection, false event detection and true event prediction could be affected by the value of maximum ETD and the maximum ETD is 80ms and opens for more values for simulation assumption. The longer of the maximum EDT, the higher of F1 score and vice versa.

Observation 2: Maximum EDT plays important role of value of F1 score calculation of AIML model.

Considering the significant impact of variables maximum ETD on the performance monitoring, it is suggested that maximum EDT is aligned between UE and network to facilitate model management decision. RAN2 can discuss whether maximum ETD is a fixed value, e.g., preconfigured in the specification, or a variable value configured by NW.

Proposal 6: For performance monitoring of indirect measurement event prediction, RAN2 discuss whether maximum ETD is a fixed value or a variable value controlled by NW for F1 score calculation.

  - Monitoring report

In the last RAN1 120bis meeting, the type of resource for the set for monitoring and the report type of inference and performance monitoring in BM was discussed, and it was agreed that:

Agreement of RAN1 For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB. Agreement of RAN1 For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support.

According to the conclusion of RAN1, it can be seen that periodic CSI-RS, semi-persistent CSI-RS and SSB are supported and aperiodic RS is not applicable to monitoring. For aperiodic RS, the measurement is one-shot and the monitoring metric may not be reliable, especially for the measurement event prediction. For AI mobility, we think the type of resource for performance monitoring should follow inference configuration.

Proposal 7: For UE-sided model monitoring with UE assistance, regarding the monitoring object, it can follow the inference configuration.

In addition, based on the above agreement, for Type 1 with Option 2 where UE calculates performance metric(s) and reports it to NW or reports an event to NW based on the performance metric(s), P/SP/AP reporting for performance monitoring is supported for BM. For performance monitoring of RRM measurement prediction and measurement event prediction, the following options can be considered:

- Option 1:Periodical reporting

- Option 2: Reporting after being triggered by NW

Proposal 8: For monitoring report, RAN2 can study the following options:

- Option A: Periodical reporting;

- Option B: Reporting triggered by NW.

For direct measurement event prediction, periodic reporting may lead to an unnecessarily signaling load since measurement events may be rarely predicted. As a result, the missed event detection, false event detection, true event prediction and F1 score may all be equal to 0. Therefore, Option B is preferred for the monitoring report of direct measurement event prediction.

Proposal 9: For direct measurement event prediction, support at least NW triggered reporting.

- Conclusion

According to the analysis in Section 2, our observations and proposals are summarized as follows:

General

NW sided AI model

Proposal 1: For NW sided AI model, the performance monitoring should be implemented at NW without any spec impact.

Proposal 2: For calculating the performance metric for network sided model, UE needs to report benchmark/reference to network.

UE sided AI model

Observation 1: For BM with a UE-side AI/ML model, the following performance monitoring operations have been agreed:

- Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW)

- Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

Proposal 3: RAN2 starts the study of performance monitoring for AI mobility of UE-sided model including the following options:

- Performance monitoring is triggered by the network:

- Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW)

- Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s)

- Performance monitoring is indicated/requested/reported by the UE

Metrics of UE-side AI/ML model

Proposal 4: For RRM measurement prediction, RAN2 to study the following options which can be used for performance monitoring metric:

- Option 1: average of actual RSRP, RSRQ, and SINR over PW window and average of predicted RSRP, RSRQ, and SINR over PW window;

- Option 2: average RSRP, RSRQ and SINR difference over PW window

Proposal 5: For measurement event prediction, F1 score can be used for performance monitoring metric.

Observation 2: Maximum EDT plays important role of value of F1 score calculation of AIML model.

Proposal 6: For performance monitoring of indirect measurement event prediction, RAN2 discuss whether maximum ETD is a fixed value or a variable value controlled by NW for F1 score calculation.

Monitoring report

Proposal 7: For UE-sided model monitoring with UE assistance, regarding the monitoring object, it can follow the inference configuration.

Proposal 8: For monitoring report, RAN2 can study the following options:

- Option A: Periodical reporting;

- Option B: Reporting triggered by NW.

Proposal 9: For direct measurement event prediction, support at least NW triggered reporting .

- References

- R2-2501702, RAN2#129bis Meeting Report;