3GPP TSG-RAN WG2 Meeting #130		R2-2503613

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post129bis][209][LP-WUS] Remaining RRC open issue for LP-WUS/WUR (vivo), one open issue is identified as below:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-12 | whether/how to enable/disable LP-WUS, e.g. by RRC/NAS As raised by Ericsson below. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |

In the email discussion on [Post129bis][208][LP-WUS] Running CR for 38.304 (CATT), two open issues are identified as below:

LP-WUS in idle/inactive mode Open issue 38304-5: FFS the UEs expecting MBS group notification should monitor its PO to receive the MBS group notification regardless of LP-WUS. Open issue 38304-8: Whether LP-WUS is only used in the last used cell or in any cell

In this contribution, we will further discuss the above issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. And other remaining issues will also be discussed.

- Discussion

  - Enable/disable LP-WUS(RRC-12)

- There are many companies are interested on whether the LP-WUS functionality could be enabled/disable for a UE using the dedicated signaling. And we have the following open issue in [1]:

- whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

On one hand, if there is no such enable/disable mechanism, which means all LP-WUS capable UEs could use the LP-WUS in the camped cell which provides corresponding LP-WUS configuration, and the network couldn’t disable the LP-WUS functionality for a UE, even if the UE may be frequently paged within a time duration. In this case it will increase the false alarm rate, thus introducing the per UE enable/disable mechanism for LP-WUS with dedicated signaling could achieve power saving gain in some special case. Besides, it will be more flexible for both network and UE if LP-WUS could be enabled/disabled per UE.

Observation 1: It could achieve some power saving gain to introduce the per UE enable/disable mechanism with dedicated signaling in some special case.

On the other hand, how to provide this enable/disable indication to UE should be determined. There are two ways based on the companies’ views. One is to rely on gNB to enable/disable the LP-WUS functionality via RRC signalling, another is to rely on CN to enable/disable the LP-WUS functionality via NAS signalling. Since the CN has more information on UE characteristics and traffic information for UEs in RRC idle/inactive, such as UE capability, UE type, paging probability, etc., we think CN is more suitable to take this responsibility. Besides, if it were relied on gNB control, new signalling between CN and gNB would be introduced to inform gNB on this control since the gNB couldn’t store the UE context for UEs in RRC idle/inactive.

Proposal 1: (RRC-12) Using NAS signalling to enable/disable the LP-WUS functionality per UE for UEs supporting LP-WUS.

In RAN2#126 meeting, it was agreed that the LP-WUS related configuration includes LP-SS configuration, LP-WUS configuration, and the entry/exit condition for LP-WUS monitoring. Besides, at least two types of LR will be introduced, i.e. one is the OOK-based WUR, another is the OFDM-based WUR. If the network supports the UEs with OOK-based WUR could use the LP-WUS in one cell, the LP-SS configuration should be configured in the LP-WUS related configuration. Otherwise, after UE with OOK-based WUR enters LP-WUS monitoring, OOK-based WUR cannot receive LP-WUS without the synchronization from LP-SS.

Proposal 2: If the network supports the OOK-based WUR, the LP-SS configuration should always be provided.

  - Coexistence between MBS group notification and LP-WUS (38304-5)

Regarding the open issue:

Open issue 38304-5: FFS the UEs expecting MBS group notification should monitor its PO to receive the MBS group notification regardless of LP-WUS.

As in Rel-17 PEI, this issue was discussed and concluded in MBS session. Since the paging subgroup for PEI is not applied to the group paging, when a group paging is transmitted, the network doesn’t indicate the subgroup that the UE which has joined the multicast session belongs to, if there is no unicast paging for the paging subgroup. In order to address this issue, it is agreed that the UEs that expect group notification ignore PEI (and just monitor paging as usual).

In our view, the similar issue will happen for LP-WUS subgrouping, thus the same principle for PEI could be reused for LP-WUS. Which is:

Proposal 3: (38304-5) UEs expecting MBS group notification should monitor its PO to receive the MBS group notification regardless of LP-WUS.

  - LP-WUS in last used cell (38304-8)

Regarding the open issue:

Open issue 38304-8: Whether LP-WUS is only used in the last used cell or in any cell.

For PEI, an indication in SIB1, i.e., lastUsedCellOnly was introduced as below to indicate whether the UE monitors PEI only if the last used cell is current camped cell.

lastUsedCellOnly When present, the field indicates that the UE monitors PEI only if the latest received RRCRelease without noLastCellUpdate is from this cell. A PEI-capable UE stores its last used cell information.

This was introduced to reduce the false alarm rate caused by the UE moving into one cell, in order to guarantee the maximum power saving gain for UE using PEI in this cell. But in LP-WUS, the situation is a bit different. As RAN1 has agreed to introduce subgroup number up to 31, which is much larger than PEI subgroup number. In this way, LP-WUS with more groupings, reduces the false alarm rate when waking up UE by LP-WUS, thereby greatly minimizing the impact on the power-saving gain of other UEs that are not in the same group as this UE in this cell. This weakens the motivation for introducing the lastUsedCellOnly.

Besides, as we proposed above, per UE enable/disable mechanism for LP-WUS functionality should be introduced. We believe that a reasonable network implementation should enable the using of LP-WUS only for UEs with relatively low traffic, i.e., those with low paging probability. UEs that may be frequently waken up or have a very high mobility state are not suitable for LP-WUS, as this could result in energy consumption overhead for other UEs. In this way, if the CN identifies the UE has high mobility state or will be frequently paged, it will disable the LP-WUS functionality for this UE directly. On the other hand, if the LP-WUS functionality is enabled, it illustrates that the false alarm brought by this mobile UE is low to other UEs whose last used cell is current cell. Thus there is no need to introduce such indication for LP-WUS and UE could use the LP-WUS in any cell if its LP-WUS functionality is enabled and there is corresponding configuration provided in this cell.

Proposal 4: (38304-8) UE could use LP-WUS in any cell, i.e. there is no need to introduce lastUsedCellOnly.

  - Value range of configured threshold (RRC-6 related)

Regarding the following EN in current RRC running CR for LP-WUS, the value range for the threshold on the measured RSRP/RSRP via LR needs to be determined.

Editor’s NOTE: FFS on the values for each RRC parameters. To be updated based on RAN1/RAN2 progress.

And similar as the RRC-6 open issue:

| RRC-6 | the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell relaxation/offloading and neighboring cell relaxation Editor’s NOTE: FFS on the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell relaxation/offloading and neighboring cell relaxation. | Issue Type: essential How to address it: can be addressed based on RAN1/RAN4 inputs or companies’ contribution |
| --- | --- | --- |

It could be observed that the threshold for LR measurement for LP-WUS monitoring should also be considered.

In current RRC running CR for LP-WUS, it is captured as follows:

Based on our observation, the receiver sensitivities of LR and MR are roughly the same. Although the receiver hardware is degraded for LR, the number of information bits carried is smaller and the data rate is lower, which results in a similar overall sensitivity compared to MR. Thus, the value range for LR measurement thresholds should reuse the range corresponding to the absolute measured values for MR. The detailed aspects can be discussed in the running CR.

Proposal 5: (RRC-6) The value range for the threshold on the measured RSRP/RSRP via LR could refer to the absolute measured values for MR. Details could be discussed in the running CR.

  - Coexistence between eDRX and LP-WUS

In RAN2#129bis meeting, we agreed that the LP-WUS is supported with eDRX, which is as below [2]:

- LP-WUS is supported with eDRX, FFS on exact impact if any

In our view, the mechanism for PEI with eDRX could be reused for LP-WUS, which means the LP-WUS is used to control PO monitoring. In details, it includes the following scenarios:

- When eDRX is configured as 5.12 and 10.24, UE monitors PO per TeDRX, CN. Samely, the UE monitors the LP-WUS before the PO, i.e, with periodicity of TeDRX, CN.

- When eDRX is configured to be longer than 10.24s, UE monitors PO within the PTW per DRX cycle, thus UE should monitor the LP-WUS per DRX cycle within the PTW.

Proposal 6: Following the same mechanism as the coexistence between PEI and eDRX, for LP-WUS:

- When eDRX is configured to be longer than 10.24s, LP-WUS is used to control PO monitoring within the PTW.

- When eDRX is configured to be 5.12s or 10.24s, LP-WUS is used to control the PO monitoring.

  - Entry/exit condition for LP-WUS monitoring

We have the following agreements in RAN2#129bis meeting on the relationship of entry/exit thresholds for RRM relaxation/offloading for different LR types:

- RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

However, the conclusion doesn’t consider the entry/exit thresholds for LP-WUS monitoring. In our understanding, there is no difference between LP-WUS monitoring and RRM measurement on this principle. Thus, the same principle for LP-WUS monitoring should also be agreed:

Proposal 7: RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

According to below RAN2 agreement:

RAN2 agreement in RAN2#127 meeting Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Different companies may have different understandings on the meaning of separate entry/exit thresholds for LP-WUS monitoring for different types of LR:

- Understanding#1: MR threshold(s) in the entry/exit condition for LP-WUS monitoring are the same for different types of LR while the LR threshold(s) in the entry/exit condition are separate for different types of LR;

- Understanding#2: Both MR threshold(s) and LR threshold(s) in the entry/exit condition for LP-WUS monitoring are separate for different types of LR.

However, according to the following agreement for RRM measurement in RAN2#129bis:

- RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

The same principle should be applied for LP-WUS monitoring, which means MR threshold(s) in entry conditions for LP-WUS monitoring can be configured separately for different types of LR.

Thus, RAN2 can confirm:

Proposal 8: RAN2 confirms the MR thresholds of entry/exit condition for LP-WUS monitoring can be configured separately for different types of LR if a cell supports both types of LRs (can revisit based on RAN1 and RAN4 progress, if any), similar as criteria for RRM relaxation.

  - Other wake up conditions

As legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data/signaling arrival.

Proposal 9: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT, due to UL data and signaling arrival when monitoring LP-WUS. And no specification impact is foreseen.

  - UE monitors both LP-WUS and PEI

One remaining issue is whether a network indication should be introduced to indicate the UE to monitor LP-WUS and PEI or not. At the network side, according to legacy design, if there is a PEI configuration in the camped cell, all UEs supporting PEI could monitor PEI, which means that network needs to always provide PEI if it provides PEI configuration. Similarly, if network supports LP-WUS and provides the LP-WUS configuration, all UEs supporting LP-WUS could monitor LP-WUS, which means that network needs to always provide LP-WUS if it provides PEI configuration. At the UE side, there is a case that PEI and LP-WUS may both be monitored at one UE when the entry condition of LP-WUS monitoring is fulfilled. In RAN1#116 meeting [3], RAN1 made the following agreement:

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

It could be observed that, when UE supports PEI and PEI is configured by the gNB, it is up to UE implementation whether to monitor PEI or not after the UE receives LP-WUS indicating wake-up.

Observation 2: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Based on the above design at the network side, regardless of whether the UE monitors PEI or not after receiving LP-WUS indicating wake-up, the network would provide PEI and LP-WUS in case it provides the PEI and LP-WUS configuration. Thus, there is no additional effort for network to provide both PEI and LP-WUS. In this way, there is no need for network to indicate the UE whether to monitor both PEI and LP-WUS or not.

Proposal 10: No need to introduce a network indication to indicate UE to monitor both LP-WUS and PEI or not.

  - Emergency PDU session

We have an FFS in RAN2#128:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

According to the CR in SA2#167 meeting [4], it removes the unnecessary restriction for emergency PDU session for PEI and LP-WUS with the following reason:

PSAP callback is delivered as normal call in IMS PDU session. There is no need for the UE and network to disable functionalities like PEIPS or LP-WUS when emergency PDU session is active

With this, RAN2 should follow the same principle as SA2, which means UE could monitor LP-WUS when UE has the emergency PDU session in RRC_IDLE/RRC_INACTIVE.

Proposal 11: Follow the same principle as SA2 on emergency PDU session with LP-WUS, i.e., UE could monitor LP-WUS when there is an active emergency PDU Session in RRC_IDLE/RRC_INACTIVE.

  - UE capability

We have the following agreement in RAN2#129bis for UE capability:

- Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR.

Considering LP-WUS is also used for CN/RAN paging, UE capability for LP-WUS should also be reported to the network like PEI. Different from PEI, besides two kinds of subgrouping methods, there are two kinds of LR receivers for LP-WUS, i.e. OOK-based WUR and OFDM-based WUR. Since RAN2 has agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs, correspondingly, it is reasonable to introduce separate UE capabilities for OFDM-based WUR and OOK-based WUR. But the final decision should be determined in RAN1.

Proposal 12: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Regarding the UE capability for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, it is straightforward to follow the capability design method for PEI. That is, the UE capabilities for supporting CN assigned subgrouping and UE_ID based subgrouping are separately indicated. And no need to introduce additional UE capability for UE_ID based subgrouping, while using the UE capability supporting the LP-WUS to indicate whether supporting the UE_ID based subgrouping as we discussed in the post email discussion[5]. Though there are separate UE capabilities for different types of LR, i.e., the UE capability supporting OFDM-based WUR and OOK-based WUR, we think no matter UE indicates which LR type among these two capabilities, it will also support the UE_ID based subgrouping.

Observation 3: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, similar as PEI design.

Observation 4: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Regarding the capability reporting for LP-WUS, we think the similar reporting mechanism for PEI could be reused directly. In detail, UE’s capability of supporting the CN assigned  subgrouping is reported to CN by NAS signalling during the registration request or TAU procedure. And for the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping is reported to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message, and gNB will report this UE capability to AMF.

Proposal 13: The capability reporting design for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

In RAN2#127 meeting, it was agreed:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

In the email discussion on UE capability [Post129bis][218][LPWUS] UE capabilities [5], it is proposed:

Proposal 2 (5/8): RRM measurement relaxation and RRM measurement fully offloading are defined as RAN2 capability. Send LS to RAN1 to inform the agreement.

According to the evaluation in study item in Rel-18 on LP-WUS/WUR, it could be observed that there is no power saving gain if UE supports LP-WUS monitoring while doesn’t support the serving cell RRM offloading or serving/neighboring cell RRM relaxation. It is unlikely for a UE only supports LP-WUS monitoring but not support any of serving/neighboring cell RRM relaxation or serving cell RRM offloading. Thus, the serving cell/neighboring cell RRM relaxation or serving cell RRM offloading should be defined as two components of the basic feature for LP-WUS, i.e. one is for serving cell/neighboring cell RRM relaxation, the other is for serving cell RRM offloading.

Proposal 14: Whether a UE supports serving cell/neighboring cell RRM relaxation and serving cell RRM offloading should be defined as two components (one is for serving cell/neighboring cell RRM relaxation, the other is for serving cell RRM offloading) of the basic feature for LP-WUS.

- Conclusion

In this contribution, we discuss the remaining issues for RRC_IDLE/ INACTIVE procedures with LP-WUR. Based on the discussion, we have the following observations and proposals.

Observation 1: It could achieve some power saving gain to introduce the per UE enable/disable mechanism with dedicated signaling in some special case.

Observation 2: According to the RAN1 agreement, when UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Observation 3: Introduce separate UE capabilities for CN assigned subgrouping and UE_ID based subgrouping for LP-WUS, similar as PEI design.

Observation 4: UE capability for supporting LP-WUS (assuming capability for OFDM-based WUR or OOK-based WUR), indicates supporting of UE_ID based subgrouping, i.e. capability supporting LP-WUS and UE_ID based subgrouping is one capability, similar as PEI design.

Enable/disable LP-WUS(RRC-12)

Proposal 1: (RRC-12) Using NAS signalling to enable/disable the LP-WUS functionality per UE for UEs supporting LP-WUS.

Proposal 2: If the network supports the OOK-based WUR, the LP-SS configuration should always be provided.

Coexistence between MBS group notification and LP-WUS (38304-5)

Proposal 3: (38304-5) UEs expecting MBS group notification should monitor its PO to receive the MBS group notification regardless of LP-WUS.

LP-WUS in last used cell (38304-8)

Proposal 4: (38304-8) UE could use LP-WUS in any cell, i.e. there is no need to introduce lastUsedCellOnly.

Value range of configured threshold (RRC-6 related)

Proposal 5: (RRC-6) The value range for the threshold on the measured RSRP/RSRP via LR could refer to the absolute measured values for MR. Details could be discussed in the running CR.

Coexistence between eDRX and LP-WUS

Proposal 6: Following the same mechanism as the coexistence between PEI and eDRX, for LP-WUS:

- When eDRX is configured to be longer than 10.24s, LP-WUS is used to control PO monitoring within the PTW.

- When eDRX is configured to be 5.12s or 10.24s, LP-WUS is used to control the PO monitoring.

Entry/exit condition for LP-WUS monitoring

Proposal 7: RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

Proposal 8: RAN2 confirms the MR thresholds of entry/exit condition for LP-WUS monitoring can be configured separately for different types of LR if a cell supports both types of LRs (can revisit based on RAN1 and RAN4 progress, if any), similar as criteria for RRM relaxation.

Other wake up conditions

Proposal 9: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT, due to UL data and signaling arrival when monitoring LP-WUS. And no specification impact is foreseen.

UE monitors both LP-WUS and PEI

Proposal 10: No need to introduce a network indication to indicate UE to monitor both LP-WUS and PEI or not.

Emergency PDU session

Proposal 11: Follow the same principle as SA2 on emergency PDU session with LP-WUS, i.e., UE could monitor LP-WUS when there is an active emergency PDU Session in RRC_IDLE/RRC_INACTIVE.

UE capability

Proposal 12: RAN2 assumes separate UE capabilities for different LR types, i.e. OFDM-based WUR and OOK-based WUR, should be introduced and reported to the network. Final decision is up to RAN1.

Proposal 13: The capability reporting design for PEI is reused for LP-WUS, i.e. UE reports capability of supporting the CN assigned subgrouping to CN during the registration request or TAU procedure; UE reports the combined capability to indicate supporting LP-WUS and UE_ID based subgrouping to RAN via UE-RadioPagingInfo IE in the RRC UECapabilityInformation message.

Proposal 14: Whether a UE supports serving cell/neighboring cell RRM relaxation and serving cell RRM offloading should be defined as two components (one is for serving cell/neighboring cell RRM relaxation, the other is for serving cell RRM offloading) of the basic feature for LP-WUS.

- References

- R2-2503765, Discussion summary and list of RRC open issue for LP-WUS WUR

- RAN2#129bis Chair minutes

- R1-2401937, Final Report of 3GPP TSG RAN WG1 #116 v1.0.0

- S2-2502428, Removing unnecessary restrictions for emergency PDU sessions

- R2-2503899, Summary of [Post129bis][218][LPWUS] UE Capabilities (Huawei)