3GPP TSG-RAN WG2 Meeting #130	R2-2503622

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	On Bj adjustments for LCH priority-adjusted data transmission

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis, there were the following conclusions regarding Bj adjustments [1]:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

According to the above agreements, the tentative work assumption regarding Bj adjustment is to be reviewed in this meeting. In this contribution, the potential benefit and the potential impact on lower priority service of Bj enhancement are investigated, based on which the related text proposals are provided in the annex.

- Discussion

For delay-awareness based scheduling, the gNB may issue UL grants to a UE to speed up the LCH priority-adjusted data transmissions based on the delay status and the intention of the UL grants is for the transmission of LCH priority-adjusted data of the UE. After receiving the UL grants, the UE may prioritize the resource allocation for the LCHs with LCH priority-adjusted data according to the additional priorities. It could happen that during the LCP procedure for a UL grant the UE has allocated radio resource for the LCHs with LCH priority-adjusted data until the corresponding Bj’s are reduced to zero, but there can be still available capacity for the UL grant and there can be still remaining LCH priority-adjusted data for some of these LCHs. According to the current LCP procedure, the UE will allocate the remaining resource of the UL grant for the other LCHs of lower priorities though the intention of the UL grant is for transmission of the LCH priority-adjusted data from the gNB perspective. After radio resource allocation for the other LCHs, there may be no or not enough remaining resources to transmit the remaining LCH priority-adjusted data.

- Fixed lower Bj bound (i.e. 0) can result in that some capacity of UL grants allocated for the intention of LCH priority-adjusted data transmission is used for the data transmission of the other LCHs of lower priorities.

To conquer that the UL grant intended for LCH priority-adjusted data transmission cannot be efficiently used for transmission of LCH priority-adjusted data due to Bj restriction, one candidate solution is to temporarily allow UE to allocate radio resources for LCH priority-adjusted data transmission even when the corresponding Bj is negative.  The data of lower priority LCHs, which can be more delay-tolerant, is deferred for future transmission when the UL grants intended for the lower priority LCHs are received.

According to the discussion in previous meetings, there is a concern that allowing a UE to allocate resource for LCH priority-adjusted data when Bj is negative may corrupt the fairness between LCHs and cause starvation of low priority service. Figure 1 below is used as an example to illustrate the related impact on the data transmission of the LCH with lower priorities. By allowing a UE to allocate resource for LCH priority-adjusted data for the first set of LCHs when the corresponding Bj’s are negative, the UE can temporarily “preempt” some radio resources for LCH priority-adjusted data transmission from the second set LCHs of lower priorities. When a later UL grant is received in case that all LCH priority-adjusted data of the first set of LCHs has been transmitted/or discarded, the Bj’s of the first set of LCHs may be negative or small positive values and all or most of the resources of the UL grant can be allocated for the second set of LCHs. In such sense, the second set of LCHs will be compensated with more radio resources when the congestions are resolved for the first set of LCHs.

Figure 1 impact on low priority data transmission due to Bj enhancements

In other words, allowance of radio resource allocation for LCH priority-adjusted data when the corresponding Bj’s are negative, can be regarded that the LCHs with LCH priority-adjusted data temporarily borrows some radio resources from the LCHs of lower priorities for LCH priority-adjusted data transmission and the borrowed resources with be returned to the LCHs of lower priorities after congestion resolution.

- By allowing LCH priority-adjusted data transmission when its Bj is negative, the LCHs with LCH priority-adjusted data temporarily can borrow some radio resources from the LCHs of lower priorities, for quick transmission of LCH priority-adjusted data.

- The borrowed radio resources by the LCHs with LCH priority adjusted-data will be returned to the LCHs of lower priorities when the congestions are resolved for the LCHs with LCH priority-adjusted data.

Based on the above discussion, temporary allowance of LCH priority-adjusted data transmission for an LCH with negative Bj can help timely LCH priority-adjusted data transmission without clearly decrease the radio resource for the data transmission of the LCHs of lower priorities. There may be certain delay increase for the data transmission of the LCHs of lower priorities, but this should be acceptable due to the LCHs of lower priorities is expected to be more delay tolerant than the LCHs with LCH priority-adjusted data.

- Support to temporarily allow UE to transmit LCH priority-adjusted data for an LCH when its Bj is negative.

To avoid the situation that a huge amount of LCH priority-adjusted data for an LCH consumes all radio resources and no radio resource is available for LCHs of lower priorities in a long period, a per-LCH negative Bj bound can be introduced for LCH priority-adjusted data transmission (see Figure 1). When the negative Bj bound is configured for an LCH, the UE is only allowed to allocate radio resource for the LCH priority-adjusted data of the LCH only when the corresponding Bj is above the negative Bj bound. In such way, the starvation risk of LCHs with lower priorities can be avoided.

- Introduce a configurable negative bound for Bj per LCH for LCH priority-adjusted data transmission.

- If a negative Bj bound is configured for an LCH, the UE can transmit the LCH priority-adjusted data of the LCH when the corresponding Bj is above its negative Bj bound.

According to the current MAC specification, the maximum Bj value for an LCH is PBR × BSD. For simplicity, the negative Bj bound for the LCH can be configured in relation to the maximum Bj value by using a coefficient within [-1.0, 0). When the UE receives the negative Bj coefficient, it derives the negative Bj bound (i.e. “negative Bj coefficient × PBR × BSD”) and applies the negative Bj bound for LCH priority-adjusted data transmission.

- The negative Bj bound of an LCH is configured via a negative coefficient (in range [-1.0, 0)) of maximum Bj value (i.e. PBR × BSD) of the LCH. The negative Bj bound for the LCH is “negative Bj coefficient × PBR × BSD”.

The related text proposals for MAC and RRC are provided based on the running CRs to illustrate the specification impact of the above proposals.

- If the above proposals are agreeable, RAN2 to consider the text proposals for RRC and MAC attached in Annex A and Annex B.

- Conclusion

In this contribution, we have discussed allowing LCH priority-adjusted data transmission for an LCH when its Bj is negative and impact on the transmission of the LCHs of lower priorities. Based on the discussions, there are the following observations:

Observation 1	Fixed lower Bj bound (i.e. 0) can result in that some capacity of UL grants allocated for the intention of LCH priority-adjusted data transmission is used for the data transmission of other LCHs of lower priorities.

Observation 2	By allowing LCH priority-adjusted data transmission when its Bj is negative, the LCHs with LCH priority-adjusted data temporarily borrows some radio resources from the LCHs of lower priorities, for quick transmission of LCH priority-adjusted data.

Observation 3	The borrowed radio resources by the LCHs with LCH priority adjusted-data will be returned to the LCHs of lower priorities when the congestions are resolved for the LCHs with LCH priority-adjusted data.

Based on the above discussions and the observations, we have the following proposals regarding LCH priority-adjusted data transmission when Bj is negative:

Proposal 1	Support to temporarily allow UE to transmit LCH priority-adjusted data for an LCH when its Bj is negative.

Proposal 2	Introduce a configurable negative bound for Bj per LCH for LCH priority-adjusted data transmission.

Proposal 3	If a negative Bj bound is configured for an LCH, the UE can transmit the LCH priority-adjusted data of the LCH when the corresponding Bj is above its negative Bj bound.

Proposal 4	The negative Bj bound of an LCH is configured via a negative coefficient (in range [-1.0, 0)) of maximum Bj value (i.e. PBR × BSD) of the LCH. The negative Bj bound for the LCH is “negative Bj coefficient × PBR × BSD”.

Proposal 5	If the above proposals are agreeable, RAN2 to consider the text proposals for RRC and MAC attached in Annex A and Annex B.

- Annex A：Text proposal for RRC

Baseline of the TP: R2-250xxxx Running RRC CR for R19 XR_v02_Rapp.docx in [2].

*************************************************start of change********************************************************

### 6.3.2	Radio resource control information elements

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- Cond PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                   OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, ms0p01-v1700, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                              OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                                 OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}               OPTIONAL,    -- Need R

[[

allowedCG-List-r16                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL,   -- Need S

allowedPHY-PriorityIndex-r16        ENUMERATED {p0, p1}                                                 OPTIONAL    -- Need S

]],

[[

logicalChannelGroupIAB-Ext-r17      INTEGER (0..maxLCG-ID-IAB-r17)                                      OPTIONAL,   -- Need R

allowedHARQ-mode-r17                ENUMERATED {harqModeA, harqModeB}                                   OPTIONAL    -- Need R

]],

[[

enhancedLCP-r19                  SEQUENCE{

priorityAdjustmentThreshold-r19          INTEGER (1..64),

additionalPriority-r19              INTEGER (1..16),

negativeBjBoundCoefficient-r19	ENUMERATED {-1dot0, -0dot8, -0dot6, -0dot4, -0dot2}

...

}                                                                                                        OPTIONAL     -- Need R

]]

}                                                                                                       OPTIONAL,   -- Cond UL

...,

[[

channelAccessPriority-r16           INTEGER (1..4)                                                      OPTIONAL,   -- Need R

bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}                                   OPTIONAL    -- Need R

]]

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

additionalPriority The additional priority that overrides the logical channel priority configured by the field priority when the logical channel adjustment condition is satisfied as specified in TS 38.321 [3]. For the same logical channel configuration, the value of the field shall be smaller than that of the field priority.

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to “allowedCG-List” as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4 and DRBs.

allowedPHY-PriorityIndex This restriction applies only when the UL grant is a dynamic grant. If the field is present and the dynamic grant has a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to the dynamic grants indicating PHY-priority index equal to the values configured by this field. If the field is present and the dynamic grant does not have a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to this dynamic grant if the value of the field is p0, see TS 38.213 [13], clause 9. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any dynamic grants. Corresponds to “allowedPHY-PriorityIndex” as specified in TS 38.321 [3].

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Corresponds to ‘allowedSCS-List’ as specified in TS 38.321 [3]. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to ‘allowedServingCells’ in TS 38.321 [3].

bitRateMultiplier Bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [3]. Value x40 indicates bit rate multiplier 40, value x70 indicates bit rate multiplier 70 and so on.

bitRateQueryProhibitTimer The timer is used for bit rate recommendation query in TS 38.321 [3], in seconds. Value s0 means 0 s, s0dot4 means 0.4 s and so on.

bucketSizeDuration Value in ms. Ms5 corresponds to 5 ms, value ms10 corresponds to 10 ms, and so on.

channelAccessPriority Indicates the Channel Access Priority Class (CAPC), as specified in TS 38.300 [2], to be used on uplink transmissions for operation with shared spectrum channel access in FR1. The network configures this field only for SRB2 and DRBs.

configuredGrantType1Allowed If present, or if the capability lcp-Restriction as specified in TS 38.306 [26] is not supported, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Otherwise, UL MAC SDUs from this logical channel cannot be transmitted on a configured grant type 1. Corresponds to ‘configuredGrantType1Allowed’ in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

logicalChannelGroup, logicalChannelGroupIAB-Ext ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to. The logicalChannelGroupIAB-Ext is only applicable to the IAB-MT. When logicalChannelGroupIAB-Ext is configured, logicalChannelGroup shall be ignored.

priorityAdjustmentThreshold Remaining time threshold for determining whether the additional logical channel priority configured by additionalPriority is applied for the logical channel, as specified in TS 38.321 [3]. Value in number of milliseconds.

logicalChannelSR-Mask Controls SR triggering when a configured uplink grant of type1 or type2 is configured. True indicates that SR masking is configured for this logical channel as specified in TS 38.321 [3].

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to false if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to “maxPUSCH-Duration” in TS 38.321 [3]. The PUSCH duration is calculated based on the same length of all symbols, and the shortest length applies if the symbol lengths are different.

negativeBjBoundCoefficient Indicates during the LCP procedure of a logical channel, a negative coefficient to calculate the negative Bj bound for transmission of the data with remaining time of the running PDCP discardTimer below the configured priorityAdjustmentThreshold of the logical channel as specified in TS 38.321 [3]. Value -1dot0 means -1.0, value -0dot8 means -0.8, value -0dot6 means -0.6, value -0dot4 means -0.4, value -0dot2 means -0.2.

Priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. Value kBps0 corresponds to 0 kiloBytes/s, value kBps8 corresponds to 8 kiloBytes/s, value kBps16 corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| PDCP-CADuplication | The field is mandatory present if the DRB/SRB associated with this logical channel is configured with PDCP CA duplication in UL in the cell group in which this IE is included (i.e. the PDCP entity is associated with multiple RLC entities belonging to this cell group). Otherwise the field is optionally present, need R. |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB or multicast MRB. It is optionally present, Need R, for a logical channel with uplink if it serves an SRB. Otherwise it is absent. |

*************************************************end of change********************************************************

- Annex B：TP for MAC

Baseline of the TP: draft R2-250xxxx R19 XR MAC running CR_final_r1.docx in [3].

*************************************************start of change********************************************************

5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2> for a logical channel configured with priorityAdjustmentThreshold, if the smallest remaining value of the running PDCP discardTimers among its data available for this transmission, evaluated at the time of the first symbol of this transmission, is below or at its priorityAdjustmentThreshold:

3>  apply additionalPriority of this logical channel;

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 and no negativeBjBoundCoefficient configured, and logical channels selected in clause 5.4.3.1.2 for the UL grant with negativeBjBoundCoefficient configured and Bj ≥ the negative Bj bound (i.e. negativeBjBoundCoefficient × PBR × BSD), are allocated resources in a decreasing priority order. For a selected logical channel with negativeBjBoundCoefficient configured, only the data with remaining time of the running PDCP discardTimers lower than priorityAdjustmentThreshold is allowed to be transmitted when its Bj is within (negativeBjBoundCoefficient × PBR × BSD, 0]. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain:

3> if a logical channel has applied additionalPriority above; and

3> if none of its remaining data available for this transmission has a running PDCP discardTimer whose remaining value, evaluated at the time of the first symbol of this transmission, is below or at its priorityAdjustmentThreshold:

4> apply priority of this logical channel;

3> all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. For this allocation of resources, logical channels applied with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query, or MAC CE for UL bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

*************************************************end of change********************************************************

- References

- RAN2#129bis Chair notes

- [POST129bis][503][XR] RRC running CR and open issues

- [POST129bis][502][XR] MAC running CR and open issues