3GPP TSG-RAN WG2 Meeting #130		R2-2503626

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on open issues for asymmetric DL sTRP/UL mTRP

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post129bis][215][MIMO_Ph5] Running CR for 38.321 (Samsung), one open issue is identified as below [1]:

MAC-4: TCI state activation MAC CE for asymmetric DL sTRP/UL mTRP Proposal 8: Discuss whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP.

We understand the TCI state activation MAC CE design to enable asymmetric DL sTRP/UL mTRP scenarios should also be discussed considering that RAN1 has concluded new TCI state activation behavior:

Agreement (RAN1#116) For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1 Conclusion (RAN1#117) For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In the email discussion on [Post129bis][214][MIMO_Ph5] Running CR for 38.331 (Ericsson), another MAC open issue is listed as below [2]:

| Asymmetric DL sTRP/UL mTRP | How to capture PL offset behaviour in RRC for handover and resume case. | To be discussed in contributions |
| --- | --- | --- |

In this contribution, we will discuss the above open issues related to asymmetric DL sTRP/UL mTRP, including MAC and RRC impacts.

- Discussion

  - TCI state activation/deactivation MAC CE (MAC-4)

Based on the conclusion drawn in RAN1 as below,

- When Rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When Rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The existing “Unified TCI state activation/deactivation MAC CE” introduced in Rel-17 can be directly reused for asymmetric DL sTRP/UL mTRP deployment scenario when Rel-17 unified TCI/ICBM is configured, because it does activate codepoint(s) each mapping to one joint TCI state for joint TCI state mode or a full set or sub-set of {one DL TCI state, one UL TCI state} for sperate TCI state mode.

For asymmetric DL sTRP/UL mTRP deployment scenario when Rel-18 unified TCI is configured, however, “Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States” and “Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States” introduced in Rel-18 cannot directly achieve the required asymmetric number of DL TCI state and UL TCI states, i.e., the TCI state codepoint mapping to the full set or subset of the following cases:

- For FR1,

  - {one joint TCI state for DL, two joint TCI states for UL} for joint TCI state mode

  - {one DL TCI state, two UL TCI state} for separate TCI state mode

- For FR2,

  - {one DL TCI state, two UL TCI state} for separate TCI state mode

This is because {two joint TCI states} or {two DL TCI states, two UL TCI states} are always kept if the Rel-18 MAC CE is used, according to the following cited paragraph in TS 38.214:

“When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states, if the UE receives a TCI codepoint mapped with a sub-set of first and second TCI-State(s) and/or a sub-set of first and second TCI-UL-State(s), the UE shall update the first/second TCI-State(s) and/or first/second TCI-UL-State(s) mapped to the TCI codepoint, when applicable, and keep the previously indicated first/second TCI-State(s) and/or first/second TCI-UL-State(s) that is/are not updated by the TCI codepoint.”

To support single joint/DL TCI state for DL, further clarification is required because it is different from Rel-18 TCI state framework.

Whether to reuse two indicated joint/DL TCI states framework in Rel-18 for the asymmetric DL sTRP/UL mTRP deployment scenario should be further discussed because it may go beyond the UE capability, even though in some cases single joint/DL TCI state can be applied for DL transmission by following configurations or indications, for example:

- For PDCCH, apply the first or the second TCI state of the two indicated joint/DL TCI states according to applyIndicatedTCI-State configured in ControlResourceSet IE

- For PDSCH scheduled or activated by DCI format 1_0,

  - apply the first or the second TCI state of the two indicated joint/DL TCI states according to applyIndicatedTCI-StateDCI-1-0 configured in BWP-DownlinkDedicated IE;

  - If the UE is not configured with applyIndicatedTCI-StateDCI-1-0, apply the first indicated joint/DL TCI state.

- For PDSCH scheduled or activated by DCI format 1_1/1_2,

  - When the UE is configured with tciSelection-PresentInDCI, apply the first or the second TCI state of the two indicated joint/DL TCI states according to the indication of TCI selection field in DCI format 1_1/1_2;

  - If the UE is not configured with tciSelection-PresentInDCI, apply both indicated joint/DL TCI states;

  - Note that support of DCI format 1_1 and if supported 1_2 configured with TCI selection field is optional with capability signaling.

The drawbacks of directly reusing Rel-18 Unified TCI States Activation/Deactivation MAC CE without any revision for Rel-19 asymmetric DL sTRP/UL mTRP deployment scenario are:

- Additional signaling is needed to distinguish between Rel-18 sDCI based mTRP and Rel-19 asymmetric DL sTRP/UL mTRP

- For joint TCI state mode, indicating two same joint TCI states does not work because same joint TCI states have to be applied to UL transmissions for different TRPs.

- For both joint TCI state mode and separate TCI state mode, indicating two different joint/DL TCI states,

  - Does not work for one or multiple PDSCH reception scheduled/activated by DCI format 1_1/1_2 without TCI selection field, e.g., when the UE does not support the capability of TCI selection field, because both different indicated joint TCI states should be applied for the PDSCH.

  - TCI selection field in DCI format 1_1/1_2 is a must and which is redundant even if the UE supports the capability of TCI selection field since there is only one indicated DL TCI state.

- For separate TCI state mode, indicating two same DL TCI states leads to some restricted or even undefined DL transmission schemes for UEs not supporting the capability of TCI selection field, e.g.,

  - When two DL TCI states are indicated and DMRS ports within two CDM groups are indicated in the DCI, which is the SDM scheme of Rel-16 sDCI-based mTRP, only single CW can be scheduled with transmission layers limited to one of 1+1, 1+2, 2+1, 2+2; in addition, DMRS ports within two CDM groups occupy more REs compared to DMRS ports within one CDM group; what’s more, according to RAN1 spec, PDSCH repetition is not allowed.

  - RAN1 does not specify the transmission scheme when two DL TCI states are indicated and DMRS ports within one CDM groups are indicated in the DCI.

To achieve a single joint/DL TCI state for DL and two joint/UL TCI states for UL, following options can be considered:

- Option 1: Introduce a new MAC CE with a full set or subset of {two joint TCI states} mapped to all codepoints for joint TCI state mode, where one of the two joint TCI states can be applied to DL, and both joint TCI states can be applied to UL; and introduce a new MAC CE with a full set or subset of {one DL TCI state, two UL TCI states} mapped to all codepoints for separate TCI state mode.

- Option 2: Revise Rel-18 “Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States” and “Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States” with a signaling to indicate that a full set or subset of {two joint TCI states} are mapped to all codepoints for joint TCI state mode, where one of the two joint TCI states can be applied to DL, and both joint TCI states can be applied to UL; or a full set or subset of {one DL TCI state, two UL TCI states} mapped to all codepoints for separate TCI state mode.

- Option 3: Revise Rel-18 “Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States” with an indication per codepoint to indicate whether the codepoint maps to a full set or subset of {two joint TCI states}, where one of the two joint TCI states can be applied to DL, and both joint TCI states can be applied to UL, or the codepoint maps to a full set or subset of {two joint TCI states} where both joint TCI states can be applied to DL and UL; and Revise Rel-18 “Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States” with an indication per codepoint to indicate whether the codepoint maps to a full set or subset of {one DL TCI state, two UL TCI states}, or the codepoint maps to a full set or subset of {two DL TCI states, two UL TCI states}.

(MAC-4) Send an LS to RAN1 to check whether to design new MAC CE(s) for the TCI States Activation/Deactivation or to extend Rel-18 Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States and Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States for the asymmetric DL sTRP/UL mTRP scenarios.

  - PL offset behavior in handover and resume case (RRC-x)

According to the WI description below: 5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 a. Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. b． Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

- We think no extra behavior should be defined for handover and resume case. UL cell definition is precluded from the WID, which means UL transmission can be scheduled towards DL/UL TRP of the target cell even though PL offset is not configured when handover or resume occurs. If the target cell is configured with PL offset,  whether to use PL offset is totally up to network scheduling.

(RRC-x) No extra behavior should be defined for handover and resume case.

- Conclusion

In this contribution, we discuss the RAN2 MAC and RRC open issues for asymmetric DL sTRP/UL mTRP. Based on the discussion above, we have the following proposals:

MAC open issue:

- (MAC-4) Send an LS to RAN1 to check whether to design new MAC CE(s) for the TCI States Activation/Deactivation or to extend Rel-18 Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States and Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States for the asymmetric DL sTRP/UL mTRP scenarios.

RRC open issues:

(RRC-x) No extra behavior should be defined for handover and resume case.

- References

- R2-2504185, Open issues for MIMO on 38.331,	Ericsson

- R2-2504210, Report of MAC open issues for MIMO, Sumsung