3GPP TSG-RAN WG2 Meeting #130	R2-2503662

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2-127bis meeting, RAN2 agreed to continue working on a Contention-Based (CB) msg3 EDT mechanism as well as the Diversity Slotted-Aloha (DSA).In RAN2-128 meeting, RAN2 progressed well on how to support CB-Msg3 in both SA and DSA cases but many FFS remain. In RAN2-129 meeting, RAN2 agreed the working assumption on the CB-Msg3 transmission window and Msg4 enhancement.In RAN2-129bis meeting, RAN2 agreed single Msg4 monitoring window and single RNTI for CB-Msg3 EDT. The corresponding agreements for CB-Msg3 enhancement can be found in the Annex.

In the post-129bis meeting email discussion[3], below open issues were identified to be further discussed.

Proposal 7: RAN2 to discuss below open issues for CB-Msg3-EDT procedure. - MAC-2: CB-RNTI calculation - MAC-7: Whether the HARQ operation is applicable to transmit CB-Msg3. - MAC-9: Whether NW/UE processing time is needed when determine the Msg4 monitoring starts. - MAC-10: FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. - MAC-11: Whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in NB-IoT CP solution - MAC-12: FFS how the multiplexing is organized for CB-MSG4. - MAC-13: FFS new MAC PDU format for CB-Msg4 - MAC-14: FFS for the detail of HARQ operation on CB-Msg4. - MAC-15: What should be the UE behavior (e.g. the can initiate the legacy 4-step RA) when the CB-Msg3 procedure fails. - MAC-17: Whether to allow multiple TBSs as in EDT. - MAC-18: How to model the CB-Msg3 response window (i.e. MSG4 monitoring window) ? Should it be a timer as in legacy RA response window, and what should be the value range.

In this contribution, we provide further view on the remaining issues for CB-msg3 EDT as well as the enhancement for Msg4.

# 2	Discussion

## 2.1	CB-Msg3 replica transmission

In RAN2-129bis meeting, RAN2 agreed both SA and DSA are mandatorily supported by UE if it supports CB-Msg3-EDT. Based on that, one single procedure to support both DSA and SA will be specified by RAN2. In other words, SA is a special setting (k=1) of the overall procedure. However, in the online discussion, it seems there was different understanding on whether the UE can send only one replica when network configures more than one replica to be transmitted by the UE within the CB-Msg3 EDT transmission window (i.e., the configured number of replicas k > 1). In our understanding, the mandatory support of DSA means the UE should always follow the network configuration on the number of replica transmissions. Otherwise, the reduction on the number of the replica transmissions may reduce the gain of DSA and cause the network resource waste. Furthermore, RAN2 agreed the Msg4 monitoring starts at the end of transmission window plus (at least) UE-eNB RTT. If the UE only sends 1 replica within a transmission window which can accommodate K>1 replica occasions, it is an unnecessary delay for the Msg4 scheduling.

Proposal 1: RAN2 confirms that the UE shall transmit the exact number of replicas configured by the network for the CB-Msg3 transmission window.

## 2.2	Maximum TBS for CB-Msg3 EDT triggering (open issue MAC-17)

It was agreed in RAN2-128 meeting that the UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS. But it is open whether the maximum TBS threshold is the same or different for different CE levels. The maximum TBS threshold for CB-Msg3 is similar to the threshold used to decide whether UE can trigger the legacy EDT transmission. As defined in TS36.331 section 5.3.3.1b, the UE can initiate legacy CP or UP EDT when the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS. The edt-TBS is defined as per-CE level, which is given in RACH-CE-LevelInfo-r13. Since the legacy EDT and CB-Msg3 EDT are similar from the Msg3 data transmission point of view, we believe the same principle used for legacy EDT can be applied to CB-Msg3 EDT. The per-CE level TBS threshold enables the flexibility to configure different thresholds considering the repetition request and resource cost for different CE levels.

edt-TBS Largest TBS for Msg3 for a CE level applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, b408 corresponds to 408 bits and so on. Additionally, value b1000or456 corresponds to 1000 bits for CE levels 0 and 1, and 456 bits for CE levels 2 and 3. See TS 36.213 [23].

Observation 1: The TBS threshold for legacy EDT is defined as per Coverage Enhancement level.

Proposal 2: The maximum TBS for CB-Msg3 can be configured on per Converage Enhancement (CE) level specific.

In legacy EDT, there is some optimization to enable UE to select smaller TBS than edt-TBS for Msg3.  The UEs can adapt their TBS selection based on their specific coverage conditions and pending data size, improving reliability and efficiency (e.g., to avoid padding bits to be transmitted in vain). If the UE selects a smaller TBS, the number of UL transmission repetitions should be adapted as well.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 in the corresponding CE level, as specified in TS 36.213 [23]. Excerpt from 36.213 section 16.3.3: … - if the UE is configured with higher layer parameter edt-SmallTBS-Enabled set to 'false', - the TBS is given by higher layer parameter edt-TBS - otherwise, - the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2 - the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where is the selected TBS for Msg3, and is given by higher layer parameter edt-TBS

For CB-Msg3, it worth to discuss whether the similar mechanism should be introduced. Or alternatively, NW may configure multiple TBS for a CE level hence UE can select the TBS based on its pending data size to avoid unnecessary bits padding.

Proposal 3: Below two options can be considered if RAN2 decides to introduce a flexible TBS selection for CB-Msg3.

- Option1: NW can configure multiple CB-Msg3 TBS for a CE level

- Option2: UE is allowed to select TBS smaller than the configured CB-Msg3 TBS for the corresponding CE level

## 2.3	CB-Msg3 Power Ramping

In RAN2-129bis meeting, RAN2 assumes power ramping should be supported for CB-Msg3-EDT for both eMTC and NB-IoT. RAN2 sent LS [2] for the RAN1 confirmation.  But for the RAN2 agreement, we noticed it is not clear the power ramping should happen between the Msg3 transmission windows or between the different replicas within a Msg3 transmission window. Typically, the UE shall increase its transmission power if it can tell the previous transmission is failed. However, for CB-Msg3, the Msg4 monitoring starts at the end of transmission window plus UE-eNB RTT. This means the Msg4 feedback can be provided to UE after the UE transmits all the replicas within the transmission window. In this sense, it is not reasonable for the UE to increase its power within a transmission window.

Proposal 4: The CB-Msg3 power ramping can only happen between CB-Msg3 transmission windows. There is no power ramping between different replicas within a CB-Msg3 transmission window.

Furthermore, for the different replicas within a CB-Msg3 transmission window, it is not clear whether the UE can change its CE level based on the measured RSRP. In principle, the UE measured RSRP may be changed before the replica transmissions hence the CE level can be changed as well. However, as the Msg4 monitoring window starts at the end of CB-Msg3 transmission window and the Msg4 monitoring window is a CE level specific configuration, it is simple to keep the CE level same within the Msg3 transmission window.

Proposal 5: The UE should keep the same CE level within a CB-Msg3 transmission window.

## 2.4	OCC for CB-Msg3

Based on the LS response from RAN1 [3], the detailed design for the OCC is still ongoing in RAN1 and whether the OCC can be applied to Msg3-EDT depends on the power imbalance between UEs. In the RAN2-129 and 129bis meeting, RAN2 discussed the power imbalance issue and the fragmentation of the PUSCH resource on the support of OCC for CB-Msg3.

RAN2-129 and 129bis meeting agreements: - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

The power imbalance issue cannot be fully addressed by defining a narrow RSRP ranges since the measured RSRP is for DL while the OCC requires small power imbalance between UEs in UL. Furthermore, the separate resource for non-OCC and OCC based CB-Msg3 will further segment the PUSCH resource on top of the per-CE level resource. In our view, RAN2 can therefore deprioritize the work on OCC for CB-Msg3, because it will increase the resource demand instead of reducing it for an unreliable feature.

Proposal 6: RAN2 to deprioritize the work on OCC for CB-Msg3.

## 2.5	CB-Msg4 monitoring window (open issue MAC-9 and 10)

In RAN2#129, it has been agreed that the Msg4 monitoring can start at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. FFS whether NW/UE processing time needs to be considered or not.

RAN2-129bis meeting agreements: - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

In TS36.321, the start time of the Msg3 response window were defined in both the legacy RACH and PUR, in which 4 ms were considered in PUR while it is not added in RACH contention resolution.

TS 36.321 5.1.5 for Contention Resolution: Once Msg3 is transmitted, the MAC entity shall: - if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: - if Msg3 is transmitted on a non-terrestrial network: … - start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission plus UE-eNB RTT. TS 36.321 5.4.7 for Msg3 response window in PUR: After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT.

For CB-Msg3, we tend to keep the specification simple and align the UE behaviour with the contention resolution timer as defined in legacy RACH procedure. Furthermore, the processing time may not be needed in order to start the transmission of CB-Msg4 earlier since the network can process the Msg3 transmission and prepare for Msg4 transmission well in advance for those UEs who transmit the Msg3 at the early part of CB-Msg3 transmission window.

Proposal 7: When the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, there is no need to consider additional delays for NW/UE processing time.

In RAN2#129bis meeting, it was discussed whether it will also be possible for the NW to configure the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. The main concern for this option is how to handle the UL/DL collision if the Msg4 monitoring window is overlapping with a replica transmission, i.e. whether the UE prioritizes the replica transmission or monitoring.

RAN2-129bis meeting agreements: - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

First of all, for full-duplex UEs, such as LTE UEs operating in enhanced coverage mode, the UE is capable enough to transmit and receive packets simultaneously. There is no UL/Msg3 and DL/Msg4 collision issue at all. Therefore, once the eNB successfully decodes one of the multiple Msg3 replicas, it may respond as early as possible (i.e., without waiting for the reception of the remaining replica(s)). The UE could stop transmitting the remaining replicas if it has received a CB-Msg4 containing a matching Contention Resolution Identity. The early termination of Msg3 replica transmissions would bring benfits such as reduing the UE power consumption, reduing latency and avoding PUSCH collisions between UEs.

Figure 1 below illustrates the scenario where the network early terminate the remaining replica transmission (planned by the UE to occur at O#4) after the network decode the first replica successfully at O#1. In this case, the network may send Msg4 for contention resolution before the end of the CB-Msg3 transmission window, therefore, the UE should start the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT.

Figure 1: Msg3 replica transmission ealry termination

Observation 2: For full-duplex UE, starting the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT does not result in any UL/DL collision. Instead, it enables early termination of Msg3 replica transmissions.

Observation 3: The early termination of Msg3 replica transmissions brings benefits such as reduced UE power consumption, reduced latency and avoidance of PUSCH collisions between UEs.

Proposal 8a: For full-duplex UEs, the UE can start the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT, without requiring additional enhancements for UL/DL collision handling.

Secondly, for half-duplex UE, it needs to be clarified what happens if the Msg4 monitoring window is overlapping with Msg3 replica transmission, i.e. whether the UE prioritizes the replica transmission or Msg4 monitoring. In our understanding, though the network is not aware of the exact Msg3 transmission occasions for a specific UE within the Msg3 transmission window, the network actually knows all the potential transmission occasions since all the CB-Msg3 PUSCH resource occasions are configured by network. The network may try to schedule Msg4 without collision to all possible Msg3 occasions. And if there is some collision between Msg4 repetitions and Msg3 replica repetitions, the UE should prioritize the UL transmission. On the contrary, if UE prioritizes DL monitoring, it is not desirable since the UE may not have a chance to transmit remaining Msg3 replicas within the duration of the Msg4 monitoring window.

Proposal 8b: For half-duplex UEs, the UE can start the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT, by prioritizing UL replica transmission when the Msg4 monitoring window overlaps with it.

If both proposals 8a and 8b are acceptable, this implies that, on top of proposal7, there are two possible options for how the UE starts the Msg4 monitoring window. However, since the UE initiates the CB-Msg3 transmission from RRC_IDLE, the network is not aware of which Msg4 window behavior the UE will employ. To avoid introducing additional signaling in CB-Msg3 to indicate the selected Msg4 window behavior, we propose that it would be simpler if the network can configure how the UE starts the Msg4 monitoring window.

Proposal 9: It is configurable for the network to define the option for how the UE starts its CB-Msg4 monitoring window.

## 2.6	CB-RNTI for CB-Msg4 monitoring (open issue MAC-2)

RAN2 has agreed to introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. For the CB-Msg3 scrambling, RAN2 has sent LS [4] to RAN1 to identify the potential RAN1 specification impact. For CB-Msg4 monitoring, RAN2 can further discuss how to derive the CB-RNTI from the transmission window.

RAN2-129bis meeting agreement: - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

There was some discussion on this issue in the post-meeting email discussion [3]. For the proposal to allocate one fixed CB-RNTI for all the CB-Msg3 transmission windows and all the CB-Msg4 monitoring windows. The main concern with this approach is that, if the CB-Msg4 windows are overlapped, with the single CB-RNTI for Msg4 monitoring, the UE may be required to decode a large number of irrelevant CB-Msg4 with response to other CB-Msg3 transmission windows. This could lead to unnecessary power consumption due to false decoding.

In our understanding, the different CB-RNTIs may still be needed for different Msg3 transmission windows if Msg4 monitoring windows corresponding to the different Msg3 transmission window are partially overlapping. For instance as illustrated in Figure 2, only different CB-RNTI is needed for monitoring Msg4 corresponding to Msgs Tx window#0 and Msg3 Tx window#1 as they are partially overlapping, but corresponding to different Msg3 Tx windows. The RNTI for monitoring Msg4 in Msg4 monitoring window #0 and #2 can be reused.

Proposal 10: The RNTI for CB-Msg4 reception can be reused by different Msg3 transmission windows if the Msg4 monitoring windows corresponding to the different Msg3 transmission windows are not overlapping.

Figure 2 Illustration of the Msg3 transmission window and Msg4 monitoring window

So the RNTI may be derived by taking into account the Msg4 monitoring window size and Msg3 transmission window periodicity. For example, the RNTI for Msg4 reception may be derived using formular:

RNTI=X + Msg3_W_index mod (ceil (Msg4_WS/Msg3_WP)) + ceil (Msg4_WS/Msg3_WP)*carrier_id,

wherein:

- X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=1 or any other value,

- Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19.

- Msg4_WS is the window size of Msg4,

- Msg3_WP: is the transmission window periodicity of Msg3.

Proposal 11: RAN2 to discuss above formula for deriving RNTI for Msg4 monitoring.

## 2.7	CB-Msg4 for Contention Resolution (open issue MAC-11)

After a CB-Msg3 transmission, the contention resolution mechanism should be supported because multiple UEs may send Msg3 to network on the same/shared PUSCH. In legacy 4-step RACH, the RACH collisions are handled using the contention resolution scheme, which relies on uniquely identifying the UEs in Msg3 (i.e., first 48 bits of CCCH SDU) and NW acknowledgement in Msg4. For CB-Msg3 transmission in shared resource, the most straightforward way is to reuse the contention resolution scheme in 4-step RACH.

Proposal 12: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

In last meeting, it was agreed L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported since it is hard to include the Contention Resolution Identification information. But there is no conclusion on whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. In our view, if the CB-Msg4 includes the Contention Resolution Identity MAC CE only, it can save some RRC message to move the UE to RRC idle, at least for CP solution.

Proposal 13: For CP solution, the CB-Msg3 procedure can be terminated by including the Contention Resolution Identity MAC CE only in CB-Msg4.

During the RAN2-129bis meeting, it was confirmed that a single CB-Msg4 can carry multiple Contention Resolution (CR) identities to support UE multiplexing. This may result in the network having to transmit multiple CB-Msg4s because the CR ID MAC CE (48 bits) of all UEs, transmitting in the CB-Msg3 window, may not fit in a single CB-Msg3.

The legacy MAC specification defines a UE shall declare the Contention Resolution not successful if the UE receives a MAC PDU addressed to its temporary C-RNTI (corresponding to the CB-RNTI for the CB-Msg3 solution) and if the MAC PDU does not contain a CR ID MAC CE matching the CCCH SDU bits of the Msg3. Given that the network may transmit multiple CB-Msg4s for a single CB-Msg3 window, this part of the specification needs to be updated to allow the UE to continue CB-Msg4 monitoring if a first received CB-Msg4 does not contain the CR ID MAC CE corresponding to the UE’s Msg3.

Observation 4: In legacy MAC specification, the UE declares Contention Resolution failure if a Msg4 addressed to the UE does not contain the CR ID MAC CE corresponding to the UE’s Msg3, but for CB-Msg3 the UE may have to receive multiple CB-Msg4s before the UE finds the matched CR ID.

As a result, a UE must decode every CB-Msg4 received throughout the Msg4 monitoring window to determine whether any of the included CR IDs matches its own. Given that multiple UEs may share the same CB-Msg3 transmission window and corresponding Msg4 monitoring window, the UE may end up decoding many irrelevant CB-Msg4 messages. False decoding of CB-Msg4 leads to significant power consumption at the UE, which is a major concern for battery-limited devices.

RAN2-129bis agreement: - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

Therefore, to save UE’s power consumption, the network may need to provide assistance information to help the UE determine whether to continue monitoring for CB-Msg4, as more than one CB-Msg4 may be sent by the network in response to more than one CB-Msg3 transmitted in the same CB-Msg3 transmission window by UEs and received successfully by the network.

Proposal 14: The network may provide assistance information to help the UE determine whether to continue monitoring for CB-Msg4 until the end of the Msg4 monitoring window.

## 2.8	CB-Msg3 EDT fallback and CE level increase (open issue MAC-15)

In the post-meeting email discussion [3], it is not clear whether UE will be in the next CE level when CB-Msg3 re-attempt number has reached a threshold.

### 2.8.1	UE initiated CE level change or fallback:

If the number of contention resolution failures for CB-Msg3 EDT reaches a threshold, the UE may perform below options:

- Option1: Increase the CE level and perform the CB-Msg3 re-attempts in new CE level

- Option2: Fallback to 4-step RACH or EDT or even PUR

In our understanding, if the contention resolution failures are caused by bad RF conditions, it is reasonable to have a higher CE level which is typically configured with higher number of repetitions to support more robust transmission scheme. However, if the transmission failures are caused by resource collision due to the limited CB-Msg3 resource for the selected CE level, it is not good to move to the next CE level since it will consume more UE energy to transmit unnecessary a larger number of repetitions and it will waste more system resource. Instead, we think it is better to fallback to 4-step RACH/EDT and select proper CE level for subsequent preamble transmissions. Considering the amount of PUSCH resources which can be allocated for a CB-Msg3 occasion is less than the preamble resources (e.g., 64) which can be allocated for a RO, the CB-Msg3 PUSCH resource collision may be worse than the legacy RACH procedure. To control the unnecessary CE level increase due to CB-Msg3 PUSCH resource collision instead of bad RF conditions, we think the UE should evaluate its latest measured RSRP to decide whether it should increase the CE level or fallback to 4-step RACH/EDT or PUR.

Proposal 15: If the number of contention resolution failures for CB-Msg3 EDT reaches a threshold, the UE should evaluate its latest measured RSRP to decide whether it should increase the CE level or fallback to 4-step RACH/EDT or PUR.

Proposal 16: If the measured RSRP is significantly better than the threshold to select the next robust CE level, the UE should not increase the CE level for CB-Msg3 re-attempts. Instead, it should fallback to 4-step RACH/EDT or PUR procedure.

Therefore, on top of the RSRP threshold for initial CE level selection, RAN2 can introduce an offset to decide whether to increase the CB-Msg3 CE level. If the latest measured RSRP is better than the threshold for next CE level plus the offset, the UE should not increase the CE level but can instead fall back to perform 4-step RACH or PUR.

Proposal 17: On top of the RSRP threshold for initial CE level selection, introduce an offset to decide whether to increase the CB-Msg3 CE level.

If the UE decides to fallback to legacy RACH or PUR procedure, there was no consensus on how to model the fallback procedure in RAN2-129bis meeting (e.g., whether RRC layer should be informed when the MAC layer has determined the CB-Msg3 procedure is failed hence an UE fallback is expected).

RAN2 agreed to reuse the existing CP-EDT and UP-EDT RRC messages for CB-Msg3 EDT. Depending on the type of fallback procedure following CB-Msg3 EDT (e.g., MO-EDT, legacy 4-step RACH, or PUR), the RRC message carried in CB-Msg3 and the one carried in Msg3 of the subsequent fallback procedure may or may not be changed.

RAN2-129bis meeting agreement: 1. RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject - For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

If the UE fall back from CB-Msg3 to MO-EDT, it seems the RRC messages included in CB-Msg3 and Msg3 for MO-EDT is not changed. If the UE fallback from CB-Msg3 to legacy 4-step RACH, the RRC message can be changed from RRCEarlyDataRequest to RRCConnectionRequest. In this sense, when the MAC layer determines CB-Msg3 has failed it shall indicate the failure to RRC layer. However, it will be beneficial if the UE maintains the CB-Msg3 MAC PDU such that the UE data can be appended to the message of the fallback procedure (4-step RA or PUR). In our view, the RRC layer can be responsible for deciding whether 4-step RA or PUR is selected for fallback. If the fallback is to 4-step RACH without MO-EDT a new RRC message may be needed, while if the fallback is to MO-EDT or PUR, the CB-Msg3 RRC message may be reused.

Proposal 18: Fallback to 4-step RA/PUR is controlled by RRC upon indication from MAC. The UE maintains the CB-Msg3 MAC SDU to include the data in the fallback transmission.

### 2.8.2	NW initiated CE level change or fallback:

In addition, from network point of view, if the network detects high overload condition in the cell (e.g., the CB-Msg3 resource is in congestion in one or all CE levels), the network may indicate UE to change a CE level or offload the UEs from re-attempting over the CB-Msg3 resource and ask UEs to fall back to legacy 4-step RACH or EDT.

In this case, the network can issue a fallback indicator in the CB-Msg4 to immediately order fallback to legacy RACH procedure. The UEs who decodes the CB-Msg4 but without matching contention resolution identity should trigger fallback. If many UEs fall back to 4-step RACH or EDT procedures, it may shift the congestion issue to the preamble domain. Similar to the backoff for preamble re-attempt, the UEs performing fallback from CB-Msg3 to preamble transmission shall apply a backoff time before transmitting the preamble. Additionally, if only one CE level resource is in congestion, the NW can indicate UE to switch to a new CE level with less resource load in the next CB-Msg3 re-attempt.

Proposal 19a: Network-indicated fallback mechanism is supported for CB-Msg3 EDT. The UEs with failed contention resolution shall fall back to legacy 4-step RACH and EDT procedure if it receives a fallback indicator in CB-Msg4.

Proposal 19b: The network can also indicate UE to change the CE level for the next CB-Msg3 re-attempt.

## 2.9	CB-Msg4 Content (including open issue MAC-14)

RAN2 agreed that a single CB-Msg4 can address multiple UEs. Some candidate content for CB-Msg4 was identified, but many details remain FFS.

RAN2 agreements: - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized. - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject - For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject. - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. - Backoff information could be included in CB-Msg4. - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

Regarding the remaining FFS on HARQ feedback for CB-Msg4, if multiple UEs are scheduled within a single CB-Msg4, each UE that successfully decodes the message needs to provide HARQ feedback to the network. However, the regular DCI used for Msg4 scheduling provides only a single PUSCH resource for feedback, which cannot support multiple UEs. Therefore, it is proposed to reuse the mechanism defined for MsgB (successRAR) in the 2-step RACH procedure. Specifically, multiple HARQ feedback resources should be indicated in the CB-Msg4 to allow multiple UEs to transmit their HARQ acknowledgments.

Proposal 20: A new HARQ-Ack resource indicator could be included in the CB-Msg4 to indicate multiple HARQ feedback resource for multiple HARQ feedback.

For the other FFS regarding timing alignment information, we think it is network implementation to decide whether to issue a UE-specific TA command to adjust the UE’s TA. Therefore, the TAC MAC CE format in RAR can be reused and the MAC CE is UE-specfic and optional presence.

Proposal 21: Reuse the existing TAC MAC CE format for CB-Msg4. The TAC MAC CE is UE-specific and optional presence.

For MAC SDU for RRC message, one issue to be discussed is whether one or multiple RRC messages can be included in one CB-Msg4. In 2-step RACH, the MsgB MAC PDU may include one MAC SDU for RRC message. For CB-Msg4, in our understanding, one or more MAC SDUs can be included in CB-Msg4 to include the RRC messages. It is up to NW implementation to decide the number of RRC messages in one CB-Msg4 MAC PDU.

Proposal 22: Multiple RRC messages (MAC SDUs) can be included into one CB-Msg4.

Additionally, as discussed in section 2.6 and 2.8.2, the network may also include more elements in CB-Msg4 such as further control information for assisting Msg4 monitoring or network initiated fallback or CE level change for CB-Msg3 reattempt. These elements can be further discussed based on conclusion reached for the proposals.

In summary, the potential CB-Msg4 content may include: the Contention Resolution ID, the RRC message, the C-RNTI, the Timing Advance information, the Backoff infomration and the HARQ feedback informtion. FFS on the other elements for NW control information.

Proposal 23: RAN2 confirms the potential CB-Msg4 content may include:

- Contention Resolution Identities

- RRC message for CB-Msg3 response

- C-RNTI

- Timing Advance information

- HARQ feedback information

- Backoff information

- FFS on other NW control information (such as information for assisting Msg4 monitoring, network initiated fallback or CE level change)

## 2.10	CB-Msg4 Structure (open issue MAC-12 and 13)

In the 2-step RACH, one or multiple UEs may be responded by network in the MsgB by including multiple successRAR MAC subPDU. For each of the UE, network can include one SuccessRAR in the MsgB. The SuccessRAR may include the CR ID, the C-RNTI, the TA information, the HARQ feedback resource and some other elements for that specific UE. Additionally, the BI indicator is a common IE which is shared by all the UEs who decode the MsgB.

Figure 6.1.5a-4: Example of a MSGB MAC PDU with MAC SDU(s)

Figure 6.1.5a-5: Example of a MSGB MAC PDU without MAC SDU(s)

Figure 6.2.3a-2: successRAR

Observation 5: For MsgB in 2-step RACH, multiple UEs can be responded in one message by including multiple successRAR in one MsgB.

Observation 6: Each SuccessRAR includes the CR ID, the C-RNTI, the TA information and the HARQ feedback resource for a specific UE.

For CB-Msg4, we think a new MAC PDU format should be defined to support multiple UEs multiplexing. Similar to the MsgB, the new CB-Msg4 MAC PDU can include the information for multiple UEs.

Proposal 24: Define a new MAC PDU format for CB-Msg4. The MsgB format in 2-step RACH can be the baseline.

To define the exact information to be included for each UE (e.g., similar to the content of successRAR), RAN2 need to first conclude the mandantory information and optional information to be deliverd from netwrok to the UE. For example, we think the UE contention resolution ID is the only mandantory element to be delvierd to UE. The other IEs (e.g., C-RNTI, TAC, HARQ feedback resource and the following RRC message) are optional presence for the specific UE.

Proposal 25: For the exact information to be included for each UE, the UE contention resolution ID is the only mandantory element to be delvierd to UE. All the other IEs are optional presence.

If RAN2 can agree the mandantory and optional IEs in the CB-Msg4, RAN2 can further discuss the MAC subheaders for the MAC PDU.

# 3	Conclusion

This document has made the following observations and proposals:

CB-Msg3 replica transmission

Proposal 1: RAN2 confirms that the UE shall transmit the exact number of replicas configured by the network for the CB-Msg3 transmission window.

Maximum TBS for CB-Msg3 EDT triggering (open issue MAC-17)

Observation 1: The TBS threshold for legacy EDT is defined as per Coverage Enhancement level.

Proposal 2: The maximum TBS for CB-Msg3 can be configured on per Converage Enhancement (CE) level specific.

Proposal 3: Below two options can be considered if RAN2 decides to introduce a flexible TBS selection for CB-Msg3.

- Option1: NW can configure multiple CB-Msg3 TBS for a CE level

- Option2: UE is allowed to select TBS smaller than the configured CB-Msg3 TBS for the corresponding CE level

CB-Msg3 Power Ramping

Proposal 4: The CB-Msg3 power ramping can only happen between CB-Msg3 transmission windows. There is no power ramping between different replicas within a CB-Msg3 transmission window.

Proposal 5: The UE should keep the same CE level within a CB-Msg3 transmission window.

OCC for CB-Msg3

Proposal 6: RAN2 to deprioritize the work on OCC for CB-Msg3.

CB-Msg4 monitoring window (open issue MAC-9 and 10)

Proposal 7: When the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, there is no need to consider additional delays for NW/UE processing time.

Observation 2: For full-duplex UE, starting the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT does not result in any UL/DL collision. Instead, it enables early termination of Msg3 replica transmissions.

Observation 3: The early termination of Msg3 replica transmissions brings benefits such as reduced UE power consumption, reduced latency and avoidance of PUSCH collisions between UEs.

Proposal 8a: For full-duplex UEs, the UE can start the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT, without requiring additional enhancements for UL/DL collision handling.

Proposal 8b: For half-duplex UEs, the UE can start the Msg4 monitoring window in the subframe containing the last PUSCH repetition of the first replica plus UE-eNB RTT, by prioritizing UL replica transmission when the Msg4 monitoring window overlaps with it.

Proposal 9: It is configurable for the network to define the option for how the UE starts its CB-Msg4 monitoring window.

CB-RNTI for CB-Msg4 monitoring (open issue MAC-2)

Proposal 10: The RNTI for CB-Msg4 reception can be reused by different Msg3 transmission windows if the Msg4 monitoring windows corresponding to the different Msg3 transmission windows are not overlapping.

Proposal 11: RAN2 to discuss below formula for deriving RNTI for Msg4 monitoring.

RNTI=X + Msg3_W_index mod (ceil (Msg4_WS/Msg3_WP)) + ceil (Msg4_WS/Msg3_WP)*carrier_id,

wherein:

- X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=1 or any other value,

- Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19.

- Msg4_WS is the window size of Msg4,

- Msg3_WP: is the transmission window periodicity of Msg3.

CB-Msg4 for Contention Resolution (open issue MAC-11)

Proposal 12: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

Proposal 13: For CP solution, the CB-Msg3 procedure can be terminated by including the Contention Resolution Identity MAC CE only in CB-Msg4.

Observation 4: In legacy MAC specification, the UE declares Contention Resolution failure if a Msg4 addressed to the UE does not contain the CR ID MAC CE corresponding to the UE’s Msg3, but for CB-Msg3 the UE may have to receive multiple CB-Msg4s before the UE finds the matched CR ID.

Proposal 14: The network may provide assistance information to help the UE determine whether to continue monitoring for CB-Msg4 until the end of the Msg4 monitoring window.

CB-Msg3 EDT fallback and CE level increase (open issue MAC-15)

Proposal 15: If the number of contention resolution failures for CB-Msg3 EDT reaches a threshold, the UE should evaluate its latest measured RSRP to decide whether it should increase the CE level or fallback to 4-step RACH/EDT or PUR.

Proposal 16: If the measured RSRP is significantly better than the threshold to select the next robust CE level, the UE should not increase the CE level for CB-Msg3 re-attempts. Instead, it should fallback to 4-step RACH/EDT or PUR procedure.

Proposal 17: On top of the RSRP threshold for initial CE level selection, introduce an offset to decide whether to increase the CB-Msg3 CE level.

Proposal 18: Fallback to 4-step RA/PUR is controlled by RRC upon indication from MAC. The UE maintains the CB-Msg3 MAC SDU to include the data in the fallback transmission.

Proposal 19a: Network-indicated fallback mechanism is supported for CB-Msg3 EDT. The UEs with failed contention resolution shall fall back to legacy 4-step RACH and EDT procedure if it receives a fallback indicator in CB-Msg4.

Proposal 19b: The network can also indicate UE to change the CE level for the next CB-Msg3 re-attempt.

CB-Msg4 Content (including open issue MAC-14)

Proposal 20: A new HARQ-Ack resource indicator could be included in the CB-Msg4 to indicate multiple HARQ feedback resource for multiple HARQ feedback.

Proposal 21: Reuse the existing TAC MAC CE format for CB-Msg4. The TAC MAC CE is UE-specific and optional presence.

Proposal 22: Multiple RRC messages (MAC SDUs) can be included into one CB-Msg4.

Proposal 23: RAN2 confirms the potential CB-Msg4 content may include:

- Contention Resolution Identities

- RRC message for CB-Msg3 response

- C-RNTI

- Timing Advance information

- HARQ feedback information

- Backoff information

- FFS on other NW control information (such as information for assisting Msg4 monitoring, network initiated fallback or CE level change)

CB-Msg4 Structure (open issue MAC-12 and 13)

Observation 5: For MsgB in 2-step RACH, multiple UEs can be responded in one message by including multiple successRAR in one MsgB.

Observation 6: Each SuccessRAR includes the CR ID, the C-RNTI, the TA information and the HARQ feedback resource for a specific UE.

Proposal 24: Define a new MAC PDU format for CB-Msg4. The MsgB format in 2-step RACH can be the baseline.

Proposal 25: For the exact information to be included for each UE, the UE contention resolution ID is the only mandantory element to be delvierd to UE. All the other IEs are optional presence.

# Reference

[1] RP-250472	Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R2-2503175	LS on CB-msg3-EDT

[3] R2-250xxxx	Report of [Post129bis][311][R19 IoT NTN] MAC CR (Mediatek)

[4] R1-2410895	Reply LS on OCC for CB-msg3 NPUSCH

# Annex: RAN2 agreements for CB-Msg3

In RAN2-127bis meeting, RAN2 agreed to continue working on a Contention-Based (CB) msg3 EDT mechanism as well as the Diversity Slotted-Aloha (DSA).

RAN2-127bis meeting Agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In RAN2-128 meeting, RAN2 progressed well on how to support CB-Msg3 in both SA and DSA cases but many FFS remain.

RAN2-128 meeting Agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)IThe UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In RAN2-129 meeting, RAN2 agreed the working assumption on the CB-Msg3 transmission window and Msg4 enhancement.

RAN2-129 meeting Agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: 1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) 2. For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicas inside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

In RAN2-129bis meeting, RAN2 agreed single Msg4 monitoring window and single RNTI for CB-Msg3 EDT.

RAN2-129bis meeting Agreements: - Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT - We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure - For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). - We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case) - We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation. - If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission. - The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource. - The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same. - RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply (CB-Msg3-EDT configuration for eMTC) - For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2. - For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed). - We will not support TDD related parameters. - For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources). - For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration - For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (CB-Msg3 configuration for NB-IoT) - For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier. - For NB-IoT, introduce below physical layer parameters in shared resources configuration as below: - Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16) - Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16) - Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set. - MCS configuration for NPUSCH (as in npusch-MCS-r16). - PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13) - The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used. NOTE: confirm with RAN1 is needed - For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration Agreements (part 2): - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure) - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring Agreements (part3): - The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific. - The Msg4 monitoring window configuration (e.g. length) is CE level specific - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized. - RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject - For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject. - RAN2 also intends to support CB-msg3-EDT for MT cases - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1 - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. - Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached. - Backoff information could be included in CB-Msg4. - L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported. - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).