3GPP TSG-RAN WG2 Meeting #130                               R2-2503665

St.Julians, Malta, May 19th – 23rd , 2025

Source:	China Telecom

Title:	A-IoT data transmission

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129-bis meeting, the following agreements regarding the A-IoT data transmission have been made[1]:

Agreements 1 AS ID is applied for Inventory + command case; 2 AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed. 3 For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits. 4 Do not specify the reader behaviour on how exactly the ASID is generated. 5 The device releases the AS ID upon power off (no stage 3 specification impact); 6 The device only keeps one AS ID at a time. 7 For CFRA, command message is used for AS ID assignment 8 For CBRA, Msg 2 is used for AS ID assignment 9 The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely. Agreements on segmentation 1 For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. 2 FFS whether the reader always includes the command for retransmission of segments. 3 1-bit indication is sufficient to indicate whether more D2R data will be sent 4 For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption. Agreements on MAC PDU format design 1 The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message 2 RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1. 3 The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message 4 The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. 5 In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS.

In this contribution, we will further discuss the general format of MAC PDU and other aspects based on the current RAN2 progress.

- Discussion

2.1 MAC PDU format design

As for the design of the A-IoT MAC header, we have agreed that “At least the following fields are required for at least for R2D in the MAC header - message type, length for SDU, and variable part(s)”. For D2R in the MAC header, we still haven’t determined whether we need a message type field. In our understanding, the device may autonomously transmit D2R messages (e.g., energy status message) for DOA traffic situations. If there is a message type field, it may be more efficient for subsequent message parsing. Thus, considering the compatibility of different Specs versions, a message type field could be set up for D2R in the MAC header. Of course, not all D2R messages need to reserve this field.​

- A D2R message type could be introduced in the MAC header.

In the last meeting, we discussed that the MAC PDU can include both MAC SDU and padding. We need to determine a D2R field to indicate how many SDU bits are present in this case. For D2R upper layer data transfer messages, the data SDU is necessary, and the padding may not exist due to the limitations of TBS. Moreover, the data SDU must be located in front of the padding. Therefore, setting a SDU length field before the MAC SDU is necessary. This approach may be simpler and can be consistent with the design of R2D in the MAC header.

- A SDU length field should be introduced to indicate how many SDU bits are present.

Regarding the reserved bits, we consider it essential to introduce them for A-IoT MAC PDU. We know that there are several message types for D2R and R2D messages. If all message types are placed in a single message -type field and the scalability of subsequent Specs versions is taken into account, we can allocate 4 bits for the message-type field and use reserved bits to expand message types in the future. Moreover, these reserved bits can also be utilized to ensure byte alignment of the MAC PDU or to extend other subsequent functions. From these perspectives, the reserved bits are necessary for A-IoT MAC PDU.

- The reserved bits are necessary for A-IoT MAC PDU.

2.2 Segmentation

For the retransmission of the first segment or unsegmented D2R message, we agree that the reader sends the R2D message with the upper - layer command included again. To simplify the design, we consider that offset zero is unnecessary for the retransmission of the first segment or unsegmented D2R message. When the device receives the retransmission message with the upper layer command, it can autonomously decide whether to send the first D2R message or the unsegmented D2R message based on the TBS indication. However, for the retransmission of the x - th segment (x > 1), it can be triggered by an R2D message containing the size of the received message. Since the device already knows the command before transmitting the first segment, it can retransmit other segments based on the offset indication without the need for the command.

- For the retransmission of the first segment or unsegmented D2R message, it is not necessary to include offset zero.

- The reader does not need to include the command for retransmitting non - first segments.

As for the D2R message size, RAN2 has agreed to introduce a 1-bit indication to indicate whether there is more data, provided that SA2 indicates that the CN can provide an estimated expected D2R message size. According to SA2’s reply [2], the CN can provide the reader with the approximate D2R message size if it is an AF request. If not, whether the CN can provide it remains under discussion. However, considering the limited TU, we can continue the discussion on the assumption that the CN cannot provide it in some cases. The sources of the approximate D2R message size are: 1) from the CN and 2) from the device. As we understand, before the reader initiates the R2D transmission procedure, it can determine whether it has obtained the size information from the CN. Then, if the reader has not received the size informaiton from the CN, it can request the device to report the approximate size information.

- RAN2 to support that the reader can request the device to report the D2R message size when the reader does not obtain such information from the CN.​

- Conclusion

This contribution makes the following proposals:

- A D2R message type could be introduced in the MAC header.

- A SDU length field should be introduced to indicate how many SDU bits are present.

- The reserved bits are necessary for A-IoT MAC PDU.

- For the retransmission of the first segment or unsegmented D2R message, it is not necessary to include offset zero.

- The reader does not need to include the command for retransmitting non - first segments.

- RAN2 to support that the reader can request the device to report the D2R message size when the reader does not obtain such information from the CN.​

# References

- 3GPP RAN2#129bis Chair notes.

- R2-2500052, LS on A-IoT Conclusions in SA WG2.