3GPP TSG-RAN WG2 Meeting #130	           R2-2503666

St.Julians, Malta, May 19th – May 23rd, 2025

Agenda item:	8.1.2.2

Source:	China Telecom

Title:	LCM for UE-sided model for BM

Document for:	Discussion

- Introduction

In this paper, we would like to further discuss LCM for UE-sided model for BM use case from the following aspects: applicability reporting, data collection, functionality management in various mobility cases, and performance monitoring.

- Discussion

2.1 Applicability functionality reporting

In last meeting, RAN2 has agreed that when UE receives the inapplicable full inference configuration in step 3, UE maintains it until NW releases it explicitly. In response to step 3, UE may send an inapplicability report to NW in step 4. From our perspective, UE can also indicates its preferred inference configuration in step4 to prevent NW from sending an inapplicable inference configuration again.

Proposal 1: If UE receives the inapplicable full inference configuration in step 3, UE can indicate its preferred inference configuration in step 4.

RAN2 has also agreed to indicate a simple cause value of inapplicability, and how to define this simple cause related to model availability is still FFS. The model availability involves

- UE including models that have been trained;

- UE including models that have been updated;

- UE including models that have been downloaded.

According to the above analysis related to model availability, the simple cause related to model availability can at least include no trained model, no updated model and no downloaded model.

Proposal 2: The simple cause related to model availability can at least include no trained model, no updated model, or no downloaded model.

2.2 Data collection

Some companies have proposed to have a prohibit timer for initiating a data collection request, we have no strong view about this. However, if such prohibit timer is supported, we have some concern about the issue of resource-wasting. When UE received an inapplicable configuration for data collection during the prohibit timer is running, UE cannot perform data collection based on received configuration, but still need to maintain the configuration and the air interface resource until it receives the release message from NW, which may result in a waste of radio resource. To address this issue, it is proposed to specify UE behaviours as follow:

- If the prohibit timer is running, UE should be allowed to send a received configuration unavailable indication to NW.

- If the prohibit timer is not running, UE can request a new data collection configuration from NW.

Proposal 3-1: If a prohibit timer for initiating a data collection request is supported, it is recommended to allow UE to send a received configuration unavailable indication to NW when the prohibit timer is running.

Proposal 3-2: If a prohibit timer for initiating a data collection request is supported and if it is not running, UE can request a new data collection configuration.

Another key issues of data collection is how to report the collected data. Take NW-side data collection procedure as a reference, UE can send a data availability indication to NW and request resource when it needs to report the collected data. As for when to report it, the following condition can be considered:

- Out of power;

- Memory has reach a threshold or full;

- Overheat.

Proposal 4: It is proposed that UE can send a data availability indication to NW and request resources when it needs to report the collected data.

Proposal 5: As for when to report the collected data, the following condition can be considered:

- out of power;

- memory has reached a threshold or full;

- overheat.

2.3 Functionality management during mobility

At the RAN2#128 meeting, RAN2 has agreed to the following agreements regarding applicability reporting.

Agreements: 4. Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Regarding functionality management during mobility, RAN2 has reached a consensus during RAN2#128 meeting that, as a baseline, the applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell. Nevertheless, it is imperative to note that diverse handover scenarios exist in practical deployments. For example, CHO allows NW to configure the handover conditions and resources of multiple target cells for UE in advance. In some complex mobility scenarios, such as high-speed mobility scenarios or areas with unstable signal coverage, the UE handover between neighboring cells may occur very rapidly and CHO can deal with the problem.

In addition, LTM, which mainly utilizes the information from the physical layer and the link layer to trigger handover, is also noticed. The physical layer and the link layer can detect signal changes more quickly, such as rapid fluctuations in signal strength or instantaneous deterioration of link quality. Compared with the handover based on higher layers (such as the RRC layer), LTM can respond more rapidly. Whether the agreed baseline can be applied to other handover scenarios needs to be discussed in order to prevent some abnormal situations that require special consideration.

Proposal 6: It is recommended that RAN2 discuss the applicable functionality reporting in other handover types, such as CHO, LTM, etc.

If Proposal 6 is supported, Handover (HO) will serve as the baseline. Assuming we are discussing the CHO scenario, the question arises as to whether the applicable functionality reporting in CHO is still supported by the same RRC procedure. For multiple potential target gNBs, consideration must be given to how to issue the network-side additional conditions and/or the inference configuration. Moreover, when the UE evaluates the execution conditions for CHO, it also needs to consider whether additional attention needs to be paid to the network-side additional conditions and/or inference configurations related to the target gNB to successfully complete the handover process.

Proposal 7: Taking Handover as the baseline, discuss whether the applicable functionality reporting in CHO is still supported by the same RRC procedure.

2.4 Performance monitoring

RAN1 has discussed performance monitoring for several meetings and the following are related agreements:

RAN1#117: Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring RAN1#118: Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#118bis: Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. RAN1#119: Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring. RAN1#120bis: Agreement For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB Agreement For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A Working Assumption At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. Working Assumption For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring. Working Assumption For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

According to RAN1 agreements, two types of performance monitoring are supported, i.e. NW-side performance monitoring and UE-assisted performance monitoring. The NW-side performance monitoring is that the UE is configured or triggered by the network to report measurement results of the resource set for monitoring, and then the network calculate the performance metric based on reported measurement results. The UE-assisted performance monitoring is that the UEs calculate the performance metric based on measurement and report the calculated performance metric to the network. Furthermore, RAN1 support reusing CSI framework to configure monitoring result reporting via L1 signaling. RAN1#120bis made further progress on performance metric calculation, monitoring report type and monitoring resource type.

From our point of view, RAN2 can discuss the network control of performance monitoring from the following aspects:

- When to report monitoring result

- Who determine whether the monitoring performance meets the requirement or not

- Performance feedback

When to report monitoring result

For NW-side performance monitoring, since UEs do not calculate the performance metric, and the content of monitoring result is L1-RSRP and/or RS index, the network control of monitoring result report can be supported with existing CSI report types. Because the network is aware of the calculated performance metric, the network can react when the performance metric is low enough.

For UE-assisted performance monitoring, UEs are aware of the calculated performance metric. It is possible that the network is only interested in the situation where the performance metric calculated by UEs is low. It is beneficial that UEs report monitoring result when the calculated performance metric is lower than threshold, even though the network has not release or deactivate the CSI report configuration dedicated for monitoring. Therefore, the network is able to configure UEs to stop monitoring in advance, which is good for UE power consumption and the overhead of monitoring result reporting. If UEs are not triggered to report the calculated performance metric in advance, the network control of monitoring result report can be supported with existing CSI report, similar to NW-side performance monitoring.

Proposal 8: For UE-assisted performance monitoring, UE report monitoring result when trigger conditions are fulfilled, such as performance metric calculated by UE is lower than a configured threshold.

Who determine whether the monitoring performance meets the requirement or not

In our understanding, the performance requirement is determined by the network, e.g. prediction accuracy. For NW-side performance monitoring, it is natural that the network determine whether the calculated performance metric meets the requirement or not. However, for UE-assisted performance monitoring, it is possible that UE or the network determines whether the calculated performance metric meets the requirement or not, given that the network can configure the required performance metric to UE in advance.

Proposal 9: For NW-side performance monitoring, the network determines whether the monitoring performance metric meets the requirement or not.

Proposal 10: For UE-assisted performance monitoring, UE or the network can determine whether the monitoring performance metric meets the requirement or not.

Performance feedback

In previous discussion related to applicable functionality for UE-sided model, it is assumed that UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. In our understanding, UE need to be aware of whether the monitoring performance metric meet the requirement or not, in order to confirm whether the model be monitored is still available or not. Furthermore, UE can confirm if there is other available model and determine whether the corresponding functionality is still applicable or not. Based on the above reason, UE need to be notified when the network determines that the monitoring performance metric does not meet the requirement based on monitoring result report.

Proposal 11: UE need to be notified when the network determines that the monitoring performance metric does not meet the requirement.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: If UE receives the inapplicable full inference configuration in step 3, UE can indicate its preferred inference configuration in step 4.

Proposal 2: The simple cause related to model availability can at least include no trained model, no updated model, or no downloaded model.

Proposal 3-1: If a prohibit timer for initiating a data collection request is supported, it is recommended to allow UE to send a received configuration unavailable indication to NW when the prohibit timer is running.

Proposal 3-2: If a prohibit timer for initiating a data collection request is supported and if it is not running, UE can request a new data collection configuration.

Proposal 4: It is proposed that UE can send a data availability indication to NW and request resources when it needs to report the collected data.

Proposal 5: As for when to report the collected data, the following condition can be considered:

- out of power;

- memory has reached a threshold or full;

- overheat.

Proposal 6: It is recommended that RAN2 discuss the applicable functionality reporting in other handover types, such as CHO, LTM, etc.

Proposal 7: Taking Handover as the baseline, discuss whether the applicable functionality reporting in CHO is still supported by the same RRC procedure.

Proposal 8: For UE-assisted performance monitoring, UE report monitoring result when trigger conditions are fulfilled, such as performance metric calculated by UE is lower than a configured threshold.

Proposal 9: For NW-side performance monitoring, the network determines whether the monitoring performance metric meets the requirement or not.

Proposal 10: For UE-assisted performance monitoring, UE or the network can determine whether the monitoring performance metric meets the requirement or not.

Proposal 11: UE need to be notified when the network determines that the monitoring performance metric does not meet the requirement.