3GPP TSG-RAN WG2 Meeting #130	R2-2503677

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.13.3

Source: 	China Telecom

Title:	Discussion on control plane aspects for NR sidelink multi-hop relay

WID/SID:	NR_SL_relay_multihop

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, control plane procedures and SRAP impact in multi-hop relay were widely discussed with much progress made. However, there are still a few open issues on SRAP configuration and SI related aspects for multi-hop U2N relay. In this contribution, we would like to share our opinions on control plane remaining issues for multi-hop U2N relay.

- Discussion

SRAP configuration

In last RAN2 meeting, the following agreements on SRAP configuration were reached.

Agreements made in RAN2#129bis 1. As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay, SRAP header is present over each hop except the PC5 hop between the remote UE and first relay UE. 2. As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay: - At the link between remote UE and the first relay UE, reuse the specified PC5 RLC channel (i.e., SL-RLC0); - At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message. 3. SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17. 4. For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs. 5. For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination. We will review next meeting if this explicit mapping functionality is needed. 6. FFS whether to introduce reflective bearer mapping as an optional feature. 7. From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information: • Local identity of the Remote UE • The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping 8. From a relay UE perspective (including first/intermediate/last relay UEs), the legacy contents of the SRAP configuration are applied for at least the directly connected child UEs. FFS (depending on outcome of the reflective mapping) if they are also applied for indirectly connected child UEs.

Based on the above agreement, RAN2 assumes that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination. It remains FFS whether to confirm this explicit mapping functionality or whether to introduce reflective bearer mapping as an optional feature. For reflective bearer mapping, the gNB doesn’t provide the downstream relay ID and the UE has to determine the next hop relay based on stored information, which may bring extra complexity on UE implementation and spec impact as well. In our opinion, we prefer to have explicit mapping functionality, which is more straightforward. And with this explicit mapping functionality, there’s no strong motivation to further introduce reflective bearer mapping as an optional feature.

Proposal 1: RAN2 confirms that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.

Proposal 2: No need to introduce reflective bearer mapping for SRAP configuration at the relay UE.

System information

For single hop U2N relay, the in-coverage L2 U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE. The L2 U2N Remote UE can also receive the system information from the L2 U2N Relay UE after PC5 connection establishment with L2 U2N Relay UE. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform L2 U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, L2 U2N Relay UE triggers on-demand SI/SIB acquisition procedure according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to L2 U2N Remote UE via PC5-RRC message.

For multi-hop U2N relay, the same principle can be followed and it was agreed that the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s) in RAN2#128 meeting. In RAN2#129 meeting, lots of agreements were reached on the details of system information handling for remote UE and relay UE. For intermediate relay UE, the following issues highlighted in yellow were left for further discussion.

Agreements and open issues in RAN2#129 - The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list. FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI. - When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE). - The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): - upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested) - FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

One issue is whether the intermediate relay UE in RRC_IDLE/RRC_INACTIVE can receive SI directly from SIB broadcast by the cell on Uu. From intermediate relay UE’s perspective, it is actually a remote UE as well. Thus, it shall follow the remote UE behaviour on SI handling as legacy. There is no strong motivation to enhance the intermediate relay UE to keep on tracking the broadcasted SI in Uu for multi-hop U2N relay, which may also bring extra complexity to the intermediate relay UE. Besides, based on the agreement made in RAN2#128 meeting, an upper bound of Uu RSRP is configured to determine the operation as an intermediate relay UE. Thus, the intermediate relay UE is very likely to be at the edge of the cell or even out of coverage. The case that the intermediate relay UE can receive SI directly from SIB broadcast by the cell on Uu is more a corner case. Moreover, to guarantee the consistency between the SIBs received directly from Uu and from the parent relay UE, further enhancement might be needed, such as the third open issue listed above. In our opinion, there is no need to support it.

However, in the post meeting discussion on FFS issues on SI, the majority companies support to allow the intermediate relay UE in RRC_IDLE/RRC_INACTIVE to receive SI directly from SIB broadcast by the cell on Uu. Regarding how to support it, there are two options. One option is left to UE implementation and the other option is to capture it in stage2 spec. In our opinion, this feature is supported based on the assumption that we can’t prevent the intermediate relay UEs from receiving SI directly via Uu if they can, vice versa. Thus, we prefer to leave it up to UE implementation.

Proposal 3: For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, regarding in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly, which is up to UE implementation. No spec impact is needed.

If proposal 3 is agreed, there is no need to support the new trigger condition on sending SI request in PC5-RRC to the parent relay when there is a change in the ability of the intermediate UE.

Proposal 4: Regarding the sending of SI request by an intermediate relay UE to the parent relay, RAN2 confirms not to support sending SI request when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

Whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE) remains FFS. If proposal 3 is agreed, there is no inconsistency issue of the SI stored in the intermediate relay UE. Allowing intermediate relay UE to forward available SI directly when it has available SI can help reduce the PC5 signalling overhead and reduce the time for remote UE to get the available SI. Assuming the remote UE requires SIBx and SIBy while the intermediate relay UE only has available SIBx. In such case, the intermediate relay UE could forward the SI request only with SIBy to the parent relay or last relay and respond the SIBx directly.

Proposal 5: RAN2 confirms that the intermediate relay UE can respond directly if it has the requested SI. For the requested SI that the intermediate relay UE doesn’t have, it forwards the requested SI list to the parent relay or last relay.

- Conclusion

In this contribution, we discuss control plane remaining issues for multi-hop U2N relay. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 confirms that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.

Proposal 2: No need to introduce reflective bearer mapping for SRAP configuration at the relay UE.

Proposal 3: For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, regarding in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly, which is up to UE implementation. No spec impact is needed.

Proposal 4: Regarding the sending of SI request by an intermediate relay UE to the parent relay, RAN2 confirms not to support sending SI request when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

Proposal 5: RAN2 confirms that the intermediate relay UE can respond directly if it has the requested SI. For the requested SI that the intermediate relay UE doesn’t have, it forwards the requested SI list to the parent relay or last relay.

- Reference

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

3GPP TS 38.300 18.2.0: "NR; Radio Resource Control (RRC) protocol specification".