3GPP TSG-RAN WG2 Meeting #130	R2-2503682

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.6.3

Source: 	China Telecom

Title:	Discussion on remaining issues of L1 event triggered measurement reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meetings, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made on measurement enhancements for LTM. However, there are still open issues on the detailed design for MR MAC CE. In this contribution, we would like to share our opinions on some of the remaining issues as per the FFS list in the post meeting discussion on MAC running CR.

- Discussion

Beam specific offset for serving beam (i.e. MAC-1)

In the current MAC running CR, there is an EN as below:

Editor’s NOTE: FFS whether the beam specific offset for serving cell Obs is needed. If needed, per-serving cell or per-beam or per-RS type. Similar as other events.

In RAN2#126 meeting, the following agreement was reached.

Agreements in RAN2#126

6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

The intention of introducing the above parameters similar to legacy L3 measurement is to help UE to provide relatively reasonable or stable L1measurement results to the gNB, so that the gNB can trigger LTM cell switch properly and avoid “Ping Pong” cell switch. In this sense, beam specific offset for serving beam is also very beneficial to have better HO performance, which will provide more flexibility to the configuration from the gNB as well.

Proposal 1: RAN2 confirms that the beam specific offset is applied to serving beam as well for LTM event evaluation.

Regarding the granularity of beam specific offset for serving beam, it could be per-cell level, which means all the beams of a serving cell shares the same beam specific offset.

Proposal 2: The granularity of beam specific offset for serving beam is configured in a per-cell level, which means all the beams of a serving cell shares the same beam specific offset.

Regarding which LTM events the beam specific offset for serving beam is used, we think all the events that involve serving beam could be considered, i.e. LTM2, LTM3 and LTM5. To introduce beam specific offset of serving beam for all the events give the most flexibility to the gNB implementation. And of course, the gNB can configure the offset value to “0”, if it doesn’t want to use it.

Proposal 3: The beam specific offset for serving beam could be used to LTM2, LTM3 and LTM5.

Content of L1 MR MAC CE triggered by LTM2 (i.e. MAC-5)

In the current MAC running CR, there is an EN as below:

Editor’s NOTE: FFS for MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

In RAN2#128 meeting, the following agreement was reached.

Agreements in RAN2#126

1. For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

2. For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

For Event LTM2, if only the current beam information is reported, the network will not get additional information of other beam(s) from candidate cell(s) or serving cell. It is obvious that allowing to report the L1 measurement result of other beams from candidate cell(s) or serving cell in the MR MAC CE triggered by LTM2 is beneficial for the NW to make cell switch decision. Thus, we suggest allowing to include measurements for LTM candidates for LTM2 if NW configures that the beam(s) not satisfying the event could be reported. Regarding which candidate beam could be selected to report, it could be up to UE implementation.

Proposal 4: For MR triggered by LTM2, the MR can include measurements for LTM candidates if the NW configures that the beam(s) not satisfying the event could be reported.

Proposal 5: It is up to UE implementation to select the candidate beam to be reported in MR triggered by LTM2.

Multiplexing of LTM MR MAC CE (i.e. MAC-7)

In RAN2#129, the following agreements were reached on how to generate and multiplex the LTM MR MAC CE.

Agreements in RAN2#129

1. LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

2. The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

3. The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

4. There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

It was agreed to include only one triggered measurement event (i.e. ltm-CSI-ReportConfigId) in one LTM MR MAC CE, which can be multiplexed in any available UL grant. However, it is possible to have multiple available L1 measurement results with different ltm-CSI-ReportConfigId at the time to transmit the MR MAC CE. In that case, the multiplexing of LTM MR MAC CE may need further discussion. The potential cases are listed as follows.

- Case 1: In one UL grant, there could be more than one normal MR MAC CEs and/or more than one truncated MR MAC CE to be multiplexed, if available.

- Case 2: In one UL grant, there could be at most one normal MR MAC CE and at most one truncated MR MAC CE to be multiplexed, if available.

- Case 3: In one UL grant, there could be at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed, if available.

For Case 1, if there is enough UL grant, multiple MR MAC CE could be multiplexed in one UL grant and each MR MAC CE includes only one triggered measurement event. Then, comparing to include multiple triggered measurement events in one MR MAC CE, it need to add more MAC subheaders, which consumes more bits. For Case 2, if there are more than one triggered measurement events to be reported, one more truncated MR MAC CE could be multiplexed in addition to one normal MR MAC CE, if available. In that case, the extra truncated MR MAC CE can help to indicate or request the gNB to allocate more UL grants for the pending measurement results. For case 3, it is restricted that at most one MR MAC CE or at most one MR MAC CE to be multiplexed in one UL grant. In that case, multiple triggered measurement events have to be reported in different MR MAC CEs in multiple UL grants, which may lead to the delay of the measurement reports.

To allow the above case 1 or case 2 can help reduce the overhead and provide more flexibility for L1 event triggered measurement reporting, we suggest RAN2 clarify whether to support the above cases.

Proposal 6: RAN2 clarifies whether to restrict at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed in one UL grant.

Proposal 7: RAN2 considers to support more than one normal or truncated MR MAC CEs to be multiplexed in one UL grant, if available.

No matter whether to restrict at most one MR MAC CE or at most one MR MAC CE to be multiplexed in one UL grant, there might be another open issue on how to select the reported measurement event among multiple triggered measurement events by the UE. For example, the measurement event that hasn’t been reported yet could be prioritised or the measurement event that has the most beams met the trigger conditions could be prioritised. Or the UE can select the event in the MR_list one by one. It is better to further clarify the UE behaviour on selecting the reported measurement event when there are multiple triggered events to be reported.

Proposal 8: RAN2 clarifies the UE behaviour on how to select the reported measurement event when there are multiple triggered events to be reported.

Whether to support padding MR MAC CE

To better utilize the UL resource, padding BSR is introduced to provide the UL data volume to the gNB when there are padding bits left. With similar motivation, it is also useful to have padding MR MAC CE so that more available L1 measurement results could be provided to the gNB on time.

Proposal 9: RAN2 considers to support padding MR MAC CE.

To support padding MR MAC CE, we need to consider the content of padding MR MAC CE. If it is restricted that at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed in one UL grant, it is meaningful to allow one extra padding MR MAC CE to be multiplexed in one UL grant when available. In that case, the padding MR MAC CE could include one event config ID and at least one triggered beam id (SSBRI or CRI) plus the corresponding L1 RSRP. If more than one normal or truncated MR MAC CEs are supported to be multiplexed in one UL grant, it is also meaningful to allow one extra padding MR MAC CE to be multiplexed in one UL grant when available, which can provide additional L1 measurement results to the gNB. Moreover, if there is no triggered event to be reported, the padding MR MAC CE could include any available L1 measurement results.

Proposal 10: The padding MR MAC CE could include one event config ID and at least one triggered beam id (SSBRI or CRI) plus the corresponding L1 RSRP, when there are triggered events to be reported.

Proposal 11: The padding MR MAC CE could include one event config ID and at least one any beam id (SSBRI or CRI) with available L1 RSRP, when there is no triggered event to be reported.

In RAN2#129 meeting, the following agreements on Logical Channel Prioritization of MR MAC CE were reached.

Agreements in RAN2#129

1.	The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

2.	The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

If padding MR MAC CE is introduced, we need further discuss the LCP of padding MR MAC CE. For the padding MR MAC CE including triggered event, it is more important than padding BSR. While for the padding MR MAC CE including any available beam which doesn’t meet the trigger condition, it is less important than padding BSR.

Proposal 12: The padding MR MAC CE with a triggered event is transmitted with a higher LCH priority than padding BSR MAC CE.

Proposal 13: The padding MR MAC CE with a non-triggered event is transmitted with a lower LCH priority than padding SL-BSR MAC CE.

- Conclusion

In this contribution, we discuss the remaining issues on L1 event triggered measurement reporting for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 confirms that the beam specific offset is applied to serving beam as well for LTM event evaluation.

Proposal 2: The granularity of beam specific offset for serving beam is configured in a per-cell level, which means all the beams of a serving cell shares the same beam specific offset.

Proposal 3: The beam specific offset for serving beam could be used to LTM2, LTM3 and LTM5.

Proposal 4: For MR triggered by LTM2, the MR can include measurements for LTM candidates if the NW configures that the beam(s) not satisfying the event could be reported.

Proposal 5: It is up to UE implementation to select the candidate beam to be reported in MR triggered by LTM2.

Proposal 6: RAN2 clarifies whether to restrict at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed in one UL grant.

Proposal 7: RAN2 considers to support more than one normal or truncated MR MAC CEs to be multiplexed in one UL grant, if available.

Proposal 8: RAN2 clarifies the UE behaviour on how to select the reported measurement event when there are multiple triggered events to be reported.

Proposal 9: RAN2 considers to support padding MR MAC CE.

Proposal 10: The padding MR MAC CE could include one event config ID and at least one triggered beam id (SSBRI or CRI) plus the corresponding L1 RSRP, when there are triggered events to be reported.

Proposal 11: The padding MR MAC CE could include one event config ID and at least one any beam id (SSBRI or CRI) with available L1 RSRP, when there is no triggered event to be reported.

Proposal 12: The padding MR MAC CE with a triggered event is transmitted with a higher LCH priority than padding BSR MAC CE.

Proposal 13: The padding MR MAC CE with a non-triggered event is transmitted with a lower LCH priority than padding SL-BSR MAC CE.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB

R2-24xxxx	Draft report of 3GPP TSG RAN WG2 meeting #127