3GPP TSG-RAN WG2 Meeting #130	R2-2503691

Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	NW side data collection

Document for:	Discussion

# 1. Introduction

Several agreements regarding network side data collection were made in previous RAN2 meetings (e.g., RAN2-126bis [1], RAN2-127 [2], RAN2-17bis [3], RAN2-128 [4], RAN2-129 [5] and RAN2-129bis [6]). The Annex lists the agreements regarding network side data collection during these meetings.

In this contribution, we address some of the open issues.

# 2. Discussion

## 2.1 Availability Indication and sending of collected data

In RAN2-129bis [6], it was agreed that:

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

That is, the UE can be configured to perform data collection not only for different use cases simultaneously. The UE may even be configured with different configurations for a single use case. Thus, in those cases where there are multiple configurations, it important for the UE to indicate in the availability indication information regarding the data collection configuration that the availability indication is corresponding to.

Proposal 1: If the UE is configured to perform different data collections simultaneously, it will include, in the availability indication, information about the corresponding data collection configuration or/and use case.

It was agreed in RAN2-127bis [3] that UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. Currently, the network sends to the UE a UEInformationRequest message indicating the type of logged measurements that it wants the UE to send, and the UE includes the requested data/measurements using corresponding fields, as shown below:

UEInformationRequest-r16-IEs ::= SEQUENCE {

idleModeMeasurementReq-r16       ENUMERATED{true}                         OPTIONAL, -- Need N

logMeasReportReq-r16             ENUMERATED {true}                        OPTIONAL, -- Need N

connEstFailReportReq-r16         ENUMERATED {true}                        OPTIONAL, -- Need N

ra-ReportReq-r16                 ENUMERATED {true}                        OPTIONAL, -- Need N

rlf-ReportReq-r16                ENUMERATED {true}                        OPTIONAL, -- Need N

mobilityHistoryReportReq-r16       ENUMERATED {true}                        OPTIONAL, -- Need N

lateNonCriticalExtension         OCTET STRING                             OPTIONAL,

nonCriticalExtension             UEInformationRequest-v1700-IEs           OPTIONAL

}

UEInformationRequest-v1700-IEs ::= SEQUENCE {

successHO-ReportReq-r17          ENUMERATED {true}                        OPTIONAL, -- Need N

coarseLocationRequest-r17        ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             UEInformationRequest-v1800-IEs           OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

flightPathInfoReq-r18            FlightPathInfoReportConfig-r18           OPTIONAL, -- Need N

successPSCell-ReportReq-r18      ENUMERATED {true}                        OPTIONAL, -- Need N

reselectionMeasurementReq-r18    ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             SEQUENCE {}                              OPTIONAL

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1700-IEs     OPTIONAL

}

UEInformationResponse-v1700-IEs ::=  SEQUENCE {

successHO-Report-r17                 SuccessHO-Report-r17                OPTIONAL,

connEstFailReportList-r17            ConnEstFailReportList-r17           OPTIONAL,

coarseLocationInfo-r17               OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1800-IEs     OPTIONAL

}

UEInformationResponse-v1800-IEs ::=  SEQUENCE {

flightPathInfoReport-r18             FlightPathInfoReport-r18            OPTIONAL,

successPSCell-Report-r18             SuccessPSCell-Report-r18            OPTIONAL,

measResultReselectionNR-r18          MeasResultIdleNR-r16                OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

Thus, there is a need to introduce a new IE/field to indicate the network is requesting a logged data for network side model training. As discussed above, if the UE is configured to perform data collection for different use cases and/or configurations simultaneously, it is important for the network to also indicate which collected data is to be reported.

Proposal 2: A new IE/field is introduced to the UEInformationRequest message to indicate the network is requesting the UE to send logged data for network sided model training. The IE may optionally include information about the corresponding data collection configuration or/and use case (if the UE is configured with multiple configurations).

Similarly, a new IE/field needs to be introduced in the UEInformationResponse that will be used to include the relevant collected data.

Proposal 3: A new IE/field is introduced to the UEInformationResponse message that will be used to contain the logged data.

## 2.2 Content of collected data

In RAN2#127bis, the following was agreed regarding the content of the collected data [3]:

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In RAN2#129bis, it was further agreed [6]:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

NR supports logging of time information in the logged MDT case, as shown below:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]]

}

As can be seen above, there is one absolute time information corresponding to the absolute time when the logged measurement configuration was provided, and for each collected sample, there is a relative timestamp from this absolute timestamp. Our understanding is that a similar approach can be adopted for the logged measurements for network side data collection, without creating too much overhead and providing detailed information to the network about the time correlation information about the samples.

Proposal 4: Time information for the collected data is kept in the same way as logged MDT:

- an absolute time corresponding to the time the data collection configuration was received, and

- for each collected sample, a relative time from the absolute time

In RAN2#129bis, it was further agreed [6]:

- Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.

It is still not clear how the UE behaves if the UE is configured to continue logging data in the target cell, before it has sent the data collected in the previous cell to the network. Even though there could be a way for the network to figure out the collected data in one cell from the sample timestamps (e.g., based on when the UE was handed over to the target cell), it is a rather complicated solution. A more scalable approach that will also allow a flexible deployment of the model training entity anywhere in the network is include the associated ID (if configured) and cell ID information in the logged data. It is not essential to include this information in every sample, but rather within a given log, the UE may have collected data partitioned for each cell, with the corresponding cell ID and associated ID for that cell during the data collection.

Proposal 5: If data collection continuation after a handover is supported (i.e., collected data contains measurements logged in a source cell and a target cell), the UE will include the following information in the collected data:

- serving cell ID (FFS if PCI is sufficient or CGI is required)

- (optionally) network side additional condition for the serving cell (e.g., associated ID)

Proposal 6: The collected data from multiple cells is partitioned by the serving cell (i.e., no need to repeat cell ID/associated ID information with each sample).

## 2.3 Data collection configuration framework

In RAN2-129, the following was agreed [5]:

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.

In RAN2-129bis, the following was proposed by the chairman regarding the data collection framework for network side model training [6]:

- Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

The support of L3 event for controlling the logging of L1 based information causes an issue, specially in the case of CU-DU split architecture, as the CSI-RS configurations are handled by the DU and L3 measurements are handled by the CU.

The following different solutions were proposed by different companies:

- solution 1: UE sends a report based on the L3 event, and network uses this information to activate/deactivate the data collection configuration using a MAC-CE [7].

- solution 2: L1 CSI framework enhanced to include logging information and information/linkage about L3 event [8]

- solution 3: L3 measurement framework enhanced to include CSI-RS logging information [9]

Solution 1 simplifies the impact on the UE side to some extent as compared to solutions 2 and 3 by making the UE not associate L3 events and L1 logging (leaving the burden on the network to activate/deactivate the L1 logging based on the reported L3 event). However, it will require some communication between the CU and DU about the event fulfilment to trigger the MAC-CE. Also, the activation/deactivation of a periodic CSI reporting configuration for logging is a new feature, as currently only aperiodic and semi-persistence configurations can be activated/deactivated dynamically.

Solution 2 requires the DU to set the parameters for the L3 event (e.g., A1/A2 threshold, TTT) and modify the CSI-MeasConfig or CSI-Report, which in our understanding is going against the functional/protocol split of the CU-DU.

Solution 3 requires some F1-AP enhancements as currently the CU transparently encapsulates the CSI-MeasConfig/CSI-Report configurations generated by the DU in an RRC message and transports it to the UE. That is, CU has now to be enabled to know the CSI configuration is for logging purposes and then associate it with a L3 measurement event.

It is desirable to have a solution that is future proof. For example, we would like to have a solution that can be easily reused for the RRM measurement prediction and measurement event prediction use cases of AI/ML mobility, where logging L3 measurements is required (maybe along with L1 measurements, depending on the RRM sub-use case). Solution 3 is more flexible to support such cases, as all that is needed will be now be to associate the L3 logging/measurement configuration with the L3 event (the same way the L1 logging/resource configuration is done for the BM case), while for solution 2 we have to redesign a new way for a L3 event to control the logging of L3 measurements.

Further discussion is required on how to exactly model solution 3. For example, the data collection configuration can be under the MeasConfig (e.g., a L3 measurement event that is now linked to a CSI-Report config) or it can be at the same level as MeasConfig (e.g., a data collection configuration that is directly under RRCReconfiguration that will have similar IEs like MeasObject and ReportConfig, but this time for logging purposes and associated with CSI configurations).

Proposal 7: The network side data collection configuration will be based on L3 measurement framework. RAN2 to discuss if the data collection configuration will be part of the MeasConfig or separate configuration directly under RRCReconfiguration.

# 4. Conclusion

In this contribution, we discussed some of the open issues regarding logging and reporting of data for the training of a network-sided model for the beam management use case, and the following proposals were made:

Proposal 1: If the UE is configured to perform different data collections simultaneously, it will include, in the availability indication, information about the corresponding data collection configuration or/and use case.

Proposal 2: A new IE/field is introduced to the UEInformationRequest message to indicate the network is requesting the UE to send logged data for network sided model training. The IE may optionally include information about the corresponding data collection configuration or/and use case (if the UE is configured with multiple configurations).

Proposal 3: A new IE/field is introduced to the UEInformationResponse message that will be used to contain the logged data.

Proposal 4: Time information for the collected data is kept in the same way as logged MDT:

- an absolute time corresponding to the time the data collection configuration was received, and

- for each collected sample, a relative time from the absolute time

Proposal 5: If data collection continuation after a handover is supported (i.e., collected data contains measurements logged in a source cell and a target cell), the UE will include the following information in the collected data:

- serving cell ID (FFS if PCI is sufficient or CGI is required)

- (optionally) network side additional condition for the serving cell (e.g., associated ID)

Proposal 6: The collected data from multiple cells is partitioned by the serving cell (i.e., no need to repeat cell ID/associated ID information with each sample).

Proposal 7: The network side data collection configuration will be based on L3 measurement framework. RAN2 to discuss if the data collection configuration will be part of the MeasConfig or separate configuration directly under RRCReconfiguration.

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[3]	RAN2-127bis Chairman Notes, Hefei, China, October 2024

[4]	RAN2-128 Chairman Notes, Orlando, USA, November 2024

[5]	RAN2-129 Chairman Notes, Athens, Greece, February 2025

[6]	RAN2-129bis Chairman Notes, Wuhan, China, April 2025

[7]	R2-2502594, Data Collection for Training of network-side ML Models, Nokia, RAN2-129bis, Wuhan, China, April 2025

[8]	R2-2502904, NW-side data collection for beam management and positioning	, Ericsson, RAN2-129bis, Wuhan, China, April 2025

[9] 	R2-2502414, Further Considerations on NW side data collection, ZTE, RAN2-129bis, Wuhan, China, April 2025

# 6. Annex (Agreements regarding network side data collection)

RAN2-126 agreements [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

RAN2-127 agreements [2]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event-based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

RAN2-127bis agreements [3]:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

RAN2-128 agreements [4]:

Agreements on NW side data collection: - Focus on the following three radio condition event-based logging: - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

RAN2-129 agreements [5]:

Agreements on NW side data collection: - Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported.FFS what to log - Low power bit indication is supported - Data availability indication is supported. FFS when this would be triggered - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - FFS how to handle idle/inactive and RLF cases and whether we have a unified.

RAN2-129bis agreements [6]:

Agreements on NW side data collection: - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS - NOTE: it is up to UE Implementation how buffer threshold reached and low power is - The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc) - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit - New SRB can be configured for NW-side data collection (with lower priority) - Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.