3GPP TSG-RAN WG2 Meeting #130	R2-2503698

St. Julians, Malta, 19 – 23 May 2025

Agenda item:	8.7.4.1

Source:	Apple

Title:	Discussions on Delay-based LCP Enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For LCH priority adjustment based on packet remaining time, RAN2 #129bis [1] has made the following agreements:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

This paper discusses our views on some of the open issues, including priority fallback with considerations of PDU Set, priority adjustment with PSI-based discarding, and Bj enhancement.

# Discussions

## Considerations of PDU Sets for Priority Fallback

During RAN2 #129bis [2], it has been agreed that in the second round of LCP procedure, the adjusted priority will always fall back to the default priority if there no more LCH priority-adjusted data.

There may be cases where the buffered packets are belonging to the same PDU Set but with different remaining times. This is evident from the discussions relevant to DSR. Thus, even if none of the remaining buffered data available has a remaining value of PDCP discard timer below priorityAdjustmentThreshold, if PDU Set discarding is configured and some remaining data in the buffer belong to a PDU Set that is already considered urgent, it is desirable to continue using the additional priority rather than fall back to the default priority. Otherwise, LCH priority adjustment may become useless from user experience perspective, as the application cannot make use of a PDU Set if only some of its packets are delivered on time. The situation is illustrated in Figure 1:

Figure 1 An illustration of a problematic case with LCH priority fallback, where PDU Set discarding is also configured.

Observation 1: When PDU Set Discarding is configured, if a LCH fallback its priority without considering the PDU Set dependency of packets that are still in the buffer, the effort of LCH priority adjustment may be in vain from Application point of view, as the already transmitted packets of a PDU Set may become useless even if they are successfully delivered on time.

In addition, according to the WID, both LCP and DSR enhancements belong to the common objective of “support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs”.

RP-250107: - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

Thus, we expect both LCP and DSR enhancements should have a unified consideration toward PDU Sets. Furthermore, PDU Set discarding is configured based on the QoS requirement of PSIHI, so the requirement should be fulfilled in both of the XR features.

Observation 2: Both LCP and DSR enhancement should have a unified considerations about PDU Set Discarding, as they belong to the same WI objective related to UL scheduling for XR, and PDU Set discarding is configured based on the QoS requirement of PSIHI.

For LCH priority fallback, it is essentially analogues to DSR cancellation where the UE exits from an operation when the condition for such operation is no longer fulfilled. In Rel-18, DSR is cancelled when all PDCP SDUs associated with the DSR are either discarded or transmitted (i.e. no longer in the buffer). Furthermore, a “PDCP SDUs associated with the DSR” is a delay-critical PDCP SDU.

TS 38.321: A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Moreover, in both Rel-18 and Rel-19 definitions, delay-critical PDCP SDUs and delay-reporting PDCP SDU should include PDCP SDUs belonging to the same PDU Set, if the PDU Set discarding is configured.

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold. Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1). If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1).

Observation 3: The LCH priority fallback procedure should follow the same principle of DSR cancellation, and DSR cancellation takes PDU Set discarding into account.

Based on the observations above, we think LCH priority fallback should also take PDU Set integrity into account. This is needed such that the features of DSR and LCH priority adjustment can truly complement each other when they are configured together.

In current running CR, the condition for LCH priority fallback is captured as following:

2> if any resources remain: 3> if a logical channel has applied additionalPriority above; and 3> if none of its remaining data available for this transmission has a running PDCP discardTimer whose remaining value, evaluated at the time of the first symbol of this transmission, is below or at its priorityAdjustmentThreshold: 4> apply priority of this logical channel;

Clearly, in the running CR the condition for LCH priority fallback only considers the status of “discardTimer”, which is running for each individual PDCP SDU, instead of a whole PDU Set (RAN2 has explicitly agreed this in Rel-18). It means, even though there are still some PDCP SDUs belonging to the urgent PDU Set, the LCH would still fallback the priority if their remaining value of PDCP discardTimer is larger than priorityAdjustmentThreshold. Hence, we think the running CR should be updated to make sure that priority fallback should take PDU Set Remaining Time into account.

Proposal 1: If PDU Set Discarding is configured, when there is still other PDCP SDUs in the buffer that belong to the same PDU Set that is already considered urgent, the LCH does not fall back to the default priority even if the remaining value of PDCP discardTimer of these PDCP SDUs are larger than priorityAdjustmentThreshold.

## Impacts of PSI-based Discarding

In Rel-18, PSI-based discarding has been introduced to cope with congestion. When this is activated for a DRB by the network, the UE should apply a special discard timer (i.e. discardTimerLowImportance) to PDCP SDUs belonging to PDU Sets that are less important. In other words, the PDCP SDUs on this DRB will be running on different discard timer values depending on their importance.

It is worth noting that, even if the LCH priority is boosted, we cannot guarantee that the more important data from this LCH will be allocated to the UL resource first. Even though we tend to think it possible to prioritize certain packets within a LCH via UE implementation, it seems RAN2 does not have a consensus that this is always the case. This is the main reason why RAN2 has agreed the data volume calculation for Rel-19 DSR may include the non-delay-critical data that is queued ahead of delay-critical data.

With this understanding, we are not so certain about the benefits of LCH priority adjustment when the congestion is present. In particular, it is questionable if we should prioritize some less-important data over data from other LCHs configured with higher default priorities. We think RAN2 can discuss the following options:

- Option 1: When PSI-based discarding is activated a DRB, priority adjustment for the corresponding LCH is disabled.

- Option 2: When PSI-based discarding is activated a DRB, priority adjustment is applied only if there is at least one more important packet in the buffer has a remaining time smaller than the threshold.

We think Option 1 is quite simple, and Option 2 is also quite straightforward if the triggering condition for LCH priority adjustment is anyway based on discardTimers instead of discardTimerLowImportance.

Proposal 2: For the potential impacts of PSI-based discarding, RAN2 can discuss the following options:

- Option 1: When PSI-based discarding is activated a DRB, priority adjustment for the corresponding LCH is disabled.

- Option 2: When PSI-based discarding is activated a DRB, priority adjustment is applied only if there is at least one more important packet in the buffer has a remaining time smaller than the threshold.

## Issues on Bj of LCHs with Adjusted Priorities

According to the LCP procedure specified in TS 38.321, each LCH maintains a Bj value, and the LCHs with negative Bj values cannot be selected for resource allocation. Thus, even if the priority of a LCH can be adjusted based on buffer delay, we cannot guarantee that the data can be transmitted from this LCH due to Bj. During RAN2 #129bis, we have made a working assumption where Bj enhancement will not be introduced. However, if the benefits are verified (e.g. based on simulation results), it may still be considered.

In our views, whether to have enhancements related to Bj also depends on if RAN2 is only targeting at cases with negative Bj value. We think it is insufficient to only consider the cases with negative Bj. Specifically, in the cases where the Bj is positive but small, even though the LCH can be selected for resource allocation, it may not be able to allocate much of the urgent data in the buffer. With considerations of PDU Sets in XR, the amount of urgent data in the buffer could be high, and so a small Bj value may be insufficient to serve the purpose. Furthermore, if LCH with negative Bj is also present, the adjusted Bj may cause some fairness issues to the LCHs with positive but small Bj. Thus, we think RAN2 should not enhance Bj handling if we cannot identify a simple solution that can handle both negative Bj and positive/small Bj.

Proposal 3: RAN2 does not enhance Bj handling in Rel-19 if we cannot identify a simple and comprehensive solution that can deal with both negative Bj and positive but small Bj.

To provide a simple and yet comprehensive solution, we think the gNB can pre-configure a Temporary Bj Value (denoted as Bj_Temp) for the LCH via RRC. Then, before the resource allocation procedure is started for a received UL grant, the UE simply set the Bj value of a LCH with adjusted priority based on the following logic:

Bj = max(Bj, Bj_Temp)

That is, the Bj of the LCH is simply set by taking the larger value between the current Bj and the configured Temporary Bj. If the current Bj value is anyway quite high, the UE will continue to use this current Bj for LCP without altering the value at all. Conversely, if the current Bj value is either negative or quite low, the UE will change the Bj value to a configured Bj_Temp value that is configured to be high enough. Note that Bj_Temp is always positive and the value of which is up to network implementation, where the factors such as delay requirements, traffic characteristics, and fairness could be taken into account by the network.

Proposal 4: If RAN2 decides to have Bj enhancement, the following solution can be considered: For a LCH with upgraded priority, the Bj of which is set based on Bj = max(Bj, Bj_Temp) before the resource allocation procedure, where Bj_Temp is pre-configured by the network.

# Conclusions

This paper discusses our views on some of the remaining issues relating to LCH priority adjustment. We have the following observations and proposals:

Observation 1: When PDU Set Discarding is configured, if a LCH fallback its priority without considering the PDU Set dependency of packets that are still in the buffer, the effort of LCH priority adjustment may be in vain from Application point of view, as the already transmitted packets of a PDU Set may become useless even if they are successfully delivered on time.

Observation 2: Both LCP and DSR enhancement should have a unified considerations about PDU Set Discarding, as they belong to the same WI objective related to UL scheduling for XR, and PDU Set discarding is configured based on the QoS requirement of PSIHI.

Observation 3: The LCH priority fallback procedure should follow the same principle of DSR cancellation, and DSR cancellation takes PDU Set discarding into account.

Proposal 1: If PDU Set Discarding is configured, when there is still other PDCP SDUs in the buffer that belong to the same PDU Set that is already considered urgent, the LCH does not fall back to the default priority even if the remaining value of PDCP discardTimer of these PDCP SDUs are larger than priorityAdjustmentThreshold.

Proposal 2: For the potential impacts of PSI-based discarding, RAN2 can discuss the following options:

- Option 1: When PSI-based discarding is activated a DRB, priority adjustment for the corresponding LCH is disabled.

- Option 2: When PSI-based discarding is activated a DRB, priority adjustment is applied only if there is at least one more important packet in the buffer has a remaining time smaller than the threshold.

Proposal 3: RAN2 does not enhance Bj handling in Rel-19 if we cannot identify a simple and comprehensive solution that can deal with both negative Bj and positive but small Bj.

Proposal 4: If RAN2 decides to have Bj enhancement, the following solution can be considered: For a LCH with upgraded priority, the Bj of which is set based on Bj = max(Bj, Bj_Temp) before the resource allocation procedure, where Bj_Temp is pre-configured by the network.

# References

- 3GPP RAN2 #129bis Chair’s Notes, Wuhan, China, April 2025.