3GPP TSG-RAN WG2 Meeting #130	R2-2503700

St. Julians, Malta, 19 – 23 May 2025

Agenda item:	8.7.5

Source:	Apple

Title:	Views on Avoidance of Unnecessary RLC Retransmissions

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For the Rel-19 XR work item objective related to RLC-AM enhancements for packets with small delay budget, RAN2 has agreed to support avoidance of unnecessary RLC transmission/retransmission. This paper provides some of our views on the remaining open issues.

# Discussions

## Impacts of Discarding to Ongoing Polling Procedures

According to TS 38.322, when a poll is triggered and sent, the transmitter should start or restart t-PollRetransmit, and set the state variable POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer.

Furthermore, the transmitter can stop and reset t-PollRetransmit when it receives a STATUS report that indicates the status of RLC SDU corresponding to POLL_SN:

- if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit.

We keep the t-PollRetransmit running because the transmitter would like to continue the polling procedure until it knows the status of each of the RLC SDU with SN up to POLL_SN. And therefore, if the transmitter gets a status report that includes at least POLL_SN, the transmitter can stop and resets t-PollRetransmit, as it is now aware of the status of each RLC SDU with SN up to POLL_SN based on such status report. Based on the information of NACK_SN in the status report, the transmitter can perform retransmission of RLC SDUs accordingly.

When a packet is discarded by the upper layers, the transmitter is no longer interested about whether the receiver has successfully received the discarded packets or not, because no retransmission will be performed for their RLC SDUs regardless of their status. Of course, the transmitter would still need to know whether the RLC SDU is positively acknowledged so it can advance the transmitting window, but the receiver will anyway provide such status report proactively when the outdated packets are detected (based on RAN2 #129bis agreement). Thus, in cases where all RLC SDUs with SN between (and including) TX_Next_Ack and POLL_SN are already either positively/negatively acknowledged or discarded, we think it is questionable whether the transmitter should keep the t-PollRetransmit running or if we can just stop the timer, because such polling procedure may no longer be needed.

We see the following problems if we allow the t-PollRetransmit to continue to run in such cases:

- It may result in excessive polling that leads to some additional signaling overhead of unnecessary or pre-mature status reports from the receiver. In particular, this may further prevent the receiver from transmitting the more critical status report (the one that is triggered by expiration of t-RxDiscard ) quickly, because t-StatusProhibit may be started more often due to excessive polling.

- When t-PollRetransmit expires, the transmitter may not have an AMD PDU to retransmit the poll, because all the RLC SDUs in the buffer have already been discarded and their retransmission are no longer permitted.

It is worth pointing out that, the situations mentioned above may not be a corner case in XR, wherein a whole bunch of packets may be discarded simultaneously due to PDU Set discarding. Hence, we think the transmitter should just simply stop and reset the running t-PollRetransmit to avoid the negative impacts mentioned above, when all RLC SDUs with SNs between (and including) TX_Next_Ack and POLL_SN are already positively/negatively acknowledged or discarded by PDCP.

Proposal 1: When all RLC SDUs with SNs between (and including) TX_Next_Ack and POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter should stop and reset the running t-PollRetransmit.

## Status Report for Indication of Abandoned RLC SDUs

Based on RAN2 agreements, once the receiver determines that one or more RLC SDUs should be abandoned based on a local timer, it can inform the transmitter to move the transmitting window forward, by deliberately acknowledging the abandoned RLC SDUs via a status report. This is also confirmed in RAN2 #129bis that a status report will be triggered upon expiration of t-RxDiscard [1]:

When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

In our understanding, RAN2 adopts the approach based on RX-initiated window synchronization (instead of TX-initiated approach) to minimize the impacts to current specification, at an expense of latency performance. In particular, the receiver needs to go through two stages before it can detect that a RLC SDU has been discarded:

- Identify a SN gap in the received RLC SDU, and then start a timer.

- Wait until the expiration of the timer.

Thus, it can take quite a while for the receiver to determine that a RLC SDU has been discarded. Furthermore, in XR we need to deal with PDU Set discarding, where a large number of packets may be discarded simultaneously. In such cases, it may take even longer for the receiver to identify which RLC SDUs can be abandoned especially when the SNs of the discarded packets are not consecutive.

Even the transmitter would stop transmission or retransmission of RLC SDUs that have been discarded, it cannot move the transmitting window forward before the receiver provides a status report. Moreover, the RX may not be able to transmit the status report immediately if the prohibit timer of the status report is already running.

With this in mind, we think the receiver transmit the status report immediately upon expiration of t-RxDiscard, even if the t-StatusProhibit is already running, in order to minimize the overall latency.

Proposal 2: The receiver should transmit the status report triggered by the expiration of t-RxDiscard immediately, even if t-StatusProhibit is still running.

# Conclusions

This paper presents some of our views on RLC-AM enhancements relating to unnecessary retransmission of discarded RLC SDUs. We have the following proposals:

Proposal 1: When all RLC SDUs with SNs between (and including) TX_Next_Ack and POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter should stop and reset the running t-PollRetransmit.

Proposal 2: The receiver should transmit the status report triggered by the expiration of t-RxDiscard immediately, even if t-StatusProhibit is still running.

# References

- 3GPP RAN2 #129bis Chair’s Notes, Wuhan, China, April 2025.