3GPP TSG-RAN WG2 Meeting #1230												             R2-250xxxx

St Julian’s, Malta, 19-23 May 2025

Agenda item:	8.2.4

Source:	Apple

Title:	Remaining issues in data transmission

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on the remaining issues of Ambient IoT MAC data transmission.

# 2   	Discussion

## 2.1 	Extensibility

The issue of A-IoT MAC extensibility (e.g. in future releases) has come up as part of some other discussions, but no agreements have been made. We think it is time for a proper discussion of this issue.

There are several mechanisms that allow extensibility listed below, which we discuss in detail:

- Spare bits

- Message type

- Protocol version indication

- Extension bit

We are likely to end up having a few spare/reserved bits, which presents an easy way to add more functionality in the future. For D2R, we’ve already agreed to introduce “more data” 1 bit indication, which is likely to go into a control field in the A-IoT MAC header. In many (if not all) cases it should be possible to design new features in such a way that the legacy devices not supporting it would always signal a specific value (e.g. 0) in the spare/reserved bits. Having said that, it can’t be the main extensibility mechanism.

Observation 1: spare/reserved bits can be used for future extensions; however we can’t rely on them exclusively for extensibility.

We’ve already agreed to introduce message type indication at least in R2D. Assuming there is sufficiently large unused “message type space”, it can be used for future extensions. It would allow introducing new messages and it can also be used to “override” the legacy messages. For example, if in the future we need say R2D data transmission or paging for that matter with radically new design requirements, we can introduce a new message type for non-backwards compatible R2D data transmission or any other message (e.g. paging), the design of which would be unrestrained by legacy.

This is in fact the primary motivation to consider D2R message type, which may not be needed otherwise.

Observation 2: message type (with sufficiently large unused message type space) can be used for future extensions; this may be an additional motivation to introduce message type in D2R.

And two “proper” ways to ensure future extensibility:

- Protocol version indication, or

- Extension bit

With the protocol version field (included in D2R and R2D messages), in the next release we would increase the version number, which would allow us unrestricted freedom of design for the next extensions.

With regards to the existing features and their control bit assignment there would be two options:

- Complete redesign of the whole header, both the legacy and the new parts

- Keep the legacy part as is, and only add new features and their corresponding control bits in e.g. the second byte

Either way, in this approach both the device and the reader would indicate their supported version in the header and the legacy devices would ignore MAC PDUs with protocol versions they don’t support. This would probably require the reader to support both versions and if (it expects multiple versions in the population of devices they need to communicate with), the reader would need to integrate and communicate with devices of different versions separately. For example, the reader would first inventory/communicate with devices of version 0 and then proceed to do the same with devices supporting version 1, etc.

Alternatively, we can allocate 1 (e.g. the last bit) to indicate that there is an extension.

In this alternative, in the first version the extension bit X would be set to 0, and to 1 in the next release. When a legacy (e.g. Rel-19) device sees that X is set to 1, it would assume that there is an extra byte in the control header that it is not expected to comprehend and process.

Proposal 1: to discuss which future extensibility mechanism to support in both D2R and R2D amongst:

- Protocol version

- Extension bit

- Message types (with sufficiently large “space”)

## 2.2 	Remaining issues (i.e. FFS from previous meetings)

In this section we discuss the remaining data transmission issues, in no particular order. We started with the ones marked “FFS” from the previous discussions, specifically:

FFS whether for D2R we need message type field, any length and need for padding

For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

FFS whether the reader always includes the command for retransmission of segments.

For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.

In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

We’ve already touched upon the issue of message type in D2R in the context of extensibility. Beyond that, it may also be useful for robustness as it may help address various cases of device mobility (e.g. a device with an active D2R granted moving into coverage of a reader other than the one that had scheduled the grant).. However, the main motivation to consider message type in D2R is the question of extensibility. This would allow us to define new D2R messages in the future, without the issue of backwards compatibility. Furthermore, since it is likely that in the future DO-A traffic will need to be supported, having message type in D2R is also beneficial from the future compatibility point of view to support DO-A, where a reader may never know whether a D2R message it receives is DO-A or from a device that had previously communicated with another reader. Therefore, the safe choice here is to adopt message type in D2R as well.

Proposal 2: to support message type in D2R direction.

On the issue of length in D2R, we’ve already agreed “to indicate how many SDU bits are present” and the only remaining FFS is whether to indicate SDU length or padding length. One motivation to signal “padding length” might be that on average, the length of padding will be smaller (in fact, probably much smaller) than the length of an SDU. This is because the CN might be able to indicate at least some estimate for the expected device response and because in many practical scenarios such response is likely to be fixed. However, since we need to for the worst-case scenario (i.e. when the indication from the CN is not available and the device has very little data to send, either way the length field would have to accommodate the maximum SDU length.

Observation 4: there is no difference whether to indicate D2R SDU length or D2R padding length, in both cases the maximum length should be equal to the maximum SDU length.

Proposal 3: select one option between SDU length or padding length; the length size (regardless of the option chosen) is equal to the maximum SDU size.

On the question of NACK for D2R data, since we’ve already agreed to introduce offset signalling for retransmission, there is little additional benefit in introducing NACK. A subsequent request for the same offset as the one previously signalled is in fact equivalent to NACK signalling, and therefore an additional explicit NACK for D2R data is not needed.

Observation 5: A subsequent request for the same offset as the one previously signalled is in fact equivalent to NACK signalling.

Proposal 4: explicit NACK for D2R data is not needed.

Finally, there is the question of whether offset 0 is included for retransmission of the first segment/unsegmented D2R message, when the reader sends the R2D message including the upper layer command again.  Frankly, either way, the design would be inconsistent. Perhaps the only way out of this is to agree that device ignores offset equal to 0. This is a strange design, but the only other option is to re-discuss the agreements we’ve made.

Proposal 5: device ignores offset equal to 0.

Finally, there is the FFS on “whether the reader always includes the command for retransmission of segments. “. Our understanding is that the FFS applies for the case of offset>0, as for the triggering the retransmission of the first segment the command is obviously needed. For the other cases though resending the command would be redundant. So assuming the MAC header design allows omitting the command, there is no needed to include it when triggering retransmissions of segments other than the first one.

Proposal 6: there is no needed to include a command when triggering retransmissions of segments other than the first one (i.e. offset>0).

## 2.3 Optionality

Since we’ve agreed to support relatively complex feature such as segmentation, that not all A-IoT devices should be expected to support, the question then becomes how to realize optionality. The usual 3GPP way of UE capability signaling is obviously out of the question, as we have agreed not to use RRC (and in fact agreed to only define MAC).

Observation 6: as there is no RRC in Ambient IoT, optional features cannot be realized through capability signalling.

We think that at least segmentation and more data indication should be considered optional.

Our proposal is to define the signalling (i.e. MAC control header bits and the associated device behavior) is such a way, that devices supporting the feature would signal the relevant indication while the devices not supporting it would signal a fixed value, e.g, 0. This should be designed in such a way that a reader would be able to communicate with either devices supporting or not supporting the feature without explicit a-priori knowledge of whether the feature is supported.

For example, the more data indication can be defined as follows:

- 1 indicates device has more (beyond the current grant) data to transmit

- 0 otherwise (i.e. this is the last segment); device not supporting the feature always signals 0

The devices that support the feature and have more segments to transmit would indicate 1, the devices that support the feature would but have no more data transmit would indicate 0 and the devices that don’t support the feature would also indicate 0. This way the reader would always know whether it can/should request for retransmission in case of failure or for more data if/when needed.

Proposal 7: optional features (e.g. segmentation and more data indication) are designed in a way that capability signaling is not required.

Proposal 8: for more data indication, 1 indicates device has more (beyond the current grant) data to transmit and 0 otherwise (or if the device doesn’t support the feature).

# 3	Proposals

Observation 1: spare/reserved bits can be used for future extensions ; however we can’t rely on them exclusively for extensibility.

Observation 2: message type (with sufficiently large unused message type space) can be used for future extensions; this may be an additional motivation to introduce message type in D2R.

Proposal 1: to discuss which future extensibility mechanism to support in both D2R and R2D amongst:

- Protocol version

- Extension bit

- Message types (with sufficiently large “space”)

Proposal 2: to support message type in D2R direction.

Observation 4: there is no difference whether to indicate D2R SDU length or D2R padding length , in both cases the maximum length should be equal to the maximum SDU length.

Proposal 3: select one option between SDU length or padding length; the length size (regardless of the option chosen) is equal to the maximum SDU size.

Observation 5: A subsequent request for the same offset as the one previously signalled is in fact equivalent to NACK signalling.

Proposal 4: explicit NACK for D2R data is not needed.

Proposal 5: device ignores offset equal to 0.

Proposal 6: there is no needed to include a command when triggering retransmissions of segments other than the first one (i.e. offset>0).

Observation 6: as there is no RRC in Ambient IoT, optional features cannot be realized through capability signalling.

Proposal 7: optional features (e.g. segmentation and more data indication) are designed in a way that capability signaling is not required .

Proposal 8: for more data indication, 1 indicates device has more (beyond the current grant) data to transmit and 0 otherwise (or if the device doesn’t support the feature).

# 5	References

[1]		RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]		TR 38.769 Study on solutions for ambient IoT (Internet of Things)