# 3GPP TSG RAN WG2 Meeting #130      	                           R2-2503712
St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.4

Source:	Apple

Title:	Remaining issues on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective of adaptation of common signal transmission includes paging adaptation, PRACH adaptation and SSB adaptation, as copied below:

In RAN2#129 [2] and RAN2#129b [3], good progress of common signal adaptation was made. However, there are still some leftover issues on paging adaptation and RACH adaptation. In this contribution, we further discuss remaining issues on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Paging adaptation

In RAN2#129 [2], the following agreements on paging adaptation were made:

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

And in RAN2#129b [3], it was further agreed on UE behavior if new PEI configuration is provided:

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

We think the main remaining issues include the below aspects:

- Details on new paging configuration

- Details of new PEI configuration

### 2.1.1 Details on new paging configuration

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle, OPTIONAL  -- Need R

…omit…

]],

[[

pagingAdaptation-NS-r19           ENUMERATED {eight,four, two, one}                 OPTIONAL,  -- Need R

pagingAdaptationNAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER (0..31)

}

]]

}

pagingAdaptation-NS Number of paging occasions per paging frame for paging adaptation

pagingAdaptationNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on.

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

In RRC running CR [4], the new paging configuration is captured as below. We discuss the two FFSs captured in EN.

FFS#1: field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions

In previous RAN2 meetings, the following restriction on paging configuration were proposed:

- Only same PF_offset as legacy paging configuration can be configured by NW.

- UE specific DRX cycle provided in NAS signaling is not considered, i.e. only defaultPagingCycle is used for Rel-19 paging.

We don’t think these restrictions are needed:

- For 1), we think the flexibility of new PF_offset is useful if NW don’t want to allow Rel-19 POs to overlap with legacy POs

- For 2), we think its main intention is to resolve the issue that number of PF<1 if N takes larger values like 64 or 256. However, since RAN2 agreed to only extend N to 32, we don’t have this issue. Then, it doesn’t make sense to ignore UE dedicated paging cycle provided in NAS signaling.

Thus, we propose:

Proposal 1: No need to have any configuration restriction on Rel-19 paging configuration (i.e. pagingAdaptationNAndPagingFrameOffset, pagingAdaptation-NS).

Proposal 2: The UE derives Rel-19 paging cycle as legacy, i.e. min(default paging cycle, UE specific DRX cycle).

FFS#2: firstPDCCH-MonitoringOccasionOfPO for paging adaptations

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                           SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                            SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}

firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE {

sCS480KHZoneEighthT                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

firstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP, see TS 38.304 [20]. The field sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS120KHZoneSixteenthT can be applied for SCS 480kHz, corresponding to sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS480KHZquarterT-SCS120KHZoneSixteenthT in IE DownlinkConfigCommonSIB respectively.

The legacy firstPDCCH-MonitoringOccasionOfPO provides symbol level offset for 1st PMO in each PO:

We identify two gaps to use legacy firstPDCCH-MonitoringOccasionOfPO to support Rel-19 PO:

- As highlighted above, legacy firstPDCCH-MonitoringOccasionOfPO only supports up to 4 PO per PF. However, Rel-19 paging adaptation supports up to 8 PO per PF. Thus, we need to extend maxPO-perPF-r19 to 8.

- As highlighted above, legacy firstPDCCH-MonitoringOccasionOfPO only covers up to 16 frames because smallest N is 1/16 in legacy paging. As N can take 1/32 in Rel-19 paging adaptation, we need to introduce new IE with extended range to cover the case of 32 frames.

An ASN.1 example to implement above changes can be found below:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                         SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

}

maxPO-perPF-r19                             INTEGER ::= 8       -- Maximum number of paging occasion per paging frame

Observation 1: There are two gaps to use legacy firstPDCCH-MonitoringOccasionOfPO to support Rel-19 PO:

- Need to extend maxPO-perPF-r19 to 8.

- Need to introduce new IE with extended range to cover the case of 32 frames (for N=1/32 case).

In previous RAN2 meetings, some company proposed to introduce slot level offset rather than existing symbol level offset in firstPDCCH-MonitoringOccasionOfPO, in order to reduce RRC overhead. However, as it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore.

Observation 2: As it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore. Thus, no need to introduce slot level offset in firstPDCCH-MonitoringOccasionOfPO.

Thus, we propose:

Proposal 3: Introduce a new optional IE firstPDCCH-MonitoringOccasionOfPO-r19 to support Rel-19 PO:

- Extend maxPO-perPF-r19 to 8.

- Redesign symbol level offset IEs to cover the case of N=1/32.

Example of ASN.1 change:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                         SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

}

maxPO-perPF-r19                             INTEGER ::= 8       -- Maximum number of paging occasion per paging frame

### 2.1.2 Details of new PEI configuration

In RRC running CR [4], the new PEI configuration is captured as below. We discuss the highlighted FFS in EN.

DownlinkConfigCommonSIB ::=     SEQUENCE {

[[

pei-Config-r17                  PEI-Config-r17                         OPTIONAL,     -- Need R

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                     OPTIONAL      -- Need R

]],

[[

frequencyInfoDL-v1800           FrequencyInfoDL-SIB-v1800              OPTIONAL      -- Need R

]],

[[

pagingAdaptationPEI-Config-r19          PEI-Config-r17                         OPTIONAL     -- Need R

]]

}

pagingAdaptationPEI-Config The PEI related configuration for paging adaptation

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                          SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE(SIZE(1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE(SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                    SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                 SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

The legacy pei-ConfigBWP include PEI SearchSpace and symbol level offset firstPDCCH-MonitoringOccasionOfPEI-O (similar to symbol offset of PMO firstPDCCH-MonitoringOccasionOfPO):

First, we think it is not necessary to introduce a separate PEI SearchSpace because legacy paging SearchSpace is reused.

Proposal 4: No need to introduce a separate PEI SearchSpace for Rel-19 PEI.

Then, similar to firstPDCCH-MonitoringOccasionOfPO, we think the legacy firstPDCCH-MonitoringOccasionOfPEI-O can’t well support the case of Ns=8 in the new PEI-O configuration:

- Legacy firstPDCCH-MonitoringOccasionOfPEI-O only supports up to 4 PO per PF. However, Rel-19 paging adaptation supports up to 8 PO per PF. Thus, we need to extend maxPEI-perPF-r19 to 8.

- Legacy firstPDCCH-MonitoringOccasionOfPEI-O only covers up to 16 frames because smallest N is 1/16 in legacy paging. As N can take 1/32 in Rel-19 paging adaptation, we need to introduce new IE with extended range to cover the case of 32 frames.

Observation 3: Similar to PMO offset, there are two gaps to use legacy firstPDCCH-MonitoringOccasionOfPEI-O to support Rel-19 PEI:

- Need to extend maxPEI-perPF-r19 to 8.

- Need to introduce new IE with extended range to cover the case of 32 frames (for N=1/32 case).

And an ASN.1 example to implement above changes can be found below:

firstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..143359)

}

maxPEI-perPF-r19                        INTEGER ::= 8       -- Maximum number of PEI occasion per paging frame

Thus, we propose:

Proposal 5: Introduce a new IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 to support Rel-19 PEI:

- Extend maxPEI-perPF-r19 to 8.

- Redesign symbol level offset IEs to cover the case of N=1/32.

Example of ASN.1 change:

firstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..143359)

}

maxPEI-perPF-r19                        INTEGER ::= 8       -- Maximum number of PEI occasion per paging frame

## 2.2 Adaptation of PRACH in time domain

### 2.2.1 Configuration of additional RACH resource for RACH adaptation

According to the latest RAN1 L1 excel, the new L1 parameters required for RACH adaptation is summarized in the following table:

| Parameter name in the spec | Description | Value range |
| --- | --- | --- |
| [addl-RACH-Config-Dyn] addl-RACH-Config-Adaptation | Configuration of additional RACH resources for DCI based adaptation of RACH in time domain. | FFS within [the same] RACH-ConfigCommon in SIB1 |
| PRACH-Config Index | RACH configuration index for additional RACH resources for DCI based adaptation of RACH in time domain. | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| msg1-FrequencyStart | Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0, at least for 4-step RACH. | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| [prach-Mask-SubsetIdentification-Dyn] prach-SubsetMask-Index-Adaptation | Single PRACH mask to identify the subset of the additional PRACH resources applicable at least for adaptation for DCI 1_0 with P-RNTI. | FFS |
| msg1-FDM | The number of PRACH transmission occasions FDMed in one time instance, at least for 4-step RACH. | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| ssb-perRACH-Occasion ssb-perRACH-OccasionAndCB-PreamblesPerSSB | Configuration of number of SSB per RO and CB preambles per SSB. | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |

As indicate in the table, addl-RACH-Config-Adaptation is configured within the same RACH-ConfigCommon in SIB1. We think there are two alternatives on signaling structure:

- Alt-1: addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

- Alt-2: addl-RACH-Config-Adaptation is placed under rach-ConfigGeneric

We prefer Alt-1 due to below 2 considerations:

- As rach-ConfigGeneric is applied to all types of RACH (including Msg1 based OD-SIB and IAB RACH which were precluded by RAN2 for RACH adaptation), Alt-2 needs to specify which RACH can’t be configured with addl-RACH-Config-Adaptation.

- It doesn’t make sense to include ssb-perRACH-OccasionAndCB-PreamblesPerSSB under rach-ConfigGeneric.

One example of Alt-1 is illustrated as follows:

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL,   -- Need M

Skip unrelated part

[[

ra-PrioritizationForSlicing-r17         RA-PrioritizationForSlicing-r17                 OPTIONAL,   -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF

FeatureCombinationPreambles-r17 OPTIONAL                                                                  -- Cond AdditionalRACH

]]

[[

addl-RACH-Config-Adaptation-r19                                                                             OPTIONAL,   -- Need R

]]

}

addl-RACH-Config-Adaptation-r19 ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

prach-SubsetMask-Index-Adaptation     FFS,

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL   -- Need M

...

}

Thus, we propose:

Proposal 6: Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1.

Meanwhile, please note that RAN2#129 agreed to support RACH adaptation for all RACH partioning features, including RedCap, SDT, Slicing, and feature combinations [2].

Agreements on RACH adaptation

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

Proposal 6 can’t support this agreement because RACH portioning features are configured in AdditionalRACH-Config which is in the same level of RACH-ConfigCommon, as illustrated in Figure.1.

Figure.1 Illustration of RRC configuration of RACH partitioning features

We think it needs to clarify that addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

Proposal 7: addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

### 2.2.2 RACH adaptation signaling

RAN1 has introduced DCI triggered RACH adaptation via DCI 1-0 with P-RNTI (paging) and DCI 1-0 with C-RNTI (PDCCH order). Furthermore, RAN1#120b agreed to use 1-bit from the Bits 5-8 within the Short Message field [5]. And RAN1 sent LS to RAN2 to confirm whether it is feasible.

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows:

- ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

Final LS in R1-2503086.

We think it is fine to use one bit in paging short message.

Proposal 7: RAN2 confirm that it is feasible to use one bit in Bits 5-8 of paging short message for RACH adaptation indication.

Furthermore, we think it is not necessary to always rely on DCI to trigger RACH adaptation because it will lead to extra paging message, which is bad for NW energy saving. Instead, we prefer to also support RACH adaptation indication in SIB1, i.e. introduce a new indication in SIB1 to activate / deactivate RACH adaptation. Furthermore, for UEs that have just accessed the network, they may not receive the Paging DCI yet in the current SI modification period, and the NW may not transmit Paging DCI for PRACH adaptation in the next SI modification period. This results in that these UEs will not be able to utilize the additional PRACH resources.

Observation 4: Only allowing DCI triggered RACH adaptation will lead to extra paging message. Furthermore, for UEs that have just accessed the network, they may not receive the Paging DCI yet in the current SI modification period. This results in that these UEs will not be able to utilize the additional PRACH resources.

Different from DCI triggered RACH adaptation, we don’t think validity duration should be introduced for SIB1 triggered RACH adaptation because BCCH modification period essentially indicates its validity duration.

Observation 5: Different from DCI triggered RACH adaptation, validate duration is not needed for SIB1 triggered RACH adaptation because BCCH modification period essentially indicates its validity duration.

Thus, we propose:

Proposal 8: Besides DCI triggered RACH adaptation, also support RACH adaptation indicated by SIB1, i.e. introduce a new indication in SIB1 to activate / deactivate RACH adaptation.

Finally, we think there is one special issue: as DCI with P-RNTI is cell common signaling, it is possible that one UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication.

Observation 6: As DCI with P-RNTI is cell common signaling, it is possible that the UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication.

We prefer the simplest solution at late stage of Rel-19: same as RRC reconfiguration of RACH resources, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

Proposal 9: If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

### 2.2.3 RA-RNTI ambiguity issue

In RAN1#120 meeting [6], there were agreements on configuring separately a Msg1-FDM for the additional PRACH resources.

Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Considering that the legacy ROs and additional ROs could be overlapping in time domain, then for the ROs which are FDMed with the same f_id, their RA-RNTI may be the same. In this case, although a legacy UE and a new UE are sending Msg 1 on different ROs, they use the same RA-RNTI to monitor RAR which increases the collision probability and also increases UE power consumption for detecting RAR not intended for itself.

Observation 7: As legacy ROs and additional ROs could be overlapping in time domain, for the ROs which are FDMed with the same f_id, their RA-RNTI are the same, which may lead to increasing RAR collision probability and UE power consumption for detecting RAR not intended for itself.

To solve the RA-RNTI ambiguity issue, the following alternatives can be considered:

- Alt 1: For RA-RNTI calculation, introduce a fixed k_offset for the additional PRACH resources similar to 2-step RACH, i.e.

RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset, where the k_offset >1.

- Alt 2: For RA-RNTI calculation, f_id for the additional PRACH resources overlapped in time domain with the legacy resources is calculated based on the number of FDMed legacy ROs, i.e. f_id_A is the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

As shown in Figure. 2, this can be applied to all the additional PRACH resources regardless of whether they overlap with legacy ones in time domain or not, or can be applied only to the additional PRACH resources that are overlapped in time domain (Alt 2-2).

Proposal 10: RAN2 discuss RA-RNTI ambiguity issue, and consider the following alternatives:

- Alt 1: Introduce a fixed k_offset for the additional PRACH resources like 2-step RACH, i.e. RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset (k_offset>1).

- Alt 2: f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

Figure.2 Illustration of Alt-2

### 2.2.4 RACH adaptation for CFRA

RACH adaptation for CFRA is discussed in post-meeting email discussion [7], and below summary proposals are made:

Proposal 1	R2 not pursue spec impact specifically for the usage of R19 additional RACH resources for CFRA initiated by

- PDCCH order for additional RACH;

- L3 HO command.

- LTM CSC MAC-CE;

- PDCCH order for LTM early UL sync;

Proposal 2	R2 not pursue spec impact specifically for the usage for R19 additional RACH resources for CFRA initiated by BFR. And taking that as a premise, R2 discuss whether UE does not base on DCI 1_0 of P-RNTI to know the availability of RACH resource indicated in rach-configGeneric for BFR.

Proposal 3	R2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

We think different companies seem to have different understanding on “spec impact specifically for the usage of R19 additional RACH resources for CFRA” in P1 and P2. To ensure an efficient online discussion, we suggest proponents of CFRA adaptation to clarify their understanding on below aspects:

- Which RACH resource is applied in CFRA adaptation (e.g. whether RACH resource configured via RACH-ConfigCommon in SIB1 or dedicated CFRA resource).

- Whether need to introduce dedicated SSB-to-RO mapping configuration for CFRA resource.

  - Please note that as legacy RACH and additional RACH may overlap, RAN1 agreed to introduce a separate mapping IE ssb-perRACH-OccasionAndCB-PreamblesPerSSB for CBRA adaptation

- Which Mask is applied in CFRA adaptation (e.g. whether new Mask configured in SIB1 or legacy Mask configured in RRC).

Thus, we propose:

Proposal 11: To ensure efficient discussion, proponents of CFRA adaptation is suggested to clarify their understanding on below aspects:

- Which additional RACH resource is applied in CFRA adaptation (e.g. whether RACH resource configured via RACH-ConfigCommon in SIB1 or dedicated CFRA resource).

- Whether need to introduce dedicated SSB-to-RO mapping config for additional CFRA resource.

- Which Mask is applied in CFRA adaptation (e.g. whether new Mask configured in SIB1 or legacy Mask configured in RRC).

# 3 Conclusion

In this contribution, we further discuss remaining issues on common signal transmission adaptation. Our observations are:

Observation 1: There are two gaps to use legacy firstPDCCH-MonitoringOccasionOfPO to support Rel-19 PO:

- Need to extend maxPO-perPF-r19 to 8.

- Need to introduce new IE with extended range to cover the case of 32 frames (for N=1/32 case).

Observation 2: As it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore. Thus, no need to introduce slot level offset in firstPDCCH-MonitoringOccasionOfPO.

Observation 3: Similar to PMO offset, there are two gaps to use legacy firstPDCCH-MonitoringOccasionOfPEI-O to support Rel-19 PEI:

- Need to extend maxPEI-perPF-r19 to 8.

- Need to introduce new IE with extended range to cover the case of 32 frames (for N=1/32 case).

Observation 4: Only allowing DCI triggered RACH adaptation will lead to extra paging message. Furthermore, for UEs that have just accessed the network, they may not receive the Paging DCI yet in the current SI modification period. This results in that these UEs will not be able to utilize the additional PRACH resources.

Observation 5: Different from DCI triggered RACH adaptation, validate duration is not needed for SIB1 triggered RACH adaptation because BCCH modification period essentially indicates its validity duration.

Observation 6: As DCI with P-RNTI is cell common signaling, it is possible that the UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication.

Observation 7: As legacy ROs and additional ROs could be overlapping in time domain, for the ROs which are FDMed with the same f_id, their RA-RNTI are the same, which may lead to increasing RAR collision probability and UE power consumption for detecting RAR not intended for itself.

Observation 8: RAN1#120 has agreed to use DCI 1-0 with C-RNTI to trigger adaptation PDCCH order RACH. However, it is not clear whether “PDCCH order” in the agreement means PDCCH order RACH in serving cell (e.g., for time alignment) or PDCCH order RACH in candidate cell (i.e. PDCCH order for early sync in LTM).

Observation 9: If supporting PDCCH order RACH adaptation for early sync in LTM, it requires candidate cell to reserve two PRACH resource sets and timely coordination between source cell and target cell. Furthermore, it requires RRC specification change to add additional RACH resource configuration in IE LTM-Candidate.

Based on observations, our proposals are:

Paging adaptation

Proposal 1: No need to have any configuration restriction on Rel-19 paging configuration (i.e. pagingAdaptationNAndPagingFrameOffset, pagingAdaptation-NS).

Proposal 2: The UE derives Rel-19 paging cycle as legacy, i.e. min(default paging cycle, UE specific DRX cycle).

Proposal 3: Introduce a new optional IE firstPDCCH-MonitoringOccasionOfPO-r19 to support Rel-19 PO:

- Extend maxPO-perPF-r19 to 8.

- Redesign symbol level offset IEs to cover the case of N=1/32.

Example of ASN.1 change:

firstPDCCH-MonitoringOccasionOfPO-r19   CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                         SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                  SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

}

maxPO-perPF-r19                             INTEGER ::= 8       -- Maximum number of paging occasion per paging frame

Proposal 4: No need to introduce a separate PEI SearchSpace for Rel-19 PEI.

Proposal 5: Introduce a new IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 to support Rel-19 PEI:

- Extend maxPEI-perPF-r19 to 8.

- Redesign symbol level offset IEs to cover the case of N=1/32.

Example of ASN.1 change:

firstPDCCH-MonitoringOccasionOfPEI-O-r19  CHOICE {

sCS15KHZoneT                                                           SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1.. maxPO-perPF-r19)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT   SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZone32T    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-SCS60KHZone32T                    SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..17919),

sCS480KHZoneEighthT-sCS120KHZone32T                      SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                  SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..71679),

sCS480KHZone32T                          SEQUENCE (SIZE (1.. maxPEI-perPF-r19)) OF INTEGER (0..143359)

}

maxPEI-perPF-r19                        INTEGER ::= 8       -- Maximum number of PEI occasion per paging frame

RACH adaptation

Proposal 6: Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1.

Proposal 7: addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

Proposal 8: Besides DCI triggered RACH adaptation, also support RACH adaptation indicated by SIB1, i.e. introduce a new indication in SIB1 to activate / deactivate RACH adaptation.

Proposal 9: If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

Proposal 10: RAN2 discuss RA-RNTI ambiguity issue, and consider the following alternatives:

- Alt 1: Introduce a fixed k_offset for the additional PRACH resources like 2-step RACH, i.e. RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset (k_offset>1).

- Alt 2: f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

Proposal 11: To ensure efficient discussion, proponents of CFRA adaptation is suggested to clarify their understanding on below aspects:

- Which additional RACH resource is applied in CFRA adaptation (e.g. whether RACH resource configured via RACH-ConfigCommon in SIB1 or dedicated CFRA resource).

- Whether need to introduce dedicated SSB-to-RO mapping config for additional CFRA resource.

- Which Mask is applied in CFRA adaptation (e.g. whether new Mask configured in SIB1 or legacy Mask configured in RRC).

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#129, Chair Notes.

[3] RAN2#129b, Chair Notes.

[4] R2-250xxxx, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[5] RAN1#120b, Chair Notes.

[6] RAN1#120, Chair Notes.

[7] R2-250xxxx, Summary of [POST129b][122][NES] (OPPO), OPPO.