3GPP TSG-RAN WG2 Meeting #130	R2-2503713

# St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running MAC CR for AI/ML for Air Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_Air-Core |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Rel-19 AI/ML for PHY to TS 38.321 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementation of agreements up to RAN2#129b and RAN1#120b. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Release-19 AI/ML for PHY is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9, 5.15, 5.18.6, 6.1.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================================CHAGNE BEGINS===================================

### 5.18.6	Activation/Deactivation of Semi-persistent CSI reporting on PUCCH

The network may activate and deactivate the configured Semi-persistent CSI reporting on PUCCH of a Serving Cell by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.16 or the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.80. The network may activate and deactivate at least one of the configured sub-configuration of a configured Semi-Persistent CSI reporting on PUCCH of a Serving Cell by sending the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.80. For each Semi-persistent CSI reporting on PUCCH configuration configured with csi-ReportSubConfigToAddModList, the network may deactivate all configured sub-configurations by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE. For each Semi-persistent CSI reporting on PUCCH configuration configured with measurement prediction, if it is reported as applicable as specified in TS 38.331 [5], the network may activate and deactivate it by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE. The configured Semi-persistent CSI reporting on PUCCH is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall:

1>	if the MAC entity receives an SP CSI reporting on PUCCH Activation/Deactivation MAC CE or an Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the SP CSI reporting on PUCCH Activation/Deactivation MAC CE or the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

=====================================NEXT CHANGE===================================

#### 6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	L: This field indciates whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for LTM or not. If ltm-CSI-ReportConfigToAddModList is not configured, R field is present instead (i.e. set to 0);

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList if L field is set to 1, or csi-ReportConfigToAddModList if L field is set to 0, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated. If the Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigToAddModList, the Si field is set to 0 to additionally indicate that all sub-configurations within csi-ReportSubConfigToAddModList shall be deactivated;

-	R: Reserved bit, set to 0.

NOTE:	If a Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigToAddModList, the corresponding Si field is not set to 1.

Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Editor’s Note: for activation / deactivation of AI/ML based SP CSI reporting on PUCCH via MAC-CE, it is FFS whether the legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE is reused or a new MAC-CE needs to be introduced.

=====================================NEXT CHANGE===================================

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	if the SCell was deactivated prior to receiving this Enhanced SCell Activation/Deactivation MAC CE and a TRS is indicated for this SCell:

3>	indicate to lower layers the information regarding the TRS.

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

[FFS 5>	CSI logging for the SCell;]

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

3>	else (i.e. firstActiveDownlinkBWP-Id is set to dormant BWP):

4>	stop the bwp-InactivityTimer of this Serving Cell, if running.

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

2>	if the active DL BWP is not the dormant BWP:

3>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

3>	trigger PHR according to clause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

[FFS 2>	not log CSI for the SCell;]

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

When the measurement reporting for fast unknown SCell activation is configured by RRC, the MAC entity shall:

1>	if SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell(s):

2>	if SCell(s) was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE:

3>	indicate to upper layers SCell(s) activation indication.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE or Enhanced SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

Editor’s Note: it is FFS whether the L1 measurement logging is stopped in the deactivated SCell.

=====================================NEXT CHANGE===================================

## 5.15	Bandwidth Part (BWP) operation

### 5.15.1	Downlink and Uplink

In addition to clause 12 of TS 38.213 [6], this clause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell except for PSCell when SCG is deactivated (see clause 5.29) or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id for PSCell when SCG is deactivated, the DL BWP is switched to the firstActiveDownlinkBWP-Id as specified in TS 38.331 [5]. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each SCell a dormant BWP may be configured with dormantBWP-Id by RRC signalling as described in TS 38.331 [5]. Entering or leaving dormant BWP for SCells is done by BWP switching per SCell or per dormancy SCell group based on instruction from PDCCH (as specified in TS 38.213 [6]). The dormancy SCell group configurations are configured by RRC signalling as described in TS 38.331 [5]. Upon reception of the PDCCH indicating leaving dormant BWP, the DL BWP indicated by firstOutsideActiveTimeBWP-Id or by firstWithinActiveTimeBWP-Id (as specified in TS 38.331 [5] and TS 38.213 [6]) is activated. Upon reception of the PDCCH indicating entering dormant BWP, the DL BWP indicated by dormantBWP-Id (as specified in TS 38.331 [5]) is activated. The dormant BWP configuration for SpCell or PUCCH SCell is not supported.

BWP for SRS for positioning Tx frequency hopping can be configured for a Serving Cell in TS 38.331 [5]. BWP for SRS Tx frequency hopping is considered as activated when it is configured. BWP switching is not applicable for BWP for SRS Tx frequency hopping.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP and the Serving Cell is not the PSCell of deactivated SCG:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

[FFS 2>	log CSI for the BWP;]

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

2>	if lbt-FailureRecoveryConfig is configured:

3>	stop the lbt-FailureDetectionTimer, if running;

3>	set LBT_COUNTER to 0;

3>	monitor LBT failure indications from lower layers as specified in clause 5.21.2.

1>	if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP:

2>	stop the bwp-InactivityTimer of this Serving Cell, if running.

2>	not monitor the PDCCH on the BWP;

2>	not monitor the PDCCH for the BWP;

2>	not receive DL-SCH on the BWP;

2>	not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

[FFS 2>	not log CSI on the BWP;]

2>	not transmit SRS on the BWP;

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

1>	if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

[FFS 2>	not log CSI for the BWP;]

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is an (e)RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is an (e)RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered consistent LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the active DL BWP is not the initialDownlinkBWP-RedCap:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a (e)RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP-RedCap:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

Editor’s Note: it is FFS whether the L1 measurement logging is stopped in the deactivated BWP.

Editor’s Note: it is FFS whether the L1 measurement logging is stopped in the dormant BWP.

Editor’s Note: it is FFS whether the L1 measurement logging is stopped in the deactivated SCG.

===============================CHANGE ENDS=========================================

# RAN2 Agreements

The RAN2 agreements are listed below. The following colors are used for the agreements that were implemented in the current version of the running CR:

- Implemented agreements

## RAN2#125bis

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model

##### General:

Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

##### Beam management & Positioning:

Agreements:

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

- FFS whether rhere is specification impact associated to gNB-side model monitoring.

- For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

#### 8.1.2.2	LCM for UE-sided model

##### Functionality granularity and capability:

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

##### Applicability/additional conditions:

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

- FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

##### Common LCM framework/signalling:

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

### 8.1.3	NW side data collection

##### Beam management:

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

## RAN2#126

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

Agreements for NW-sided model for Beam Management

1	For the network-side model, required network side additional condition is left up to the network implementation

2	RAN2 will wait for RAN1 for any required UE side additional conditions.

3	For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

#### 8.1.2.3	LCM for Positioning use case

Agreements for NWside model

1	RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

### 8.1.3	NW side data collection

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

## RAN2#127

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

#### 8.1.2.3	LCM for Positioning use case

### 8.1.3	NW side data collection

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

## RAN2#127bis

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

#### 8.1.2.3	LCM for Positioning use case

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

### 8.1.3	NW side data collection

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

## RAN2#128

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Agreements

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

4.	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

5.	Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

6.	For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

#### 8.1.2.3	LCM for Positioning use case

Agreements 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

### 8.1.3	NW side data collection

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

## RAN2#129

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

#### 8.1.2.3	LCM for Positioning use case

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

### 8.1.3	NW side data collection

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

### 8.1.4	UE side data collection

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

## RAN2#129b

### 8.1.2	Functionality based LCM

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

#### 8.1.2.2	LCM for UE-sided model for Beam Management use case

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

#### 8.1.2.3	LCM for Positioning use case

Agreements for positioning case 1

- LMF is responsible for functionality management

- UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

- Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

- An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

- For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

### 8.1.3	NW side data collection

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

- New SRB can be configured for NW-side data collection  (with lower priority)

# RAN1 Agreements

Some RAN1 agreements are listed below. The following colors are used for the agreements that were implemented in the current version of the running CR:

- Implemented agreements

## RAN1#119

    - Specification support for beam management

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3