# 3GPP TSG RAN WG2 Meeting #130      	                           R2-2503714
St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.1.2.2

Source:	Apple

Title:	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

By RAN2#129b [2], most of essential issues on LCM framework for UE side model have been closed. However, there are some leftover issues to be addressed. In this contribution, we discuss the following remaining issues on LCM procedure of AI/ML based beam management:

- Remaining issues on data collection configuration (Open issue RRC-3)

- Remaining issues on option A

  - Processing requirement of initial report in RRCReconfigurationComplete (Open issue RRC-17)

  - UE behavior for non-applicable CSI-ReportConfig(s) (Open issue RRC-4/5)

  - Details on UAI for applicability update report (Open issue RRC-2)

  - Whether associated ID is mandatory or optional (Open issue RRC-16)

- Remaining issues on option B

  - Details on initial applicability report in RRCReconfigurationComplete (Open issue RRC-7)

  - Co-existence of option A and option B (Open issue RRC-8)

- Simple cause for non-applicability (Open issue RRC-1)

- How to handle configuration in IDLE/INACTIVE/RLF (Open issue RRC-6)

- LCM of CSI prediction (Open issue RRC-13)

# 2 Discussion

## 2.1 Remaining issues on data collection configuration (RRC-3)

RAN2#129b agreed that UE can use UAI to report preferred configuration from a list of candidate configurations provided by the NW [2]. However, the signaling details are FFS.

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Agreement

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

As RAN2 agreed to use UAI message to send UE request, it is straight forward to include whether to allow send start/stop dicaiton and the candidate configurations in OtherConfig of RRCReconfiguraiton. On the detailed content of candidate configurations, we think the simplest way is to follow the following RAN1#120 agreement [3] to use a list of CSI-ReportConfig without CSI report which includes two CSI-ResourceConfigId for Set A and Set B respectively and one or two associated IDs:

Thus, we propose:

Proposal 1: To support UE request on data collection via UAI, include the following two configurations in OtherConfig of RRCReconfiguration:

- Indicaiton on whether to allow send start/stop request

- A list of candidate CSI-ReportConfig(s) without CSI report which at least includes two CSI-ResourceConfigId for Set A/Set B and one or two associated IDs according to RAN1#120 agreement.

Then, we discuss the detailed reporting format of UE preferred configuration. As we know, one CSI-ReportConfig is uniquely identified by CSI-ReportConfigId. Thus, we can use a list of CSI-ReportConfigId(s) to indicate UE preferred data collection configuration. Meanwhile, similar to applicability reporting, ServCellIndex needs to be included to support CA.

Proposal 2: The detailed reporting format of UE preferred data collection configuration includes ServCellIndex and a list of CSI-ReportConfigId(s) among the candidate CSI-ReportConfig(s) configured in OtherConfig.

Finally, we discuss the following FFS of RAN2#128 [3]:

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In our understanding, this FFS is intended to discuss whether it is necessary to introduce NW initiated UE request signaling (e.g. via RRCReconfigurationComplete message as response to data collection configuration in RRCReconfiguartion).

Please note that RAN2#129b agreed to use UAI for UE request, and it is up to UE implementation when to send the request. Then, if the UE can’t perform data collection based on received configuration, it can send the request via UAI with indication of stopping data collection and suggestion on applicable CSI resource configuration. Thus, we think UAI should be sufficient and there is no strong need to introduce NW initiated UE request signaling on top of UE initiated UAI message. Thus, we propose:

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

## 2.2 Remaining issues on option A

In RAN2#129 [5], it was agreed to support both Option A and Option B on applicability reporting configuration. And some agreements on how Option A works were also made:

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

We discuss the remaining issues on Option A in this section.

### Processing requirement of report via RRCReconfigurationComplete (RRC-17)

According to Section 12 of TS 38.331 [6], there are two processing latency requirements (10ms vs 16ms) between reception of RRCReconfiguration and reporting RRCReconfigurationComplete, depending on whether it is related to CA/DC operation.

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (Intra-NR mobility with LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration | DLDedicatedMessageSegment | RRCReconfigurationComplete | 16+( Nseg -1)*10 | Nseg is number of RRC segments |

For Option A, it was agreed to report initial applicability report in RRCReconfigurationComplete. However, it is not clear what is its processing latency requirement because the applicability reporting is a new reporting different from CA/DC configuration.

As Rel-19 is the first release of AI/ML, we expect new / unpredicted challenges for the UE to handle AI/ML operation. Thus, we prefer to use relaxed RRC processing latency requirements (i.e. 16ms) which is a safer option.

Proposal 4: As Rel-19 is the first release to introduce AI/ML, relaxed RRC processing timing requirement is applied for reporting applicability in RRCReconfigurationComplete (i.e. within 16ms after reception of Step 3).

### UE behaviour for non-applicable CSI-ReportConfig(s) (RRC-4/5)

Open issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

The issue is:

Open Issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

Issue description: It is ambiguous what the UE should report to the NW when its periodic beam prediction configuration becomes inapplicable and whether the model would output anything at all when if, e.g., the input distribution no longer matches what is expected.

We need to determine whether the configuration should cease reporting, or send the input (measurements) to the NW once it becomes inapplicable, or continue sending (inaccurate) predictions.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In our understanding, when the periodic beam prediction is inapplicable, the UE doesn’t have valid AI/ML model to perform correct inference. Then, there are 3 possible behaviors:

- Alt-1: The UE stops periodic beam prediction reporting until it becomes applicable

- Alt-2: The UE sends the input (measurements) to the NW once it becomes inapplicable

- Alt-3: The UE continue sending inaccurate predictions (e.g. use old AI/ML model)

We think Alt-2 is actually UE autonomous fallback to non-AI operation, which needs further spec impacts and should not be supported at late stage of Rel-19. Alt-3 is not preferred because inaccurate predication may mislead NW. Thus, we prefer Alt-1.

Proposal 5: To avoid misleading NW, the UE stops periodic beam prediction reporting after successfully transmitting UAI on its inapplicability.

Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

In RAN2#129 [5], it was agreed that UE autonomously activates the applicable periodic CSI-ReportConfig upon reporting applicable functionalities via RRCReconfigurationComplete.

- Agreements applicability reporting and management If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

RAN2#129b further agreed that UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly [2].

- Agreements on applicability reporting

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

However, the UE behavior is not clear on when the UE can activate the inapplicable inference configurations when it becomes applicable and reports in UAI. We think it needs to separately discuss for two cases:

- Periodic CSI reporting: following the same logic of RAN2#129 agreement [5], we think the UE can autonomously activate it upon reporting it becoming applicability via UAI. Otherwise, we need to introduce extra indication in RRC or lower layer indication to activate the periodic CSI reporting, which is unnecessary and goes against of the intention of introducing periodic reporting.

- SP/AP CSI reporting: it is straight forward to reuse RAN2#129 agreement [5], i.e. the UE waits legacy DCI / MAC-CE to activate SP / AP CSI reporting upon reporting it becoming applicability via UAI.

Thus, we propose:

Proposal 6: For the non-applicable CSI-ReportConfig, when it becomes applicable:

- The UE autonomously activates periodic CSI reporting upon reporting it becoming applicable via UAI.

- The UE waits legacy DCI / MAC-CE to activate SP/AP CSI reporting upon reporting it becoming applicable via UAI.

Please note that the opposite direction (i.e. when one applicable CSI-ReportConfig becomes non-applicable) was already agreed in RAN2#128 [4], i.e. the UE reports UAI on it becoming non-applicable and NW is expected deactivate this non-applicable CSI-ReportConfig:

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

### Details on UAI for applicability update report (RRC-2)

In RAN2#129 [5], it was agreed that UAI can be sent to update applicability.

Agreements

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

In legacy RRC [6], the UE reports UAI only if the NW required information are configured in OtherConfig of RRCReconfiguration. However, as Option A is configured in CSI-ReportConfig which is outside of OtherConfig of RRCReconfiguration, it is not clear how to specify the new UAI reporting behavior for applicability update.

Observation 1: In legacy RRC, the UE reports UAI only if the NW required information are configured in OtherConfig of RRCReconfiguraiton. However, as Option A is configured in CSI-ReportConfig which is outside of OtherConfig, it is not clear how to specify the new UAI reporting behavior for applicability update.

There are the following two alternatives to resolve the issue:

- Alt-1: Specify the UE to implicitly associate all the CSI-ReportConfig with beam prediction configuration (e.g. Set A/B configuration) with UAI for applicability update reporting.

- Alt-2: Introduce a new list of CSI-ReportConfigId in OtherConfig to explicitly configure the UE to report applicability update with UAI.

We think both alternatives can work, but Alt-2 is unnecessarily making things complex because it needs to introduce both ASN.1 change and the new UE behavior in Section 5.7.4 of TS 38.331 [6]. Thus, from specification perspective, Alt-2 has no benefit over Alt-1, and we propose:

Proposal 7: For all the CSI-ReportConfig(s) with beam prediction configuration (e.g. Set A/B configuration) in RRCReconfiguration, the UE implicitly associates them with UAI for applicability update reporting.

### 2.2.4 Whether associated ID is mandatory or optional (RRC-16)

This issue was discussed in RAN1, but RAN1 can’t achieve consensus and will further discuss this issue as indicated in RAN1 reply LS R1-2410898 [7]:

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

Figure.1: Issue illustration of model ambiguity between UE and NW if associated ID is absent in Step 3

We share our view from RAN2 perspective. We prefer the associated ID is mandatory provided in Step 3 because absence of associated ID may cause model ambiguity between UE and NW. As we know, one AI/ML functionality may include multiple NW-side additional conditions corresponding to multiple AI/ML models. Then, the UE may not know which model should be used for inference if the NW doesn’t indicate the current NW-side additional condition (i.e. associated ID) to the UE. We provide an example in Figure. 1, where the UE has model 1 (corresponding to associated ID 1) and model 2 (corresponding to associated ID 2) available for the same functionality A while the current NW condition is associated ID = 2. Then, after UE reports that the functionality is applicable under either associated ID=1 or ID=2 in step 4, the NW activates the functionality A in step 5, but the UE doesn’t know whether it should use model 1 or model 2 for inference. If the UE chooses to use model 1, it will lead to non-consistency with current NW condition.

Observation 2: Because applicability reporting is per inference configuration (CSI-ReportConfig) in Option A but the UE may train multiple models for one inference configuration based on its implementation, absence of associated ID may cause model ambiguity between UE and NW.

Thus, we propose that associated ID should be mandatory provided in Step 3 from RAN2 perspective. Otherwise, it may cause model ambiguity between UE and NW

Proposal 8: RAN2 assume that associated ID is mandatory provided in Step 3. Otherwise, it may cause model ambiguity issue between UE and NW.

## 2.3 Remaining issues on option B

In RAN2#129 [5], although it was agreed to support both Option A and Option B, no agreement on Option B can be made because companies had different views on how Option B works. And in RAN2#129b [2], the following WA on option B was agreed.

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Based on this WA, we discuss the UE behaviour when option B is configured.

### 2.3.1 Details on initial applicability report in RRCReconfigurationComplete (RRC-7)

As agreed in RAN2#129b [2], option B is configured in OtherConfig of RRCReconfiguration, but RRCReconfigurationComplete is used for its initial applicability reporting. In legacy RRC, RRCReconfigurationComplete is not used to report the feedback for configuration in OtherConfig. Thus, it is not clear how to fill this gap in RRC spec.

Observation 3: Option B is configured in OtherConfig of RRCReconfiguration, but RRCReconfigurationComplete is used for its initial applicability reporting. As RRCReconfigurationComplete is not used to report the feedback for configuration in OtherConfig in legacy RRC. Thus, it is not clear how to fill this gap in RRC spec.

There are the following two alternatives to resolve the issue:

- Alt-1: Specify the UE to implicitly associate all the sets of inference related parameters configured in OtherConfig of RRCReconfiguration with RRCReconfigurationComplete for initial applicability reporting.

- Alt-2: Introduce a new list of set indices of inference related parameters in RRCReconfiguration to explicitly configure the UE to report initial applicability report in RRCReconfigurationComplete.

Similar to the discussion on using UAI for applicability update report in Option A, we prefer Alt-1 because Alt-2 unnecessarily makes things complex to have both ASN.1 change and the new UE behavior in TS 38.331. Thus, from specification perspective, Alt-2 has no benefit over Alt-1. Thus, we propose

Proposal 9: For all the sets of inference related parameters configured in OtherConfig of RRCReconfiguration, the UE implicitly associates them with RRCReconfigurationComplete for initial applicability reporting.

Meanwhile, one followed issue is how the NW can differentiate whether the reported applicability/non-applicability is for full inference configuration (Option A) or for set of inference related parameter (Option B), which requires non-overlapping ID space to identify Option A and Option B.

Observation 4: The NW needs to differentiate whether the reported applicability/non-applicability is for full inference configuration (Option A) or for set of inference related parameter (Option B). Thus. non-overlapping ID space is required to identify Option A and Option B.

Please note that the latest RRC running CR introduces an IE ApplicabilityReport-r19 which includes a list of CSI-ReportConfigId and its status (i.e. applicable or inapplicable).

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..FFS)) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19               ServCellIndex,

applicabilityReportConfigIdList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports)) OF ApplicabilityReportConfigIdList-r19     OPTIONAL,

...

}

ApplicabilityReportConfigIdList-r19 ::=    SEQUENCE {

applicabilityReportConfigId-r19                CSI-ReportConfigId,

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

inapplicabilityCause-r19                       ENUMERATED {FFS}              OPTIONAL,

...

}

The design of IE ApplicabilityReport-r19 basically follows the RAN2#129 agreement on Option A, but it can’t support Option B because inference related parameter sets are configured in RRCReconfiguraitonOtherConfig which is different from the place to configure Option A (i.e. in RRCReconfiguraitonCSI-MeasConfig).

We think there are the following two alternatives to resolve the issue:

- Alt-1: Introduce CSI-ReportConfigId under the set of inference related parameters as identifier of the set.

  - It allows to use the common IE ApplicabilityReport-r19 to report applicability of both Option A and Option B.

  - Its RRC structure can be illustrated in Figure. 2.

- Alt-2: Introduce a new set index (e.g. named as InferenceSetIndex) under the set of inference related parameters as identifier of the set.

  - As InferenceSetIndex may be equal to CSI-ReportConfigId, we have to introduce a separate IE to report the list of applicable / inapplicable InferenceSetIndex for Option B.

Figure.2: Illustration of co-existence of Option A and Option B

Observation 5: If we introduce a new set index (e.g. InferenceSetIndex) to identify the set of inference related parameters, a separate IE from ApplicabilityReport-r19 is needed to report the list of applicable / inapplicable InferenceSetIndex for Option B due to possible ambiguity between InferenceSetIndex and CSI-ReportConfigId.

As we agreed to aim for a unified framework for Option A and Option B, we prefer Alt-1.

Proposal 10: To allow the common IE ApplicabilityReport-r19 to report applicability of both Option A and B, introduce CSI-ReportConfigId under the set of inference related parameters as the set identifier of Option B.

Finally, after the reporting of applicability of option B, it is clear that the UE can’t autonomously start inference because option B is not full inference configuration. Thus, it seems to be straight forward that the UE needs to wait RRCReconfiguration message with full inference configuration in CSI-ReportConfig before starting inference. Thus, we propose:

Proposal 11: For Option B, after reporting applicability, the UE waits NW to send RRCReconfiguration message with full inference configuration in CSI-ReportConfig before starting inference.

### 2.3.2 Co-existence of option A and option B (RRC-8)

With RAN2#129b agreement [2] and Proposal 9/10/11 in this contribution, we think that option A and option B have the unified applicability reporting procedure. Therefore, option A can co-exist with option B.

Meanwhile, we understand that some company may have concern that simultaneous configuration of option A and option B in one RRCReconfiguration message may bring extra UE complexity. Thus, to allow more flexibility, we propose to introduce separate UE capability for option B.

Proposal 12: RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. And a separate UE capability is introduced for option B to allow more flexibility.

## 2.4 Simple cause indication of in-applicability (RRC-1)

In RAN2#129b [2], it was discussed whether UE can explicitly report cause for non-applicability. Due to diverse option, RAN2 made the following compromised agreement:

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

In our understanding, the cause value to the NW is intended to indicate whether NW can expect that the inapplicable CSI-ReportConfig will become applicable after some time duration, so that NW can just wait the model to be available in local device for inference (without need of reconfiguration of inference).

Observation 6: The cause value to the NW is intended to indicate whether NW can expect that the inapplicable CSI-ReportConfig will become applicable after some time duration, so that NW can just wait the model to be available in local device for inference (without need of reconfiguration of inference).

Figure.3 Different stage of AI model storage and corresponding transfer time requirement

Our understanding on how the model becomes available in local device for inference operation is illustrated in Figure. 3. The AI model inference timeline depends on factors such as memory structure and size, model parameter size and inference delay. Due to the large storage requirement of different model parameters, the model can be stored in various locations and different time is required to move the model for inference:

- Most trained AI models are stored in the cloud. For each use case, different models can be trained based on factors like association ID, scenarios, configurations, and UE side additional conditions such as UE speed.

- Model(s) are downloaded to UE devices based on different use cases. The models are first stored in the non-volatile memory (NVM). The download process from the cloud to the device may requires up to several hundreds of ms or longer.

- When the model is activated, the UE moves the model parameters from NVM to off-chip memory such as DRAM. The time required for this operation depends on the model size and memory’s read/write speed and may require up to several tens of ms or longer.

- Model inferencing operation is performed with on-chip memory, and it typically much faster to transfer model from off-chip memory to on-chip memory, and this process usually takes on the order of ms.

According to Figure.3, we think that the UE may report the simple cause value between the time that model is available in server and the time before the model is available in its on-chip memory for inference because the UE can have a well estimation on how long such duration is. It doesn’t make sense to report the cause value before the model is available in server because the time to complete model training is difficult to estimate.

Observation 7: Between the time that model is available in server and the time before the model is available in its on-chip memory for inference, the NW can expect that the inapplicable CSI-ReportConfig will become applicable shortly. While it is difficult for the UE to estimate when the model training is complete (i.e. available in server).

Thus, to facilitate NW taking suitable action, we propose to introduce the following 1-bit simple cause value:

- “1” indicate that model training is complete (i.e. at least available in UE server) but requires additional time before ready for inference in local device (e.g. download in on-chip memory). And NW can expect the UE will update its applicability in UAI.

- “0” indicate that model training is not complete. NW may not expect the UE will update its applicability in UAI.

Proposal 13: Together with non-applicability, the UE can report the following 1-bit simple cause value:

- “1” indicate that model training is complete but requires additional time before ready for inference in local device. NW can expect the UE will update its applicability in UAI.

- “0” indicate that model training is not complete. NW may not expect the UE will update its applicability in UAI.

## 2.5 How to handle configuration in IDLE/INACTIVE/RLF (RRC-6)

Open issue RRC-6: Handling of inference, applicability reporting and UE data collection preference configurations when UE goes to RRC_IDLE/INACTIVE state and during RRCReestablishment

Issue description: RAN2 made following agreements in RAN2#129bis meeting:

Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

It is not clear whether UE releases inference configurations when UE goes to RRC_IDLE/CONNECTED state or upon network configuration via RRCRelease, etc.

Furthermore, handling of applicability configurations and UE data collection preference configuration (applicabilityReportConfig and dataCollectionPreferenceConfig in RRC running CR) during RRCReestablishment and in transition to RRC_INACTIVE state needs to be specified.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The issue is:

In TS 38.331 [6], whether the UE shall release or keep some specific RRC configuration when entering IDLE/INACTIVE/RLF is clearly specified. For example, in case of entering INACTIVE state:

From Section 5.3.8.3 of TS 38.331:

Skip unrelated part

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

-	aerial-Config, if configured;

-	cellDTX-DRX-Config, if configured;

Skip unrelated part

In the example of entering INACTIVE state, CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations) are stored in UE’s Inactive AS context.

Thus, we think the legacy UE behaviour for the RRC configuration in CSI-Meaconing (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations) can be reused. No extra RRC spec impacts are foreseen.

Proposal 14: On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).

## 2.6 LCM of CSI prediction (RRC-13)

Agreement

For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

- Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion

- Detailed parameter name is upto RAN2

In RAN1#120b [9], one agreement with RAN2 impact was made:

The issue is that the CSI-ReportConfig of Rel-19 AI/ML based CSI prediction is same as the CSI-ReportConfig of Rel-18 “non-AI CSI prediction” (i.e. CSI prediction with typeII-Doppler-r18). Thus, the UE needs to differentiate whether one CSI-ReportConfig is for Rel-18 or Rel-19 CSI prediction.

Observation 8: The CSI-ReportConfig of Rel-19 AI/ML based CSI prediction is same as the CSI-ReportConfig of Rel-18 “non-AI CSI prediction” (i.e. CSI prediction with typeII-Doppler-r18). Thus, the UE needs to differentiate whether one CSI-ReportConfig is for Rel-18 or Rel-19 CSI prediction.

We think the simplest solution is to add one indication under CSI-ReportConfig for this purpose. Thus, we propose:

Proposal 15: Introduce one indication under CSI-ReportConfig for the UE to differentiate whether it is for Rel-19 AI/ML based CSI prediction or Rel-18 “non-AI CSI prediction” (i.e. CSI prediction with typeII-Doppler-r18). In details, it is for Rel-19 CSI prediction when this indication is present. Otherwise, it is for Rel-18 CSI prediction.

Then, we discuss applicability reporting of CSI prediction. We think the applicability reporting specified for AI/ML based beam management can be reused as baseline. In addition, the configuration of applicability reporting of CSI prediction in Step 3 is expected to be simpler than beam management:

- Associated ID is not needed for CSI prediction because RAN1 has no consensus on introducing associated ID for CSI prediction.

- As May RAN1 meeting is the last meeting of Rel-19 AI/ML for PHY, there is no sufficient TU to study Option A and Option B similar to AI/ML based beam management. We think RAN2 can assume applicability reporting of CSI prediction only supports Option A (i.e. one or more CSI-ReportConfig on CSI prediction are configured for applicability reporting).

Proposal 16: Reuse applicability reporting of AI/ML based management as baseline of CSI prediction. On the configuration of applicability reporting, only support Option A without associated ID (i.e. one or more CSI-ReportConfig on CSI prediction can be configured for UE to report applicability via RRCReconfigurationComplete / UAI).

# 3 Conclusion

In this contribution, we discuss key remaining issues on LCM procedure of UE-sided model for AI/ML based beam management. Our observations are:

Observation 1: In legacy RRC, the UE reports UAI only if the NW required information are configured in OtherConfig of RRCReconfiguraiton. However, as Option A is configured in CSI-ReportConfig which is outside of OtherConfig, it is not clear how to specify the new UAI reporting behavior for applicability update.

Observation 2: Because applicability reporting is per inference configuration (CSI-ReportConfig) in Option A but the UE may train multiple models for one inference configuration based on its implementation, absence of associated ID may cause model ambiguity between UE and NW.

Observation 3: Option B is configured in OtherConfig of RRCReconfiguration, but RRCReconfigurationComplete is used for its initial applicability reporting. As RRCReconfigurationComplete is not used to report the feedback for configuration in OtherConfig in legacy RRC. Thus, it is not clear how to fill this gap in RRC spec.

Observation 4: The NW needs to differentiate whether the reported applicability/non-applicability is for full inference configuration (Option A) or for set of inference related parameter (Option B). Thus. non-overlapping ID space is required to identify Option A and Option B.

Observation 5: If we introduce a new set index (e.g. InferenceSetIndex) to identify the set of inference related parameters, a separate IE from ApplicabilityReport-r19 is needed to report the list of applicable / inapplicable InferenceSetIndex for Option B due to possible ambiguity between InferenceSetIndex and CSI-ReportConfigId.

Observation 6: The cause value to the NW is intended to indicate whether NW can expect that the inapplicable CSI-ReportConfig will become applicable after some time duration, so that NW can just wait the model to be available in local device for inference (without need of reconfiguration of inference).

Observation 7: Between the time that model is available in server and the time before the model is available in its on-chip memory for inference, the NW can expect that the inapplicable CSI-ReportConfig will become applicable shortly. While it is difficult for the UE to estimate when the model training is complete (i.e. available in server).

Observation 8: The CSI-ReportConfig of Rel-19 AI/ML based CSI prediction is same as the CSI-ReportConfig of Rel-18 “non-AI CSI prediction” (i.e. CSI prediction with typeII-Doppler-r18). Thus, the UE needs to differentiate whether one CSI-ReportConfig is for Rel-18 or Rel-19 CSI prediction.

Based on observations, our proposals can be found below:

Remaining issue of data collection configuration

Proposal 1: To support UE request on data collection via UAI, include the following two configurations in OtherConfig of RRCReconfiguration:

- Indicaiton on whether to allow send start/stop request

- A list of candidate CSI-ReportConfig(s) without CSI report which at least includes two CSI-ResourceConfigId for Set A/Set B and one or two associated IDs according to RAN1#120 agreement.

Proposal 2: The detailed reporting format of UE preferred data collection configuration includes ServCellIndex and a list of CSI-ReportConfigId(s) among the candidate CSI-ReportConfig(s) configured in OtherConfig.

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

Remaining issues on Option A

Proposal 4: As Rel-19 is the first release to introduce AI/ML, relaxed RRC processing timing requirement is applied for reporting applicability in RRCReconfigurationComplete (i.e. within 16ms after reception of Step 3).

Proposal 5: To avoid misleading NW, the UE stops periodic beam prediction reporting after successfully transmitting UAI on its inapplicability.

Proposal 6: For the non-applicable CSI-ReportConfig, when it becomes applicable:

- The UE autonomously activates periodic CSI reporting upon reporting it becoming applicable via UAI.

- The UE waits legacy DCI / MAC-CE to activate SP/AP CSI reporting upon reporting it becoming applicable via UAI.

Proposal 7: For all the CSI-ReportConfig(s) with beam prediction configuration (e.g. Set A/B configuration) in RRCReconfiguration, the UE implicitly associates them with UAI for applicability update reporting.

Proposal 8: RAN2 assume that associated ID is mandatory provided in Step 3. Otherwise, it may cause model ambiguity issue between UE and NW.

Remaining issues on Option B

Proposal 9: For all the sets of inference related parameters configured in OtherConfig of RRCReconfiguration, the UE implicitly associates them with RRCReconfigurationComplete for initial applicability reporting.

Proposal 10: To allow the common IE ApplicabilityReport-r19 to report applicability of both Option A and B, introduce CSI-ReportConfigId under the set of inference related parameters as the set identifier of Option B.

Proposal 11: For Option B, after reporting applicability, the UE waits NW to send RRCReconfiguration message with full inference configuration in CSI-ReportConfig before starting inference.

Proposal 12: RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. And a separate UE capability is introduced for option B to allow more flexibility.

Simple cause value of non-applicability

Proposal 13: Together with non-applicability, the UE can report the following 1-bit simple cause value:

- “1” indicate that model training is complete but requires additional time before ready for inference in local device. NW can expect the UE will update its applicability in UAI.

- “0” indicate that model training is not complete. NW may not expect the UE will update its applicability in UAI.

How to handle configuration in IDLE/INACTIVE/RLF

Proposal 14: On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).

LCM of CSI prediction

Proposal 15: Introduce one indication under CSI-ReportConfig for the UE to differentiate whether it is for Rel-19 AI/ML based CSI prediction or Rel-18 “non-AI CSI prediction” (i.e. CSI prediction with typeII-Doppler-r18). In details, it is for Rel-19 CSI prediction when this indication is present. Otherwise, it is for Rel-18 CSI prediction.

Proposal 16: Reuse applicability reporting of AI/ML based management as baseline of CSI prediction. On the configuration of applicability reporting, only support Option A without associated ID (i.e. one or more CSI-ReportConfig on CSI prediction can be configured for UE to report applicability via RRCReconfigurationComplete / UAI).

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#129b, Chair Notes.

[3] RAN1#120, Chair Notes.

[4] RAN2#128, Chair Notes.

[5] RAN2#129, Chair Notes.

[6] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[7] R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model.

[8] R2-250xxxx, Draft running RRC CR for AIML PHY, Ericsson.

[9] RAN1#120b, Chair Notes.