# 3GPP TSG RAN WG2 Meeting #130      	                        R2-2503716
St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.1.3

Source:	Apple

Title:	Remaining issues on NW-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. One of its sub-objectives is to specify data collection for NW-sided model, as highlighted copied below:

In RAN2#129b [2], most of essential issues on NW-side data collection have been closed. However, there are some leftover issues. In this contribution, we discuss the following remaining issues on NW-sided data collection from below aspects:

- Remaining RRC open issues on NW-sided data collection

- Remaining MAC open issues on NW-sided data collection

- UE capability of NW-sided data collection

- Collected data type of NW-sided data collection

- Collected data type of UE-sided data collection

# 2 Discussion

## Remaining RRC open issues on NW-side data collection

In this section, we share our view on the open issues suggested by RRC CR Rapporteur and some important open issues raised by company in [3].

### Open issue RRC-18/35: NW control on retaining logged data at HO

Issue description: The signaling details of the network control on how data should be retained at handover are FFS, based on the RAN2#129 bis agreement:

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

The issue is captured as an editor’s note in the running CR, in clause 5.3.5.3 and 6.2.2.

It the rapporteur’s view, it should be clarified Whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. From the rapporteur perspective, it is not possible for the source gNB to add the 1-bit indication during HO, in the same RRCReconfiguration that encapsulates the RRCReconfiguration from the target gNB. Thus, the rapporteur sees the following two possible solutions for this issue:

- The source gNB sends the 1-bit indication to the UE before HO.

- The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

In current RRC running CR [11], it has captured 1-bit indication retainLoggedMeasurements-r19 under the RRCReconfiguration.

RRCReconfiguration-v19xy-IEs ::=        SEQUENCE {

otherConfig-v19xy                       OtherConfig-v19xy                                       OPTIONAL, -- Need M

retainLoggedMeasurements-r19            ENUMERATED {true}                                     OPTIONAL, -- Need N

nonCriticalExtension                    SEQUENCE {}                                              OPTIONAL

}

retainLoggedMeasurements If present, it indicates that the UE shall retain the logged measurements available in VarCSI-LogMeasReport upon completing the handover execution. Editor's Note: FFS signaling details.

First, we think this 1-bit indication is sufficient, and no further ASN.1 work is needed:

- We understand that the RAN2#129b agreement [2] is intended to cover both cases of “before HO” and “during HO” rather than down-selection between them. And it is up to source cell implementation whether to send it “before HO” or “during HO” (i.e. in same RRC message to carry ReconfigurationWithSync).

- We understand that source cell determines this indication according to the PLMN/vendor information of target cell. However, as PLMN/vendor info is sensitive, we agree with Rapporteur that source cell determines based on its implementation without spec change on inter-node message.

Proposal 1: RAN2 confirm the following understanding on NW control on retaining logged data at HO:

- It is up to source cell implementation to determine whether to send the indication retainLoggedMeasurements-r19 to the UE without spec change on inter-node message.

- It is up to source cell implementation whether to send the indication retainLoggedMeasurements-r19 “before HO” or “during HO” (i.e. in same RRC message to carry ReconfigurationWithSync).

Then, we think one missed issue is when the target cell is legacy cell (e.g. pre-Rel19 NR cell or LTE cell). In this case, it is clear that the UE should discard the logged measurements because target cell doesn’t support NW-side data collection. To reflect this, the simplest way is to specify that the UE shall discard the logged measurement upon HO if this field is absent.

Observation 1: When the target cell is legacy cell (e.g. pre-Rel19 NR cell or LTE cell), the UE should discard the logged measurements because target cell doesn’t support NW-side data collection.

Thus, we propose:

Proposal 2: To cover the legacy target cell case (e.g. pre-Rel19 NR cell or LTE cell), specify the UE shall discard the logged measurement upon HO if absent. The field description can be updated as follows:

If present, it indicates that the UE shall retain the logged measurements available in VarCSI-LogMeasReport upon completing the handover execution. If absent, it indicates that the UE shall discard the logged measurements available in VarCSI-LogMeasReport (if any) upon completing the handover execution.

### Open issue RRC-19/29: Reporting assistance info on logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated?

This issue is related to the RAN2#129bis agreement:

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

The issue is captured as an editor’s notes in the running CR, in clause 5.3.5.9, 5.7.4.3, 6.2.2, and 6.3.4.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

In current RRC running CR [3], it has captured 3 separate indications on buffer threshold, full buffer and low power:

OtherConfig-v19xy ::=                   SEQUENCE {

applicabilityReportConfig-r19   SetupRelease {ApplicabilityReportConfig-r19}       OPTIONAL, -- Need M

dataCollectionPreferenceConfig-r19   SetupRelease {DataCollectionPreferenceConfig-r19}    OPTIONAL, -- Need M

loggedDataCollectionAssistanceConfig-r19    SetupRelease {LoggedDataCollectionAssistanceConfig-r19}        OPTIONAL  -- Need M

}

LoggedDataCollectionAssistanceConfig-r19 ::= SEQUENCE {

loggedDataCollectionBufferThreshold-r19               ENUMERATED {FFS}                        OPTIONAL, -- Need R

loggedDataCollectionFullBuffer-r19                     ENUMERATED {true}                       OPTIONAL, -- Need R

loggedDataCollectionPowerLow-r19                       ENUMERATED {true}                        OPTIONAL  -- Need R

}

First, we support the current ASN.1 coding to keep the 3 separate indications on buffer threshold, full buffer and low power. Although it may save some signaling overhead if we can introduce some joint indication (e.g. trigger indication when both in low power state and buffer > threshold), the benefit is quite marginal (i.e. up to 2bit reduction in RRC). Meanwhile, the joint indication will lead to further RAN2 work and increased complexity in both UE and gNB. Thus, we don’t think the optimization is worth it.

Proposal 3: On UAI reporting related to NW-side data collection, keep the 3 separate indications on buffer threshold, full buffer and low power state (i.e. no need of optimization to introduce joint indication like when the UE has data and low power state).

Secondly, we notice that the type of buffer threshold is still FFS. According to our understanding, there are 3 alternatives proposed in the previous meetings:

- Alt-1: absolute data amount in the logging buffer (e.g. 4 KB)

- Alt-2: percentage of logging buffer size (e.g. 10% of buffer)

- Alt-3: absolute remaining logging buffer size (e.g. 4 KB)

As buffer management is UE implementation, it doesn’t make sense to adopt Alt-2 and Alt-3 because how to determine remaining buffer size depends on different UE implementation. Thus, we prefer Alt-1.

Proposal 4: The buffer threshold is specified as absolute data amount in the logging buffer (e.g. X KB).

### Open issue RRC-20: Further procedures for UE assistance info related to logging

Issue description: It has not yet been discussed whether further procedures for UE reporting assistance information related to logging is need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer, etc.

The issue is captured as an editor’s note in the running CR, in clause 5.7.4.2 and 5.7.4.3.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

We think the current 3 indications are sufficient without need of further optimization. Some technique arguments on the list provided in [3]:

- Prohibit timers: we understand that all the existing 3 indications are not expected to trigger frequently. Thus, no strong motivation to introduce prohibit timers.

- Indication that battery state is not low any longer: upon reception of low power indication from the UE, we think that the reasonable NW behaviour is to configure another UE for NW-side data collection, i.e. it doesn’t make sense to just collect data from one particular UE. Thus, no strong motivation to introduce this new indication.

- Indication that the memory is not full any longer: upon reception of full buffer indication from the UE, we think that the NW may retrieve data from the UE, and NW can estimate the buffer status afterwards based on its implementation. Thus, no strong motivation to introduce this new indication.

Thus, we propose:

Proposal 5: RAN2 not pursue further optimization on UAI reporting related to NW-side data collection (e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer).

### Open issue RRC-27: How to set logging periodicity

The description of open issue can be found as follows:

Issue description: Regardless of the chosen logging framework, further discussion is needed on how to set the logging periodicity for each logging RS.

To provide flexibility in the logging timing, two options could be considered for logging periodicity:

(i) aligning with the RS transmission periodicity, or

(ii) introducing an optional logging interval setting.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

First, it is not technique correct to align with the RS transmission periodicity. In existing L1 CSI measurement framework, the time to perform L1 measurement is not equal to CSI-RS transmission periodicity but instead depends on detailed RAN1/RAN4 requirement (e.g. the reference CSI-RS as specified in TS 38.214 [4]).

Secondly, we think that the measurement and logging are two separately UE behaviours. It doesn’t make sense to mix them together.

Thus, we think the logging interval should be mandatory configured.

Proposal 6: Logging interval is mandatory configuration for NW-side data collection.

### Open issue RRC-21: Time related content of logged data

Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity).

This issue refers to the RAN2#129bis agreement:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

The issue is captured as an editor’s note in the running CR, in clause 5.7.10.3 and 6.2.2.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

As there are only two remaining RAN2 meetings for Rel-19 AI/ML, we don’t think RAN2 have sufficient time to study a fancy solution. The only option is to reuse the legacy timestamp mechanism of logged MDT as baseline:

- After reception of logging configuration, the UE adds one absolute timestamp with format of 48-bit string (YY-MM-DD HH:MM:SS).

- For each logged sample, add relative timestamp compared with the absolute timestamp.

During online discussion of RAN2#129b [2], the main concern on above legacy mechanism is the signalling overhead of adding timestamp for each logging sample.

Figure. 1 Illustration of how to add timestamp for logged measurements

If company really have concern on signalling overhead, we think the concern can be addressed by below ways:

- Adopt a short format for absolute timestamp (e.g. 8-bit string with only hour-level time info).

- Relative timestamp of some periodic logging samples can be skipped (e.g. the samples which follow the configured logging interval with the previous one).

    - In the example of Figure.1, the relative timestamp of 2nd and 3rd logging samples can be skipped.

Thus, we propose:

Proposal 7: Legacy timestamp mechanism of logged MDT is reused as baseline of NW-side data collection:

- After reception of logging configuration, the UE adds one absolute timestamp. FFS detailed format.

- For each logged sample, add relative timestamp compared with the absolute timestamp. FFS whether some of relative timestamps can be skipped to reduce signaling overhead.

### Open issue RRC-22: RAN1 involvement for logged data for NW-side and UE-side data collection

Issue description: Procedures for performing the L1 measurement results are captured in RAN1 specification, i.e. TS 38.214. Rapporteur assumes that the same should be applied for the case of radio measurements logging for the NW-side data collection and UE-side data collection. Hence RAN1 involvement is expected to capture procedures related to the radio measurements logging, e.g. in the UE variable VarCSI-LogMeasReport for the case of NW-side data collection.

From the rapporteur’s perspective, an LS should be sent to RAN1.

The issue is captured as an editor’s note in the running CR, in clause 5.7.10.3.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

Firstly, we don’t think RAN1 agreed to reuse logged data for UE-side data collection. What RAN1 agreed is just how to reuse L1 CSI framework for UE-side data collection, but they never touched logged measurement for UE-side data collection.

Secondly, due to diverse view, UE-side data collection was not agreed to enter normative phase in Rel-19. Currently, SA2/SA5 are evaluating UP solution of UE-side data collection option 2 and option 3 which is a different direction from NW-side data collection specified in Rel-19 (i.e. CP solution).

Observation 2: RAN1 didn’t agree to reuse logged data for UE-side data collection. Furthermore, SA2/SA5 are evaluating UP solution of UE-side data collection option 2 and option 3 which is a different direction from NW-side data collection specified in Rel-19 (i.e. CP solution).

Thus, to avoid potential misalignment with UP solution of SA2/SA5, we don’t think RAN2 can extend logged measurement framework to UE-side data collection. At this stage, RAN2 can wait for RAN1 LS.

Proposal 8a: As NW-side data collection is RAN2-led objective, no need of RAN1 involvement on logged measurement.

Proposal 8b: To avoid potential misalignment with UP solution of UE-side data collection being studied in SA2/SA5, RAN2 do not extend logged measurement framework to UE-side data collection.

### Open issue RRC-23: Cell ID stored with logged data for NW-side data collection

Issue description: It has not been clarified what type of cell ID the UE needs to log along with the logged data, in order to unambiguously identify the cell in which the UE performed the data logging, e.g. CGI, PCI-ARFCN etc.

The issue is captured as an editor’s note in the running CR, in clause 6.2.2.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

As the intention to include Cell ID is to support CA, we think it is sufficient to use ServCellIndex. Please note that the mapping from ARFCN and PCI to ServCellIndex is already configured in ServingCellConfigCommon.

Observation 3: As the intention to include Cell ID is to support CA, it is sufficient to use ServCellIndex. Please note that the mapping from ARFCN and PCI to ServCellIndex is configured in ServingCellConfigCommon.

Thus, we propose:

Proposal 9: As the intention is to support CA, ServCellIndex is logged together with the logged measurements.

### Open issue RRC-24: Where to include the logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3.

This issue was discussed in RAN2#129bis and the following outcome was captured:

Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

The issue is captured as an editor’s note in the running CR, in clause 6.3.2.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

Our view can be found in the RAN2 contribution co-sourced with ZTE [5].

### Open issue-25/30: Dynamic activation/deactivation of data collection for logging

In RAN2#128 [8], it was agreed not to support measurements on aperiodic CSI resources. However, it was not concluded whether to support measurements on Semi-Persistent (SP) CSI resource.

Agreements on NW side data collection - Measurements on aperiodic CSI resources are not reported for NW sided data collection.

We think it is not reasonable to support measurements on SP CSI resources. Enhanced MDT is intended to support data collection for model training which doesn’t have latency requirement. If NW needs the UE to perform data collection only for one particular time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Observation 4: If NW needs the UE to perform data collection only for one particular time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Thus, we propose not to support dynamic activation / deactivation of logged L1 measurements on SP CSI resources because it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources.

Proposal 10: As target for offline training without latency requirement, no need to support dynamic activation/deactivation of logged L1 measurements on SP CSI resources (i.e. it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources).

### Open issue RRC-26: Multiplexing legacy logged data and AIML logged data in new SRB

Issue description: RAN2#129bis agreed that

2. New SRB can be configured for NW-side data collection (with lower priority)

Given the agreement on the new SRB for the transmission of the UEInformationResponse, Rapporteur’s understanding is that the new SRB may be used also for the transmission of the legacy SON/MDT reports (e.g. logged MDT measurements, RLF-Report, RA-reports, successful HO reports, etc.), whenever the UEInformationResponse carries both a legacy SON/MDT report and AIML logged data. However, this would impact how legacy SON/MDT reporting is performed which can only be at the moment on SRB1 or SRB2.

From the specification perspective, this affects primarily RRC clause 5.7.10.3 and it would impact also other specifications, e.g. TS 37.320.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

The description of open issue can be found as follows:

In our understanding, the motivation to introduce a new SRB for AI/ML logged data is exactly to avoid mixing with other SRB, including SRB2 and SRB4. Thus, we don’t see the need to multiplex legacy logged data (i.e. data of logged MDT) and AI/ML logged data in the new SRB.

Proposal 11: No need to support multiplexing legacy logged data (i.e. data of logged MDT) and AI/ML logged data in the new SRB.

### Open issue RRC-32: UE behaviour when L3 measurement event trigger is met

Issue description: In RAN2 Athens meeting, RAN2 confirmed to support L3 measurement event triggered data logging method. But it’s still unclear what is the UE behavior during the period that L3 measurement triggered data logging event fulfills.

Proposed resolution: We understand MDT-like solution can be considered as the baseline, i.e. UE performs data logging periodically during the period that L3 measurement triggered data logging event fulfills. If L3 measurement triggered data logging event does not fulfill, UE just stops data logging.

OPPO added this open issue in [3], and its description can be found as follows:

In RAN2#129 [7], it was agreed to support event triggered logging with FFS on what to log.

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.   FFS what to log

We understand that the L3 RRM based event triggered logging is intended to restrict the area where the UE collects L1 measurement. When the event condition is met (e.g. L3 RSRP > threshold), the UE starts logging the L1 measurement until the condition is not met. Thus, the logging metric is same as periodically logging.

Proposal 12: When L3 measurement event trigger condition is met, the UE periodically performs L1 measurement logging until the trigger condition is no longer met.

### Open issue RRC-38: Minimum AS layer memory size supported by the UE.

Issue description:

We made the following agreement in RAN2#127 that we will specify a minimum AS layer memory size across all use cases, similar to logged MDT. The specific value of the memory size is FFS, but it has not been addressed yet.

Agreements

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

We added this open issue in [3], and its description can be found as follows:

In legacy NR, logged MDT and QoE measurements (including paused measurements and IDLE/INACTIVE measurements) had specified the same minimum AS layer memory size, which is 64kB according to TS 38.306 [6]:

Logged MDT (From Section 4.2.18 of TS 38.306)

| loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB. | UE | No | No | No |
| --- | --- | --- | --- | --- |

QoE (from Section 6 of TS 38.306)

| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |
| --- | --- |
| AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE | It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports" |

Observation 5: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64KB according to TS 38.306.

Meanwhile, RAN1 Reply LS (R1-2310681) [7] provided typical data size for model training in beam management. According to the highlighted parts, we can estimate that the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) = 2800 bits=0.35KB.

From RAN1 reply LS (R1-2310681) LCM purpose; UE-side/NW-side models; Data content; Typical data size (per data sample); Typical latency requirement; Notes. Training; UE-side, NW-side; L1-RSRPs and/or beam-IDs; See Note 1 for L1-RSRPs; Relaxed. ==omit table rows on “Inference” and “Monitoring” === Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed. Note 2: There are no agreements on the reporting type. Note 4: Feasibility and necessity of the monitoring schemes listed in the table are under discussion. Note 5: For BM Case 2, the typical value of the number of history measurement time instance used in evaluations is up to 8 and typical value of the number of predicted future time instance is 1~4.

Based on above analysis, we think it is sufficient to reuse the same minimum AS layer memory size of logged MDT and QoE. Thus, we propose:

Proposal 13: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

## Remaining MAC open issues on NW-side data collection

In this section, we share our view on remaining MAC issues on NW-side data collection.

In existing TS 38.321 [10], the L1 CSI reporting is stopped in the following cases:

- The SCell is deactivated (in Section 5.9	)

- BWP is deactivated (in Section 5.15.1)

- BWP is dormant BWP (in Section 5.15.1)

- SCG is deactivated (in Section 5.15.1)

The reason for stopping L1 CSI reporting behind these specifications is because the UE can’t perform L1 CSI measurement on resource configured in deactivated SCell/BWP/SCG.

Observation 6: In existing TS 38.321, L1 CSI reporting is stopped in the following cases because the UE can’t perform L1 CSI measurement on resource configured in deactivated SCell/BWP/SCG:

- The SCell is deactivated (in Section 5.9)

- BWP is deactivated (in Section 5.15.1)

- BWP is dormant BWP (in Section 5.15.1)

- SCG is deactivated (in Section 5.15.1)

Following the same logic, we think that the UE stops L1 CSI measurement logging for the L1 resources which are configured in deactivated SCell/BWP/SCG.

Proposal 14: The UE stops L1 measurement logging in the following cases:

- The SCell is deactivated

- BWP is deactivated

- BWP is dormant BWP

- SCG is deactivated

This UE behaviour should be captured in MAC spec, and TP in Appendix is provided. Meanwhile, we also capture it in the running CR of TS 38.321 [12].

## UE capability on NW-side data collection

As the feature of NW-side data collection was introduced by RAN2, we think RAN2 should define its UE capability.

Because RAN2 agreed immediate MDT is the baseline of logged L1 measurement, we need to check how UE capability on legacy immediate MDT is specified. According to TS 38.306 [6], no dedicated UE capability is defined for immediate MDT, which instead includes one basic L3 measurement capability and several separate UE capabilities on specific measurement metrics:

- As immediate MDT reuses L3 RRM, the following intra-frequency and inter-frequency L3 measurement is reused as baseline of immediate MDT:

From Section 4.2.9 of TS 38.306:

| intraAndInterF-MeasAndReport Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to SN configured measurement when (NG)EN-DC is configured. For NR SA, MN and SN configured measurement when NR-DC is configured, and MN configured measurement when NE-DC is configured, this feature is mandatory supported. | UE | Yes | Yes | No |
| --- | --- | --- | --- | --- |

- On top of it, the following separate UE capabilities on specific measurement metrics for immediate MDT (e.g. WLAN, BT, barometer, etc.) are defined in Section 4.2.18.

From Section 4.2.18 of TS 38.306:

| barometerMeasReport-r16 Indicates whether the UE supports uncompensated barometeric pressure measurement reporting upon request from the network. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| excessPacketDelay-r17 Indicates whether the UE supports the UL PDCP excess packet delay measurement per DRB as specified in TS 38.314 [26]. A UE that supports the UL PDCP excess packet delay measurement shall also support the measurement configuration and reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| gnss-Location-r16 Indicates whether the UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON, MDT, and NTN related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE state. A UE shall set this field to supported if it indicates the support of nonTerrestrialNetwork-r17. | UE | CY | No | No |
| immMeasBT-r16 Indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED state. | UE | No | No | No |
| immMeasWLAN-r16 Indicates whether the UE supports WLAN measurements in RRC_CONNECTED state. | UE | No | No | No |
| orientationMeasReport-r16 Indicates whether the UE supports orientation information reporting upon request from the network. | UE | No | No | No |
| ulPDCP-Delay-r16 Indicates whether the UE supports UL PDCP Packet Average Delay measurement (as specified in TS 38.314 [26]) and reporting in RRC_CONNECTED state. | UE | No | No | No |

Observation 7: In TS 38.306, no dedicated UE capability is defined for immediate MDT, which instead includes one basic L3 measurement capability (intraAndInterF-MeasAndReport) and several separate UE capabilities on specific measurement metrics for immediate MDT (e.g. WLAN, BT, barometer, etc.).

Then, on UE capability of L1 measurement logging, we can use similar way as legacy immediate MDT:

- Introduce a general NW-side data collection capability with forward compatibility (e.g. loggedMeasurements-dataCollection-r19) under UE-BasedPerfMeas-Parameters. For example:

| loggedMeasurements-dataCollection-r19 Indicates whether the UE supports logged measurements for NW-side data collection in RRC_Connected state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of logged measurements for NW-side data collection is 64KB. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

- The specific measurement to log is separate capability. For example, L1 RSRP for Set A/B are defined for Rel-19 AI based beam management and L3 RSRP for RRM prediction in Rel-20.

Thus, we propose:

Proposal 15: Define the following UE capability of NW side model data collection, which is independent of UE capability of immediate MDT:

- Introduce a general NW-side data collection capability with forward compatibility (e.g. loggedMeasurements-dataCollection-r19) under UE-BasedPerfMeas-Parameters. For example:

| loggedMeasurements-dataCollection-r19 Indicates whether the UE supports logged measurements for NW-side data collection in RRC_Connected state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of logged measurements for NW-side data collection is 64KB. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

- The specific measurement quantities to log are separate UE capability (e.g., L1 RSRP for Set A/B are defined for Rel-19 AI based beam management).

## 2.4 Collected data type of NW-sided data collection

NW-sided data collection includes NW-side model of AI/ML based management and case 3a)/3b) of AI/ML based positioning. We think RAN2 can wait RAN1 L1 excel and no RAN2 immediate work is needed:

Proposal 16: On required data type for NW-sided data collection (including AI/ML based beam management and AI/ML based positioning case 3a/3b), RAN2 wait RAN1 L1 parameter excel.

## 2.4 Collected data type of UE-sided data collection

UE-sided data collection includes UE-side model for AI/ML based beam management and positioning case 1. As the UE-side model is deployed at UE side, the basic assumption is that UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. Therefore, when UE uses UE-side AI/ML model for inference, the UE can generate the measurement for AI/ML model inference using the same type or format of the measurement as training phase by implementation. 3GPP should not restrict the data type used for input of AI/ML model training. Thus, we do not think it is necessary to specify the data type of AI/ML model input. 3GPP only need to specify model output for UE-side model, and reference signal (RS) used for model input measurement and its configuration.

Observation 8: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Thus, for UE-side data collection, we propose the specific data used for model input and their intermediate processing is up to UE implementation, which should check with RAN1 on any issue.

Proposal 17: On UE-side data collection, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation. Send LS to RAN1 to check any issue.

# 3 Conclusion

In this contribution, we discuss remaining issues of NW-sided data collection. Our Observation are:

Observation 1: When the target cell is legacy cell (e.g. pre-Rel19 NR cell or LTE cell), the UE should discard the logged measurements because target cell doesn’t support NW-side data collection.

Observation 2: RAN1 didn’t agree to reuse logged data for UE-side data collection. Furthermore, SA2/SA5 are evaluating UP solution of UE-side data collection option 2 and option 3 which is a different direction from NW-side data collection specified in Rel-19 (i.e. CP solution).

Observation 3: As the intention to include Cell ID is to support CA, it is sufficient to use ServCellIndex. Please note that the mapping from ARFCN and PCI to ServCellIndex is configured in ServingCellConfigCommon.

Observation 4: If NW needs the UE to perform data collection only for one particular time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Observation 5: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64KB according to TS 38.306.

Observation 6: In existing TS 38.321, L1 CSI reporting is stopped in the following cases because the UE can’t perform L1 CSI measurement on resource configured in deactivated SCell/BWP/SCG:

- The SCell is deactivated (in Section 5.9)

- BWP is deactivated (in Section 5.15.1)

- BWP is dormant BWP (in Section 5.15.1)

- SCG is deactivated (in Section 5.15.1)

Observation 7: In TS 38.306, no dedicated UE capability is defined for immediate MDT, which instead includes one basic L3 measurement capability (intraAndInterF-MeasAndReport) and several separate UE capabilities on specific measurement metrics for immediate MDT (e.g. WLAN, BT, barometer, etc.).

Observation 8: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Based on observations, our proposals can be found below:

Remaining RRC open issues on NW-side data collection

Proposal 1: RAN2 confirm the following understanding on NW control on retaining logged data at HO:

- It is up to source cell implementation to determine whether to send the indication retainLoggedMeasurements-r19 to the UE without spec change on inter-node message.

- It is up to source cell implementation whether to send the indication retainLoggedMeasurements-r19 “before HO” or “during HO” (i.e. in same RRC message to carry ReconfigurationWithSync).

Proposal 2: To cover the legacy target cell case (e.g. pre-Rel19 NR cell or LTE cell), specify the UE shall discard the logged measurement upon HO if absent. The field description can be updated as follows:

If present, it indicates that the UE shall retain the logged measurements available in VarCSI-LogMeasReport upon completing the handover execution. If absent, it indicates that the UE shall discard the logged measurements available in VarCSI-LogMeasReport (if any) upon completing the handover execution.

Proposal 3: On UAI reporting related to NW-side data collection, keep the 3 separate indications on buffer threshold, full buffer and low power state (i.e. no need of optimization to introduce joint indication like when the UE has data and low power state).

Proposal 4: The buffer threshold is specified as absolute data amount in the logging buffer (e.g. X KB).

Proposal 5: RAN2 not pursue further optimization on UAI reporting related to NW-side data collection (e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer).

Proposal 6: Logging interval is mandatory configuration for NW-side data collection.

Proposal 7: Legacy timestamp mechanism of logged MDT is reused as baseline of NW-side data collection:

- After reception of logging configuration, the UE adds one absolute timestamp. FFS detailed format.

- For each logged sample, add relative timestamp compared with the absolute timestamp. FFS whether some of relative timestamps can be skipped to reduce signaling overhead.

Proposal 8a: As NW-side data collection is RAN2-led objective, no need of RAN1 involvement on logged measurement.

Proposal 8b: To avoid potential misalignment with UP solution of UE-side data collection being studied in SA2/SA5, RAN2 do not extend logged measurement framework to UE-side data collection.

Proposal 9: As the intention is to support CA, ServCellIndex is logged together with the logged measurements.

Proposal 10: As target for offline training without latency requirement, no need to support dynamic activation/deactivation of logged L1 measurements on SP CSI resources (i.e. it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources).

Proposal 11: No need to support multiplexing legacy logged data (i.e. data of logged MDT) and AI/ML logged data in the new SRB.

Proposal 12: When L3 measurement event trigger condition is met, the UE periodically performs L1 measurement logging until the trigger condition is no longer met.

Proposal 13: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Remaining MAC open issues on NW-side data collection

Proposal 14: The UE stops L1 measurement logging in the following cases:

- The SCell is deactivated

- BWP is deactivated

- BWP is dormant BWP

- SCG is deactivated

UE capability on NW-side data collection

Proposal 15: Define the following UE capability of NW side model data collection, which is independent of UE capability of immediate MDT:

- Introduce a general NW-side data collection capability with forward compatibility (e.g. loggedMeasurements-dataCollection-r19) under UE-BasedPerfMeas-Parameters. For example:

| loggedMeasurements-dataCollection-r19 Indicates whether the UE supports logged measurements for NW-side data collection in RRC_Connected state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of logged measurements for NW-side data collection is 64KB. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

- The specific measurement quantities to log are separate UE capability (e.g., L1 RSRP for Set A/B are defined for Rel-19 AI based beam management).

Collected data type of NW-sided data collection

Proposal 16: On required data type for NW-sided data collection (including AI/ML based beam management and AI/ML based positioning case 3a/3b), RAN2 wait RAN1 L1 parameter excel.

Collected data type of UE-sided data collection

Proposal 17: On UE-side data collection, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation. Send LS to RAN1 to check any issue.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#129b, Chair Notes.

[3] R2-250xxxx, Remaining RRC open issues in feature AIML PHY, Ericsson.

[4] TS 38.214-v18.6.0, Physical layer procedures for data, 2025-03.

[5] R2-250xxxx, Considerations On the NW Side Data Collection RRC Framework, ZTE.

[6] TS 38.306-v18.0.0, User Equipment (UE) radio access capabilities, 2023-12.

[7] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[8] RAN2#128, Chair Notes.

[9] RAN1#120b, Chair Notes.

[10] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[11] R2-250xxxx, Draft running RRC CR for AIML PHY, Ericsson.

[12] R2-2503713, Running MAC CR for AI/ML for Air Interface, Apple (Rapporteur).

# Appendix (38.321 TP)

============================== CHAGN BEGINS==================================

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	if the SCell was deactivated prior to receiving this Enhanced SCell Activation/Deactivation MAC CE and a TRS is indicated for this SCell:

3>	indicate to lower layers the information regarding the TRS.

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	CSI logging for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

3>	else (i.e. firstActiveDownlinkBWP-Id is set to dormant BWP):

4>	stop the bwp-InactivityTimer of this Serving Cell, if running.

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

2>	if the active DL BWP is not the dormant BWP:

3>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

3>	trigger PHR according to clause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not log CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

When the measurement reporting for fast unknown SCell activation is configured by RRC, the MAC entity shall:

1>	if SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell(s):

2>	if SCell(s) was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE:

3>	indicate to upper layers SCell(s) activation indication.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE or Enhanced SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

================================== NEXT CHANGE===================================

## 5.15	Bandwidth Part (BWP) operation

### 5.15.1	Downlink and Uplink

In addition to clause 12 of TS 38.213 [6], this clause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell except for PSCell when SCG is deactivated (see clause 5.29) or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id for PSCell when SCG is deactivated, the DL BWP is switched to the firstActiveDownlinkBWP-Id as specified in TS 38.331 [5]. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each SCell a dormant BWP may be configured with dormantBWP-Id by RRC signalling as described in TS 38.331 [5]. Entering or leaving dormant BWP for SCells is done by BWP switching per SCell or per dormancy SCell group based on instruction from PDCCH (as specified in TS 38.213 [6]). The dormancy SCell group configurations are configured by RRC signalling as described in TS 38.331 [5]. Upon reception of the PDCCH indicating leaving dormant BWP, the DL BWP indicated by firstOutsideActiveTimeBWP-Id or by firstWithinActiveTimeBWP-Id (as specified in TS 38.331 [5] and TS 38.213 [6]) is activated. Upon reception of the PDCCH indicating entering dormant BWP, the DL BWP indicated by dormantBWP-Id (as specified in TS 38.331 [5]) is activated. The dormant BWP configuration for SpCell or PUCCH SCell is not supported.

BWP for SRS for positioning Tx frequency hopping can be configured for a Serving Cell in TS 38.331 [5]. BWP for SRS Tx frequency hopping is considered as activated when it is configured. BWP switching is not applicable for BWP for SRS Tx frequency hopping.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP and the Serving Cell is not the PSCell of deactivated SCG:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

2>	log CSI for the BWP;

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

2>	if lbt-FailureRecoveryConfig is configured:

3>	stop the lbt-FailureDetectionTimer, if running;

3>	set LBT_COUNTER to 0;

3>	monitor LBT failure indications from lower layers as specified in clause 5.21.2.

1>	if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP:

2>	stop the bwp-InactivityTimer of this Serving Cell, if running.

2>	not monitor the PDCCH on the BWP;

2>	not monitor the PDCCH for the BWP;

2>	not receive DL-SCH on the BWP;

2>	not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

2>	not log CSI on the BWP;

2>	not transmit SRS on the BWP;

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

1>	if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not log CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is an (e)RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is an (e)RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered consistent LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the active DL BWP is not the initialDownlinkBWP-RedCap:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a (e)RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP-RedCap:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

============================. CHANGE ENDS=========================================