3GPP TSG-RAN WG2 Meeting #130	R2-2503728

St. Julians, Malta, 19-23 May 2025

Agenda item:	8.0

Source:	Apple, OPPO, Huawei, HiSilicon, Samsung

Title:	Discussion on CT1 Reply LS (C1-252546) on RAT restrictions

WID/SID:	ECRATU – Release 19

Document for:	Discussion and Decision

# 1 Introduction

RAN2 has sent a Reply LS [2] to CT1 in RAN2#129 meeting in regards of CT1’s request to study the impact of the WID ECRATU (Enhancement of controlling RAT utilization) on the RAN2 specifications. RAN2 has endorsed the draftCRs [3][4][5][6] for 4G/5G specifications, but has indicated that there is some concerns whether it is feasible to update 2G/3G to support this feature (i.e., the impacted functions including cell selection and reselection for 2G/3G cell if restricted).

In the reply LS [1] , CT1 believes that “the support of the GERAN/UTRAN restrictions is useful for GERAN/UTRAN network sunset and would also like to highlight that non-support of the GERAN/UTRAN restrictions can result in some implementations in an inferior user experience if the UE is wasting time with futile registration attempts on these RATs”

In this paper, we discuss the RAN2 action to support GERAN/UTRAN restrictions.

# 2 Discussion

Regarding the sunset of 2G/3G network, it would be helpful to restrict UEs not to camp on legacy 2G/3G RAN, when CN can inform the GERAN/UTRAN restrictions to UE with upper layer signaling. However, it could be a burden for UE to update 2G/3G radio implementations (e.g., cell selection and reselection) to only assist the sunset of 2G/3G network. Also, the majority of the 2G/3G devices, which are legacy devices in the field, won’t support this ECARTU feature. So, it is unclear how critical to mandate every Rel-19 UE which plans to support ECRATU feature to support the GERAN/UTRAN restrictions, especially if operators intend to use ECRATU to indicate restrictions for 4G or 5G. Hence, we think 3GPP may take a compromise approach to make the AS layer support of GERAN/ETRAN restriction optional. To be more specific, instead of using “shall” in normative requirements, “should” or “may” can be used instead. If a UE supporting ECRATU would take the decision to not implement the changes in 2G or 3G access stratum, it is expected that when the UE camps of the restricted cell, the UE NAS layer would then consider the cell as suitable and would trigger the registration. As the access technology is restricted the registration would be rejected (i.e. with MM cause code #15) in the same manner as UEs not supporting ECRATU would be treated in this PLMN.

Proposal 1	RAN2 confirms that AS layer support of ECARTU feature (i.e., cell selection and reselection procedure) by GERAN/UTRAN is optional.

With this compromise approach, we provide the 25.304 and 43.022 CR drafts in the appendix. RAN2 can check and endorse those CRs so that they can be formerly submitted to RAN plenary for agreement, as TS 25.304 and TS 43.022 have been transferred to RAN Plenary when RAN6 WG was closed in 2020.

Proposal 2	RAN2 endorse the RAN Plenary CRs of GERAN/UTRAN in the Appendix.

# 3 Conclusion

In this contribution, based on the incoming Reply CT1 LS, we discuss 2G/3G aspects on RAT utilization control and have the following proposals:

Proposal 1	RAN2 confirms that AS layer support of ECARTU feature (i.e., cell selection and reselection procedure) by GERAN/UTRAN is optional.

Proposal 2	RAN2 endorse the RAN plenary CRs of GERAN/UTRAN in the Appendix.

# 4 References

[1] C1-252546 Reply LS on UE usage of the RAT restrictions

[2] R2-2501556 Reply LS on UE usage of the RAT restrictions

[3] R2-2501531, Rel-19 draftCR TS 36.304, “Restriction on RAT utilization”

[4] R2-2501532, Rel-19 draftCR TS 38.304, “Restriction on RAT utilization”

[5] R2-2500967, Rel-19 draftCR TS 36.300, “Restriction on RAT utilization”

[6] R2-2500438, Rel-19 draftCR TS 38.300, “Restriction on RAT utilization”

# Appendix A : draft CR for 25.304

3GPP TSG-RAN Plenary Meeting #xxx	RP-25xxxx

xxx, xxx, xxx 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.304 | CR | XXXX | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction on RAT utilization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the LS "UE usage of the RAT restrictions" from CT1 (C1-247193), R2 is informed that network may indicate the access technology utilization restriction information to UE with NAS EMM and 5GMM procedures. And UE will maintain a list of "PLMNs with associated access technology restrictions" which is applicable for the PLMN and optionally for EPLMN: - not consider any PLMN/access technology combination for which an access technology restriction is configured in the list of "PLMNs with associated access technology restrictions" as a candidate PLMN/access technology for PLMN selection purpose. - If an access technology is restricted in a PLMN, not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service. In summary, this feature requires UE to avoid selecting or re-selecting a cell which is deployed in an access technology for which an access technology restriction is applicable for the PLMN or, if applicable EPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a description associated with access technology utilization restriction information including the following aspects: - The usage of the list of "PLMNs with associated access technology restrictions" during cell selection and cell reselection. - The impact on the suitable cell definition. (NOTE: The restrictions are not applicable if the UE is selecting a cell for limited service.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell selection and reselection taking access technology utilization restriction information as received over NAS is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 4.3, 5.2.1, 5.2.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR has been discussed by RAN2 in R2-250xxxx |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 4.2	Functional division between AS and NAS in Idle mode

Table 1 presents the functional division between UE non-access stratum (NAS) and UE access stratum (AS) in idle mode. The NAS part is specified in [5] and the AS part in the present document. Examples of different idle mode procedures are presented in Clause 10.

Table 1: Functional division between AS and NAS in idle mode

| Idle Mode Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| PLMN Selection | Maintain the list of allowed PLMN types. It can be GSM-MAP only, ANSI-41 only or both. Maintain a list of PLMNs in priority order according to [5]. Select a PLMN using automatic or manual mode as specified in [5] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs from AS for PLMN selection. Maintain a list of equivalent PLMN identities. | Search for available PLMNs. If associated RAT(s) is (are) set for the PLMN, search in this (these) RAT(s) and other RAT(s) for that PLMN as specified in [5]. Perform measurements to support PLMN selection. Synchronise to a broadcast channel to identify found PLMNs. Report available PLMNs with associated PLMN type and RAT to NAS on request from NAS or autonomously. It shall respect allowed PLMN types indications from NAS. |
| Cell Selection | Control cell selection for example by indicating RAT(s) associated with the selected PLMN to be used initially in the search of a cell in the cell selection. NAS is also maintaining lists of forbidden registration areas, a list of "PLMNs with associated access technology restrictions and a Permitted CSG list. NAS provides these lists to AS. Maintain a list of equivalent PLMN identities and provide the list to AS. Maintain a list of "PLMNs with associated access technology restrictions" and provide the list to AS. | Perform measurements needed to support cell selection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Search for a suitable cell. The cells broadcast their 'PLMN identity' (GSM-MAP) or 'SID' and may broadcast a CSG ID and CSG indicator in the system information. Respond to NAS whether such cell is found or not. If associated RATs is (are) set for the PLMN, perform the search in this (these) RAT(s) and other RATs for that PLMN as specified in [5]. If such a cell is found, the cell is selected to camp on. |
| Cell Reselection | Control cell reselection by for example, maintaining lists of forbidden registration areas. Maintain a list of equivalent PLMN identities and provide the list to AS. Maintain a list of "PLMNs with associated access technology restrictions" and provide the list to AS. Maintain a list of forbidden LAs and provide the list to AS. Maintain a Permitted CSG list and provide the list to AS. | Perform measurements needed to support cell reselection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Change cell if a more suitable cell is found. |
| Location registration | Register the UE as active after power on. Register the UE's presence in a registration area, for instance regularly or when entering a new registration area. Maintain lists of forbidden LAs. Deregister UE when shutting down. | Report registration area information to NAS. |
| MBMS | Maintain priority information for various services and participate in service selection. Indicate to AS if an offered session has previously been received. | Maintain a list of TMGIs of activated services. Notify NAS of service availability and request NAS to prioritise in case of service conflicts. Report to NAS when an activated service requires action. Receive and handle the MCCH and MTCH, and, if applicable, the MSCH and MICH. |
| Support for manual CSG selection | Provide request to search for available CSG(s). Evaluate reports of available CSG(s) from AS for CSG selection. Select a CSG and request AS to select a suitable cell belonging to this CSG. | Search for cells with a CSG ID. Read the HNB name from system information if a cell with a CSG ID is found. Report CSG ID(s) of the found cells broadcasting a CSG ID together with the HNB name and PLMN(s) to NAS. On selection of a CSG by NAS, select any cell belonging to the selected CSG fulfilling the cell selection criteria and not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and give an indication to NAS that access is possible (for the registration procedure). |

Next change

## 4.3	Service type in Idle and Connected Mode

This clause defines the level of service that may be provided by the network to a UE in Idle mode and Connected Mode.

The action of camping on a cell is necessary to get access to some services. Three levels of services are defined for UE:

-	Limited service (emergency calls, ETWS if supported, and CMAS if supported on an acceptable cell)

-	Normal service (for public use on a suitable cell)

-	Operator service (for operators only on a reserved cell)

Furthermore, the cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls, receive ETWS if supported, and receive CMAS if supported). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS primary notification and ETWS secondary notification and to receive CMAS notification in a UTRAN network:

-	The cell is not barred, see subclause 5.3.1.1;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.1.2;

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell should fulfil the “access technology restriction” requirement as below and shall fulfil all the other following requirements.

-	The cell shall be part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

-	For a CSG cell, the cell is a CSG member cell for the UE;

According to the latest information provided by the NAS.

-	The cell is not barred, see subclause 5.3.1.1;

-	The cell is part of at least one LA that is not part of the list of "forbidden LAs for roaming" [9], which belongs to a PLMN that fulfills the first bullet above;

-	The access technology of the cell is not marked as restricted for the PLMN that fulfils the first bullet above in the list of "PLMNs with associated access technology restrictions" (TS 23.122 [5]);

-	The cell selection criteria are fulfilled, see subclause 5.2.3.1.2.

If the IE "Multiple PLMN List" [4] is broadcast in the cell, the cell is considered to be part of all LAs with LAIs constructed from the PLMN identities in the "Multiple PLMN List" and the LAC broadcast in the cell.

barred cell:

A cell is barred if it is so indicated in the system information [4].

reserved cell:

A cell is reserved if it is so indicated in system information [4].

Following exceptions to these definitions are applicable for UEs:

-	camped on a cell that belongs to a LA that is forbidden for regional provision of service, and during emergency calls.

-	as an outcome of the manual CSG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and inform NAS that access is possible (for location registration procedure).

NOTE:	UE is not required to support manual search and selection of CSG(s) while in RRC CONNECTED state.

A cell that belongs to a LA that is forbidden for regional provision service ([5], [16]) is suitable but provides only limited service.

If a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for cell reselection for the duration of the emergency call.

suitable MBSFN cluster:

A cell is part of an MBSFN cluster if it is indicated in system information [4]. An MBSFN cluster provides exclusively MBMS services. A suitable MBSFN cluster shall fulfil all the following requirements:

-	For FDD, 3.84 Mcps TDD IMB and 3.84/7.68 Mcps TDD the MBSFN cluster shall be part of either:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

The MBSFN cluster shall be indicated as:

-	providing MBSFN only service

For FDD, 3.84 Mcps TDD IMB and 3.84/7.68 Mcps TDD MBSFN cluster selection/reselection process for MBSFN clusters operating on the same frequency shall be based on reception quality. Details of the process are not specified.

Next change

### 5.2.1	Introduction

As stated in clause 1, the present document applies to UEs that support at least UTRA.

Different types of measurements are used in different RATs and modes for the cell selection and reselection. The performance requirements for the measurements are specified in [10] and [11].

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, by maintaining a list of forbidden registration area(s), and by maintaining a list of "PLMNs with associated access technology restrictions" and a list of equivalent PLMNs. The UE shall select a suitable cell and the radio access mode based on idle mode measurements and cell selection criteria.

In order to speed up the cell selection process, stored information for several RATs may be available in the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in [10] and [11].

The NAS is informed if the cell selection and reselection results in changes in the received system information.

For normal service, the UE has to camp on a suitable cell, tune to that cell's control channel(s) so that the UE can:

-	Receive system information from the PLMN;

-	Receive registration area information from the PLMN, e.g., location area and routing area; and

-	Receive other AS and NAS Information;

-	If registered:

-	receive paging and notification messages from the PLMN; and

-	initiate call setup for outgoing calls or other actions from the UE.

Next change

##### 5.2.6.1.3	Highest ranked cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in subclause 5.2.6.1.4, or for the best cell according to absolute priority reselection criteria specified in subclause 5.2.6.1.4a, the UE shall check if the access is restricted according to the rules in subclause 5.3.1.1.

If that cell and other cells have to be excluded from the candidate list, as stated in subclause 5.3.1.1, the UE shall not consider these as candidates for cell reselection. This limitation is removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of forbidden LAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300s. If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to its access technology being restricted based on the list of "PLMNs with associated access technology restrictions" (TS 23.122 [5]), the UE should not consider this cell and other cells of the restricted access technology on the same frequency, as candidates for reselection for a maximum of 300s. If the UE has to perform an any cell selection procedure any limitation shall be removed. If the UE is redirected under UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden LAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to its access technology being restricted based on the list of "PLMNs with associated access technology restrictions" (TS 23.122 [5]), the UE should not consider this cell and other cells of the restricted access technology on the same frequency as candidates for reselection for a maximum of 300s. If the UE has to perform an any cell selection procedure any limitation shall be removed. If the UE is redirected under UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

End of change

# Appendix B: draft CR for 43.022

3GPP TSG-RAN Plenary Meeting #xxx	RP-25xxxx

xxx, xxx, xxx 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 43.022 | CR | xxxx | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction on RAT utilization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-5-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the LS "UE usage of the RAT restrictions" from CT1 (C1-247193), R2 is informed that network may indicate the access technology utilization restriction information to UE with NAS EMM and 5GMM procedures. And UE will maintain a list of "PLMNs with associated access technology restrictions" which is applicable for the PLMN and optionally for EPLMN: - not consider any PLMN/access technology combination for which a access technology restriction is configured in the list of "PLMNs with associated access technology restrictions" as a candidate PLMN/access technology for PLMN selection purpose. - If an access technology is restricted in a PLMN, not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service. In summary, this feature requires UE to avoid selecting or re-selecting a cell which is deployed in an access technology for which an access technology restriction is applicable for the PLMN or, if applicable EPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a description associated with access technology utilization restriction information including the following aspects: - The usage of the list of "PLMNs with associated access technology restrictions" during cell selection and cell reselection. - The impact on the suitable cell definition. (NOTE: The restrictions are not applicable if the UE is selecting a cell for limited service.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell selection and reselection taking access technology utilization restriction information as received over NAS is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR has been discussed by RAN2 in R2-250xxxx |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 3.2	Camping on a cell

### 3.2.1	Normal camping

For normal service, the MS has to camp on a suitable cell, tune to that cell's control channel(s), and possibly register within the PLMN so that the MS can:

a)	Receive system information from the PLMN , e.g., the cell options ;

b)	Receive paging messages from the PLMN, e.g., when there is an incoming call for the MS;

c)	Initiate call setup for outgoing calls or other actions from the MS (where possible, see subclauses 3.5.3 and 3.5.4).

The choice of such a suitable cell for the purpose of receiving normal service is referred to as "normal camping". There are various requirements that a cell must satisfy and the “access technology restriction” requirement a cell should satisfy before an MS can perform normal camping on it:

i)	It should be a cell of the selected PLMN or, if the selected PLMN is equal to the last registered PLMN, an equivalent PLMN;

ii)	It should not be "barred" (see subclause 3.5.1);

iii)	It should not be in an LA which is in the list of "forbidden LAs for roaming";

iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated as shown in subclause 3.6;

v)	It should not be a SoLSA exclusive cell to which MS does not subscribe.  This requirement is only valid for MSs supporting SoLSA.

iv)	The access technology of the cell is not marked as restricted for the PLMN that fulfils the first bullet above in the list of "PLMNs with associated access technology restrictions" (TS 23.122 [31]).

Initially, the MS looks for a cell which satisfies these 5 constraints ("suitable cell") by checking cells in descending order of received signal strength. If a suitable cell is found, the MS camps on it and performs any registration necessary. Cells can have two levels of priority, suitable cells which are of low priority are only camped on if there are no other suitable cells of normal priority. (This is called "cell selection").

When camped on a cell the MS regularly looks to see if there is a better cell in terms of a cell re-selection criterion, and if there is, the better cell is selected. Also if one of the other criteria changes, (e.g., the current serving cell becomes barred), or there is a downlink signalling failure (see subclause 3.6), a new cell is selected. (This is called "cell reselection"). A MS that has enabled PEO or EC operation has relaxed requirements for how often it verifies the suitability of its serving cell and the suitability of neighbour cells for re-selection (see 3GPP TS 45.008 [25] and 3GPP TS 44.018 [23]).

In order to speed up these processes, a list of the RF channels containing BCCH or CPBCCH carriers of the same PLMN is broadcast in the system information messages, see subclause 4.8. Also, the MS does not need to search all possible RF channels to find a suitable cell. If, after searching the number of RF channels, given for each frequency band below, with the strongest received signal level, a BCCH or CPBCCH carrier has been found but no suitable cell of the selected PLMN has been found, the MS can stop the attempt to find a suitable cell of the selected PLMN.

The number of channels to be searched are 15 for T-GSM 380, 15 for T-GSM 410, 15 for GSM 450, 15 for GSM 480, 25 for GSM 710, 25 for GSM 750, 25 for T-GSM 810, 30 for GSM 850 Band, 30 for GSM 900, 15 for T-GSM 900 and 40 for DCS 1800 and PCS 1900.

End of change