3GPP TSG-RAN WG2 Meeting #130	R2-2503734

St. Julians, Malta, May 19th– 23rd, 2025

Agenda Item:	8.1.2.2

Source:	KT Corporation

Title:	Discussion on applicability reporting

WID/SID:	NR_AIML_air-Core (Release-19)

Document for:	Discussion and Decision

Introduction

In the RAN WG2 #129bis meeting, RAN WG2 made the following agreements on applicability reporting for the UE-side model [1].

Agreements on applicability reporting 1 Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec 2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. 3 No prohibit timer is introduced

This paper discusses remaining issues and additional considerations for applicability reporting for the UE-side model.

Discussion

As an AI/ML LCM procedure [2], applicability reporting with its cause provides several important benefits to mobile operators. This helps improve the efficiency of network operations, improve user experience, reduce radio resources, support data-driven network optimization, and enhance interoperability. Especially, the improvement of network efficiency would be significant. For example, when a UE is unable to use an AI/ML model, the network can reduce unnecessary resource allocation to increase network efficiency by fully utilizing a received cause information. Furthermore, the collected information from the UE can be analyzed and used for mobile network planning and optimization. Therefore, applicability reporting with its detailed cause is essential for AI/ML management.

Observation 1: Applicability reporting with its cause provides several important benefits to mobile operators, so that it can mprove the efficiency of network operations, improve user experience, reduce radio resources, support data-driven network optimization, and enhance interoperability.

Proposal 1: The applicability reporting with its explicit cause should be sent to network.

The applicability reporting can be sent by the existing RRC messages RRCReconfigurationComplete or UEAssistanceInformation, and a new IE such as ‘AI/ML Applicability Change Cause’, to inform the network the cause should be defined.

Proposal 2: An applicability change cause information should be introduced and sent to the network.

Furthermore, if the applicability and cause reporting is supported by UE, feature (FG), model as well as functionality units, the network will be able to receive more precise information and respond to specific UEs, features (FGs), functionalities, or models for reducing radio resource waste. In other words, it explains how to report at the UE, feature, and model levels in addition to functionality to support more granularity for reporting causes.

Proposal 3: The applicability and cause reporting should be supported by UE, feature (FG), model as well as functionality units.

Finally, the causes can provide detailed information for inapplicability, so that the network can correctly manage AI/ML operations. However, for simplicity, they can be grouped into categories such as those related to model/functionality, UE (UE-side server, if used) system and power, radio conditions, etc.

Proposal 4: The inapplicability causes can be grouped into categories for simplicity, such as those related with model/functionality, UE (UE-side server) system and power, radio conditions, etc.

Conclusion

In this contribution, we have the following proposals for applicability reporting with cause for UE-side model:

Proposal 1: The applicability reporting with its explicit cause should be sent to network.

Proposal 2: An applicability change cause information should be introduced and sent to the network.

Proposal 3: The applicability and cause reporting should be supported by UE, feature (FG), model as well as functionality units.

Proposal 4: The inapplicability causes can be grouped into categories for simplicity, such as those related with model/functionality, UE (UE-side server) system and power, radio conditions, etc.

References

[1] Draft Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

[2] 3GPP TR 38.843 V18.0.0 (2023-12): “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.