3GPP TSG-RAN WG2 Meeting #130	R2-2503739

St. Julians, Malta, May 19th– 23rd, 2025

Agenda Item:	8.1.4

Source:	KT Corporation

Title:	Discussion on LCM for UE-sided model

WID/SID:	NR_AIML_air-Core (Release-19)

Document for:	Discussion and Decision

Introduction

In the RAN WG2 #129bis meeting, RAN WG2 made the following agreements on data collection configuration for the UE-side model for beam management case [1].

Agreements on data collection configuration The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request

This paper discusses remaining issues and additional considerations for data collection configuration for the UE-side model.

Discussion

2.1	Motivations

For mobile operators, operating various AI/ML models, ensuring model performance, ensuring AI/ML interoperability between various RAN and UE devices, accuracy of data collection, efficient use of wireless network resources, reducing energy consumption, and reducing radio traffic congestion are critical factors for reducing network operating costs and improving service reliability.

For example, by providing model training phase and performance information, the network can predict data collection patterns and optimize resource allocation.

In addition, AI/ML LCM [2] in wireless network can be performed optimally based on sufficient information about UE as well as network, and it is very important for network automation and operation management through related big data.

Therefore, optimization and refinement of AI/ML data collection procedures are essential.

Observation 1: The LCM for AI/ML air interface is key issue for mobile operators for reducing network operating costs and improving service reliability as well as for improving radio resource utilization.

Proposal 1: AI/ML data collection configuration procedure and information should be optimized and refined for perfect interoperability, reliable AI/ML performance, and improved resource utilization.

However, the configuration for training data collection through AI/ML air interfaces is currently being discussed, but the detailed procedure for configuring data collection, how to request configuration for data collection from the UE, and the details of the data collection indication and the causes for the inability to collect data when the UE is unable to collect data according to the configuration received from the network are still unclear and lacking.

Observation 2: The detailed procedure for configuring data collection, how to request configuration for data collection from the UE, and the details of the data collection indication and the causes for the inability to collect data when the UE is unable to collect data according to the configuration received from the network are still unclear and lacking.

2.2	UE Capability Information

The network checks UE's support capabilities through UECapabilityEnquiry, and then UE responds with UECapabilityInformation to support data collection and model training. When the UE responds, the network can prepare an appropriate data collection configuration by proactively reporting the UE's capability and status such as the support of AI/ML model/functionality, the model training stage (e.g., initial training, retraining, updating, etc.), the types and purpose of data collection, etc.

Proposal 2: UECapabilityInformation can include additional information for appropriate data collection configuration at the network.

2.3	Data Collection Configuration Preference from UE

The existing RRC procedure does not sufficiently reflect the dynamic requirements of UE-side model training (e.g., data requirements for each training stage, model performance monitoring), and therefore lacks response to radio resource efficiency and model performance optimization.

Therefore, additional information for data collection preference can be reported, such as data collection request type of modify, data collection preferred configuration sets and priority, UE context/status information, data collection purpose, data collection request priority/urgency, model training phase, model performance, data quality criteria, etc.

When the network receives the above configuration preference request, it can configure the optimal data collection configuration for the UE.

Proposal 3: Additional information for data collection configuration preference can be requested by UE.

2.4	Data Collection Feasibility Indication and Causes

UE determines data collection feasibility (or configuration applicability) based on received configuration information. The criteria for determining the feasibility can be UE power/system/processor conditions, channel conditions, etc. If data collection is not possible, an initial report is made via the RRCReconfigurationComplete message. If it is not possible to collect all/part of the data, it can be reported together including the corresponding data item (list).

If data collection is not possible from the UE, the network can take more appropriate action depending on the cause. Therefore, the existing RRC messages RRCReconfigurationComplete and UEAssistanceInformation can be extended to add a new IE such as "AI/ML Data Collection Inability Change Cause". The examples for the causes are UE power/battery, UE resource/system (including UE-side server), radio condition, model related, etc.

Proposal 4: Data collection feasibility indication and inability cause should be reported to network.

Conclusion

In this contribution, we have the following proposals for data collection configuration for UE-side model:

Proposal 1: AI/ML data collection configuration procedure and information should be optimized and refined for perfect interoperability, reliable AI/ML performance

Proposal 2: UECapabilityInformation can include additional information for appropriate data collection configuration at the network.

Proposal 3: Additional information for data collection configuration preference can be requested by UE.

Proposal 4: Data collection feasibility indication and inability cause should be reported to network.

References

[1] Draft Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

[2] 3GPP TR 38.843 V18.0.0 (2023-12): “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.