3GPP TSG-RAN-WG2 Meeting #130	R2-2503745

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item	:		8.6.3

Source	: 	Sharp

Title	: 	Discussion on issues for supporting L1 event triggered measurement reporting

Document for	:	Discussion and Decision

Introduction

In this paper, we discuss the issue related to L1 event triggered measurement reporting.

Discussion

The content of L1 MR MAC CE triggered by LTM2

The contents of L1 MR MAC CE triggered by LTM2 is FFS and should be discussed.

Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

In the previous meeting, RAN2 agreed to support Event LTM2~5 as below:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For event LTM2, UE evaluates only the beam of serving cell and doesn’t evaluate beams of candidate cells. And In the current MAC CR[1], there is a following description:

### 5.x.2	Performing measurement

For the LTM candidate cell(s) with the candidate ID not configured in ltm-CandidateReportConfigList, the UE is not required to perform measurement and event evaluation in 5.x.3 on the reference signaling belonging to the candidate ID for the corresponding event.

This means that for LTM2, the UE is not required to perform measurement for candiate cell(s). However, if the UE is configured any other event and perfoms measurment for candidate cell(s), the candidate beam information is useful for the network to decide the LTM cell switch execution and/or early sync. Therefore, for MR triggered by LTM2, the MR MAC CE should be able to include candidate information.

Proposal 1. For MR triggered by LTM2, the MR MAC CE can include measurements for LTM candidates.

Cancellation of MR

During the MAC CR email discussion[2], some issues are raised as below:

- MAC-15: Whether the triggered MR should be cancelled in the below case: if all the triggered beam(s), i.e. Type#00 and Type#01, have been reported, no matter whether truncated MR MAC CE or regular MR MAC CE is used before, the trigger MR should be cancelled.

- MAC-16: Whether the triggered MR should be cancelled in the below two cases:
Case 1: When a beam (or multiple beam) meets the entry condition for TTT (Type#00), a MR will be triggered. Before getting the UL grant, if this beam (or all these beams) meets the leaving condition, then, whether this MR is still triggered or should be cancelled? 
Case 2: When a beam (or multiple beam) in the reporting list meets the leaving condition for TTT (Type#01), a MR will be triggered. Before getting the UL grant, if this beam (or all these beams) meets the entry condition again, then, whether this MR is still triggered or should be cancelled?

For the MAC-15 case, when all the triggered beam(s) have been reported, all remaining beams have not satisfied the condition for TTT and these beams are may be helpful but not important for the network to decide the LTM cell switch execution and early sync, therefore MR should be cancelled.

Proposal 2. The triggered MR should be cancelled if all the triggered beam(s) have been reported.

For the case 1 in MAC-16, information on beams that have already fulfilled the leaving condition is not necessary for the network, even if they have satisfied the entry condition once. For the case 2 in MAC-16, reporting on a beam that once met the leaving condition again is not necessary and should be cancelled, since the network will just continue to assume that the quality is good.

Proposal 3. The triggered MR should be cancelled when a beam (or multiple beam) meets the entry condition for TTT and before getting the UL grant this beam (or all these beams) meets the leaving condition.

Proposal 4. The triggered MR should be cancelled when a beam (or multiple beam) meets the leaving condition for TTT and before getting the UL grant this beam (or all these beams) meets the entry condition.

Conclusion

Proposal 1. For MR triggered by LTM2, the MR MAC CE can include measurements for LTM candidates.

Proposal 2. The triggered MR should be cancelled if all the triggered beam(s) have been reported.

Proposal 3. The triggered MR should be cancelled when a beam (or multiple beam) meets the entry condition for TTT and before getting the UL grant this beam (or all these beams) meets the leaving condition.

Proposal 4. The triggered MR should be cancelled when a beam (or multiple beam) meets the leaving condition for TTT and before getting the UL grant this beam (or all these beams) meets the entry condition.

Reference

[1] R2-250xxxx_Running MAC CR for Mob Ph4_v21_Rapp3.docx

[2] R2-250xxxx_Discussion summary and list of MAC open issue for mobility_v14_Rapp3.docx