3GPP TSG-RAN WG2 Meeting #130                                     R2-2503749

St.Julians, Malta,  May 19th – 23rd , 2025

Source: 			OPPO

Title: 	Discussion on providing MBS service area in NTN network

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

Agreements:

- RAN2 understands the intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

- If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH.

- If no intended service area is explicitly indicated (e.g., in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

- UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

FFS whether and how the association between intended service area and frequencies is acquired.

In the last RAN2#129bis meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

In this paper, we would like to further present our opinions on the topic.

- Discussion

- 2.1 how to link the service area for each MBS session

In the RAN2#129 meeting, we have made an agreement that it shall be possible to signal multiple area IDs to one MBS service (we insert a list of service area IDs in MCCH). Note that also in the RAN2#128 meeting, we made an agreement that a signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s), which means that in the MCCH, not only the intended service area of the current serving cell, but also those of neighbour cells are needed to be included. Since in the last RAN2 #129bis meeting, we have agreed to prioritize the frequency whose intended service area associated with the interested MBS service covers the UE’s current position, so it seems necessary to specify intended service area separately for each interested MBS service. To achieve this, we need to isolate the intended service area of the current cell from the service areas of neighbour cells belonging to each MBS session and attach different IDs with these service areas. Subsequently, only the ID of the service area of the current cell is included in the mbs-SessionInfoList-r17 IE included in the MBSBroadcastConfiguration message, and the IDs of the service areas of neighbour cells for the same MBS session are included in other IEs, e.g., MBS-NeighbourCellList.

Proposal 1: RAN2 to agree to capture the intended service area ID for the current cell in the mbs-SessionInfoList-r17 IE in MBSBroadcastConfiguration.

Proposal 2: RAN2 to agree to capture the intended service area ID for the neighbour cells in the MBS-NeighbourCellList IE in MBSBroadcastConfiguration.

- 2.2 MBS interest indication

According to TS 38.331, the purpose of this procedure is to inform the network that the UE in RRC_Connected is receiving or is interested in receiving MBS broadcast service(s) and/or to inform the network of the priority of MBS broadcast versus unicast and multicast MRB reception. Even if the UE is not entering any intended service area, the UE could still perform the MBS interest indication procedure towards the network. Subsequently, if the network would like to provide the MBS service to the UE, no matter where the UE is, the network could just send the MBS content towards the UE via unicast. As a result, we think the legacy mechanism of MBS interest indication could be followed by the NTN cell, and no other additional enhancement such as UE indicating the MBS interest indication only when the UE enters one intended service area is needed.

Observation 1: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast.

Proposal 3: does not introduce any additional restriction on when to initialize the MBS interest indication, on the top of the legacy UE behaviour.

- 2.3 Whether the association between intended service area and frequencies is needed in SIB21?

According to the section 2.1, intended service area per-cell is suggested by us to be included in the mbs-SessionInfoList-r17 and MBS-NeighbourCellList. Since via the mtch-NeighbourCell IE included in the mbs-SessionInfoList IE, the UE can obtain the information of which neighbour cell supports which MBS session service, and considering the MBS-NeighbourCellList already includes the information of the carrier frequency information, the UE can obtain the mapping relationship between intended service area and the frequency information via reading the MCCH, and there is no need to specify the intended service area ID information in SIB21.

Observation 2: mapping relationship between intended service area ID and the frequency can be obtained by the UE via decoding the mtch-NeighbourCell IE and MBS-NeighbourCellList IE in the mbs-SessionInfoList-r17 IE, if the intended service area ID can be agreed to be embedded in the MBS-NeighbourCellList.

Proposal 4: intended service area ID shall not be included in the SIB21.

Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Proposal 1: RAN2 to agree to capture the intended service area ID for the current cell in the mbs-SessionInfoList-r17 IE in MBSBroadcastConfiguration.

Proposal 2: RAN2 to agree to capture the intended service area ID for the neighbour cells in the MBS-NeighbourCellList IE in MBSBroadcastConfiguration.

Observation 1: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast.

Proposal 3: does not introduce any additional restriction on when to initialize the MBS interest indication, on the top of the legacy UE behaviour.

Observation 2: mapping relationship between intended service area ID and the frequency can be obtained by the UE via decoding the mtch-NeighbourCell IE and MBS-NeighbourCellList IE in the mbs-SessionInfoList-r17 IE, if the intended service area ID can be agreed to be embedded in the MBS-NeighbourCellList.

Proposal 4: intended service area ID shall not be included in the SIB21.