3GPP TSG-RAN WG2 Meeting #130										R2-2503763

St Julian, Malta, 19th – 23rd May, 2025                               		Revision of R2-2501960

Agenda item:		8.4.2

Title: 	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure and configuration of LP-WUS for IDLE/INACTIVE modes and give our proposals.

- Discussion

2.1 About LP-WUS measurement configuration

In RAN2#129bis meeting [1], there is an FFS as follows:

- All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations.

In RAN2#129bis meeting [1], there are some discussion on whether/how reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading, and there is the following agreements:

- Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

And some further discussion is suggested for direction 3 as follows [1]:

- Direction 3a. merge LPWUS monitoring and serving cell offloading - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

During the LP_WUS study phase, it is indicated that LP-WUS monitoring without RRM Relaxation/Offloading will not bring much gain [2].

Meanwhile, if only RRM Relaxation/Offloading is enabled, UE need monitoring PDCCH for paging frequently, which is also not beneficial for UE power saving.

Observation 1: More power saving gain can be achieved if LP-WUS monitoring and RRM Relaxation/Offloading are performed simultaneously.

Therefore, it is suggested to merge the RRM Relaxation/Offloading condition and LP-WUS monitoring condition. Based on previous RAN2 agreements, network might configure Relaxation, Offloading, or both, and UE will work only in one state (e.g. either in RRC Relaxation, in offloading state, or none), UE can monitoring LP-WUS only when it is in serving cell offloading state or in Rel-19 RRM relaxation state.

Proposal 1: UE monitors LP-WUS only when it is in serving cell offloading state or in Rel-19 RRM relaxation state, i.e. the entry/exit condition for serving cell offloading and relaxation can be reused for LP-WUS monitoring without configuring separate thresholds.

## 2.2 Activation/deactivation of LP-WUS

Since it has been agreed in RAN2#125bis that LP-WUS related configuration for IDLE/INACTIVE state is provided via system information, the provision of LP-WUS configuration via SIB can be seen as an implicit activation to apply LP-WUS configuration for all the UEs supporting this function. Meanwhile, if the LP-WUS configuration is no longer broadcast via SIB, it can be assumed that application of LP-WUS configuration (and also the LP-WUS monitoring) is deactivated.

However, it’s also possible that the UE would not apply LP-WUS configuration immediately on reception of LP-WUS configuration and the application of LP-WUS configuration will be activated later, e.g., per-UE via a dedicated signaling, e.g., RRC connection release message. During SI stage, such process has been discussed but no decision has been made.

The main motivation to activate/deactivate LP-WUS for a specific UE or part of UEs via dedicated signaling is that it may be not suitable to always activate the LP-WUS monitoring for all the UEs as soon as the LP-WUS configuration is broadcast. For example, LP-WUS can be applied to diverse UE types, e.g., IIoT UE, (e) Redcap UE, NR UE with XR traffic and so on, the device type or traffic type of UE may be another consideration for activating or deactivating LP-WUS for a UE. For example, the UEs with delay-sensitive services may not be suitable to monitor LP-WUS considering the possible large wake up latency caused by LP-WUS, which may have unexpected impacts on the delay-sensitive service. The exact latency that may be increased by LP-WUS is still under evaluation in RAN1, e.g., to take into account the LP-WUS processing time, MR transition time for ramp up, Time/frequency synchronization of MR etc. Per our understanding, if the additional latency caused by LP-WUS is far larger than the duration that random access procedure may use, e.g. if the increased latency is several seconds or more, the LP-WUS in IDLE/INACTIVE modes should be activated/deactivated by UE dedicated signaling, e.g., to activate LP-WUS only for the UE without delay-sensitive services or to deactivate LP-WUS when the UE starts to perform delay-sensitive services.

In a summary, from RAN2 perspective, we can see the need to activate/deactivate the application of LP-WUS configuration via dedicated signaling.

Observation 2: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

In the last meeting, some companies propose and/or support that “CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signaling; the absence of indication means UE is allowed to use LP-WUS functionality”. But considering that CN does not know the radio resource configuration status (e.g. DRX cycle) and UE’s capability (e.g. time Offset between LP_WUS and PDCCH), it is difficult for CN to decide whether the LP-WUS is suitable for the UE. And SA2 has deleted the description related to PEI not used for emergency session, it is impossible that SA2 specifies something that LP-WUS does not apply some UEs.

Observation 3: CN does not know the radio resource configuration status and UE’s capability, it is difficult for CN to decide whether the LP-WUS is suitable for the UE.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by dedicated signaling is supported, the gNB firstly needs to be aware of the UE capability of supporting LP-WUS in IDLE/INACTIVE modes and then can be able to activate/deactivate the LP-WUS for the UE by dedicated signaling.

Meanwhile, for the paging gNB, to only know the UE capability would be not sufficient as UE may not have been activated the use of LP-WUS, it also needs to know whether the LP-WUS is activated for this UE. Therefore, whether the LP-WUS monitoring has been activated for this UE (which can only be performed on UE supporting LP-WUS) needs to be transmitted to the paging gNB by Paging or RAN Paging.

Proposal 2a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, send LS to RAN3 to provide LP-WUS activation/deactivation indication to the paging gNB, e.g., via NGAP Signaling, NGAP PAGING and RAN PAGING message.

2.3 Time offset between LP-WUS and PO

As agreed in RAN1, UE reports one wake-up delay (e.g. one of candidate values) via UE capability reporting, and wake-up delay is minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. If UE reports, it can implicitly indicate that UE supports LP-WUS in RRC_IDLE/RRC_INACTIVE state. From the network perspective, as agree in RAN1, time offset value(s) between a LO and a reference PO/PF is/are configured. In order to support the different values for wake up delay, network can configure different time offset values, and a time offset value is larger than or equal to a reported wake-up delay + PEI offset if PEI is configured. However, the different time offset values may cause the multiple LP-WUS occasions for same PO scattered over the time domain, too many LP-WUS occasions per PO is not convenient for LP-WUS resource scheduling, network can configure some time offsets that can be used. In which case, UE should determine the time offset based on its reported wake-up delay from the time offsets configured by network, e.g. the UE selects the minimum time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Proposal 3a: Introduce UE reporting wake-up delay as part of the UE capability, which is also an implicit indication of UE supporting LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 3b: Network configures time offset(s) that can be used between LO and MR PDCCH monitoring.

Proposal 3c: UE determine the time offset to be used based on its reported wake-up delay as part of the UE capability and the time offset(s) configured by network, e.g. select the minimal time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Considering that the LP-WUS time offset capability will be used for paging gNB to determine the LP-WUS occasion, the LP-WUS wake-up delay capability should be included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, since the ue-RadioPagingInfo-r17 IE is already included in UERadioPagingInformation container(e.g. which is included in RAN3 IE of UE Radio Capability for Paging IE) and is transparently passed from gNB to AMF, from AMF to gNB and from one gNB to another gNB, the LP-WUS capability will not be lost if the gNB is rel-17 or later.

Proposal 4: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message so that it can be delivered to paging gNB transparently.

In RAN1#120bis meeting, it has been agreed that:

Agreement The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4.

That means, one LO may associated with multiple continuous POs, and the LO position is determined based on one of the associated POs, e.g. with time offset before the reference PO, and how to determine the reference PO, PF and i-s can be used as a baseline, e.g.

The reference PF associated with the LP-WUS is determined by (SFN for PF) - floor (iPO/Ns)*T/N, where iPO=UE_IDmodN⋅NS+i_smod poNumPerLPWUS is a reference paging occasion index;

The reference i_s associated with the LP-WS  = i_s mod poNumPerLPWUS.

where poNumPerLPWUS is the number of POs associated with one LP-WUS.

But how to determine the reference PO need wait for RAN1 conclusion.

Proposal 5: When one LO associates with multiple continuous POs, the time offset is relative to the reference PO. And how to determine the reference PO require more RAN1 input.

2.3 LP-WUS and eDRX

In RAN2#129bis meeting[1], it has been agreed that：

- LP-WUS is supported with eDRX, FFS on exact impact if any

Considering that eDRX is used for high latency communication case, CN will not send paging and/or DL signaling to gNB outside the PTW, UE need only monitor PDCCH paging during the PTW.

LP-WUS during the PTW and/or shortly before the start of the PTW. For simplicity, UE can decide the LP-WUS monitoring occasion based on the reference PDCCH monitoring occasion, e.g. UE monitors LP-WUS with a time offset before the reference PO, whichever the eDRX is configured or not.

Proposal 6: When eDRX is configured, UE only monitors LP-WUS during the PTW and/or shortly before the start of the PTW, e.g. with a time offset before the reference PO.

Proposal 6a: UE monitors LP-WUS with a time offset before the reference PO, no matter whether the eDRX is configured or not.

2.5 Whether LP-WUS is only used in the last used cell or in any cell

Although LP-WUS false wake-up case is less than PEI false wakeup case (e.g. there is at most 31 LP_WUS subgroups, and UE may monitor PEI after the UE detects it’s LP-WUS), if the LP-WUS is sent in Tracking Area, and one Tracking Area includes many cells and/or there are large amount of UEs in one tracking area, LP-WUS false wake-up may also occur. Similar as PEI, a network indication (e.g. lastUsedCellOnly_LPWUS) can be used to indicate whether UE monitors LP-WUS only in the cell where the latest RRCRelease without noLastCellUpdate is received, or in all the cells, which can be configured by network based on implementation (e.g. how many cells included in one Tracking Area and/or the paging probability statistics in one tracking area).

Proposal 7: An indication (e.g. lastUsedCellOnly_LPWUS) is introduced to control whether UE only monitors LP-WUS in the cell where the latest RRCRelease without noLastCellUpdate is received.

- Conclusion

The following proposals are given based on the above analysis:

Provision of LP-WUS related configuration

Observation 1: More power saving gain can be achieved if LP-WUS monitoring and RRM Relaxation/Offloading are performed simultaneously.

Proposal 1: UE monitors LP-WUS only when it is in serving cell offloading state or in Rel-19 RRM relaxation state, i.e. the entry/exit condition for serving cell offloading and relaxation can be reused for LP-WUS monitoring without configuring separate thresholds.

Activation/deactivation of LP-WUS

Observation 2: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Observation 3: CN does not know the radio resource configuration status and UE’s capability, it is difficult for CN to decide whether the LP-WUS is suitable for the UE.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE is supported.

Proposal 2a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, send LS to RAN3 to provide LP-WUS activation/deactivation indication to the paging gNB, e.g., via NGAP Signaling, NGAP PAGING and RAN PAGING message.

Time offset between LP-WUS and PO

Proposal 3a: Introduce UE reporting wake-up delay as part of the UE capability, which is also an implicit indication of UE supporting LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Proposal 3b: Network configures time offset(s) that can be used between LO and MR PDCCH monitoring.

Proposal 3c: UE determine the time offset to be used based on its reported wake-up delay as part of the UE capability and the time offset(s) configured by network, e.g. select the minimal time offset configured by network that is larger than or equal to its reported wake-up delay. If there is no time offset configured by network that is larger than or equal to its reported wake-up delay, the UE does not monitor LP-WUS.

Proposal 4: The UE reported wake-up delay is included in ue-RadioPagingInfo-r17 IE of UECapabilityInformation message, so that it can be delivered to paging gNB transparently.

Proposal 5: When one LO associates with multiple continuous POs, the time offset is relative to the reference PO. And how to determine the reference PO require more RAN1 input.

LP-WUS and eDRX

Proposal 6: When eDRX is configured, UE only monitors LP-WUS during the PTW and/or shortly before the start of the PTW, e.g. with a time offset before the PO to be monitored.

Proposal 6a: UE monitors LP-WUS with a time offset before the reference PO, no matter whether the eDRX is configured or not.

Whether LP-WUS is only used in the last used cell or in any cell

Proposal 7: An indication (e.g. lastUsedCellOnly_LPWUS) is introduced to control whether UE only monitors LP-WUS in the cell where the latest RRCRelease without noLastCellUpdate is received.

- References

- Draft_RAN2_129bis_Meeting_Report_v2

- 3GPP TR 38.869 V18.0.0 (2023-12), Study on low-power wake-up signal and receiver for NR