3GPP TSG-RAN WG2 Meeting #130										R2-2503764

St Julian, Malta, 19th – 23rd May, 2025							Revision of R2-2501961

Agenda item:		8.4.4

Title: 	Procedures for LP-WUS in RRC_CONNECTED

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure of LP-WUS for RRC_CONNECTED state and give our proposals.

- Discussion

2.1 Activation/deactivation for LP-WUS monitoring in RRC_CONNECTED state

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. Considering that the coverage of LP-WUS may be less than or equal to the cell coverage, even the LP-WUS related configuration is provided, the LP-WUS may not be monitored, e.g. when the UE is at the cell edge. The LP-WUS monitoring activation/deactivation should be considered.

Usually, there are two solutions for LP-WUS monitoring activation/deactivation:

- Solution 1: similar as RRC_IDLE/RRC_INACTIVE state, a cell edge condition (e.g. RSRP threshold) is configured to UE, and UE monitors LP-WUS in RRC_CONNECTED state when it is not at the cell edge. For this solution, Network cannot know whether the UE is monitoring the LP-WUS and should always send LP-WUS, whichever the UE monitors it or not. In this case, for Option 1-1, Network couldn’t know whether UE starts the drx-onDurationTimer.

- Solution 2: Network activate or deactivate the LP-WUS explicitly, e.g. with RRC message, DCI or MAC CE. For this solution, UE need monitor LP-WUS only when it is activated by network, and network can activate or deactivate the LP-WUS based on the CQI info or UE assistance information reported by UE. So that network can determine whether and when to activate or deactivate the LP-WUS monitoring in RRC_CONNECTED state, LP-WUS monitoring entry and exit condition (e.g. RSRP and optional RSRQ threshold) should be configured to UE(e.g. by RRCReconfiguration message), and UE can report LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message .

To keep alignment between UE LP-WUS monitoring and gNB LP-WUS transmission, solution 2 is preferred. For light signaling load and not impact RAN1 on DCI design, MAC CE is suggested to be used for network to activate or deactivate the LP-WUS explicitly.

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

## 2.2 LP-WUS monitoring schemes with C-DRX configured

In the current specification, secondary DRX Group can be configured for UE power saving when both FR1 and FR2 cells are configured in one cell group, in which case the UE power consumption can be reduced if a separate drx-InactivityTimer and drx-onDurationTimer in secondary DRX Group is configured for the FR2 cells enabling FR2 to go to sleep more quickly. In RAN2#129bis [1], there is an FFS as follows:

=> FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group.

In RAN1#120bis meeting [2], the following conclusion has been made:

Conclusion For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

The main concern for option 1-1 to configure LP-WUS per DRX group is that: the C-DRX cycle is common for dual DRX group, only drx-onDurationTimer and drx-InactivityTimer can be configured for DRX group and secondary DRX group separately, and the drx-onDurationTimer is always started simultaneously for the two DRX group. But based on the TS 38.321[3], the drx-onDurationTimer is started per DRX group, and the DRX Active Time is maintained per DRX group as follows. Thus, if LP_WUS is configured per DRX group, using LP-WUS to indicate which DRX group to start drx-onDurationTimer does not impact the MAC specification architecture. It is feasible technically.

## 5.7 Discontinuous Reception (DRX) ... When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or ... When DRX is configured, the MAC entity shall: ... 1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer for each DRX group; 2> stop drx-InactivityTimer for each DRX group. ... 1> if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or 1> if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number drx-StartOffset] modulo (drx-NonIntegerShortCycle)) = 0: 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset: 2> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3: 3> if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 2> else: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

Observation 1: In the current MAC specification, the drx-onDurationTimer is started per DRX group, and DRX Active Time is maintained per DRX group. It is feasible technically to configure LP-WUS with secondary DRX group simultaneously for option 1-1.

Since both secondary DRX Group and LP-WUS are used for UE power saving, with LP-WUS per DRX group, network can indicate which DRX group(e.g. primary DRX group, secondary DRX group or both) enters Active Time based on different use cases, which can save UE power more.

Proposal 2: The LP-WUS can be configured with secondary DRX group simultaneously for option 1-1.

With same consideration as for option 1-1, LP-WUS can also be configured with secondary DRX group simultaneously for option 1-2.

Proposal 3: The LP-WUS can be configured with secondary DRX group simultaneously for option 1-2.

When LP-WUS is configured with secondary DRX Group simultaneously, how to correlate LP-WUS with DRX group should be considered:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, and different LP-WUS subgroup for different DRX group is configured to indicate which DRX group is triggered

Option 2 is preferred due to less signaling overhead. Considering that multiple LP-WUS subgroups of one LP-WUS can be configured for a UE in RRC_CONNECTED state, one LP-WUS subgroup can be configured for one DRX group, and another LP-WUS subgroup can be configured for another DRX group, UE in RRC_CONNECTED state need only monitor one LP-WUS.

Proposal 4: When LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS subgroup can only be configured for one DRX group, i.e. one LP-WUS subgroup is configured with one DRX group while another LP-WUS subgroup is configured for the other DRX group, so that the LP-WUS will only activate one DRX group.

Considering that legacy, in case DCP is monitored but not detected, the legacy drx-onDurationTimer is not started and UE does not enter into active time. Same mechanism can be reused for LP-WUS in RRC_CONNECTED, e.g. If LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started and the UE does not enter into active time.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. lpwus_PDCCHMonitoringTimer) is not started for option 1-2.

For Option 1-2, as ‘UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)’ in RAN1, UE can determine the LP-WUS monitoring occasion using the similar formula as that to start legacy drx-onDurationTimer. Considering that LP-WUS monitoring consumes much less UE power than PDCCH monitoring, and smaller LP-WUS cycle can decrease service delay, one LP-WUS cycle with smaller value can be configured, which does not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6: For Option 1-2, RAN2 confirms that one single LP-WUS cycle is defined to determine the LP-WUS monitoring occasion.

In Rel-18, non-integer C-DRX cycle is defined to match the non-integer traffic periodicity (e.g. XR periodicity), considering the low power consumption, small granularity of LP-WUS cycle (e.g. value of 1ms) can be used to match the non-integer C-DRX periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

For option1-2, when LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS monitoring configuration (e.g. LP-WUS monitoring cycle, time offset) is configured and UE monitors one LP-WUS, LP-WUS subgroup(s) are used to indicate for which DRX group the lpwus_PDCCHMonitoringTimer starts.

Similar as drx-onDurationTimer, lpwus_PDCCHMonitoringTimer can be configured per DRX group.

Proposal 7: For option1-2, when LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS monitoring configuration (LP-WUS cycle, timeOffset) is configured per cell group, LP-WUS subgroup number and lpwus_PDCCHMonitoringTimer is configured per DRX group.

In RAN2#117 meeting, it has been agreed that “For Option 1-2,The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring.”, that means that a timeOffset should be defined for the GAP between LP-WUS monitoring and the start of newly defined timer (e.g. lpwus_PDCCHMonitoringTimer). Since legacy drx-startOffset is defined to determine the PDCCH monitoring start occasion, drx-startOffset - timeOffset can be used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion.

Proposal 8:  For Option 1-2, RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. the new timer triggered by LP-WUS).

Similar as the drx-onDurationTimer start occasion, For Option 1-2, the LP-WUS monitoring occasion can be determined with the LP-WUS cycle and LP-WUS start offset (e.g. drx-startOffset - timeOffset) as follows: if LP-WUS is configured and activated, the UE monitors the LP-WUS at the SFN and subframe determined by:

[(SFN × 10) + subframe number - ( drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) =0:

Proposal 9: For Option 1-2, the LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

- Conclusion

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CEl.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

Observation 1: In the current MAC specification, the drx-onDurationTimer is started per DRX group, and DRX Active Time is maintained per DRX group. It is feasible technically to configure LP-WUS with secondary DRX group simultaneously for option 1-1.

Proposal 2: The LP-WUS can be configured with secondary DRX group simultaneously for option 1-1.

Proposal 3: The LP-WUS can be configured with secondary DRX group simultaneously for option 1-2.

Proposal 4: When LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS subgroup can only be configured for one DRX group, i.e. one LP-WUS subgroup is configured with one DRX group while another LP-WUS subgroup is configured for the other DRX group, so that the LP-WUS will only activate one DRX group.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. lpwus_PDCCHMonitoringTimer) is not started for option 1-2.

Proposal 6: For Option 1-2, RAN2 confirms that one single LP-WUS cycle is defined to determine the LP-WUS monitoring occasion.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 7: For option1-2, when LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS monitoring configuration (LP-WUS cycle, timeOffset) is configured per cell group, LP-WUS subgroup number and lpwus_PDCCHMonitoringTimer is configured per DRX group.

Proposal 8: For Option 1-2, RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. the new timer triggered by LPWUS).

Proposal 9: For Option 1-2, The LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

Observation 1: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

- References

- Draft_RAN2_129bis_Meeting_Report_v2

- Draft_Minutes_report_RAN1#120b_v010

- 3GPP TS 38.321 V18.4.0 (2024-12),NR; Medium Access Control (MAC) protocol specification