3GPP TSG-RAN WG2 Meeting #130		R2-2503773

Malta, May 19th – 23rd, 2025

Source:	Baicells

Title:		Discussion on Conditional intra-CU LTM remaining issues

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

The following agreements to support the Conditional intra-CU LTM are made in the RAN2#129[1] and RAN2#129bis[2]:

RAN2#129 Agreements on C-LTM: 1. From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. 2. The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. 3. For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. 7. Do not support DG-based first UL transmission for RACH-less conditional LTM. 8. For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. 9. For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. 10. Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA)

RAN2#129bis Agreements on C-LTM: 1. Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. 2. C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer. 3. RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. 4. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this contribution, we will discuss the remaining open issues and further details of conditional LTM, including:

- Which beam to perform the CLTM cell switch for L3 based C-LTM

- Discussion

  - Conditional LTM Execution

Which beam to perform the CLTM cell switch for L3 based C-LTM

In the R2#129 meeting, the following agreements for beam selection for RACH-less C-LTM  in L1 based C-LTM case are as follows:

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

However, the beam selection for RACH-less C-LTM in L3 based C-LTM needs to be considered too. If the cell switching is triggered by the L3 execution conditions to a CLTM candidate cell, and there are activated TCI states for this CLTM candidate cell, UE select one of the activated TCI states for accessing the cell. However, if there is no activated TCI state for this CLTM candidate cell, then the L3 beam level measurement results of this CTLM candidate cell can be used by UE for selecting the best beam as the target beam for accessing the target candidate cell.

Proposal1: If the CLTM cell switching is triggered by the L3 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the best beam as the target beam for accessing the target candidate cell.

- Conclusion

In this contribution, we discussed the issues related to Conditional LTM. We have the following Proposal:

Proposal1: If the CLTM cell switching is triggered by the L3 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the best beam as the target beam for accessing the target candidate cell.

References

- RAN2 #129 chairman notes

- RAN2 #129bis chairman notes