3GPP RAN WG2 Meeting #130	R2-2503777

St.Julians, Malta,  May 19th – 23rd, 2025

Agenda Item:	8.1.4

Source:	Mediatek Inc.,Nokia,Qualcomm Incorporated,Xiaomi,InterDigital Inc.

Title:	Discussion on the Necessity of Non-Standardized Data

Document for:	Discussion, Decision

Introduction

In the RAN#106 meeting, the Way forward on AIML UE sided data collection was approved [1]:

For SA From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfilement of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5. RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 to study the transfer of data over CP for now. For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed General - RAN will make a final decision on way forward taking into account SA inputs when available. RAN will have a checkpoint in Q2, 2025

This contribution argues for supporting non-standardized data content in Solutions 2/3, balancing MNO visibility needs with AI/ML flexibility.

Discussion

2.1 Standardized vs. Non-Standardized Data Trade-offs

For the standardized data contents, the UE performs measurement and data logging if supported according to the network’s configuration. The standardized data content can be fully visible to MNO if the information conveyed, the values held, the data types used, and the data formats structured, as well as the encoding and decoding rules applied are standardized, irrespective of CP or UP based transport used as in positioning.

Observation 1: Full visibility of standardized data content means that all attributes, including the information conveyed, the values held, the data types used, and the data formats structured, are specified. Conversely, no standardized visibility indicates that none of these attributes are defined. Partial visibility for partially standardized data content means only certain attributes are specified.

However, it is important to consider the benefits of non-standardized data content, which can still be made accessible to MNOs through alternative methods such as SLAs, or business contracts.

- Flexibility in Model Training and enhanced performance: AI/ML algorithms often require tailored datasets that are specific to the particular use case or environment in which they are deployed. Not standardizing data content allows for the customization of data collection and model training processes to suit specific needs, enhancing the effectiveness of the AI solutions and the performance of these models.

- Decreased Standardization Burden: The effort and resources required to standardize vast and varied data types across multiple industries and use cases can be immense. By not standardizing data content, this burden is significantly reduced, allowing standardization efforts to focus on more critical areas where universal standards can provide clear benefits.

- Scalability and Evolution: As AI technologies evolve and the variety of use cases expands, the need for data will similarly diversify. It is impractical to anticipate and standardize data for all potential future needs. Keeping data content non-standardized allows for greater scalability and adaptability as new requirements and use cases emerge.

- Vendor-Specific Innovation: Allowing for vendor-dependent information and proprietary data formats can spur innovation, as different vendors may develop unique, optimized ways of handling data that could lead to more advanced AI solutions.

In RAN Plenary LS RP-242389 [2], the full visibility requirement for UE side data collection is only agreed for standardized data. Therefore, the transport for the UE-side data collection should be allowed to carry both standardized and non-standardized data. Full visbility is achieved for the standardized data contents as agreed in [2] and different level of visibility can be ensured for non-standardized data contents depending upon the MNO requirements as discussed in next section.

The non-standardized contents allow for the tailoring of data to specific AI/ML use cases and environments, enhancing model performance and effectiveness. This approach also reduces the burden of having to standardize every aspect of data, which can be resource-intensive and impractical given the vast and varied nature of data types needed.

Proposal 1: Support non-standardized and partially standardized data contents in Solution 2 and/or Solution 3. The choice of solution will depend on the outcome of the data transfer study.

2.2 Addressing Privacy and MNO Concerns

Critics may raise privacy risks, but these are mitigated through three key safeguards:

- User Consent Enforcement

As long as the non-standadrized data contents are of the same type and used for the similar puspose as the standardized data contents, the data collection of standardized and non-standardized data is covered by the existing 3GPP consent mechanism.

- Data Scope Limitation

The collected data is strictly radio/sensor-generated and non-identifiable, including: radio metrics: RSRP, RSRQ, SINR, latency, beam failure rates and sensor data, such as UE mobility patterns (e.g., velocity, direction), ambient RF environment measurements, etc. There may be modem-generated logs, such as HARQ retransmission statistics, channel occupancy, interference levels.

Unlike subscriber metadata (e.g., IMSI) or behavioral data (e.g., app usage), radio/sensor data cannot directly identify individuals. Furthermore,the data can be systematically anonymized to prevent traceability to individual users by removing identifier before storage/transmission or applying data masking.

- MNO Governance

Operators retain full governance over non-standardized data via e.g. policy-based filtering. The MNO can block non-standardized data assuming that the non-standardized data is contained in a transparent container. The MNO can also approve whilelist of data types through NEF or OAM configurations. .

Proposal 2: Adopt the following principles to mitigate privacy risks and other principles are not excluded:

- User consent enforcement: Reuse the existing 3GPP User consent mechanism for UE sided data collection.

- Data scope limitation: Restrict collection to non user identifiable information, such as radio/sensor-generated metrics.

- MNO governance: Allow operators to enable/disable non-standardized data collection.

Conclusion

Standardized vs. Non-Standardized Data Trade-offs

Observation 1: Full visibility of standardized data content means that all attributes, including the information conveyed, the values held, the data types used, and the data formats structured, are specified. Conversely, no standardized visibility indicates that none of these attributes are defined. Partial visibility for partially standardized data content means only certain attributes are specified.

Proposal 1: Support non-standardized and partially standardized data contents in Solution 2 and/or Solution 3. The choice of solution will depend on the outcome of the data transfer study.

Addressing Privacy and MNO Concerns

Proposal 2: Adopt the following principles to mitigate privacy risks and other principles are not excluded:

- User consent enforcement: Reuse the existing 3GPP User consent mechanism for UE sided data collection.

- Data scope limitation: Restrict collection to non user identifiable information, such as radio/sensor-generated metrics.

- MNO governance: Allow operators to enable/disable non-standardized data collection.

Reference

[1] RP-243307, Way forward on AIML UE sided data collection, RAN2 Chair (InterDigital)

[2] RP-242389, LS on AIML data collection, RAN plenary